Supporting Information

Determination of atmospherically deposited microplastics in moss samples 

Mike Wenzel1,2, Björn Fischer3, Carmen Wolf1, Christine Kube1,
Stefan Nickel4, Annekatrin Dreyer5, Barbara Völksen4, Winfried Schröder4,
Gerrit Renner2,7, Jürgen Schram6, Torsten C. Schmidt2,7, Jochen Tuerk1, 7*

SI 1. Sampling site specifications

Table 1-1 shows detailed sampling site information such as geographical and altitude above sea level, distance thresholds for the respective type of emission source, distance to the next monitoring station, and moss species collected.

Table1-1: Sampling site coordinates and altitude above sea level, potential emission sources, distance threshold for potential emission sources, distance to environmental monitoring stations equipped with measuring devices, and moss species sampled.

| Sampling site [No.] | Sampling site coordinates Altitude above sea level [m] | Potential emission source | Distance threshold for potential emission source [m] | Distance to environmental monitoring stations [m] | Moss species |
| --- | --- | --- | --- | --- | --- |
| 1 | Latitude: 49,281622 |  |  |  |  |
|  | Longitude: 6,84455 | Agricultural areas | 300 | 270 | Hypnum cupressiforme |
|  | Altitude: 289 |  |  |  |  |
| 2 | Latitude: 50,92509 |  |  |  |  |
|  | Longitude: 9,33503 | Agricultural areas | 300 | 550 | Hypnum cupressiforme |
|  | Altitude: 399 |  |  |  |  |
| 3 | Latitude: 51,357992 | Single houses | 300 |  |  |
|  | Longitude: 12,326555 | Town | 600 | 2766 | Hypnum cupressiforme |
|  | Altitude:214 |  |  |  |  |

SI 2. Analytical methods

Detailed setup parameters and device manufacturer specifications about the thermogravimetric furnace used by performing TED-GC-MS analysis can be seen in Table 2-1. Detailed setup parameters and device manufacturer specifications about gas chromatograph and mass spectrometer, as well as the adsorption/desorption units and injection parameters used during TED-GC-MS analysis, are shown in Table 2-2.

Table 2-1: Detailed setup parameters used and device manufacturer specifications concerning the thermogravimetric furnace applied by performing TED-GC-MS analysis.

| Parameter | Information |
| --- | --- |
| Model | TGA 2 (Mettler Toledo GmbH, Gießen, Germany) |
| Purge Gas (N2) | 30 mL/min |
| Total Exhaust Flow | 50 mL/min |
| Heating Range | 25-600 °C |
| Heating Rate | 10 °C/min |
| Crucibles | Material: Alumina; V = 600 µL (Mettler Toledo GmbH, Gießen, Germany) |

Table 2-2: Detailed setup parameters and device manufacturer specifications concerning the adsorption/desorption unit, gas chromatograph, and mass spectrometer applied by performing TED-GC-MS analysis.

| Parameter | Information |  |
| --- | --- | --- |
| Adsorption | Model (Adsorption unit) | Adsorption Unit TDU 3.5+ (Gerstel GmbH & Co KG, Mülheim an der Ruhr, Germany) |
|  | Adsorbent | Twister®; Material: Polydimethylsiloxane (Gerstel GmbH & Co KG, Mülheim an der Ruhr, Germany) |
|  | Interface Temperatures | 240°C / 40°C |
| Desorption | Model | Desorption Unit TDU 3.5+ (Gerstel GmbH & Co KG, Mülheim an der Ruhr, Germany) |
|  | Temperature Programme | 50°C-200°C; Heating rate: 40°C/min; Isotherm at 200°C for 5 minutes |
|  | Mode | Splitless |
| Cryofocus and Injection | Model | CIS 4 (Gerstel GmbH & Co KG, Mülheim an der Ruhr, Germany) |
|  | Modus | Solvent Vent |
|  | Temperature Programme | -100°C-270°C; Heating Rate: 12°C/s; Isotherm at 270°C for 3 min (-100°C was achieved with liquid nitrogen) |
| Gas Chromatography | Model | GC7890 (Agilent, Santa Clara, California, USA) |
|  | Column | HP-5MS; length: 30 m × outer diameter: 250 μm; film thickness: 0.25 μm |
|  | Helium Flow Rate | 1 mL/min |
|  | Temperature Programme | 40°C-300°C; Isotherm at 40°C for 1 minute; Isotherm at 300°C for 5 minutes |
| Mass spectrometry | Model | 5977B MSD (Agilent, Santa Clara, California, USA) |
|  | Temperatures | Interface: 300°C; Ion Source: 230°C; Quadropole: 150°C |
|  | Ionisation | EI (70 eV) |
|  | Mode | Scan, 35-350 m/z |

SI 3 Data evaluation

SI 3.1 Pyrolysis products

Pyrolysis products (qualifier and quantifier) used, their retention times (tR) and quantifier ions used are presented in Table 3-1.

Table 3-1: Overview of qualifier and quantifier products, their retention time (tR), and the fragment ions (quantifier ion) used for data evaluation during TED-GC-MS analysis.

| Polymer | Pyrolysis products (qualifier products) |  | Retention time (tR) [min] | Quantifier product |  | Quantifier Ion (m/z) |
| --- | --- | --- | --- | --- | --- | --- |
| Polystyrene (PS) | 2,4-Diphenyl-1-butene |  | 28.78 | 2,4-Diphenyl-1-butene |  | 208 |
|  | 2,4,6-Triphenyl-1-hexene |  | 42.12 |  |  |  |
| Polypropylene (PP) | 2,4,6,8-Tetramethylundec-1-ene (isomer 1) |  | 18.73 | 2,4,6,8-Tetramethylundec-1-ene (isomer 3) |  | 111 |
|  | 2,4,6,8-Tetramethylundec-1-ene (isomer 2) |  | 18.95 |  |  |  |
|  | 2,4,6,8-Tetramethylundec-1-ene (isomer 3) |  | 19.18 |  |  |  |
| Polyethylene terephthalate (PET) | Vinyl benzoate |  | 13.97 | Vinyl benzoate |  | 105 |
|  | Ethyl benzoate |  | 14.89 |  |  |  |
|  | Benzoic acid |  | 15.29 |  |  |  |
| Polyethylene (PE) | 1,14 Pentadecadiene |  | 23.18 | 1,16 Heptadecadiene |  | 96 |
|  | 1,16 Heptadecadiene |  | 27.84 |  |  |  |
| Styrene butadiene rubber (SBR) | Cyclopentylbenzene |  | 16.35 | Cyclohexenylbenzene |  | 104 |
|  |  | Cyclohexenylbenzene | 19.30 |  |  |  |

SI 3.2 Quality assurance

Process blank: The sample preparation procedure was performed without a moss sample. Therefore, all devices and materials were proven with regard to possible laboratory, material, and device contamination using TED-GC-MS.

System blank: Measurement of an empty crucible between each sample performing TED-GC-MS measurements

Quality control sample: Measurement of each polymer investigated by using the polymer test material (Bundesanstalt für Materialforschung und -prüfung (BAM), Berlin, Germany). This allows the verification of the calibration used concerning TED-GC-MS analysis.

SI 3.4 Limit of detection (LOD) and Limit of quantification (LOQ)

The calculation is adapted from reference [45] shown in the manuscript. The instrumental limit of detection (LOD) and limit of quantification (LOQ) in µg (absolute) were determined using the signal-to-noise ratio (S/N). Therefore, the extracted Ion chromatogram of the quantifier ions of the quantifier product was used. For the LOD a factor of 3, and for LOQ a factor of 10, were considered. The noise was calculated by averaging noisy data points next to the peak signal. The calculation was performed using ChemStation software (Version: F.01.03.2357, Manufacturer: Agilent Technologies, Inc.). In addition, the LOD and LOQ in µg/g for the determination of microplastics in moss (using µPEEL) were determined. Therefore, the resulting average recoveries achieved concerning each polymer, the instrumental LOD and LOQ, and the initial sample mass 1 g were considered. An overview of the LOD´s and LOQ´s concerning the polymers (polystyrene (PS), polypropylene (PP), polyethylene terephthalate (PET), and polyethylene (PE)) is shown in Table SI 3-2. With regard to styrene-butadiene-rubber, no recoveries were determined. Therefore, only the instrumental LOD and LOQ are present.

Table 3-2: Determined instrumental limit of detection (LOD) and limit of quantification (LOQ) in µg (absolute) and determined LOD and LOQ for the determination of microplastics in moss [µg/g] in regard to TED-GC-MS analysis. The LOD´s and LOQ´s were determinate concerning the polymers polystyrene (PS), polypropylene (PP), polyethylene terephthalate (PET) and polyethylene (PE). With regard to styrene-butadiene-rubber, no recoveries were determined. Therefore, only the instrumental LOD and LOQ are present.

| Polymer | Quantifier product | Quantifier Ion (m/z) | LOD [µg] | LOQ [µg] | LOD [µg/g] | LOQ [µg/g] |
| --- | --- | --- | --- | --- | --- | --- |
| PS | 2,4-Diphenyl-1-butene | 208 | 0.09 | 0.29 | 0.13 | 0.42 |
| PP | Tetramethylundec-1-ene (isomer 3) | 111 | 0.13 | 0.44 | 0.20 | 0.66 |
| PET | Vinyl benzoate | 105 | 0.87 | 2.92 | 0.99 | 3.31 |
| PE | 1,16 Heptadecadiene | 96 | 4.21 | 14.04 | 5.40 | 17.99 |
| SBR | Cyclohexenylbenzene | 104 | 0.71 | 2.37 | - | - |

SI 3.5 µRaman analysis

Table 3-3 shows the not extrapolated number of polymers identified concerning each sampling site and a measured filter area of approx. 2-10 %. Identification was performed using µRaman measurements.

Table 3-3: The actual number of polymers identified within a filter surface area of 2 – 10 % concerning the three sampling sites investigated. Identification was performed using µRaman measurements.

|  | Polymers |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling site [No.] | PC | PDMS | PE | PVC | PP | PTFE | PET | PA |
| 1 | 3 | 2 | 13 | 1 | 4 | 3 | 1 | - |
| 2 | - | - | 15 | 4 | 1 | - | 2 | - |
| 3 | 5 | 3 | 4 | 1 | 2 | 1 | - | 1 |