SUPPLEMENTARY MATERIAL

Machine Learning Algorithms to Detect Patient-Ventilator Asynchrony. A Feasibility Study.

Jeffrey Williams, MD1

Jalil Ahari, MD2

Kendrew Wong, MD3

Arun Jose, MD4

Guillermo Gutierrez, MD, PhD5

1. Department of Anesthesiology, The George Washington University

2. Department of Medicine, The George Washington University

3. Department of Medicine, NYU Grossman School of Medicine

4. Department of Medicine, University of Cincinnati

5. Emeritus Professor of Medicine, Anesthesiology and Engineering,
    The George Washington University

| Table 1E. Patient Demographics and Admission Data |  |
| --- | --- |
| Demographics: |  |
| Age (Years) | 62.5 [57.0,70.8] |
| Gender | 50% female |
| Ethnicity (%): |  |
| Black | 68.2 |
| Caucasian | 18.2 |
| Hispanic | 11.4 |
| Asian | 2.3 |
| Admission diagnosis: |  |
| Pulmonary | 52.3% |
| Sepsis or septic shock | 18.2% |
| Neurologic | 15.9% |
| Cardiac | 4.5% |
| Gastroenterological | 4.5% |
| Oncological | 2.3% |
| Renal | 2.3% |
| Reason for intubation: |  |
| Airway protection | 45.5% |
| Hypoxemia | 34.1% |
| Hypercapnia | 20.5% |
| Other admission data: |  |
| SOFA | 7.0 [6,9] |
| SAPS | 54.5 [47.0,63.8] |
| Glasgow scale* | 7 [6,9] |
| BMI (kg/m2) | 28.9 [25.9,36.0] |

- Ventilator modified Glasgow scale with a maximum of 10 points.

Data acquisition system

We acquired ventilator data continuously with a Raspberry 3B microcontroller (Raspberry Pi) connected to the ventilator’s RS-232 port via a DB9 cable. The microprocessor was programmed to interact with the Servo_i ventilator Computer Interface Emulator. Data gathering begins with the microprocessor signaling the ventilator to acquire, in parallel, 4096 samples of the airway flow and pressure signals at 31.25 Hz, a rate that satisfies the Nyquist-Shannon criterion to avoid aliasing of data. Once signal sampling is completed (131.07 seconds, or approximately 2.2 minutes), frequency spectra for airway pressure and the inspiratory and expiratory phases of the airway flow signals were computed by the microprocessor using the Fast Fourier Transform (FFT) algorithm (Duhamel P, Vetterli M: Fast Fourier transforms: a tutorial review and a state of the art. Signal Processing 1990, 19:259-299). Spectral frequency granularity was 0.458 cycles per minute.

Figure 1E: An epoch's dashboard, displaying demographic data, ventilator settings, and measured patient data. A slider provides access to data from different days and times, while radio buttons are used for epoch classification. The lower panel shows flow and pressure signals sampled at 31.25 Hz over a duration of 131 seconds.

Figure 2E. Examples of the different breathing patterns used in epoch classification.

Figure 2E

| Table 2E |  | Epoch classification for each enrollee according to asynchrony type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient | Synch | Variable | Double | Ineffective | Cycling | Complex | Auto |  | Monitored | # of | % Time |
| Number |  | Breathing | Trigger | Efforts |  |  | Trigger |  | Time (hr) | Asynchs | Asynch |
| 1820022301 | 0 | 80 | 56 | 6 | 341 | 13 | 1 |  | 20.7 | 416 | 84% |
| 1820022601 | 207 | 588 | 17 | 1 | 272 | 9 | 2 |  | 45.7 | 299 | 27% |
| 1820030701 | 414 | 595 | 49 | 4 | 22 | 23 | 5 |  | 46.3 | 98 | 9% |
| 1820030801 | 5 | 1 | 3 | 35 | 0 | 1 | 0 |  | 1.9 | 39 | 87% |
| 1820030802 | 102 | 638 | 6 | 1 | 354 | 22 | 2 |  | 46.9 | 383 | 34% |
| 1820030803 | 0 | 0 | 0 | 1 | 54 | 0 | 0 |  | 2.3 | 55 | 100% |
| 1820030804 | 35 | 273 | 29 | 55 | 108 | 8 | 1 |  | 21.2 | 200 | 39% |
| 1820031601 | 197 | 854 | 49 | 397 | 470 | 277 | 9 |  | 93.9 | 1193 | 53% |
| 1820031602 | 689 | 318 | 42 | 3 | 546 | 47 | 9 |  | 68.9 | 638 | 39% |
| 1820032001 | 413 | 37 | 30 | 536 | 65 | 55 | 11 |  | 47.8 | 686 | 60% |
| 1820032101 | 143 | 43 | 6 | 8 | 301 | 6 | 5 |  | 21.3 | 321 | 63% |
| 1820032201 | 927 | 663 | 3 | 9 | 16 | 43 | 6 |  | 69.5 | 71 | 4% |
| 1820032301 | 80 | 0 | 7 | 4 | 3 | 7 | 1 |  | 4.3 | 21 | 21% |
| 1820032401 | 417 | 138 | 6 | 30 | 35 | 13 | 2 |  | 26.7 | 84 | 13% |
| 1820033101 | 28 | 521 | 1 | 0 | 25 | 24 | 3 |  | 25.1 | 50 | 8% |
| 1820040101 | 5 | 851 | 8 | 59 | 1101 | 24 | 9 |  | 85.7 | 1192 | 58% |
| 1820040201 | 0 | 404 | 1 | 32 | 137 | 33 | 2 |  | 25.4 | 203 | 33% |
| 1820040501 | 393 | 195 | 0 | 20 | 22 | 24 | 0 |  | 27.3 | 66 | 10% |
| 1820040502 | 459 | 517 | 9 | 13 | 87 | 32 | 3 |  | 46.7 | 141 | 13% |
| 1820040601 | 680 | 490 | 10 | 2 | 620 | 7 | 8 |  | 75.7 | 639 | 35% |
| 1820041301 | 217 | 760 | 11 | 24 | 88 | 26 | 6 |  | 47.2 | 149 | 13% |
| 1820042401 | 0 | 214 | 2 | 3 | 368 | 44 | 0 |  | 26.3 | 417 | 66% |
| 1820042701 | 303 | 149 | 14 | 5 | 16 | 8 | 6 |  | 20.9 | 43 | 9% |
| 1820050801 | 75 | 229 | 66 | 25 | 91 | 48 | 2 |  | 22.3 | 230 | 43% |
| 1820051401 | 65 | 304 | 5 | 104 | 108 | 13 | 3 |  | 25.1 | 230 | 38% |
| 1820051501 | 243 | 935 | 24 | 15 | 344 | 59 | 23 |  | 68.5 | 442 | 27% |
| 1820053001 | 534 | 317 | 136 | 212 | 825 | 161 | 10 |  | 91.5 | 1334 | 61% |
| 1820061301 | 0 | 917 | 313 | 31 | 338 | 61 | 6 |  | 69.4 | 743 | 45% |
| 1820071201 | 216 | 154 | 32 | 0 | 313 | 4 | 3 |  | 30.1 | 349 | 48% |
| 1820071801 | 279 | 281 | 11 | 28 | 27 | 18 | 6 |  | 27.1 | 84 | 13% |
| 1820072401 | 74 | 318 | 40 | 1 | 50 | 27 | 4 |  | 21.4 | 118 | 23% |
| 1820073101 | 0 | 34 | 4 | 434 | 12 | 0 | 2 |  | 20.3 | 450 | 93% |
| 1820080101 | 18 | 218 | 1 | 274 | 1692 | 229 | 14 |  | 101.9 | 2196 | 90% |
| 1820081601 | 352 | 1292 | 32 | 86 | 712 | 32 | 9 |  | 104.8 | 862 | 34% |
| 1820082201 | 410 | 615 | 35 | 728 | 262 | 19 | 32 |  | 87.5 | 1044 | 50% |
| 1820082801 | 163 | 26 | 9 | 230 | 55 | 15 | 0 |  | 20.8 | 309 | 62% |
| 1820090401 | 124 | 281 | 149 | 252 | 813 | 44 | 6 |  | 69.5 | 1258 | 75% |
| 1820091301 | 88 | 418 | 1478 | 11 | 407 | 124 | 11 |  | 105.7 | 2020 | 80% |
| 1820103101 | 490 | 179 | 36 | 201 | 169 | 36 | 0 |  | 46.3 | 442 | 40% |
| 1820110101 | 183 | 1085 | 0 | 0 | 346 | 54 | 4 |  | 69.7 | 400 | 24% |
| 1820110801 | 426 | 885 | 31 | 120 | 115 | 25 | 11 |  | 67.2 | 291 | 18% |
| 1820111601 | 703 | 1355 | 43 | 150 | 129 | 35 | 12 |  | 101.1 | 357 | 15% |
| 1820121101 | 89 | 209 | 0 | 188 | 150 | 33 | 3 |  | 28.0 | 371 | 55% |
| 1820121201 | 387 | 147 | 9 | 2 | 325 | 26 | 3 |  | 37.5 | 362 | 40% |
|  |  |  |  |  |  |  |  |  | MEAN | VALUES |  |
|  | No | TI | DT | IE | DC | CM | DX |  | Time (hr) | Asynchs | % Time |
| Total | 10633 | 18528 | 2813 | 4340 | 12334 | 1809 | 255 |  | 48.0 | 484.0 | 42.0% |
|  | 21.1% | 36.7% | 5.6% | 8.6% | 24.4% | 3.6% | 0.5% |  | 30.2 |  |  |
| Modes of Ventilation |  |  |  |  |  |  | Median |  | 46.0 | 353.0 | 38.9% |
| Total | 50712 |  |  | Total Epochs = | 50,712 |  | 1st Q |  | 24.4 | 135.3 | 20.0% |
| PRVC | 30994 | 61.1% |  |  |  |  | 3rd Q |  | 69.5 | 638.8 | 59.3% |
| PC | 15769 | 31.1% |  |  |  |  | min |  | 1.9 | 21.0 | 4.3% |
| VC | 1054 | 2.1% |  |  |  |  | max |  | 105.7 | 2196.0 | 100.0% |
| PSPC | 2895 | 5.7% |  |  |  |  |  |  |  |  |  |

MACHINE LEARNING ALGORITHMS

We used scikit-learn, an open source machine learning library for the Python programming language, to develop the classification supervised machine learning algorithms. We tested several algorithms to determine the best predictor. These were Random Forest, k-Nearest Neighbors, Convolutional Neural Networks, Quadratic Discriminant Analysis, Logistic Regression, Gradient Boosted Decision Trees, Ada Boost, Naïve Bayes Classifiers and Support Vector Machines. For the purpose model development, the classified epochs were distributed randomly into a Train data set encompassing 70% of the instances. The remaining 30% were assigned to a Test data set and used to evaluate model performance.

Algorithm comparison:  We initially identified 33 variables related to patient demographics and ventilator data as model features. From these, we further selected 19 having ≥ 5% correlations with the predicted asynchronies. The Random Forest algorithm yielded the highest accuracy score and Support Vector Machines the worst for all three models. The k-Nearest Neighbors and Convolutional Neural Networks and ADA Boost algorithms also performed well.

Table 3E. Overall accuracies (%) of machine-learning algorithms when applied to the Test dataset.

| Classifier Algorithm | Model 1 Asynchrony Present | Model 2 Asynchrony Identification | Model 3 Severity | Model 4 AutoPEEP Present |
| --- | --- | --- | --- | --- |
| Random Forest | 91.1 | 81.7 | 87.4 | 92.8 |
| k-Nearest Neighbors, n= 3 | 85.3 | 74.8 | 80.6 | 88.7 |
| Neural Networks | 81.2 | 68.5 | 0.80 | 87.2 |
| Quadratic Discriminant | 73.3 | 59.7 | 75.1 | 42.9 |
| Logistic Regression | 75.9 | 65.0 | 79.6 | 87.1 |
| Decision Tree | 79.7 | 64.8 | 81.1 | 87.5 |
| ADA Boost | 82.0 | 61.3 | 80.2 | 88.5 |
| Naïve Bayes | 74.4 | 58.2 | 72.7 | 70.4 |
| Support Vector Machines | 58.2 | 40.5 | 57.3 | 73.8 |

Comparison of different machine-learning algorithms used to train Model 2.

No null values found in the database

Study     Day  Epoch  ... Med Dx_6 Gender_0  Gender_1

0  1820022601  180227    134  ...    False     True     False

1  1820022601  180227    143  ...    False     True     False

2  1820022601  180227    168  ...    False     True     False

3  1820022601  180227    176  ...    False     True     False

4  1820022601  180227    188  ...    False     True     False

5  1820022601  180227    190  ...    False     True     False

6  1820022601  180227    192  ...    False     True     False

7  1820022601  180227    211  ...    False     True     False

8  1820022601  180227    215  ...    False     True     False

9  1820022601  180227    221  ...    False     True     False

0 = Synchrony

1 = Variable breathing

2 = Double triggering

3 = Ineffectual efforts

4 = Cycling

5 = Complex

6 = Auto triggering

[10 rows x 46 columns]

Number of 0 cases:  18528 (20.97%)

Number of 1 cases:  18528 (36.54%)

Number of 2 cases: 2813 (5.55%)

Number of 3 cases:  4340 (8.56%)

Number of 4 cases: 12334 (24.32%)

Number of 5 cases:  1809 (3.57%)

Number of 6 cases:  255 (0.50%)

Creating train and test sets

70.00% in training set

30.00% in test set

Original 0  : 10633 (20.97%)

Original 1  : 18528 (36.54%)

Original 2 : 2813 (5.55%)

Original 3 : 4340 (8.56%)

Original 4 : 12334 (24.32%)

Original 5 : 1809 (3.57%)

Original 6 : 255 (0.50%)

Training 0  : 7356 (20.72%)

Training 1  : 13047 (36.75%)

Training 2 : 1933 (5.45%)

Training 3 : 3029 (8.53%)

Training 4 : 8679 (24.45%)

Training 5 : 1261 (3.55%)

Training 6 : 193 (0.54%)

Test 0      : 3277 (21.54%)

Test 1      : 5481 (36.03%)

Test 2     : 880 (5.78%)

Test 3     : 1311 (8.62%)

Test 4     : 3655 (24.02%)

Test 5     : 548 (3.60%)

Test 6     : 62 (0.41%)

Features used to train the algorithm

Total instances 50,712

H1DCe = H1/DC for the expiratory portion of the flow signal frequency spectrum

H1DCi = H1/DC for the inspiratory portion of the flow signal frequency spectrum

H1DCp = H1/DC for the pressure signal frequency spectrum

Ins/Exp = H1DCi/ H1DCe

Pre/Exp = H1DCp/ H1DCe

Pre/Ins = H1DCp/ H1DCi

FIO2 = Fraction inspired oxygen

RR = Respiratory rate

TV = Tidal volume

MV = Minute ventilation

Peak P = Peak pressure

PEEP = Positive End Expiratory Pressure

COV Ttot = Covariance of the total breathing cycle time (Ttotal)

COV PPre = Covariance of peak pressure

Age = Patient’s age

SpecHead = Mean value of the first five cpm of the inspiratory frequency spectrum

Insp H2DC = H2/DC for the inspiratory portion of the flow signal frequency spectrum

Insp H3DC = H3/DC for the inspiratory portion of the flow signal frequency spectrum

Area = Inspiratory spectrum’s area

Correlation matrix

H1DCe         -0.502009

H1DCi          -0.491316

H1DCp         -0.434080

Ins/Exp         0.080394

Pre/Exp       -0.108517

Pre/Ins        -0.263684

FIO2              0.084131

RR                  0.195331

TV                 -0.105130

MV                 0.059583

Peak P          -0.214461

PEEP             -0.065990

COV Ttot       0.426755

COV PPre      0.460087

Age                0.149975

SpecHead     0.387298

Insp H2DC   -0.448347

Insp H3DC   -0.366241

Area              0.324640

Random Forest Classifier

Accuracy with training data

Accuracy: 1.0000

Accuracy with test data

Accuracy: 0.8169

Classification Report

precision    recall  f1-score   support

0       0.92      0.91      0.91      3277

1       0.80      0.86      0.83      5481

2       0.84      0.65      0.74       880

3       0.83      0.66      0.73      1311

4       0.76      0.81      0.78      3655

5       0.71      0.54      0.61       548

6       1.00      0.73      0.84        62

accuracy           0.82     15214

macro avg         0.84      0.74      0.78     15214

weighted avg       0.82      0.82      0.81     15214

===========================================================================

Naive Bayes Gaussian model

Accuracy with training data

Accuracy: 0.5771

Accuracy with test data

Accuracy: 0.5815

===========================================================================

KNeighbors model x 3

Accuracy with training data

Accuracy: 0.5771

Accuracy with test data

Accuracy: 0.7483

===========================================================================

QuadraticDiscriminantAnalysis model

Accuracy with training data

Accuracy: 0.5771

Accuracy with test data

Accuracy: 0.5972

macro avg         0.56      0.59      0.55     15138

weighted avg       0.62      0.60      0.57     15138

===========================================================================

LogisticRegression

Accuracy with training data

Accuracy: 0.5771

Accuracy with test data

Accuracy: 0.6500

===========================================================================

ADA Boost Classifier

Accuracy with training data

Accuracy: 0.5771

Accuracy with test data

Accuracy: 0.6127

===========================================================================

Decision Tree Classifier

Accuracy with training data

Accuracy: 0.5771

Accuracy with test data

Accuracy: 0.6476

===========================================================================

Neural network model

Accuracy with training data

Accuracy: 0.5771

Accuracy with test data

Accuracy: 0.6848

macro avg          0.71      0.57      0.60     15138

weighted avg        0.70      0.68      0.67     15138

Examples of Epoch Classification