Supporting Information

This study employs a material flow analysis (MFA) approach to comprehensively quantify the inventory, transactions, and movements of materials within the Chinese aluminium industry, encompassing various phases such as mining, smelting, electrolysis, Processing and manufacturing, Recycling management, and Regeneration, over a duration of 33 years from 1990 to 2022. This investigation delineates potential variables of change, computes the carbon emissions associated with both primary aluminium （referred, “Pri-Al”）and secondary aluminium（referred, “Sec-Al”）, and applies the Logarithmic Mean Divisia Index (LMDI) methodology to disaggregate the sector's carbon dioxide emissions into principal factors, including carbon emission coefficient, energy composition, energy efficiency, product mix, and aggregate output. Furthermore, it evaluates the temporal evolution and contribution percentages relative to the baseline year of 1990 and the preceding year, performs correlation analysis, and elucidates the cumulative impact of long-standing policies as well as the influence of short-term policy adjustments on the carbon footprint of the aluminium sector.

Table S1: China's Aluminum Industry Output (1990-2022)/ Unit:10kt

| Tache | Mining | Smelting | Electrolysis | Processing and manufacturing |  | Recycling management |  | Regeneration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Bauxite | Alumina | Primary aluminum | Aluminum alloy | Aluminum | New aluminium scrap | Old scrap aluminum | Secondary aluminum |
| 1990 | 567 | 146 | 85 | 15 | 39 | 0 | 10 | 1 |
| 1991 | 614 | 108 | 96 | 17 | 0 | 0 | 12 | 1 |
| 1992 | 664 | 126 | 110 | 20 | 0 | 0 | 13 | 1 |
| 1993 | 719 | 148 | 125 | 23 | 0 | 0 | 15 | 1 |
| 1994 | 778 | 173 | 150 | 26 | 0 | 0 | 18 | 4 |
| 1995 | 842 | 220 | 187 | 29 | 174 | 0 | 20 | 19 |
| 1996 | 911 | 255 | 190 | 34 | 162 | 0 | 23 | 13 |
| 1997 | 218 | 294 | 218 | 39 | 175 | 0 | 27 | 14 |
| 1998 | 1067 | 323 | 244 | 44 | 0 | 0 | 31 | 10 |
| 1999 | 1155 | 378 | 281 | 51 | 0 | 0 | 36 | 21 |
| 2000 | 1250 | 443 | 299 | 58 | 217 | 0 | 41 | 20 |
| 2001 | 1352 | 518 | 358 | 66 | 234 | 18 | 47 | 65 |
| 2002 | 1464 | 606 | 451 | 76 | 400 | 75 | 55 | 130 |
| 2003 | 1584 | 709 | 596 | 87 | 544 | 82 | 63 | 145 |
| 2004 | 1714 | 830 | 669 | 99 | 533 | 94 | 72 | 166 |
| 2005 | 1741 | 842 | 781 | 93 | 648 | 113 | 81 | 194 |
| 2006 | 1898 | 1326 | 926 | 132 | 879 | 140 | 95 | 235 |
| 2007 | 2045 | 1947 | 1228 | 196 | 1251 | 170 | 105 | 275 |
| 2008 | 2518 | 2303 | 1318 | 251 | 1621 | 165 | 110 | 275 |
| 2009 | 2921 | 2379 | 1289 | 243 | 1593 | 478 | 100 | 578 |
| 2010 | 3684 | 2893 | 1619 | 362 | 1991 | 230 | 170 | 400 |
| 2011 | 3600 | 3417 | 1960 | 378 | 2743 | 225 | 215 | 440 |
| 2012 | 4522 | 4214 | 2230 | 480 | 3074 | 213 | 270 | 483 |
| 2013 | 2801 | 4775 | 2653 | 593 | 4028 | 232 | 295 | 527 |
| 2014 | 6950 | 5380 | 2832 | 635 | 4674 | 195 | 370 | 565 |
| 2015 | 6079 | 5898 | 3152 | 629 | 5237 | 178 | 400 | 578 |
| 2016 | 6500 | 6103 | 3270 | 750 | 5719 | 187 | 443 | 630 |
| 2017 | 7000 | 7033 | 3667 | 792 | 5832 | 190 | 500 | 690 |
| 2018 | 7900 | 7457 | 3645 | 797 | 5023 | 185 | 510 | 695 |
| 2019 | 7500 | 7230 | 3598 | 942 | 5225 | 118 | 607 | 725 |
| 2020 | 6000 | 7313 | 3708 | 964 | 5779 | 39 | 701 | 740 |
| 2021 | 8500 | 7748 | 3850 | 1068 | 6105 | 46 | 784 | 830 |
| 2022 | 7781 | 8186 | 4005 | 1218 | 6222 | 180 | 602 | 782 |

Sources: National Bureau of Statistics, The Yearbook of Nonferrous Industry of China, China Industrial Statistical Yearbook, General Administration of Customs, China Recycling Industry Development Report, Ministry of Commerce

Table S2: China's Primary Aluminum/Electrolytic Aluminum Energy Standard Coal Conversion Factors and Carbon Emission Conversion Factors (CI)

| Energy varieties | Coal | Oil | Natural gas | Electricity |
| --- | --- | --- | --- | --- |
| Carbon emission factor | 0.7143 | 0.9714 | 13.3 | 0.1229 |
| Low calorific value(GJ/t or GJ/104Nm3) | 26.7 | 41.816 | 389.31 | - |
| Carbon content per unit calorific value（*10-3 tC/GJ） | 27.4 | 20.1 | 15.3 | - |
| Oxidation rate（%） | 94 | 98 | 99 | - |

Sources: GB/T 32150.4 Greenhouse Gas Emission Accounting and Reporting Requirements - Part 4: Aluminum Smelting Enterprises

Table S3: GHG emission factor (EF) of thermal power generation in China (Unit: kgCO2/MWh)

| Year | Primary aluminum |  | Secondary aluminum |  |
| --- | --- | --- | --- | --- |
|  | Grid electricity | Self-generated electricity | Grid electricity | Self-generated electricity |
| 1990 | 0.79 | 1.06 | 1.33 | 1.44 |
| 1991 | 0.78 | 1.05 | 1.29 | 1.40 |
| 1992 | 0.77 | 1.04 | 1.26 | 1.36 |
| 1993 | 0.75 | 1.02 | 1.22 | 1.31 |
| 1994 | 0.74 | 1.01 | 1.19 | 1.27 |
| 1995 | 0.75 | 1.02 | 1.16 | 1.24 |
| 1996 | 0.74 | 1.02 | 1.13 | 1.20 |
| 1997 | 0.74 | 1.02 | 1.10 | 1.16 |
| 1998 | 0.73 | 1.00 | 1.07 | 1.13 |
| 1999 | 0.72 | 0.99 | 1.04 | 1.09 |
| 2000 | 0.71 | 0.97 | 1.01 | 1.06 |
| 2001 | 0.69 | 0.96 | 0.99 | 1.03 |
| 2002 | 0.69 | 0.95 | 0.96 | 0.99 |
| 2003 | 0.68 | 0.94 | 0.93 | 0.96 |
| 2004 | 0.67 | 0.93 | 0.91 | 0.93 |
| 2005 | 0.66 | 0.92 | 0.88 | 0.91 |
| 2006 | 0.66 | 0.91 | 0.88 | 0.87 |
| 2007 | 0.64 | 0.89 | 0.91 | 0.92 |
| 2008 | 0.62 | 0.86 | 0.92 | 0.91 |
| 2009 | 0.61 | 0.85 | 0.83 | 0.81 |
| 2010 | 0.59 | 0.83 | 0.68 | 0.68 |
| 2011 | 0.58 | 0.82 | 0.68 | 0.68 |
| 2012 | 0.57 | 0.81 | 0.65 | 0.65 |
| 2013 | 0.57 | 0.80 | 0.74 | 0.73 |
| 2014 | 0.57 | 0.79 | 0.72 | 0.73 |
| 2015 | 0.56 | 0.78 | 0.66 | 0.72 |
| 2016 | 0.55 | 0.78 | 0.64 | 0.66 |
| 2017 | 0.55 | 0.77 | 0.64 | 0.66 |
| 2018 | 0.54 | 0.76 | 0.60 | 0.58 |
| 2019 | 0.53 | 0.74 | 0.61 | 0.58 |
| 2020 | 0.52 | 0.73 | 0.61 | 0.58 |
| 2021 | 0.51 | 0.72 | 0.60 | 0.54 |
| 2022 | 0.50 | 0.71 | 0.60 | 0.54 |

Sources: National Grid Carbon Emission Factors, Calculation and Forecast of National Grid Carbon Emission Factors from 2005 to 2035

Table S4: China's Aluminum Industry Energy Structure (%)

| Year | Primary aluminum |  |  |  |  |  |  | Secondary aluminum |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | coal | oil | gas | Grid electricity | Other traditional energy sources | hydropower | Other renewable energy sources | coal | gas | Grid electricity |
| 1990 | 78 | 31 | 11 | 5 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1991 | 78 | 3 | 11 | 6 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1992 | 78 | 3 | 11 | 6 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1993 | 78 | 3 | 11 | 6 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1994 | 77 | 3 | 11 | 6 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1995 | 77 | 3 | 10 | 6 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1996 | 77 | 3 | 10 | 7 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1997 | 77 | 4 | 10 | 7 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1998 | 76 | 4 | 10 | 7 | 1 | 2 | 0 | 28 | 53 | 19 |
| 1999 | 76 | 4 | 10 | 7 | 1 | 2 | 0 | 28 | 53 | 19 |
| 2000 | 76 | 4 | 10 | 7 | 1 | 2 | 0 | 28 | 53 | 19 |
| 2001 | 77 | 3 | 10 | 7 | 1 | 2 | 0 | 28 | 53 | 19 |
| 2002 | 77 | 3 | 9 | 7 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2003 | 78 | 3 | 9 | 6 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2004 | 78 | 3 | 8 | 6 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2005 | 78 | 3 | 8 | 6 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2006 | 78 | 3 | 9 | 6 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2007 | 77 | 3 | 9 | 7 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2008 | 77 | 3 | 9 | 7 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2009 | 78 | 3 | 9 | 7 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2010 | 78 | 3 | 9 | 7 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2011 | 78 | 3 | 9 | 7 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2012 | 78 | 3 | 9 | 6 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2013 | 78 | 3 | 8 | 6 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2014 | 79 | 3 | 8 | 6 | 1 | 3 | 0 | 28 | 53 | 19 |
| 2015 | 79 | 3 | 8 | 6 | 1 | 4 | 0 | 0 | 95 | 5 |
| 2016 | 79 | 3 | 8 | 6 | 1 | 4 | 0 | 0 | 95 | 5 |
| 2017 | 79 | 3 | 8 | 6 | 1 | 4 | 0 | 0 | 95 | 5 |
| 2018 | 79 | 3 | 8 | 6 | 1 | 3 | 1 | 0 | 95 | 5 |
| 2019 | 75 | 3 | 8 | 6 | 1 | 4 | 4 | 0 | 95 | 5 |
| 2020 | 76 | 3 | 7 | 6 | 1 | 6 | 2 | 0 | 95 | 5 |
| 2021 | 81 | 4 | 3 | 5 | 0 | 6 | 1 | 0 | 95 | 5 |
| 2022 | 78 | 4 | 3 | 5 | 0 | 8 | 2 | 0 | 95 | 5 |

Sources: IAI

Table S5: Output and product mix of primary and secondary aluminum products in China（1990-2022）

| Year | Production |  |  | Structure |  |
| --- | --- | --- | --- | --- | --- |
|  | Pri-Al | Sec-Al | Total | Pri-Al | Sec-Al |
| 1990 | 85 | 1 | 86 | 99.16% | 0.84% |
| 1991 | 96 | 1 | 97 | 99.25% | 0.75% |
| 1992 | 110 | 1 | 110 | 99.47% | 0.53% |
| 1993 | 125 | 1 | 126 | 99.01% | 0.99% |
| 1994 | 150 | 4 | 153 | 97.64% | 2.36% |
| 1995 | 187 | 19 | 206 | 90.62% | 9.38% |
| 1996 | 190 | 13 | 203 | 93.61% | 6.39% |
| 1997 | 218 | 14 | 232 | 93.82% | 6.18% |
| 1998 | 244 | 10 | 253 | 96.07% | 3.93% |
| 1999 | 281 | 21 | 302 | 93.03% | 6.97% |
| 2000 | 299 | 20 | 318 | 93.87% | 6.13% |
| 2001 | 358 | 65 | 423 | 84.62% | 15.38% |
| 2002 | 451 | 130 | 581 | 77.63% | 22.37% |
| 2003 | 596 | 145 | 741 | 80.44% | 19.56% |
| 2004 | 669 | 166 | 835 | 80.12% | 19.88% |
| 2005 | 781 | 194 | 975 | 80.10% | 19.90% |
| 2006 | 926 | 235 | 1161 | 79.77% | 20.23% |
| 2007 | 1228 | 275 | 1504 | 81.70% | 18.30% |
| 2008 | 1318 | 275 | 1593 | 82.73% | 17.27% |
| 2009 | 1289 | 578 | 1867 | 69.04% | 30.96% |
| 2010 | 1619 | 400 | 2019 | 80.19% | 19.81% |
| 2011 | 1960 | 440 | 2400 | 81.67% | 18.33% |
| 2012 | 2230 | 483 | 2713 | 82.21% | 17.79% |
| 2013 | 2653 | 527 | 3181 | 83.42% | 16.58% |
| 2014 | 2832 | 565 | 3396 | 83.37% | 16.63% |
| 2015 | 3152 | 578 | 3729 | 84.51% | 15.49% |
| 2016 | 3270 | 630 | 3900 | 83.85% | 16.15% |
| 2017 | 3667 | 690 | 4357 | 84.16% | 15.84% |
| 2018 | 3645 | 695 | 4340 | 83.98% | 16.02% |
| 2019 | 3598 | 725 | 4323 | 83.23% | 16.77% |
| 2020 | 3708 | 740 | 4448 | 83.36% | 16.64% |
| 2021 | 3850 | 830 | 4680 | 82.27% | 17.73% |
| 2022 | 4005 | 782 | 4787 | 83.66% | 16.34% |

Sources: The Yearbook of Nonferrous Industry of China

Table S6: Energy Intensity and Carbon Emissions of Primary Aluminum and Secondary Aluminum ( 1990-2022)

| Year | Energy intensity(kgce/t) |  | Carbon emissions(CO2-eq) |  |
| --- | --- | --- | --- | --- |
|  | Pri-Al | Sec-Al | Pri-Al | Sec-Al |
| 1990 | 4299 | 4299 | 2036 | 2 |
| 1991 | 4218 | 4218 | 2237 | 2 |
| 1992 | 4051 | 4051 | 2436 | 1 |
| 1993 | 3882 | 3882 | 2655 | 3 |
| 1994 | 3866 | 3866 | 3140 | 7 |
| 1995 | 3685 | 3685 | 3716 | 39 |
| 1996 | 3630 | 3630 | 3718 | 26 |
| 1997 | 3671 | 3671 | 4270 | 28 |
| 1998 | 3654 | 3654 | 4663 | 19 |
| 1999 | 3596 | 3596 | 5297 | 40 |
| 2000 | 3718 | 3718 | 5793 | 36 |
| 2001 | 3508 | 3508 | 6470 | 113 |
| 2002 | 3381 | 3381 | 7798 | 226 |
| 2003 | 2969 | 2969 | 8894 | 252 |
| 2004 | 2964 | 2964 | 9780 | 287 |
| 2005 | 2930 | 2930 | 11313 | 334 |
| 2006 | 3110 | 3110 | 14416 | 386 |
| 2007 | 3389 | 3389 | 21298 | 448 |
| 2008 | 3352 | 3352 | 22544 | 433 |
| 2009 | 3239 | 3239 | 20992 | 951 |
| 2010 | 3302 | 3302 | 26788 | 620 |
| 2011 | 3355 | 3355 | 32736 | 643 |
| 2012 | 3343 | 3343 | 36772 | 671 |
| 2013 | 2888 | 2888 | 38062 | 713 |
| 2014 | 2688 | 2688 | 37935 | 962 |
| 2015 | 2636 | 2636 | 41013 | 1502 |
| 2016 | 2562 | 2562 | 40901 | 1601 |
| 2017 | 2793 | 2793 | 49706 | 1901 |
| 2018 | 2586 | 2586 | 46590 | 1889 |
| 2019 | 2557 | 2557 | 43820 | 1963 |
| 2020 | 2412 | 2412 | 42928 | 1979 |
| 2021 | 2356 | 2356 | 33186 | 2219 |
| 2022 | 2318 | 2318 | 33108 | 2091 |

Sources: 1990-1999&2021-2022，The Yearbook of Nonferrous Industry of China；2000-2020，Central South University

Table S7: LMDI for GHG emissions in the Chinese aluminum industry（Based on 1990s）

| Year | Carbon emission factor(CI) |  | Energy structure(ES) |  | Energy intensity(EI) |  | Product Structure(QS) |  | Total production（Q） |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Variable value/10kt | Contribution rate/% | Variable value/10kt | Contribution rate/% | Variable value/10kt | Contribution rate/% | Variable value/10kt | Contribution rate/% | Variable value/10kt | Contribution rate/% |
| 1991 | -30 | 7.42% | -41 | 9.96% | 75 | 18.33% | 2 | 0.40% | 260 | 63.89% |
| 1992 | -61 | 3.48% | -133 | 7.51% | 152 | 8.63% | 6 | 0.36% | -1412 | 80.01% |
| 1993 | -98 | 11.09% | -238 | 26.95% | 233 | 26.47% | -3 | 0.37% | 310 | 35.12% |
| 1994 | -142 | 12.86% | -270 | 24.52% | 332 | 30.09% | -36 | 3.22% | 323 | 29.31% |
| 1995 | -136 | 8.48% | -431 | 26.83% | 422 | 26.27% | -223 | 13.91% | 394 | 24.50% |
| 1996 | -157 | 8.72% | -473 | 26.33% | 593 | 32.99% | -143 | 7.98% | 431 | 23.98% |
| 1997 | -184 | 8.45% | -478 | 21.97% | 744 | 34.23% | -149 | 6.84% | 619 | 28.50% |
| 1998 | -236 | 8.80% | -516 | 19.29% | 978 | 36.51% | -90 | 3.34% | 859 | 32.06% |
| 1999 | -304 | 9.74% | -611 | 19.55% | 1028 | 32.90% | -193 | 6.16% | 989 | 31.65% |
| 2000 | -387 | 11.65% | -523 | 15.77% | 1048 | 31.58% | -175 | 5.27% | 1185 | 35.73% |
| 2001 | -493 | 12.63% | -785 | 20.13% | 760 | 19.48% | -532 | 13.64% | 1331 | 34.12% |
| 2002 | -590 | 12.11% | -1040 | 21.35% | 529 | 10.86% | -902 | 18.50% | 1812 | 37.18% |
| 2003 | -685 | 11.51% | -1732 | 29.10% | -381 | 6.40% | -818 | 13.74% | 2337 | 39.26% |
| 2004 | -790 | 11.06% | -1847 | 25.85% | -936 | 13.10% | -878 | 12.29% | 2693 | 37.69% |
| 2005 | -962 | 11.57% | -2089 | 25.11% | -1012 | 12.17% | -958 | 11.51% | 3298 | 39.65% |
| 2006 | -1176 | 12.72% | -2067 | 22.35% | -396 | 4.28% | -1154 | 12.48% | 4453 | 48.17% |
| 2007 | -1689 | 12.86% | -1975 | 15.04% | 1071 | 8.16% | -1345 | 10.25% | 7049 | 53.69% |
| 2008 | -2011 | 14.34% | -2147 | 15.31% | 936 | 6.67% | -1313 | 9.36% | 7618 | 54.32% |
| 2009 | -2149 | 15.45% | -2341 | 16.83% | -3 | 0.02% | -2406 | 17.29% | 7011 | 50.40% |
| 2010 | -2885 | 17.45% | -2569 | 15.53% | -10 | 0.06% | -1712 | 10.35% | 9361 | 56.61% |
| 2011 | -3434 | 17.03% | -2784 | 13.80% | -267 | 1.32% | -1816 | 9.00% | 11869 | 58.85% |
| 2012 | -3877 | 16.53% | -3068 | 13.08% | -946 | 4.03% | -1912 | 8.15% | 13653 | 58.21% |
| 2013 | -4004 | 14.56% | -4950 | 18.00% | -2103 | 7.65% | -1779 | 6.47% | 14664 | 53.32% |
| 2014 | -4134 | 14.31% | -5799 | 20.07% | -2777 | 9.61% | -1681 | 5.82% | 14506 | 50.20% |
| 2015 | -4637 | 14.46% | -6424 | 20.03% | -4106 | 12.80% | -1436 | 4.48% | 15468 | 48.23% |
| 2016 | -4801 | 14.10% | -6789 | 19.93% | -5430 | 15.94% | -1489 | 4.37% | 15550 | 45.65% |
| 2017 | -5655 | 14.86% | -6512 | 17.11% | -5328 | 14.00% | -1659 | 4.36% | 18904 | 49.67% |
| 2018 | -5603 | 15.14% | -7315 | 19.76% | -4695 | 12.69% | -1580 | 4.27% | 17817 | 48.14% |
| 2019 | -5735 | 14.89% | -7157 | 18.58% | -7257 | 18.84% | -1560 | 4.05% | 16803 | 43.63% |
| 2020 | -5849 | 14.86% | -7839 | 19.92% | -7561 | 19.21% | -1499 | 3.81% | 16607 | 42.20% |
| 2021 | -5140 | 13.00% | -6809 | 17.22% | -13173 | 33.31% | -1150 | 2.91% | 13279 | 33.57% |
| 2022 | -5283 | 12.63% | -6977 | 16.68% | -15195 | 36.32% | -1030 | 2.46% | 13353 | 31.92% |

Table S8: Driving forces for GHG emissions in the Chinese aluminum industry（Based on previous year/N-1）

| Year | Carbon emission factor(CI) |  | Energy structure(ES) |  | Energy intensity(EI) |  | Product Structure(QS) |  | Total production（Q） |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Variable value/10kt | Contribution rate/% | Variable value/10kt | Contribution rate/% | Variable value/10kt | Variable value/10kt | Contribution rate/% | Variable value/10kt | Contribution rate/% | Variable value/10kt |
| 1991 | -37 | 7.79% | 81 | 17.21% | -94 | 20.11% | 5 | 1.03% | 253 | 53.87% |
| 1992 | -39 | 7.26% | 87 | 16.02% | -108 | 19.89% | -11 | 1.97% | 298 | 54.87% |
| 1993 | 24 | 4.95% | 72 | 14.49% | -12 | 2.45% | -36 | 7.34% | 350 | 70.78% |
| 1994 | -28 | 2.35% | 227 | 18.76% | -164 | 13.58% | -229 | 18.91% | 562 | 46.40% |
| 1995 | -20 | 1.47% | 137 | 10.23% | -56 | 4.21% | 108 | 8.09% | 1019 | 76.01% |
| 1996 | -61 | 13.61% | 276 | 61.41% | 44 | 9.78% | 8 | 1.81% | -60 | 13.39% |
| 1997 | -73 | 9.58% | -35 | 4.53% | -21 | 2.77% | 95 | 12.49% | 539 | 70.63% |
| 1998 | -103 | 13.27% | -54 | 6.98% | -80 | 10.34% | -144 | 18.57% | 393 | 50.84% |
| 1999 | -122 | 6.77% | -573 | 31.83% | 185 | 10.29% | 45 | 2.49% | 875 | 48.62% |
| 2000 | -58 | 3.20% | -533 | 29.24% | -361 | 19.79% | -573 | 31.46% | 297 | 16.30% |
| 2001 | -86 | 1.96% | -1710 | 39.19% | -262 | 6.01% | -552 | 12.65% | 1754 | 40.19% |
| 2002 | -123 | 2.56% | -1006 | 20.98% | -1083 | 22.58% | 264 | 5.51% | 2319 | 48.36% |
| 2003 | -191 | 8.12% | 28 | 1.17% | -17 | 0.73% | -33 | 1.39% | 2086 | 88.58% |
| 2004 | -115 | 3.85% | 1594 | 53.56% | -124 | 4.16% | -2 | 0.06% | 1142 | 38.37% |
| 2005 | -350 | 5.61% | 3406 | 54.64% | 748 | 12.00% | -48 | 0.77% | 1681 | 26.97% |
| 2006 | -664 | 12.50% | -458 | 8.61% | 1508 | 28.38% | 381 | 7.17% | 2303 | 43.33% |
| 2007 | -563 | 6.93% | -2395 | 29.49% | -255 | 3.13% | 249 | 3.06% | 4661 | 57.38% |
| 2008 | -1046 | 15.81% | -15 | 0.22% | -713 | 10.79% | -3551 | 53.71% | 1287 | 19.47% |
| 2009 | -332 | 4.04% | -685 | 8.35% | 411 | 5.01% | 3213 | 39.12% | 3570 | 43.48% |
| 2010 | -454 | 8.73% | -1900 | 36.53% | 431 | 8.28% | 493 | 9.48% | 1923 | 36.97% |
| 2011 | -45 | 0.49% | -3445 | 37.91% | -151 | 1.66% | 211 | 2.32% | 5237 | 57.62% |
| 2012 | -361 | 2.82% | -2103 | 16.44% | -5498 | 42.98% | 499 | 3.90% | 4330 | 33.85% |
| 2013 | -684 | 5.16% | -3964 | 29.88% | -2532 | 19.08% | -20 | 0.15% | 6067 | 45.73% |
| 2014 | -601 | 6.92% | -4183 | 48.18% | -899 | 10.36% | 450 | 5.19% | 2549 | 29.36% |
| 2015 | -374 | 4.44% | 2807 | 33.25% | -1198 | 14.18% | -259 | 3.07% | 3804 | 45.06% |
| 2016 | -805 | 9.81% | 1337 | 16.29% | 4033 | 49.11% | 136 | 1.66% | 1900 | 23.13% |
| 2017 | -1178 | 6.10% | -9128 | 47.24% | -3731 | 19.31% | -85 | 0.44% | 5200 | 26.91% |
| 2018 | -626 | 21.12% | -1314 | 44.35% | -513 | 17.32% | -317 | 10.69% | -193 | 6.52% |
| 2019 | -764 | 2.86% | -23148 | 86.65% | -2554 | 9.56% | 55 | 0.21% | -193 | 0.72% |
| 2020 | -494 | 5.39% | -6115 | 66.72% | -890 | 9.71% | -368 | 4.01% | 1298 | 14.16% |
| 2021 | -30 | 0.98% | 75 | 2.43% | -548 | 17.85% | 381 | 12.43% | 2034 | 66.30% |
| 2022 | -31 | 3.32% | 78 | 8.25% | -41 | 4.30% | 2 | 0.17% | 793 | 83.96% |