Towards all-inorganic antimony sulphide semi-transparent solar cells

Abstract. NiO, a wide band gap hole-transporting material (HTM), is gaining attention in photovoltaics due to its optical transparency, chemical stability, and favourable band alignment with absorber. This study uses NiOx nanoparticle-based HTM in semi-transparent Sb2S3 solar cells via a simple chemical precipitation method. We optimized NiOx layer by varying precursor solution concentration and studied its impact on optical and structural properties, composition of nanoparticles and subsequent effect on the performance of semi-transparent Sb2S3 solar cell. NiOx nanoparticles, deposited from nickel(II)nitrate hexahydrate (precursor solution concentrations of 0.2 M to 1.2 M), were thermally treated by two steps at 90°C for 6 h and 270°C for 3 h. Nanoparticles with crystallite sizes of 6 – 9 nm had band gaps (Eg) of ca 3.65 – 3.70 eV. Using 1.2 M concentration yielded the largest crystallites (9 nm), lowest Eg (3.65 eV) while retaining the most organic residues. The highest power conversion efficiency (2.65 %) was achieved with NiOx from a 0.5 M precursor, a 60 % improvement over HTM-free cells. The effect of precursor solution concentration on the solar cell parameters (efficiency, fill factor, open circuit voltage and short circuit current) are discussed. Present work paves a path toward stable, efficient, and cost-effective all-inorganic Sb2S3 solar cells using NiOx HTM instead of organic counterparts.

Keywords: Hole transport layer, Sb2S3 solar cells, NiOx nanoparticles, Chemical precipitation, NiO thin film.