Supplemental Materials

text-s1

Contrast-enhanced CT:

CT examination was conducted with a MDCT (Cohort 1, SOMATOM Definition AS+ & Dual Energy Force, Siemens, Forchheim, Germany; Cohort 2, Revolution CT, GE Healthcare, Waukesha, Wisconsin, US; Cohort 3, iCT, Philips Healthcare, Amsterdam, Netherlands) in three institutions before the surgical treatment. Precontrast CT scans were obtained before contrast media injection. Non-ionic contrast medium (Ultravist; 300 mgI/mL, Bayer, Berlin, Germany) was injected into the antecubital vein with a dose of 60-110 ml (1.5 ml per kilogram of body weight) and an average injection rate of 3.0 ml/s through a venous indwelling needle (20 or 22 gauge). By using the automated scan-triggering software (Care-Bolus; Siemens Medical Systems, Iselin, NJ), the arterial phase and portal venous phase scan started automatically with a 15 s delay and a 50 s delay after the attenuation value of abdominal aorta reached 100 HU, respectively. After the acquisition of the portal venous phase images, delayed phase images were acquired with a delay of 180 s. CT scans were obtained at a tube current-time product of 180mAs and a peak voltage of 120kVp. Whole-liver scanning was completed in 4 to 8seconds while patients held their breath. Images were reconstructed with a 5 mm slice thickness.

Gd-based contrast-enhanced MRI

354 patients were examined on a 1.5T MR scanner (HDxt Signa, GE medical system, USA) and a 3.0 T MR scanner (Skyra, Siemens Healthcare). The Gd-EOB-DTPA MRI of the liver in our institute were performed with the following sequences: axial T1-weighted dual-echo in-phase and opposed-phase gradient-echo (GRE) sequence and T2-weighted single-shot fast spin echo sequence (SS-FSE). A single-shot echo-planar imaging sequence (b-value of 0 and 800 s/mm2) was used to obtain diffusion-weighted imaging (DWI). Finally, a total of 0.025 mmol/kg of Gd-EOB-DTPA (Primovist; Bayer, Berlin, Germany) or 0.1 mmol/kg Gd-DTPA (Magnevist, Bayer, Berlin, Germany) was injected via the antecubital vein at a rate of 2.0 to 3.0 mL/s and followed by a 20-ml saline flush. Breath-hold serial axial T1-weighted 3D spoiled GRE fat-suppressed images with slice interpolation was repeated before and after the injection of Gd-based contrast agents. In addition to a pre-contrast acquisition, the dynamic imaging phases including arterial phase (AP), porta-venous phase (PVP), the transitional phase (TP) and hepatobiliary phase (HBP) were obtained at 20 sec, 45 sec, 3 min, and 15 min after the agent injection, respectively.

Supplemental figure legends:

Suppl fig. 1. The flowchart of the pipeline for Liver Tumor Imaging Staging (LTIS) tool development. In first stage, individual images within the derivation cohort of study are interpreted by a panel of radiologists (a). This first-stage procedure produces an expert-based structural report for HCC (b). Subsequently, in second stage, the dedicative imaging features are modelled with a multivariate regression analysis approach, and the factors significantly connected to HCC aggressiveness are selected and clustered (c). In last step, the findings of the regression analysis are simplified and presented as a decision tree and a five-score diagnostic scheme (d), which is intuitive and easier for the interpretation of HCC aggressiveness.

Suppl fig. 2. The incidence of tumor PG1 to PG3 stratified by LTIS assignment in center 1 (a), center 2 (b) and center 3 (c).

Suppl fig. 3. Forest plots of RFS (a) and OS (b) with a Cox’s proportional hazard analysis of clinical variables, LTIS score, AJCC stage and histopathological aggressive grade. RFS = recurrence-free survival; OS = overall survival. *, p < 0.05; **, p < 0.01; ***, p < 0.001.