# Three-Dimensional Optical Trapping with a Low-NA Objective using a Flat-top Beam

# Laurynas Lialys1 and Shima Fardad1,2,*

1Department of Electrical Engineering & Computer Science, University of Kansas, Lawrence, 66045, USA

2I2S, Institute for Information Sciences, University of Kansas, Lawrence, 66045, USA

*sfardad@ku.edu

# Extinction Cross-Section Calculation

For a sphere, the extinction cross-section Cext can be expressed in terms of the Mie coefficients an  and bn as [1]:

where k = 2πn2/λ is the wavenumber in the surrounding medium, λ is the vacuum wavelength, and n2 is the refractive index of the medium. The Mie coefficients an and bn describe the electric and magnetic multipole scattering amplitudes for a sphere of radius r, relative refractive index m = n1/n2, and size parameter q = 2πRn2/λ, where: r is the radius of the sphere. The number of terms N to retain in the Mie expansion is accurately estimated via the widely used empirical rule [2] N ≈ q + 4q1/3 + 2 rounded up to the next integer. This ensures accurate computation of the scattering and extinction efficiencies while maintaining computational efficiency. For our experiments, we considered polystyrene spheres (n1 = 1.58) in water (n2 = 1.33) illuminated at a wavelength of λ = 532nm. The number of coefficients to be retained is then N = 23. The computed coefficients are shown in Fig. S1. The resulting extinction cross-section is Cext ≈ 2.2µm2.

Fig. S1. Magnitude of Mie coefficients |an| and |bn| as a function of size parameter x. Peaks correspond to electric and magnetic resonances of different orders.

# References

- Craig F Bohren and Donald R Huffman. Absorption and scattering of light by small particles. John Wiley & Sons, 2008.

- Warren J Wiscombe. Mie scattering calculations: Advances in technique and fast, vectorspeed computer codes, volume 10. National Technical Information Service, US Department of Commerce, 1979.