Supplementary Information for:

UV light combined with nitrate remove bisphenol A: kinetics, transformation pathways, and acute toxicity assessment

Zhiquan Lia, Congwei Luoa,b*,Fengxun Tana, Daoji Wua,b, Shishun Wanga, Xiaoxiang Chenga, Fumiao Zhanga, Mingli Lia, Qiao Mac

a School of Municipal and Environmental Engineering, Shandong Jianzhu University, Jinan, 250101, P. R. China.

b Resources and Environment Innovation Institute，Shandong Jianzhu University, Jinan, 250101, P. R. China.

c National Engineering Lab of Coal-Fired Pollution Emission Reduction, School of Energy and Power Engineering, Shandong University, Jinan 250061, P. R. China

*Corresponding authors contact details:

Doc. Congwei Luo, Phone: 86-531-86367291, email: luocongwei2009@163.com

Table of Contents

Text S1 Details of LC/MS/MS and the acute toxicity test method.

Text S2 Calculating the contributions from UV, ·OH, and RNS.

Table S1 [·OH]ss at different concentrations of NO3− in UV/nitrate process.

Table S2 Calculations of the contributions from UV, ·OH, and RNS in UV/nitrate process (Taking control experiment as an example).

Table S3 Actual waters quality information.

Table S4 Detected intermediate products of bisphenol A in UV/nitrate process.

Figure S1 Removal ratios of BPA in sole NO3−, direct UV and UV/nitrate process.

Figure S2 Removal ratios of UV, ·OH, and RNS in UV/nitrate process.

Figure S3 Changes of TOC before and after reaction in UV/nitrate process.

Figure S4 LC/MS/MS results of BPA degradation in UV/nitrate process.

Text S1 Details of LC/MS/MS and the acute toxicity test method.

The LC/MS/MS system was composed with Waters alliance 2695 XELC/MS/MS (Waters, Watford, UK), Zobax SB-C18 (5μm, 4.6 mm × 250 mm, Agilent, USA). Separation was accomplished with a gradient mode: mobile phase A, water; mobile phase B, ACN; flow rate 0.3 mL/min, the ratio of A to B, 0~3min, 70:30; 3~4min, 70:30 to 95:5; 4~6min, 95:5 to 100:0; 6~25min, 100:0; 25~30min, 100:0 to 70:30. This method utilized ESI-LC/MS/MS operating in MRM mode. The Waters alliance 2695 Quattro Premier XE was used in negative ion ESI. The ESI settings were the following: capillary voltage, 3.5kV; cone voltage, 40V; the flow of desolvation gas (Argon gas), 800L/h; the flow of cone gas, 20L/h; collision energy, 20V.

In terms of the acute toxicity test method, 75 μL osmotic pressure adjusting solution was added into 675 μL sample and then 250 μL resuscitated bacterial solution was added to each sample. Samples were mixed and measured after a certain interval of time. Note that methanol used in sample concentration for toxicity analysis was blown away and diluted in phosphate buffer.

Text S2 Calculating the contributions from UV, ·OH, and RNS.

(1)RUV=0tkUV, BPABPAdt=kUV, BPA0t1BPAdt+…+tn-1tnBPAdt

(2)R∙OH=0tk∙OH, BPA∙OHssBPAdt=k∙OH, BPA∙OHss0tBPAdt

=k∙OH, BPA∙OHss0t1BPAdt+…+tn-1tnBPAdt

Where tn-1tnBPAdt=BPA×tn-tn-1, and BPA is the average concentration of BPA between time tn and tn-1.

(3)RRNS=R-RUV-R∙OH

Table S1 [·OH]ss at different concentrations of NO3− in UV/nitrate process.

| NO3- (mM) | 0 | 10 | 20 | 30 | 50 |
| --- | --- | --- | --- | --- | --- |
| kobs, NB | 4.87510-5 | 5.09510-5 | 5.32410-5 | 6.92510-5 | 7.78010-5 |
| R2 | 0.991 | 0.998 | 0.980 | 0.998 | 0.998 |
| [·OH]ss (M) | - | 5.6410-16 | 1.1510-15 | 5.2610-15 | 7.4510-15 |

Table S2 Calculations of the contributions from UV, ·OH, and RNS in UV/nitrate process (Taking control experiment as an example).

| Time (s) | [BPA] (μM) | kUV, BPA ×tn-1tnBPAdt | RUV (μM) | k∙OH, BPA∙OHss ×tn-1tnBPAdt | R·OH (μM) | RRNS (μM) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 30 | 0 | 0 | 0 | 0 | 0 |
| 300 | 29.03221 | 0.036898 | 0.036898 | 0.474755 | 0.474755 | 0.456137 |
| 600 | 27.63855 | 0.035422 | 0.072320 | 0.455764 | 0.930519 | 1.358611 |
| 1200 | 26.07389 | 0.067146 | 0.139466 | 0.863944 | 1.794463 | 1.992181 |
| 1800 | 24.79445 | 0.063591 | 0.203057 | 0.818198 | 2.612661 | 2.389833 |
| 2400 | 23.30352 | 0.060127 | 0.263184 | 0.773637 | 3.386298 | 3.046998 |
| 3000 | 22.00291 | 0.056638 | 0.319821 | 0.728737 | 4.115034 | 3.562234 |
| 3600 | 20.8828 | 0.053611 | 0.373433 | 0.689800 | 4.804835 | 3.938933 |

Table S3 Actual waters quality information.

| actual waters | surface water | groundwater | drinking water |
| --- | --- | --- | --- |
| TOC (mg-C/L) | 4.115 | 1.008 | 2.254 |
| total alkalinity (mg CaCO3/L) | 230.63 | 252.41 | 154.01 |
| NO3- (mg-N/L) | 10.418 | 11.41 | 3.67 |
| NO2- (mg-N/L) | 0.1 | nd | 0.003 |
| pH | 7.81 | 7.61 | 8.21 |

Table S4 Detected intermediate products of bisphenol A in UV/nitrate process.

| Compound | RT (time) | [M-H]- |  | Chemical Formula | Proposed Structure |
| --- | --- | --- | --- | --- | --- |
|  |  | Theoretical m/z | Observed m/z |  |  |
| B79 | 6.789 | 79.13 | 78.9 | C6H8 |  |
| B92 | 29.135 | 92.09 | 91.9 | C6H5O |  |
| B93 | 29.135 | 93.1 | 93.0 | C6H6O |  |
| B97 | 6.789 | 97.14 | 96.8 | C6H10O |  |
| B131 | 7.428 | 131.16 | 131 | C9H8O |  |
| B135 | 29.135 | 135.19 | 134.9 | C9H12O |  |
| B136 | 29.135 | 136.14 | 135.9 | C7H7O2N |  |
| B151 | 6.789 | 151.19 | 151.1 | C9H12O2 |  |
| B181 | 7.428 | 181.17 | 181.1 | C9H10O4 |  |
| B199 | 7.428 | 199.11 | 199.2 | C6H4O6N2 |  |

Figure S1 Removal ratios of BPA in sole NO3−, direct UV and UV/nitrate process. Experimental conditions: [BPA] = 30 μM, [NO3-] = 30 mM, pH = 7.0, [phosphate] = 5 mM, time = 3600 s, I = 0.172 mW cm-2.

Figure S2 Removal ratios of UV, ·OH, and RNS in UV/nitrate process. Experimental conditions:[BPA] = 30 μM, [NO3-] = 30 mM, pH = 7.0, [phosphate] = 5 mM, time = 3600 s, I = 0.172 mW·cm-2.

Figure S3 Changes of TOC before and after reaction in UV/nitrate process. Experimental conditions:[BPA] = 30 μM, [NO3-] = 30 mM, pH = 7, [phosphate] = 5 mM, time = 3600 s, I = 0.172 mW cm-2.

Figure S4 LC/MS/MS results of BPA degradation in UV/nitrate process. (a) LC chromatogram of BPA in UV/nitrate process. (b) The mass spectra of B79, B97, B151. (c) The mass spectra of B92, B93, B135, B136. (d) The mass spectra of B131, B181, B199.