SUPPLEMENTARY MATERIAL

Subcutaneous pressure measurement protocol

Measurements were performed by a single operator (AD). Subcutaneous interstitial pressure (SIP) was measured on the leg at the same location in all patients, 2–3 cm externally from the tibial crest and 5 cm below the anterior tibial tuberosity. An anesthetic patch (Emlapatch) was applied before each measurement. The SIP was measured using a sterile, single-use, dedicated transducer-tipped pressure probe (Micro-Cath ®, Millar Inc., USA) with 3.5 French diameter. The pressure probe was inserted subcutaneously through a 30 mm, 16G venous catheter. The catheter was then removed from the skin (along the probe length), and the probe was finally removed by 1 cm to allow space around the probe and fixed using an adhesive transparent dressing. The probe was connected to a monitor (Powerlab®, AD Instruments, New Zealand) and data were recorded using dedicated software (Labchart®, AD Instruments, New Zealand). The probe was zeroed in saline at room temperature prior to introduction. SIP was measured after a stabilization phase of 20-30 minutes, averaged over 5 min. After probe removal, pressure readings were used to correct for the measurement drift. In a subgroup of patients, the catheter was left for 24h to measure the temporal changes in SIP.

Prospective reporting of bleeding and hematoma at the insertion site was planned (as a potential confounding factor for SIP leading to falsely increased measurements).

Ischemic interstitial pressure

Experimentally, the best evaluation of interstitial pressure is given when transcapillary and lymphatic flows are stopped (e.g., after cardiac arrest in experimental studies). Thus, in a subset of patients, we evaluated the effect of interrupting blood flow on SIP by using a tourniquet for a short period. A large blood pressure cuff was placed above the knee and rapidly inflated to 180-190 mmHg of pressure for 5 min. The times of inflation and deflation were reported in the pressure recording.

Secondary outcomes

1) Central venous pressure (CVP) and perfusion pressure were measured simultaneously with SIP. When central venous access was not available, CVP was approximated using inferior vena cava ultrasound imaging.

2) Daily and cumulated fluid balance (day 1 to 3)

Assessed daily by the difference in 24-hour fluid intake and diuresis (or losses by renal replacement therapy).

3) Edema, and stage of severity (day 1 to 3)

A validated edema score was used (24), ranging from 0 (no edema) to 4 (severe edema) (Appendix 1). The maximum severity score observed during the study period was used.

4) Sequential Organ Failure Scores (SOFA) at admission and day 3

5) Lactate level at admission

6) ICU mortality