Biological Activity of Wastewater Assessed Using In Vitro Cell-Based Assays

## ADAMO RICCARDO PETOSA1, MONICA NOWIERSKI2 and VIVIANE YARGEAU*, 1

1 Department of Chemical Engineering, McGill University

2 Ontario Ministry of the Environment, Conservation and Parks

# SUPPLEMENTAL MATERIAL

Table S1 Characteristics of the wastewater treatment plants included in the present study

| Wastewater Treatment Plant | Treatment Type | Sample Type | Sample Collection Method | Sample Collection Dates | Plant Capacity (m3/day) |
| --- | --- | --- | --- | --- | --- |
| WWTP1 | Secondary treatment, conventional activated sludge, non-nitrifying, continuous chlorination | influent, effluent | 24 hour composite sampling | April - August 2014 | 409,000 |
| WWTP2 | Tertiary treatment, nitrifying, seasonal chlorination | influent, effluent | 24 hour composite sampling | April - August 2014 | 18,180 |
| WL | Lagoon (secondary treatment equivalent) with seasonal discharge | influent, effluent, surface water downstream of wastewater discharge | grab sampling | May, June, September 2014 | 1,364 |

Table S2 Target analytes quantified by each analytical method used

| METHOD 1 (LC-HRMS, ESI, + Mode) | METHOD 2 (LC-HRMS, ESI, - Mode) | METHOD 3 (LC-MS/MS, ESI, + Mode) |
| --- | --- | --- |
| Androstenedione | Gemfibrozil | 17β-Estradiol |  |
| Carbamazepine | Ibuprofen | Estrone |  |
| Sulfamethoxazole | Triclosan | 17α-Ethinylestradiol |  |
| Trimethoprim | 2,4-D |  |  |
| Atrazine | MCPA |  |  |
| DEET | Bentazon |  |  |
|  | Acesulfame K |  |  |

Table S3 Limits of detections (LODs) and limits of quantification (LOQs) for the target analytes

| COMPOUND | LOD (ng/L) | LOQ (ng/L) |
| --- | --- | --- |
| Acesulfame K | 7.6 | 22.5 |
| Androstenedione | 3.6 | 10.5 |
| Atrazine | 11.6 | 35.3 |
| Bentazon | 24.7 | 74.2 |
| Carbamazepine | 10.2 | 30.9 |
| 2,4-D | 5.8 | 17.5 |
| DEET | 16.4 | 49.5 |
| 17β-Estradiol | 2.5 | 9 |
| Estrone | 1.5 | 5 |
| 17α-Ethinylestradiol | 2 | 7 |
| Gemfibrozil | 33.8 | 101.5 |
| Ibuprofen | 14.9 | 44.4 |
| MCPA | 17.1 | 51.6 |
| Sulfamethoxazole | 9.1 | 27.6 |
| Triclosan | 32.4 | 96.7 |
| Trimethoprim | 11.3 | 34.2 |