Continuous measurement of pulse wave velocity under general anesthesia by carotid and femoral Doppler

Supplementary Material

Théophile Harlé1,2*, Jona Joachim1,2, Pierre Boutouyrie3, Joaquim Mateo1,2, Jade Perdereau1,2, Alexandre Mebazaa1,2, Jérome Cartailler1,2 and Fabrice Vallée1,2**

1Université Paris Cité, Inserm UMRS 942 Mascot, F-75006, Paris, France. 2Department of Anesthesia and Critical Care, Lariboisière Hospital, APHP, F-75010, Paris, France. 3Pharmacology, Inserm PARCC U970, Georges-Pompidou European Hospital, Paris-Cité University, Assistance publique-Hôpitaux de Paris, Paris, France.

* Corresponding author email: theophile.harle@aphp.fr

** This author was equally involved in the realization of this work and should be considered as senior author.

Short running title: Arterial stiffness under general anaesthesia and cardiovascular risk

Supplementary Material 1. Description of anesthesia protocol.

Protocol steps:

- Anaesthesia consultation: 2-30 days before

- Perioperative monitoring: electrocardiogram, heart rate (HR), pulsed oxygen saturation, non-invasive blood pressure measured by cuff oscillometry and depth of anaesthesia was monitored using the bispectral index (BIS)

- General anaesthesia induced and maintained by the intravenous route with a BIS objective between 40 and 60. Drugs used for anesthesia: - Propofol (75-150 μg / kg / min) - Remifentanil (0.2-0.5 μg / kg / min) - Atracurium Besilate (0.5mg/kg) was used for intubation

- Norepinephrine diluted at 5 µg/mL administered continuously via an electric syringe to restore optimal MAP ( > 15% of pre-induction MAP) after induction anesthesia

- Mechanical ventilation: volume 6ml / kg and a EtCO2 objective at［35-38］mmHg

Supplementary Material 2. Description of the study protocol. A: WAKIe R3 2TC® monitor connected directly to IntelliVue MX850® monitor for centralized monitoring of carotid and femoral Doppler signals. B: IntelliVue MX850 ® monitor screen recording all intraoperative signals (carotid and femoral Doppler signals are framed in red).

Supplementary Material 3. Measurement of TTcf and PWV using Graphysio software.   A: TTcf measurement using the "foot-by-foot" method (the maximum second derivative function of each signal is used to determine the wave foot). B: Graphysio software representation of carotid and femoral Doppler signals, MAP, TTcf and PWV (in cm.s-1) over time

Supplementary material 4. Flow chart of the study. 43 patients were included in the study. 2 patients were excluded due to insufficient Doppler signal quality. Of the 41 analyzed patients, 32 received both PWVAW and PWVGA evaluation and 38 patients received at least one continuous dynamic PWVGA evaluation with a total of 54 episodes of MAP changes (24 MAP increases and 30 MAP decreases).

Analysed patients

n = 41

Included patients

n=43

PWVAW and PWVGA assessment

32 patients

Excluded patients

n=2

(poor signal quality)

Dynamic PWVGA assessment

38 patients

54 dynamic PWVGA measurements

24 MAP increases

30 MAP decreseases

Supplementary Material 5. PWVAW (A) and PWVGA (B) according to cardiovascular risk status. ***p<0.001, ****p<0.0001 and NS=not significant.

Supplementary Material 6. Variations in PWVGA, MAP and HR during MAP increases (A) and decreases (B). 0: start of MAP variation. 1: end of MAP variation. ****p<0.0001 and NS=not significant.