Supplement:

Computed Tomography Imaging Protocols

The typical imaging parameters were: 120 kV, 200 effective mAs, axial scan mode, 22-cm display field of view (FOV), 50-cm large-body scan FOV, pitch of 1, gantry rotation time of 0.5 seconds, detector collimation of 128 x 0.6 mm, and 3-mm axial and coronal reconstructed slice thickness with a soft-tissue algorithm reconstruction. Real-time automatic tube current modulation software (CARE Dose4D; Siemens) was used to regulate the tube current according to the patient’s anatomical structures. Acquisition of CT images began 70 seconds after the injection of 140 mL of intravenous iopamidol (Isovue-370; Bracco) at 2.5 mL/s.

Magnetic Resonance Imaging Protocols

All axial T1- (repetition time/echo time [TR/TE] = 790/11 ms) and T2-weighted images (WIs) (TR/TE = 5470/85 ms) were acquired with a FOV = 190 x 190 mm2 , matrix size = 448 x 291, and 3-mm slice thickness without gap. Diffusionweighted imaging (DWI) was performed using single-shot echo-planar imaging before injecting the contrast agent. The DWI sequence parameters were: TR/TE, 5450/62 ms; b value, 0 and 1000 sec/mm2 ; section thickness/gap, 4/0 mm; FOV, 192 x 192 mm; and acquisition time, approximately 5 minutes. The apparent diffusion coefficient maps were acquired automatically within the manufacturer’s console. Dynamic contrast-enhanced (DCE) MRI was performed in an axial plane with three-dimensional controlled aliasing in parallel imaging results in a higher acceleration (CAIPIRINHA) technique before, during, and after the administration of a standard single-bolus administration of 0.1 mmoL of gadoterate meglumine (Dotarem; Guerbet) per kilogram of patient body weight dose at 4 mL/s. Dynamic acquisition was performed with a temporal resolution of 3.2 seconds. Contrast was administered after 11 baseline dynamics (144 dynamics in total). Axial (TR/TE, 650/12 ms) and coronal (TR/TE, 540/11 ms) contrast-enhanced (CE)-T1WIs with fat suppression were obtained after DCE-MRI. Axial (TR/TE = 650/12 ms) and coronal (TR/TE = 540/11 ms) images. CE-T1WIs with fat suppression were obtained after DCE-MRI. The detailed imaging parameters for DCE-MR imaging were: slice thickness = 3 mm without gap; 20 slices; z-axis coverage = 60 mm; spatial in-plane resolution = 184 x 160; TR/TE = 6.3/3.1 ms; flip angle = 15°; FOV = 19 cm; total acquisition time = 7 minutes 24 second