Supporting Information for

Effect of exhaust gas recirculation (EGR) on diesel engine carbonaceous PM emissions

Xinling Lia,b, Pengcheng Zhaob

a College of Smart Energy, Shanghai Jiao Tong University, China
b Institute of Eco-Chongming (IEC), Shanghai, 202162, China

Corresponding author. E-mail: lxl@sjtu.edu.cn (Xinling Li).

Number of figures: 1

Number of tables: 4

List of contents of the supporting information:

Figure

Fig. S1. Diagram of the system and instrumentation for the diesel engine combustion, particulate sampling and chemical analysis experiments

Tables

Table S1. Engine specifications

Table S2. Engine operation conditions

Table S3. Engine combustion, performance, and gaseous emission parameters

Table S4. Characteristic ions and retention time of target PAHs

PAHs & n-alkanes OC(OC1-OC4), EC(EC1-EC3) EGR

Table S1. Engine specifications

| Number of cylinders | 4 |
| --- | --- |
| Fuel injection system | Common-rail injection system |
| Injection strategy | One main injection |
| Suction type | Turbo intercooler |
| Displacement | 2.8 L |
| Bore×stroke | 94 mm×100 mm |
| Maximum power | 110 kW/3000 rpm |
| Maximum torque | 360 N m/2200 rpm |

Fig.S1 Diagram of the system and instrumentation for the diesel engine combustion, particulate sampling and chemical analysis experiments

Table S1. Engine specifications

| Number of cylinders | 4 |
| --- | --- |
| Fuel injection system | Common-rail injection system |
| Injection strategy | One main injection |
| Suction type | Turbo intercooler |
| Displacement | 2.8 L |
| Bore×stroke | 94 mm×100 mm |
| Maximum power | 110 kW/3000 rpm |
| Maximum torque | 360 N m/2200 rpm |

Table S2. Engine operation conditions

| Speed (rpm) | Load (%) | EGR rate (%) | Injection timing (°CA ATDC) | Injection pressure |
| --- | --- | --- | --- | --- |
| 1400 | 25 | 0, 10, 20, 30, 40, 45 | -6 | 120 |
|  | 50 | 0, 8, 16, 22, 30 |  |  |
| 2200 | 25 | 0, 10, 20, 30, 40, 45 |  |  |
|  | 50 | 0, 5, 10, 15, 20 |  |  |

Table S3. Engine combustion, performance, and gaseous emission parameters

| Speed (rpm) | Load (%) | EGR (%) | Pmax (MPa) | HRRmax (kJ/m3 deg) | CD (℃A) | Texh (K) | BSFC (g/kWh) | Air-fuel ratio | HC (g/kWh) | CO (g/kWh) | NOX (g/kWh) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1400 | 25 | 0 | 8.3 | 214 | 12 | 224 | 255 | 47.3 | 0.54 | 3.3 | 8.0 |
|  |  | 10 | 7.7 | 228 | 10 | 232 | 256 | 39.4 | 0.51 | 2.9 | 5.2 |
|  |  | 20 | 7.7 | 213 | 15 | 245 | 258 | 33.8 | 0.46 | 2.5 | 3.2 |
|  |  | 30 | 7.3 | 230 | 17 | 259 | 254 | 29.0 | 0.44 | 2.5 | 1.6 |
|  |  | 40 | 5.7 | 145 | 27 | 283 | 262 | 23.5 | 0.44 | 2.8 | 0.5 |
|  |  | 45 | 5.8 | 146 | 32 | 316 | 269 | 19.7 | 0.49 | 6.6 | 0.9 |
|  | 50 | 0 | 9.7 | 130 | 17 | 316 | 213 | 30.1 | 0.19 | 0.2 | 6.7 |
|  |  | 8 | 9.7 | 133 | 23 | 335 | 212 | 25.6 | 0.14 | 0.2 | 2.2 |
|  |  | 16 | 9.3 | 140 | 30 | 352 | 218 | 22.8 | 0.14 | 0.4 | 1.2 |
|  |  | 22 | 9.5 | 133 | 32 | 364 | 218 | 20.9 | 0.15 | 0.5 | 1.5 |
|  |  | 30 | 9.0 | 135 | 43 | 382 | 226 | 19.4 | 0.15 | 2.5 | 0.7 |
| 2200 | 25 | 0 | 8.0 | 177 | 13 | 223 | 259 | 50.4 | 0.53 | 3.1 | 4.4 |
|  |  | 10 | 7.7 | 149 | 20 | 235 | 259 | 43.2 | 0.45 | 2.1 | 2.7 |
|  |  | 20 | 7.8 | 127 | 23 | 250 | 258 | 37.5 | 0.39 | 1.7 | 1.7 |
|  |  | 30 | 7.3 | 123 | 24 | 265 | 262 | 31.8 | 0.36 | 1.6 | 1.1 |
|  |  | 40 | 6.8 | 131 | 27 | 272 | 272 | 25.4 | 0.33 | 2.1 | 0.5 |
|  |  | 45 | 6.7 | 130 | 29 | 317 | 276 | 21.2 | 0.31 | 3.5 | 0.3 |
|  | 50 | 0 | 9.1 | 118 | 18 | 310 | 214 | 35.7 | 0.23 | 0.4 | 4.1 |
|  |  | 5 | 9.0 | 116 | 24 | 322 | 216 | 32.5 | 0.21 | 0.4 | 2.9 |
|  |  | 10 | 8.9 | 108 | 30 | 317 | 215 | 30.4 | 0.22 | 0.4 | 2.7 |
|  |  | 15 | 9.0 | 111 | 33 | 336 | 218 | 28.5 | 0.20 | 0.4 | 2.0 |
|  |  | 20 | 9.2 | 109 | 45 | 346 | 218 | 27.5 | 0.20 | 0.4 | 1.6 |

EGR: Exhaust gas recirculation; Pmax: Maximum in-cylinder pressure; HRRmax: Maximum heat release rate; CD: Combustion duration; Texh: Exhaust temperature; BSFC: Brake specific fuel consumption

Table S4. Characteristic ions and retention time of target PAHs

| Abbr. | Compounds | Ring number | Characteristic ions (m/z) | Retention time (min) |
| --- | --- | --- | --- | --- |
| Nap | Naphthalene | 2 | 128 | 8.26 |
| Acy | Acenaphthylene | 3 | 152 | 10.22 |
| Ace | Acenaphthene | 3 | 154 | 10.44 |
| Fl | Fluorene | 3 | 166 | 11.12 |
| Phe | Phenanthrene | 3 | 178 | 12.90 |
| Ant | Anthracene | 3 | 178 | 13.02 |
| Flu | Fluoranthene | 4 | 202 | 15.95 |
| Pyr | Pyrene | 4 | 202 | 16.56 |
| BcP | Benzo[c]phenanthalene | 4 | 228 | 19.32 |
| BaA | Benzo[a]anthracene | 4 | 228 | 20.06 |
| Chr | Chrysene | 4 | 228 | 20.18 |
| BbF | Benzo[b]fluoranthene | 5 | 252 | 23.03 |
| BkF | Benzo[k]fluoranthene | 5 | 252 | 23.10 |
| BeP | Benzo[e]pyrene | 5 | 252 | 23.58 |
| Per | Perylene | 5 | 252 | 23.81 |
| BaP | Benzo[a]pyrene | 5 | 252 | 23.82 |
| IcdP | Indeno[1,2,3-cd] pyrene | 6 | 276 | 23.85 |
| DBA | Dibenzo[a, h]anthracene | 5 | 278 | 24.34 |
| BghiP | Benzo[g,h,i]perylene | 6 | 276 | 24.77 |
| Cor | Coronene | 7 | 300 | 28.10 |