Supplementary Information to “Accounting for Albedo in Carbon Market Protocols”

Table S1. Summary of albedo deductions and benefits by biomes. The minimum, maximum, and median project albedo deductions/benefits refer to the project within each biome that was found to have the minimum, maximum, and median project albedo deduction/benefit. We excluded four biomes with 1 or no projects located within them. Biomes are sourced from Dinerstein et al, 2017. Negative values for minimum, maximum, and median indicate albedo benefits, and positive values for minimum, maximum, and median indicate albedo deductions.

| Biome | Project Count | Project Hectares | Projected Credits (million metric tons of CO2) | Project Albedo Deduction (+)/ Benefit (-) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Minimum | Maximum | Median |
| Deserts & Xeric Shrublands | 10 | 451,993 | 15.5 | -13.8% | 10,000% a | 76.0% |
| Mangroves | 8 | 159,130 | 0.5 | -13.8% | 62.0% | 3.1% |
| Mediterranean Forests, Woodlands & Scrub | 5 | 19,971 | 4.0 | -100.9% | 8,285.8% | 48.0% |
| Montane Grasslands & Shrublands | 21 | 449,288 | 136.5 | -16.1% | 10,000% a | 38.9% |
| Temperate Broadleaf & Mixed Forests | 18 | 1,192,567 | 164.0 | 2.9% | 304.0% | 27.8% |
| Temperate Conifer Forests | 6 | 7,887 | 5.5 | 17.7% | 10,000% a | 165.0% |
| Tropical & Subtropical Coniferous Forests | 4 | 14,989 | 0.3 | 2.9% | 16.9% | 6.9% |
| Tropical & Subtropical Dry Broadleaf Forests | 22 | 11,629,253 | 15.9 | -11.6% | 10,000% a | 17.7% |
| Tropical & Subtropical Grasslands, Savannas & Shrublands | 62 | 6,353,644 | 107.2 | -11.4% | 10,000% a | 20.8% |
| Tropical & Subtropical Moist Broadleaf Forests | 101 | 29,949,181 | 346.2 | -24.0% | 10,000% a | 11.3% |

a The Hasler et al, 2024 dataset truncates high albedo offsets at 10,000% to avoid +/- infinity (2004).

Table S2. VCM standards and protocols assessed with summaries of the total projected credits (in million metrics tons), minimum project albedo median, median project albedo median, and maximum project albedo median across all projects using each respective protocol. The minimum, maximum, and median project albedo deductions/benefits refer to the project from each protocol that was found to have the minimum, maximum, and median project albedo deduction/benefit.

| VCM Registry | Protocols | Project Count | Projected Credits (million metric tons CO2) | Project Albedo Deduction/ Benefit |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Minimum | Maximum | Median |
| American Carbon Registry (ACR) (https://acrcarbon.org/registry/) | AR-ACM0001 | 1 | 0.4 | 23.5% | 23.5% | 23.5% |
| Climate Action Reserve (CAR) (https://www.climateactionreserve.org/) | ARB Compliance Offset Protocol - U.S. Forest Projects | 1 | 0.0 | 26.1% | 26.1% | 26.1% |
| Ecoregistry (ER) (https://www.ecoregistry.io/projects-list/cercarbono-co2) | AR-ACM0003 | 12 | 17.3 | 0.8% | 33.0% | 9.9% |
|  | CERCARBONO | 1 | 1 | 7.0% | 7.0% | 7.0% |
| Gold Standard (GS) (https://registry.goldstandard.org/projects) | AR-AM0003 | 1 | 0.8 | 11.3% | 11.3% | 11.3% |
|  | AR-AM0004 | 1 | 1.8 | 383.6% | 383.6% | 383.6% |
|  | Gold Standard: Afforestation/Reforestation | 8 | 4.6 | -23.9% | 8,285.8% | 17.9% |
| Verified Carbon Standard (VCS) (https://registry.verra.org/app/search/VCS) | AR-ACM0001 | 17 | 34.1 | -20.9% | 81.9% | 44.7% |
|  | AR-ACM0002 | 1 | 0.3 | 72.3% | 72.3% | 72.3% |
|  | AR-ACM0003 | 72 | 551.5 | -100.9% | 10,000% a | 18.4% |
|  | AR-ACM0014 | 1 | 1 | 62.0% | 62.0% | 62.0% |
|  | AR-AM0003 | 1 | 0.2 | -2.3% | -2.3% | -2.3% |
|  | AR-AM0005 | 2 | 4.8 | 3.7% | 7.0% | 5.3% |
|  | AR-AM0014 | 1 | 1.4 | 20.5% | 20.5% | 20.5% |
|  | AR-AMS0001 | 14 | 16.9 | -1.6% | 87.8% | 12.3% |
|  | AR-AMS0002 | 2 | 0.0 | 63.7% | 192.3% | 128.0% |
|  | AR-AMS0005 | 2 | 0.2 | 65.5% | 116.6% | 91.1% |
|  | AR-AMS0006 | 1 | 0.6 | 17.5% | 17.5% | 17.5% |
|  | AR-AMS0007 | 8 | 14.5 | -16.1% | 109.6% | 13.7% |
|  | VM0007 | 1 | 62.9 | 15.8% | 15.8% | 15.8% |
|  | VM0017 | 1 | 0.7 | 60.8% | 60.8% | 60.8% |
|  | VM0026 | 1 | 17.7 | 1,548.8% | 1,548.8% | 1,548.8% |
|  | VM0047 | 22 | 50.5 | -11.6 | 84.1% | 18.6% |

a The Hasler et al, 2024 dataset truncates high albedo offsets at 10,000% to avoid +/- infinity (2004).

Testing the Albedo Radiative Forcing Change Method

As an alternative to discounting a project’s credits by its albedo deduction (see section “Applying the Albedo Deduction” in Methods), we assessed the potential of subtracting the radiative forcing change (in terms of CO2e) due to albedo changes from the ex-ante projected credits (also in terms of CO2e). In addition to the percentage deductions/benefits, Hasler et al, 2024 provide albedo radiative forcing change layers (“AlbedoRadiativeForcing_median.tif”), which we downloaded and applied to the same projects as the albedo deduction method described and used above.

This approach produced disparate and unrealistic results – for example, one project with ex-ante projected credits of about 800,000 showed a revised credit production of negative 35 million metrics tons CO2e after subtracting the median albedo radiative forcing change from the projects’ reported ex-ante projected credits. We suspect these results are due to the differences in assumptions made between the Hasler et al, 2024 data products—which assume maximum reforestation—and the VCM projects’ design, which may have different species, planting densities, or maintenance regimes, among other attributes. This informed the choice of discounting the albedo deduction as a percentage in this analysis to meet our goal of providing a first order approximation. The analysis in our paper assumes that the CO2 removal to albedo change ratio, as provided in the albedo deduction percentage, stays reasonably constant when assessed over the course of a project’s lifetime.