# Appendix 1:

# Import rasterio, pandas and numpy libraries

import rasterio

import numpy as np

import pandas as pd

# Define the input raster files

slopePath = "slopeReclass.tif"

soilPath = "soilReclass.tif"

drainageDensityPath = "draindensReclass.tif"

lineamentDensityPath = "lineadensReclass.tif"

lulcPath = "lulcReclass.tif"

morphologyPath = "morphologyReclass.tif"

lithoPath = "lithoReclass.tif"

# Define the output raster file

output_file = "dharGWRZ.tif"

# Define the reclassification values for each input layer

# The values are in the range of 1 to 10, where higher values indicate higher suitability or preference

slope_reclass = {1: 10, 2: 8, 3: 5, 4: 3, 5: 1, 6:-1}

#Slopeclass 1: 0-2, 2:2-6, 3:6-12, 4:12-20, 5: 20-52

drainage_reclass = {1: 2, 2: 3, 3: 5, 4: 7, 5: 9, 6:-1}

soil_reclass = {1: 6, 2: 6, 3: 7, 4: 6, 5: 8, 6: 8, 7: 7, 8: 7, 9: 6, 10: -1}

#'Chromic Haplusterts':1, 'Entic Haplusterts':2, 'Fluventic Haplustepts':3, 'Lithic Haplustepts':4, 'Lithic Ustorthents':5, 'Typic Haplustalfs': 6, 'Typic Haplustepts': 7,  'Typic Ustorthents':8, 'Vertic Haplustepts':9

lineament_reclass = {1: 1, 2: 3, 3: 5, 4: 7, 5: 9, 6:-1}

lulc_reclass = {1: 8, 2: 6, 3: 6, 4: 5, 5: 1, 6:4, 7:10 , 8:-1}

#LuLc Class 1: TreeCover, 2: Shrubland, 3: Grassland, 4: Cropland, 5: buildup, 6: bare/sparse land, 7: waterbody

morpho_reclass = {1: 7, 2: 1, 3: 4, 4: 8, 5: 10, 6: 1, 7:-1}

#morpho Class 1: Slope pidment complex, 2: hills/Dykes, 3: Plateau, 4: plain, 5: water, 6: Anthro

litho_reclass = {1: 10, 2: 1, 3: 1, 4: 2, 5: 6, 6: 8, 7: 7, 8: 1, 9: 1, 10: 1, 11: 1, 12: 7, 13: 7, 14: 1, 15: 1, 16: 3, 17:8, 18: 2, 19: -1}

# Alluvium:1, Amphibolite: 2, Basalt:3, Cherty Dolomitic Marble: 4, Cherty Limestone: 5, Conglomerate/Limestone/Sandstone: 6, Coralline Limestone:7, Dolerite: 8

#Gabbro: 9, Granite: 10, Granite Gneiss: 11, Limestone: 12, Nodular Limestone: 13, Phyllite: 14, Pyroxenite: 15, Quartzite: 16, Sandstone: 17, Slate: 18

# Define the weights for each input layer

# The weights are in the range of 0 to 1, where higher values indicate higher importance or influence

slope_weight = 0.447

soil_weight = 0.013

drainageDen_weight = 0.059

lineamDen_weight = 0.229

lulc_weight = 0.054

morpho_weigth = 0.13

litho_weight = 0.069

# Read the input raster layers as numpy arrays using rasterio.open()

with rasterio.open(slopePath) as src:

slope = src.read(1)

#print("slope metadata",src.meta)

profile = src.profile # Save the profile for writing the output file

#drainage density

with rasterio.open(drainageDensityPath) as src:

drainageDens = src.read(1)

drainageDens = np.where((drainageDens >5), 6, drainageDens)

drainageDens = np.where((drainageDens <1), 6, drainageDens)

#print("drainage metadata",src.meta)

#lineament density

with rasterio.open(lineamentDensityPath) as src:

lineaDen = src.read(1)

lineaDen = np.where((lineaDen >5), 6, lineaDen)

drainageDens = np.where((drainageDens <1), 6, drainageDens)

#print("lineament metadata",src.meta)

#lulc

with rasterio.open(lulcPath) as src:

lulc = src.read(1)

lulc = np.where((lulc >7), 8, lulc)

lulc = np.where((lulc <1), 8, lulc)

#print("lulc metadata",src.meta)

#morpho

with rasterio.open(morphologyPath) as src:

morpho = src.read(1)

morpho = np.where((morpho >6), 7, morpho)

morpho = np.where((morpho <1), 7, morpho)

#print("morpho metadata",src.meta)

#soil

with rasterio.open(soilPath) as src:

soil = src.read(1)

soil = np.where((morpho >9), 10, soil)

soil = np.where((morpho <1), 10, soil)

#print("morpho metadata",src.meta)

#litho

with rasterio.open(lithoPath) as src:

litho = src.read(1)

litho = np.where((litho >18), 19, litho)

litho = np.where((litho <1), 19, litho)

#print("morpho metadata",src.meta)

# Reclassify the values in each input layer to a common scale using numpy.vectorize()

slope_reclass_func = np.vectorize(lambda x: slope_reclass.get(x, 0))

slope_reclass = slope_reclass_func(slope)

drainageDens_reclass_func = np.vectorize(lambda x: drainage_reclass.get(x, 0))

drainageDens_reclass = drainageDens_reclass_func(drainageDens)

lineaDen_reclass_func = np.vectorize(lambda x: lineament_reclass.get(x, 0))

lineaDen_reclass = lineaDen_reclass_func(lineaDen)

lulc_reclass_func = np.vectorize(lambda x: lulc_reclass.get(x, 0))

lulc_reclass = lulc_reclass_func(lulc)

morpho_reclass_func = np.vectorize(lambda x: morpho_reclass.get(x, 0))

morpho_reclass = morpho_reclass_func(morpho)

litho_reclass_func = np.vectorize(lambda x: litho_reclass.get(x, 0))

litho_reclass = litho_reclass_func(litho)

soil_reclass_func = np.vectorize(lambda x: soil_reclass.get(x, 0))

soil_reclass = soil_reclass_func(soil)

# Multiply each input layer by its weight and add them together to get the weighted overlay output layer

weighted_overlay = slope_weight * slope_reclass + drainageDen_weight * drainageDens_reclass + lineamDen_weight * lineaDen_reclass + lulc_weight * lulc_reclass + morpho_weigth * morpho_reclass + litho_weight * litho_reclass + soil_weight * soil_reclass

# Write the output layer to a new raster file using rasterio.write()

with rasterio.open(output_file, "w", **profile) as dst:

dst.write(weighted_overlay, 1)