Supplementary Material

Harnessing Magnesium Ion for Innovative Fenton-Like Processes: Greener Multi-Oxidants Strategy to Depollute Water

Aswin Kottapurath Vijay, a,b  Gifty Sara Rolly, c  Vered Marks, d Virender K. Sharma,e

Dan Meyerstein* a,c

a Chemistry Department, Ben-Gurion University, Beer-Sheva, Israel

Email: danm@ariel.ac.il

b Department of Sanitary Engineering, Faculty of Civil and Environmental Engineering, Gdańsk University of Technology, Gdańsk, Poland

c Department of Chemical Sciences and The Radical Research Center, Ariel University,

Ariel, Israel.

d Department of Chemical Sciences, Ariel University, Ariel, Israel

e Program for the Environment and Sustainability, Department of Environmental and Occupational Health, Texas A&M University, College Station, Texas 77843, United States

Text S1 : Experimental methods

S1.1.Materials

All chemicals were of analytical grades and were used without further purification. MgCl2.6H2O, acetamidophenol, NaHCO3, NaOH, and HClO4 were acquired from Sigma Aldrich ((Rehovot, Israel). H2O2 was obtained from J. T. Baker. (CH3)2SO was purchased from TCI. D2O was bought from Tzamal D-Chem Laboratories Ltd. Water purified with a Milli-Q system with a resistivity > 15 MΩ·cm-1 was used throughout the experiments.

S1.2. Reaction of MgIIaq + H2O2 with DMSO in the presence and absence of bicarbonate

Most of the experiments were conducted at a near-neutral pH. The pH was adjusted to 7.40 ± 0.05 using NaOH. The pH measurements were made using a Schott Instrument Lab 850 pH meter. Stock solutions of H2O2 were prepared in water. Stock solutions of MgCl2, 5.0 mM, in H2O were made. All the solutions were purged with argon in glass syringes during the preparation and before carrying out studies. Several difficulties arose in the study of the effect of [HCO3-] on the reaction. At pH 7.40, CO2 is also present in the solution. Removing the O2 by bubbling with an inert gas also drives CO2 out of the solution and decreases the HCO3- concentration considerably. To overcome this problem the argon gas was passed through a gas-washing bottle containing a solution of HCO3- at the same concentration. This method and its effectiveness were previously reported (Illés et al. 2019).

To analyze the products formed by the oxidation of substrates by a solution containing Mg2+aq and H2O2,  DMSO ((CH3)2SO) was added to the solutions, and the products were monitored by 1H NMR spectroscopy. DMSO was chosen as a substrate as its oxidation processes are well documented (Song et al. 2019; Lerner et al. 2021).  Specifically, 1H NMR measurements were performed on a 400 MHz Bruker Avance spectrometer. All samples were dissolved in solutions of H2O (90%)/D2O (10%) and the NMR experiments were performed at 300 K. The products formed by the oxidation of DMSO via the oxidation by a mixture of Mg2+aq and H2O2 were measured in the absence and presence of bicarbonate.

S1.3. Reaction of MgIIaq + H2O2 with acetamidophenol in the presence and absence of bicarbonate

Transformation of acetamidophenol in the Mg2+aq + H2O2/bicarbonate system was investigated in 20 mL EPA vials fitted with poly-tetrafluoroethylene lined caps as batch reactors at 250C. The final concentrations of acetamidophenol and NaHCO3 were 0.25 mM and 10 mM, respectively. Argon bubbled solution of Mg2+ and acetamidophenol was added to a mixture of bicarbonate and hydrogen peroxide solutions.  The solution pH was adjusted to 7.40 by H2SO4 (0.10 M) or NaOH (0.10 M). Six sets of experiments were conducted: Bicarbonate-acetamidophenol (HCO3- + ACP), magnesium-acetamidophenol (Mg2+ + ACP), hydrogen peroxide - acetamidophenol (H2O2 + ACP), magnesium-hydrogen peroxide-acetamidophenol (Mg2+ + H2O2 + ACP), magnesium-bicarbonate-acetamidophenol (Mg2+ + HCO3- + ACP), and magnesium-hydrogen peroxide-bicarbonate-acetamidophenol (Mg2+ + H2O2 + HCO3- + ACP). All experiments were carried out in duplicates and data were averaged.

S1.4. Quantification of acetamidophenol

Concentrations of acetamidophenol in the samples were analyzed by HPLC-UV which was performed using a Dionex Ultimate 3000 equipped with a Diode Array Detector (Thermo Scientific), Hypersil GOLD (dim. (mm)- 150 × 4.6, particle size- 3μm) column. The detection wavelength was set at 243 nm. The temperature of the column was set at 250C, and a flow rate of 0.50 ml/min. The substrates were eluted using 0.3% H3PO4/CH3OH = 70:30.

References

Illés E, Mizrahi A, Marks V, Meyerstein D (2019) Carbonate-radical-anions, and not hydroxyl radicals, are the products of the Fenton reaction in neutral solutions containing bicarbonate. Free Radic Biol Med 131:1–6. https://doi.org/doi.org/10.1016/j.freeradbiomed.2018.11.015

Lerner A, Kornweitz H, Zilbermann I, et al (2021) Radicals in ‘biologically relevant’ concentrations behave differently: Uncovering new radical reactions following the reaction of hydroxyl radicals with DMSO. Free Radic Biol Med 162:555–560. https://doi.org/10.1016/j.freeradbiomed.2020.11.012

Song S, Li X, Wei J, et al (2019) DMSO-catalysed late-stage chlorination of (hetero)arenes. Nat Catal 3:107–115. https://doi.org/10.1038/s41929-019-0398-0