Table S1 Liquid phase conditions

| Instrument model | Thermo Scientific Vanquish ultra-high performance liquid chromatograph |  |
| --- | --- | --- |
| Column | ACQUITY UPLC® HSS T3, 1.8 µm, 2.1×100 mm |  |
| mobile phase | mobile phase B: ACN Mobile phase A: H2O + 0.1% FA (volume fraction) Column temperature: 35℃ Flow rate: 0.35 mL/min Sample volume : 10 µL Gradient elution as specified in the table below: |  |
| Time (min) | Mobile phase A (％) | Mobile phase B (％) |  |
| 0.0 | 98 | 2 |  |
| 0.5 | 98 | 2 |  |
| 1.5 | 80 | 20 |  |
| 4.0 | 35 | 65 |  |
| 11.0 | 5 | 95 |  |
| 15.0 | 5 | 95 |  |
| 15.1 | 98 | 2 |  |
| 20.0 | 98 | 2 |  |

Table S2 Mass spectrometry conditions

| Instrument Model | Quadrupole/electrostatic field orbit trap high-resolution mass spectrometer Q-Exactive |
| --- | --- |
| Ionization mode | ES+/- |
| Capillary temperature (°C) | 320 |
| Spray voltage (V) | 3.7k (+) 3.0(-) |
| Auxiliary gas heater temperature (°C) | 350 |
| Capillary (kV) | 3.0 |
| S-lens RF level | 60 |
| Sheath gas flow rate (Arb) | 35 |
| Auxiliary gas flow rate (Arb) | 10 |

Table S3 Mass spectrometry acquisition methods

| Ionization mode | Full MS/dd-MS2 (Top N) |  |
| --- | --- | --- |
| General |  |  |
| Runtime | 0–20 min |  |
| Polarity | Positive/negative |  |
| Default charge state | 1 |  |
| Full MS |  |  |
| Resolution |  | 70,000 |
| AGC target |  | 1e6 |
| Maximum IT |  | 100 ms |
| Scan range |  | 70–1000 m/z |
| dd-MS2 |  |  |
| Resolution |  | 17,500 |
| AGC target |  | 1e5 |
| Maximum IT |  | 50 ms |
| Loop count |  | 10 |
| TopN |  | 10 |
| Isolation window |  | 1.5 m/z |
| （N）CE/stepped NCE |  | 20, 40, 60 |
| dd Settings |  |  |
| Minimum AGC target |  | 8.00 e3 |
| Apex trigger |  | 4–8s |
| Exclude isotopes |  | on |
| Dynamic exclusion |  | 10.0 s |