# Supporting information

ICP-MS/MS Feasibility study

For the ICP-MS/MS feasibility study, 25 grab surface water samples were collected in the Belgian Campine region (Mol and Dessel), where there is a legacy of contamination by non-ferrous industries. The sampling of surface water was conducted on 28/05/2018 in the main current at a depth of 10–15 cm.

Table 1: Identification and location of surface water samples

| Vito Code | Sampling point | Location | X coordinates | Y coordinates |
| --- | --- | --- | --- | --- |
| 180530-0036 | 1 | Eindergatloop, vlak voor Dommel | 223534,5 | 214347,3 |
| 180530-0037 | 2 | Eindergatloop, halverwege brug en Dommel | 223113,5 | 214400,1 |
| 180530-0038 | 3 | Eindergatloop, thv brug achter zandvang | 222680,1 | 214461,7 |
| 180530-0039 | 4 | Dommel na eindergatloop | 223556,7 | 214356,4 |
| 180530-0040 | 5 | Dommel voor Eindergatloop | 223551 | 214316,1 |
| 180530-0041 | 6 | Kanaal Herentals-Bocholt, thv Dommel | 223539,7 | 214398,6 |
| 180530-0042 | 7 | Dommel 500m na Eindergatloop | 223410,3 | 214805,4 |
| 180530-0043 | 8 | Dommel 500m voor Eindergatloop | 223524,6 | 213835,6 |
| 180530-0044 | 9 | Scheppelijke Nete | 204550,8 | 210114,7 |
| 180530-0045 | 10 | Scheppelijke Nete | 203284,5 | 209393,2 |
| 180530-0046 | 11 | Molse Nete | 202478,1 | 208624,2 |
| 180530-0048 | 13 | Molse Nete | 201608,8 | 207695,9 |
| 180530-0049 | 14 | Molse Nete | 198792,5 | 207110,4 |
| 180530-0050 | 15 | Molse Nete | 196172,6 | 206264,6 |
| 180530-0051 | 16 | Kanaal naar Beverlo | 209886,1 | 214363,8 |
| 180530-0052 | 17 | Kanaal Herentals-Bocholt, voor kanaal naar Beverlo | 209869,6 | 214711,2 |
| 180530-0053 | 18 | Postelvaartje | 209578,2 | 214824,1 |
| 180530-0055 | 20 | Eindergatloop | 220730,3 | 214593,6 |
| 180530-0056 | 21 | Eindergatloop | 217075 | 212350,5 |
| 180530-0057 | 22 | Kanaal Herentals-Bocholt, na kanaal naar Beverlo | 209390,2 | 214538,2 |
| 180530-0058 | 23 | Vijver thv Kristalijn | 208977,8 | 213663,6 |
| 180530-0059 | 24 | Congovaart thv TAP | 199424,7 | 212208,6 |
| 180530-0060 | 25 | Grote laguna thv HAL 196 | 201083,2 | 212709,4 |
| 180530-0061 | 26 | Scheppelijke Nete | 209133,9 | 211515,3 |
| 180530-0063 | 28 | Onbenoemde waterloop | 209897,5 | 211128,6 |

Figure 1: Location of surface water samples.

Non-targeted monitoring of element (bio-)availability in surface water

For the non-targeted monitoring of elements in surface water, 50 surface waters were sampled during 2023 across the Flanders Region as part of the existing monitoring network operated by the Flemish Environmental Agency (VMM).

Table 2: Identification and location of surface water samples sampling for non-targeted monitoring.

| Sampling place code | Watercourse | Sample number code | Date of sampling |
| --- | --- | --- | --- |
| 25000 | Zwinnevaart, Oostkerke | L-2023-09901 | 30/05/2023 |
| 6010 | Isabellavaart, Heist | L-2023-09908 | 30/05/2023 |
| 816000 | Boudewijnkanaal, Dudzele | L-2023-09912 | 30/05/2023 |
| 881000 | Zuidervaartje, Damme | L-2023-09916 | 30/05/2023 |
| 632520 | Gaverbeek, Harelbeke | L-2023-10061 | 01/06/2023 |
| 633000 | Gaverbeek, Harelbeke | L-2023-10063 | 01/06/2023 |
| 888500 | Kerkebeek, Sint-Michiels, Brugge | L-2023-10076 | 01/06/2023 |
| 900000 | Rivierbeek, Oostkamp | L-2023-10082 | 01/06/2023 |
| 613000 | Devebeek, Tielt | L-2023-10538 | 08/06/2023 |
| 72000 | Mark, Kersel, NL | L-2023-10754 | 12/06/2023 |
| 80000 | Merkske, Minderhout | L-2023-10764 | 12/06/2023 |
| 970000 | Kemmelbeek, Diksmuide | L-2023-10839 | 13/06/2023 |
| 274000 | Kleine Nete, Herentals | L-2023-10849 | 13/06/2023 |
| 257200 | Grote Nete, Zammel | L-2023-10950 | 14/06/2023 |
| 258500 | Grote Nete, Winkelomheide | L-2023-10951 | 14/06/2023 |
| 275500 | Kleine Nete, Kasterlee | L-2023-10957 | 14/06/2023 |
| 325000 | Grote Laak, Veerle | L-2023-10961 | 14/06/2023 |
| 329000 | Molse Nete, Winkelom | L-2023-10969 | 14/06/2023 |
| 333000 | Molse Nete, Balen | L-2023-10970 | 14/06/2023 |
| 289000 | Aa, Sassenhout | L-2023-11056 | 15/06/2023 |
| 289400 | Aa, Tielen | L-2023-11058 | 15/06/2023 |
| 279000 | Molenbeek, Viersel | L-2023-11310 | 20/06/2023 |
| 333750 | Scheppelijke Nete, Mol | L-2023-11406 | 21/06/2023 |
| 122040 | Grensmaas, Stevensweert, NL | L-2023-16822 | 25/09/2023 |
| 91000 | Dommel, Grote Heide | L-2023-16833 | 25/09/2023 |
| 93000 | Dommel, Kleine-Brogel- | L-2023-16834 | 25/09/2023 |
| 95000 | Eindergatloop, Neerpelt | L-2023-16835 | 25/09/2023 |
| 99000 | Bollisenbeek, Wijchmaal | L-2023-16837 | 25/09/2023 |
| 99550 | Bollisenbeek, Wijchmaal | L-2023-16839 | 25/09/2023 |
| 630000 | Gaverbeek, Sint-Eloois-Vijve | L-2023-16909 | 28/09/2023 |
| 948010 | Bollaerdbeek, Ieper | L-2023-17124 | 02/10/2023 |
| 672013 | Despierrebeek, Ieper | L-2023-17133 | 02/10/2023 |
| 439500 | Grote Gete, Utsenaken | L-2023-17201 | 03/10/2023 |
| 439900 | Grote Gete, Tienen | L-2023-17202 | 03/10/2023 |
| 442480 | Borggracht, Tienen | L-2023-17203 | 03/10/2023 |
| 442490 | Grote Gete, Tienen | L-2023-17204 | 03/10/2023 |
| 439700 | Grote Gete, Tienen | L-2023-17245 | 03/10/2023 |
| 72000 | Mark, Kersel, NL | L-2023-17366 | 04/10/2023 |
| 80000 | Merkske, Minderhout | L-2023-17380 | 04/10/2023 |
| 492000 | Durme, Tielrode | L-2023-17589 | 09/10/2023 |
| 690015 | BergenVaart, Dunkerque, FR | L-2023-17675 | 10/10/2023 |
| 172100 | Scheldekanaal, Zwijnaarde | L-2023-17717 | 10/10/2023 |
| 934000 | Stenensluisvaart, Woumen | L-2023-17778 | 11/10/2023 |
| 257200 | Grote Nete, Zammel | L-2023-17809 | 11/10/2023 |
| 258500 | Grote Nete, Winkelomheide | L-2023-17810 | 11/10/2023 |
| 325000 | Grote Laak, Veerle | L-2023-17817 | 11/10/2023 |
| 329000 | Molse Nete, Winkelom | L-2023-17818 | 11/10/2023 |
| 333000 | Molse Nete, Balen | L-2023-17819 | 11/10/2023 |
| 991000 | Heidebeek, Poperinge | L-2023-17886 | 12/10/2023 |
| 955000 | Martjevaart, Merkem | L-2023-17900 | 12/10/2023 |
| 990000 | Heidebeek, Haringe | L-2023-17916 | 12/10/2023 |
| 289000 | Aa, Sassenhout | L-2023-18097 | 16/10/2023 |
| 289400 | Aa, Tielen | L-2023-18098 | 16/10/2023 |
| 197 | Voorhaven, Oostende | L-2023-18132 | 17/10/2023 |
| 274000 | Kleine Nete, Herentals | L-2023-18180 | 17/10/2023 |
| 690900 | Kreek Van Nieuwendamme, Sint-Joris | L-2023-18195 | 17/10/2023 |
| 860000 | Moerdijkvaart, Gistel | L-2023-18202 | 17/10/2023 |
| 289000 | Aa, Sassenhout | L-2023-18334 | 18/10/2023 |

Data analysis – geo chemical modelling

In this study, the PHREEQC model (PHase Requilibrium Equations for Evaluating the Chemical Quality of Water, version 3; Parkhurst and Appelo, 1999) was used to model the speciation of inorganic elements in the surface water. In this study, the wateq4f.dat database was used, commonly applied to natural water systems. This database is consistent with the standard PHREEQC database (PHREEQC.dat), but it includes a broader set of elements. For elements not defined in wateq4f.dat, an alternative database, llnl.dat, was applied. In Table 2, an overview is given of the elements and database used for the PHREEQC calculations for the NTA study. In the last columns an overview is given of the elements for which database is available but that were not considered in the PHREEQC calculations.

Table 2:  Overview of database used per element

| element | Database used for NTA geo chemical modelling | element | Not considered in NTA geo chemical modelling |
| --- | --- | --- | --- |
| Ag | wateq4f | Bi | No database available |
| Al | wateq4f | Er | llnl |
| As | wateq4f | Eu | llnl |
| Au | llnl | Ga | llnl |
| B | wateq4f | Gd | llnl |
| Ba | wateq4f | Ge | llnl |
| Be | llnl | Hf | llnl |
| Ca | wateq4f | Ho | llnl |
| Cd | wateq4f | In | llnl |
| Ce | llnl | La | llnl |
| Co | llnl | Li | wateq4f |
| Cr | llnl | Lu | llnl |
| Cs | wateq4f | Nb | No database available |
| Cu | wateq4f | Nd | llnl |
| Dy | llnl | Pd | llnl |
| Fe | wateq4f | Pr | llnl |
| Hg | llnl | Pt | No database available |
| K | wateq4f | Re | llnl |
| Mg | wateq4f | Ru | llnl |
| Mn | wateq4f | Sb | llnl |
| Mo | llnl | Sc | llnl |
| Na | wateq4f | Sm | llnl |
| Ni | wateq4f | Ta | No database available |
| P | wateq4f | Tb | llnl |
| Pb | wateq4f | Te | No database available |
| Rb | wateq4f | Tl | llnl |
| S | wateq4f | Tm | llnl |
| Se | wateq4f | W | llnl |
| Si | wateq4f | Y | llnl |
| Sn | llnl | Yb | llnl |
| Sr | wateq4f | Zr | llnl |
| Th | llnl |  |  |
| Ti | llnl |  |  |
| U | wateq4f |  |  |
| V | llnl |  |  |
| Zn | wateq4f |  |  |

Bio availability of elements

A comparison of the measured dissolved concentration, the PHREEQC calculated fraction of the free metal ion, the calculated risk characterization ratio (RCR) by Bio-MET and PNEC Pro are summarised in the tables hereunder.

|  | Measured | PHREEQC | Bio-MET RESULTS (Zinc) with EQSbioav = 20 µg/L |  |  |  | PNEC-Pro |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Number | Zinc conc (dissolved) [µg/L] | Zn2+ (µg/L) | Local HC5 (dissolved) [µg/L] | BioF | Bioavailable Zinc Conc (µg/L) | RCR | PNEC | RCR |
| 202309908 | 3,75 | 0,21 | 108,90 | 0,10 | 0,38 | 0,02 | 53,50 | 0,07 |
| 202310063 | 7,50 | 1,98 | 46,55 | 0,23 | 1,76 | 0,09 | 21,30 | 0,35 |
| 202310538 | 20,00 | 2,91 | 71,10 | 0,15 | 3,07 | 0,15 | 29,48 | 0,68 |
| 202310754 | 3,75 | 1,46 | 69,98 | 0,16 | 0,58 | 0,03 | 25,42 | 0,15 |
| 202310764 | 3,75 | 1,94 | 61,41 | 0,18 | 0,67 | 0,03 | 22,40 | 0,17 |
| 202310839 | 3,75 | 0,20 | 61,50 | 0,18 | 0,66 | 0,03 | 26,47 | 0,14 |
| 202310849 | 3,75 | 2,25 | 38,53 | 0,28 | 1,06 | 0,05 | 18,19 | 0,21 |
| 202310950 | 7,50 | 2,02 | 44,94 | 0,24 | 1,82 | 0,09 | 25,12 | 0,30 |
| 202310951 | 36,00 | 22,74 | 39,32 | 0,28 | 9,98 | 0,50 | 15,07 | 2,39 |
| 202310957 | 3,75 | 1,82 | 45,53 | 0,24 | 0,90 | 0,04 | 25,45 | 0,15 |
| 202310961 | 7,50 | 5,84 | 28,48 | 0,38 | 2,87 | 0,14 | 16,64 | 0,45 |
| 202310969 | 87,00 | 51,66 | 38,36 | 0,28 | 24,72 | 1,24 | 14,79 | 5,88 |
| 202311056 | 3,75 | 1,73 | 50,92 | 0,21 | 0,80 | 0,04 | 20,26 | 0,19 |
| 202311058 | 3,75 | 1,51 | 52,72 | 0,21 | 0,78 | 0,04 | 21,51 | 0,17 |
| 202311310 | 3,75 | 1,68 | 45,86 | 0,24 | 0,89 | 0,04 | 18,91 | 0,20 |
| 202316822 | 3,75 | 0,57 | 27,49 | 0,40 | 1,49 | 0,07 | 17,90 | 0,21 |
| 202316833 | 62,00 | 43,45 | 29,09 | 0,37 | 23,23 | 1,16 | 12,91 | 4,80 |
| 202316834 | 32,00 | 25,65 | 31,35 | 0,35 | 11,13 | 0,56 | 11,82 | 2,71 |
| 202316835 | 230,00 | 121,97 | 27,81 | 0,39 | 90,14 | 4,51 | 31,50 | 7,30 |
| 202316837 | 33,00 | 27,91 | 27,52 | 0,40 | 13,07 | 0,65 | 11,22 | 2,94 |
| 202316839 | 163,00 | 127,56 | 32,83 | 0,33 | 54,12 | 2,71 | 12,08 | 13,49 |
| 202316909 | 7,50 | 2,26 | 59,52 | 0,18 | 1,37 | 0,07 | 26,76 | 0,28 |
| 202317124 | 3,75 | 0,32 | 77,71 | 0,14 | 0,53 | 0,03 | 29,41 | 0,13 |
| 202317366 | 7,50 | 3,84 | 63,55 | 0,17 | 1,29 | 0,06 | 24,36 | 0,31 |
| 202317380 | 3,75 | 2,13 | 79,36 | 0,14 | 0,52 | 0,03 | 27,43 | 0,14 |
| 202317589 | 3,75 | 0,83 | 44,77 | 0,24 | 0,91 | 0,05 | 32,01 | 0,12 |
| 202317675 | 3,75 | 1,01 | 76,92 | 0,14 | 0,53 | 0,03 | 70,05 | 0,05 |
| 202317717 | 3,75 | 0,60 | 37,11 | 0,29 | 1,10 | 0,06 | 19,62 | 0,19 |
| 202317778 | 3,75 | 0,93 | 68,70 | 0,16 | 0,60 | 0,03 | 32,02 | 0,12 |
| 202317809 | 82,00 | 39,83 | 42,38 | 0,26 | 21,09 | 1,05 | 19,84 | 4,13 |
| 202317810 | 188,00 | 124,89 | 39,60 | 0,28 | 51,75 | 2,59 | 16,17 | 11,63 |
| 202317817 | 18,80 | 14,98 | 20,46 | 0,53 | 10,01 | 0,50 | 11,14 | 1,69 |
| 202317818 | 370,00 | 219,25 | 42,76 | 0,25 | 94,31 | 4,72 | 17,17 | 21,55 |
| 202317886 | 7,50 | 0,71 | 57,22 | 0,19 | 1,43 | 0,07 | 37,81 | 0,20 |
| 202317900 | 7,50 | 0,54 | 66,07 | 0,16 | 1,24 | 0,06 | 46,30 | 0,16 |
| 202317916 | 3,75 | 0,06 | 66,96 | 0,16 | 0,61 | 0,03 | 43,02 | 0,09 |
| 202318097 | 7,50 | 5,25 | 42,05 | 0,26 | 1,94 | 0,10 | 17,24 | 0,43 |
| 202318098 | 16,10 | 11,53 | 41,51 | 0,26 | 4,23 | 0,21 | 16,79 | 0,96 |
| 202318132 | 3,75 | 0,78 | 26,22 | 0,42 | 1,56 | 0,08 | 598,58 | 0,01 |
| 202318180 | 3,75 | 2,56 | 44,04 | 0,25 | 0,93 | 0,05 | 18,32 | 0,20 |
| 202318195 | 3,75 | 0,42 | 66,96 | 0,16 | 0,61 | 0,03 | 47,58 | 0,08 |
| 202318202 | 3,75 | 1,46 | 57,22 | 0,19 | 0,71 | 0,04 | 26,06 | 0,14 |
| 202318334 | 3,75 | 1,96 | 44,43 | 0,25 | 0,92 | 0,05 | 17,54 | 0,21 |

Table 3: comparison of the measured dissolved concentration, the PHREEQC calculated fraction of the free metal ion, the calculated risk characterization ratio (RCR) by Bio-MET and PNEC for nickel.

|  | Measured | PHREEQC | Bio-MET RESULTS (Nickel) with EQSbioav = 4 µg/L |  |  |  | PNEC-Pro |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Number | Nickel (dissolved) [µg/L] | Ni2+ (µg/L) | Local HC5 (dissolved) [µg/L] | BioF | Bioavailable Nickel Conc (µg/L) | RCR | PNEC | RCR |
| 202309908 | 1,60 | 0,00 | 20,94 | 0,19 | 0,31 | 0,08 | 23,88 | 0,07 |
| 202310063 | 6,40 | 0,10 | 15,70 | 0,25 | 1,63 | 0,41 | 15,19 | 0,42 |
| 202310538 | 6,30 | 0,05 | 17,85 | 0,22 | 1,41 | 0,35 | 16,66 | 0,38 |
| 202310754 | 6,10 | 0,17 | 20,15 | 0,20 | 1,21 | 0,30 | 21,33 | 0,29 |
| 202310764 | 6,10 | 0,30 | 22,19 | 0,18 | 1,10 | 0,27 | 23,39 | 0,26 |
| 202310839 | 1,60 | 0,00 | 12,56 | 0,32 | 0,51 | 0,13 | 7,84 | 0,20 |
| 202310849 | 4,30 | 0,49 | 18,03 | 0,22 | 0,95 | 0,24 | 21,59 | 0,20 |
| 202310950 | 3,20 | 0,05 | 13,83 | 0,29 | 0,93 | 0,23 | 11,76 | 0,27 |
| 202310951 | 3,30 | 0,34 | 17,68 | 0,23 | 0,75 | 0,19 | 18,38 | 0,18 |
| 202310957 | 4,40 | 0,27 | 17,54 | 0,23 | 1,00 | 0,25 | 19,19 | 0,23 |
| 202310961 | 1,60 | 0,83 | 20,80 | 0,19 | 0,31 | 0,08 | 22,89 | 0,07 |
| 202310969 | 3,60 | 0,37 | 19,99 | 0,20 | 0,72 | 0,18 | 19,90 | 0,18 |
| 202311056 | 3,20 | 0,13 | 18,50 | 0,22 | 0,69 | 0,17 | 18,76 | 0,17 |
| 202311058 | 1,60 | 0,05 | 17,40 | 0,23 | 0,37 | 0,09 | 17,13 | 0,09 |
| 202311310 | 1,60 | 0,06 | 17,60 | 0,23 | 0,36 | 0,09 | 17,74 | 0,09 |
| 202316822 | 0,80 | 0,01 | 7,29 | 0,55 | 0,44 | 0,11 | 2,13 | 0,38 |
| 202316833 | 8,00 | 1,78 | 16,60 | 0,24 | 1,93 | 0,48 | 21,37 | 0,37 |
| 202316834 | 11,70 | 3,80 | 18,60 | 0,22 | 2,52 | 0,63 | 23,65 | 0,49 |
| 202316835 | 5,40 | 0,83 | 16,10 | 0,25 | 1,34 | 0,34 | 20,28 | 0,27 |
| 202316837 | 10,80 | 6,57 | 19,58 | 0,20 | 2,21 | 0,55 | 32,67 | 0,33 |
| 202316839 | 52,00 | 25,75 | 23,50 | 0,17 | 8,85 | 2,21 | 32,64 | 1,59 |
| 202316909 | 3,30 | 0,06 | 18,80 | 0,21 | 0,70 | 0,18 | 18,97 | 0,17 |
| 202317124 | 3,30 | 0,01 | 15,34 | 0,26 | 0,86 | 0,22 | 11,32 | 0,29 |
| 202317366 | 5,40 | 0,27 | 21,80 | 0,18 | 0,99 | 0,25 | 23,47 | 0,23 |
| 202317380 | 8,40 | 0,57 | 27,42 | 0,15 | 1,23 | 0,31 | 29,58 | 0,28 |
| 202317589 | 1,60 | 0,02 | 11,65 | 0,34 | 0,55 | 0,14 | 12,25 | 0,13 |
| 202317675 | 3,50 | 0,06 | 25,10 | 0,16 | 0,56 | 0,14 | 31,73 | 0,11 |
| 202317717 | 1,60 | 0,01 | 10,81 | 0,37 | 0,59 | 0,15 | 8,44 | 0,19 |
| 202317778 | 5,30 | 0,07 | 19,13 | 0,21 | 1,11 | 0,28 | 20,01 | 0,26 |
| 202317809 | 1,60 | 0,07 | 14,93 | 0,27 | 0,43 | 0,11 | 15,32 | 0,10 |
| 202317810 | 1,60 | 0,20 | 17,58 | 0,23 | 0,36 | 0,09 | 19,90 | 0,08 |
| 202317817 | 3,40 | 1,96 | 15,60 | 0,26 | 0,87 | 0,22 | 22,93 | 0,15 |
| 202317818 | 3,40 | 0,30 | 18,06 | 0,22 | 0,75 | 0,19 | 17,93 | 0,19 |
| 202317886 | 1,60 | 0,01 | 17,04 | 0,23 | 0,38 | 0,09 | 16,41 | 0,10 |
| 202317900 | 6,90 | 0,03 | 15,58 | 0,26 | 1,77 | 0,44 | 15,18 | 0,45 |
| 202317916 | 1,60 | 0,00 | 13,50 | 0,30 | 0,47 | 0,12 | 7,88 | 0,20 |
| 202318097 | 1,60 | 0,22 | 20,10 | 0,20 | 0,32 | 0,08 | 23,13 | 0,07 |
| 202318098 | 1,60 | 0,27 | 21,00 | 0,19 | 0,30 | 0,08 | 25,36 | 0,06 |
| 202318132 | 0,80 | 0,02 | 7,59 | 0,53 | 0,42 | 0,11 | 104,89 | 0,01 |
| 202318180 | 3,30 | 0,46 | 19,05 | 0,21 | 0,69 | 0,17 | 21,23 | 0,16 |
| 202318195 | 1,60 | 0,01 | 13,50 | 0,30 | 0,47 | 0,12 | 12,86 | 0,12 |
| 202318202 | 3,30 | 0,09 | 18,30 | 0,22 | 0,72 | 0,18 | 17,34 | 0,19 |
| 202318334 | 0,80 | 0,04 | 18,54 | 0,22 | 0,17 | 0,04 | 17,67 | 0,05 |

Table 4: comparison of the measured dissolved concentration, the PHREEQC calculated fraction of the free metal ion, the calculated risk characterization ratio (RCR) by Bio-MET and PNEC for cupper.

|  | Measured | PHREEQC | Bio-MET RESULTS (Copper) with EQSbioav = 7 µg/L |  |  |  | PNEC-Pro |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Number | Copper (dissolved) [µg/L] | Cu2+ (µg/L) | Local HC5 (dissolved) [µg/L] | BioF | Bioavailable Copper (µg/L) | RCR | PNEC | RCR |
| 202309908 | 1,00 | 0,00 | 50,31 | 0,02 | 0,02 | 0,00 | 26,53 | 0,04 |
| 202310063 | 0,50 | 0,00 | 34,81 | 0,03 | 0,01 | 0,00 | 5,63 | 0,09 |
| 202310538 | 3,40 | 0,00 | 41,68 | 0,02 | 0,08 | 0,01 | 14,04 | 0,24 |
| 202310754 | 2,30 | 0,00 | 51,81 | 0,02 | 0,04 | 0,01 | 33,14 | 0,07 |
| 202310764 | 1,00 | 0,00 | 58,73 | 0,02 | 0,02 | 0,00 | 35,12 | 0,03 |
| 202310839 | 2,60 | 0,00 | 17,15 | 0,06 | 0,15 | 0,02 | 4,77 | 0,55 |
| 202310849 | 1,00 | 0,00 | 35,57 | 0,03 | 0,03 | 0,00 | 29,87 | 0,03 |
| 202310950 | 0,50 | 0,00 | 28,55 | 0,04 | 0,02 | 0,00 | 16,43 | 0,03 |
| 202310951 | 0,50 | 0,00 | 26,29 | 0,04 | 0,02 | 0,00 | 20,94 | 0,02 |
| 202310957 | 0,50 | 0,00 | 39,90 | 0,03 | 0,01 | 0,00 | 29,24 | 0,02 |
| 202310961 | 0,50 | 0,00 | 7,00 | 0,14 | 0,07 | 0,01 | 14,32 | 0,03 |
| 202310969 | 0,50 | 0,00 | 28,64 | 0,03 | 0,02 | 0,00 | 16,48 | 0,03 |
| 202311056 | 1,00 | 0,00 | 39,87 | 0,03 | 0,03 | 0,00 | 23,92 | 0,04 |
| 202311058 | 0,50 | 0,00 | 35,65 | 0,03 | 0,01 | 0,00 | 23,55 | 0,02 |
| 202311310 | 0,50 | 0,00 | 36,81 | 0,03 | 0,01 | 0,00 | 18,99 | 0,03 |
| 202316822 | 1,00 | 0,00 | 7,35 | 0,14 | 0,14 | 0,02 | 6,10 | 0,16 |
| 202316833 | 0,50 | 0,00 | 21,67 | 0,05 | 0,02 | 0,00 | 26,09 | 0,02 |
| 202316834 | 0,50 | 0,00 | 24,51 | 0,04 | 0,02 | 0,00 | 31,71 | 0,02 |
| 202316835 | 0,50 | 0,00 | 21,22 | 0,05 | 0,02 | 0,00 | 20,36 | 0,02 |
| 202316837 | 1,00 | 0,00 | 21,04 | 0,05 | 0,05 | 0,01 | 40,29 | 0,02 |
| 202316839 | 1,00 | 0,00 | 21,04 | 0,05 | 0,05 | 0,01 | 38,42 | 0,03 |
| 202316909 | 1,00 | 0,00 | 42,06 | 0,02 | 0,02 | 0,00 | 21,33 | 0,05 |
| 202317124 | 3,20 | 0,00 | 25,61 | 0,04 | 0,12 | 0,02 | 14,43 | 0,22 |
| 202317366 | 1,00 | 0,00 | 64,86 | 0,02 | 0,02 | 0,00 | 34,78 | 0,03 |
| 202317380 | 1,00 | 0,00 | 76,25 | 0,01 | 0,01 | 0,00 | 45,21 | 0,02 |
| 202317589 | 3,50 | 0,00 | 23,27 | 0,04 | 0,15 | 0,02 | 10,59 | 0,33 |
| 202317675 | 0,50 | 0,00 | 69,02 | 0,01 | 0,01 | 0,00 | 23,92 | 0,02 |
| 202317717 | 1,00 | 0,00 | 19,51 | 0,05 | 0,05 | 0,01 | 2,20 | 0,45 |
| 202317778 | 0,50 | 0,00 | 46,30 | 0,02 | 0,01 | 0,00 | 28,93 | 0,02 |
| 202317809 | 1,00 | 0,00 | 34,76 | 0,03 | 0,03 | 0,00 | 24,34 | 0,04 |
| 202317810 | 1,00 | 0,00 | 35,34 | 0,03 | 0,03 | 0,00 | 26,59 | 0,04 |
| 202317817 | 0,50 | 0,00 | 7,00 | 0,14 | 0,07 | 0,01 | 15,68 | 0,03 |
| 202317818 | 0,50 | 0,00 | 33,29 | 0,03 | 0,02 | 0,00 | 20,62 | 0,02 |
| 202317886 | 1,00 | 0,00 | 46,44 | 0,02 | 0,02 | 0,00 | 10,92 | 0,09 |
| 202317900 | 1,00 | 0,00 | 34,33 | 0,03 | 0,03 | 0,00 | 23,05 | 0,04 |
| 202317916 | 1,00 | 0,00 | 17,60 | 0,06 | 0,06 | 0,01 | 14,43 | 0,07 |
| 202318097 | 2,50 | 0,00 | 39,32 | 0,03 | 0,06 | 0,01 | 33,60 | 0,07 |
| 202318098 | 1,00 | 0,00 | 36,18 | 0,03 | 0,03 | 0,00 | 35,46 | 0,03 |
| 202318132 | 1,00 | 0,00 | 11,75 | 0,09 | 0,09 | 0,01 |  |  |
| 202318180 | 1,00 | 0,00 | 41,44 | 0,02 | 0,02 | 0,00 | 30,68 | 0,03 |
| 202318195 | 0,50 | 0,00 | 21,29 | 0,05 | 0,02 | 0,00 | 15,89 | 0,03 |
| 202318202 | 0,50 | 0,00 | 35,65 | 0,03 | 0,01 | 0,00 | 25,74 | 0,02 |
| 202318334 | 0,50 | 0,00 | 34,49 | 0,03 | 0,01 | 0,00 | 19,04 | 0,03 |

Table 5: comparison of the measured dissolved concentration, the PHREEQC calculated fraction of the free metal ion, the calculated risk characterization ratio (RCR) by Bio-MET and PNEC for lead.

|  | Measured | PHREEQC | Bio-MET RESULTS (Lead) with EQSbioav = 1,2 µg/L |  |  |  | PNEC-Pro |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Number | Lead (dissolved) [µg/L] | Pb2+ (µg/L) | Local HC5 (dissolved) [µg/L] | BioF | Bioavailable Lead (µg/L) | RCR | PNEC | RCR |
| 202309908 | 0,25 | 0,00 | 38,78 | 0,03 | 0,01 | 0,01 | 36,10 | 0,01 |
| 202310063 | 0,25 | 0,00 | 14,99 | 0,08 | 0,02 | 0,02 |  |  |
| 202310538 | 0,25 | 0,00 | 24,98 | 0,05 | 0,01 | 0,01 | 19,81 | 0,01 |
| 202310754 | 0,25 | 0,00 | 26,64 | 0,05 | 0,01 | 0,01 | 27,82 | 0,01 |
| 202310764 | 0,25 | 0,01 | 24,88 | 0,05 | 0,01 | 0,01 | 27,62 | 0,01 |
| 202310839 | 0,25 | 0,00 | 20,26 | 0,06 | 0,01 | 0,01 | 12,82 | 0,02 |
| 202310849 | 0,25 | 0,01 | 16,16 | 0,07 | 0,02 | 0,02 | 20,87 | 0,01 |
| 202310950 | 0,25 | 0,00 | 14,47 | 0,08 | 0,02 | 0,02 | 16,73 | 0,01 |
| 202310951 | 0,25 | 0,01 | 12,72 | 0,09 | 0,02 | 0,02 | 15,32 | 0,02 |
| 202310957 | 0,25 | 0,01 | 17,70 | 0,07 | 0,02 | 0,01 | 23,26 | 0,01 |
| 202310961 | 0,25 | 0,06 | 5,43 | 0,22 | 0,06 | 0,05 | 9,44 | 0,03 |
| 202310969 | 0,25 | 0,01 | 12,31 | 0,10 | 0,02 | 0,02 | 13,17 | 0,02 |
| 202311056 | 0,25 | 0,00 | 16,36 | 0,07 | 0,02 | 0,02 | 19,69 | 0,01 |
| 202311058 | 0,25 | 0,00 | 17,63 | 0,07 | 0,02 | 0,01 | 20,21 | 0,01 |
| 202311310 | 0,25 | 0,00 | 14,95 | 0,08 | 0,02 | 0,02 | 16,50 | 0,02 |
| 202316822 | 0,25 | 0,00 | 8,54 | 0,14 | 0,04 | 0,03 | 7,64 | 0,03 |
| 202316833 | 0,25 | 0,02 | 11,43 | 0,10 | 0,03 | 0,02 | 15,86 | 0,02 |
| 202316834 | 0,25 | 0,04 | 14,54 | 0,08 | 0,02 | 0,02 | 19,02 | 0,01 |
| 202316835 | 0,25 | 0,02 | 11,14 | 0,11 | 0,03 | 0,02 | 18,63 | 0,01 |
| 202316837 | 0,25 | 0,08 | 21,20 | 0,06 | 0,01 | 0,01 | 23,99 | 0,01 |
| 202316839 | 0,25 | 0,06 | 18,36 | 0,07 | 0,02 | 0,01 | 23,63 | 0,01 |
| 202316909 | 0,25 | 0,00 | 20,16 | 0,06 | 0,01 | 0,01 | 21,63 | 0,01 |
| 202317124 | 0,25 | 0,00 | 26,41 | 0,05 | 0,01 | 0,01 | 20,55 | 0,01 |
| 202317366 | 0,25 | 0,01 | 25,83 | 0,05 | 0,01 | 0,01 | 28,23 | 0,01 |
| 202317380 | 0,25 | 0,01 | 32,02 | 0,04 | 0,01 | 0,01 | 36,31 | 0,01 |
| 202317589 | 0,25 | 0,00 | 14,90 | 0,08 | 0,02 | 0,02 | 16,11 | 0,02 |
| 202317675 | 0,25 | 0,00 | 27,26 | 0,04 | 0,01 | 0,01 | 36,88 | 0,01 |
| 202317717 | 0,25 | 0,00 | 12,11 | 0,10 | 0,02 | 0,02 | 7,67 | 0,03 |
| 202317778 | 0,25 | 0,00 | 22,72 | 0,05 | 0,01 | 0,01 | 27,50 | 0,01 |
| 202317809 | 0,25 | 0,00 | 15,68 | 0,08 | 0,02 | 0,02 | 18,64 | 0,01 |
| 202317810 | 0,25 | 0,01 | 15,70 | 0,08 | 0,02 | 0,02 | 18,73 | 0,01 |
| 202317817 | 0,25 | 0,07 | 5,10 | 0,24 | 0,06 | 0,05 | 8,10 | 0,03 |
| 202317818 | 0,25 | 0,01 | 13,30 | 0,09 | 0,02 | 0,02 | 16,07 | 0,02 |
| 202317886 | 0,25 | 0,00 | 19,25 | 0,06 | 0,02 | 0,01 | 19,84 | 0,01 |
| 202317900 | 0,25 | 0,00 | 23,06 | 0,05 | 0,01 | 0,01 | 28,63 | 0,01 |
| 202317916 | 0,25 | 0,00 | 21,15 | 0,06 | 0,01 | 0,01 | 23,36 | 0,01 |
| 202318097 | 0,25 | 0,01 | 18,14 | 0,07 | 0,02 | 0,01 | 23,27 | 0,01 |
| 202318098 | 0,25 | 0,02 | 19,34 | 0,06 | 0,02 | 0,01 | 24,22 | 0,01 |
| 202318132 | 0,25 | 0,00 | 6,74 | 0,18 | 0,04 | 0,04 | 146,46 | 0,00 |
| 202318180 | 0,25 | 0,02 | 17,94 | 0,07 | 0,02 | 0,01 | 21,88 | 0,01 |
| 202318195 | 0,25 | 0,00 | 22,34 | 0,05 | 0,01 | 0,01 | 25,54 | 0,01 |
| 202318202 | 0,25 | 0,00 | 19,25 | 0,06 | 0,02 | 0,01 | 23,03 | 0,01 |
| 202318334 | 0,25 | 0,01 | 14,32 | 0,08 | 0,02 | 0,02 | 15,94 | 0,02 |

Table 6: comparison of the measured dissolved concentration, the PHREEQC calculated fraction of the free metal ion, the calculated risk characterization ratio (RCR) by Bio-MET for cobalt.

|  | Measured | PHREEQC | Bio-MET RESULTS (Cobalt) with EQSbioav = 0,5 µg/L |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sample Number | Cobalt (dissolved) [µg/L] | Co2+ (µg/L) | Local HC5 (dissolved) [µg/L] | BioF | Bioavailable Cobalt (µg/L) | RCR |
| 202309908 | 0,25 | 0,25 | 3,11 | 0,51 | 0,13 | 0,26 |
| 202310063 | 0,87 | 0,86 | 3,16 | 0,51 | 0,44 | 0,88 |
| 202310538 | 2,70 | 2,69 | 3,09 | 0,52 | 1,40 | 2,80 |
| 202310754 | 1,62 | 1,62 | 3,57 | 0,45 | 0,73 | 1,45 |
| 202310764 | 2,60 | 2,60 | 3,60 | 0,44 | 1,16 | 2,31 |
| 202310839 | 0,42 | 0,42 | 2,18 | 0,73 | 0,31 | 0,62 |
| 202310849 | 1,36 | 1,36 | 3,33 | 0,48 | 0,65 | 1,31 |
| 202310950 | 10,70 | 10,65 | 2,83 | 0,56 | 6,04 | 12,08 |
| 202310951 | 0,79 | 0,79 | 3,44 | 0,47 | 0,37 | 0,74 |
| 202310957 | 1,28 | 1,27 | 3,25 | 0,49 | 0,63 | 1,26 |
| 202310961 | 0,45 | 0,44 | 4,31 | 0,37 | 0,17 | 0,33 |
| 202310969 | 0,62 | 0,62 | 3,57 | 0,45 | 0,28 | 0,56 |
| 202311056 | 0,71 | 0,71 | 3,46 | 0,46 | 0,33 | 0,66 |
| 202311058 | 0,37 | 0,37 | 3,32 | 0,48 | 0,18 | 0,36 |
| 202311310 | 0,95 | 0,95 | 3,38 | 0,47 | 0,45 | 0,90 |
| 202316822 | 0,03 | 0,02 | 1,96 | 0,82 | 0,02 | 0,04 |
| 202316833 | 3,10 | 3,09 | 3,34 | 0,48 | 1,49 | 2,97 |
| 202316834 | 4,60 | 4,60 | 3,45 | 0,46 | 2,13 | 4,27 |
| 202316835 | 3,90 | 3,84 | 3,34 | 0,48 | 1,87 | 3,74 |
| 202316837 | 5,80 | 5,79 | 4,73 | 0,34 | 1,96 | 3,92 |
| 202316839 | 19,60 | 19,56 | 4,33 | 0,37 | 7,25 | 14,50 |
| 202316909 | 1,57 | 1,56 | 3,45 | 0,46 | 0,73 | 1,46 |
| 202317124 | 0,50 | 0,50 | 2,54 | 0,63 | 0,31 | 0,63 |
| 202317366 | 0,81 | 0,81 | 3,60 | 0,44 | 0,36 | 0,72 |
| 202317380 | 0,84 | 0,84 | 4,14 | 0,39 | 0,32 | 0,65 |
| 202317589 | 0,18 | 0,18 | 2,65 | 0,60 | 0,11 | 0,22 |
| 202317675 | 0,11 | 0,11 | 3,77 | 0,42 | 0,05 | 0,09 |
| 202317717 | 0,03 | 0,02 | 2,54 | 0,63 | 0,02 | 0,03 |
| 202317778 | 0,40 | 0,40 | 3,42 | 0,47 | 0,19 | 0,37 |
| 202317809 | 5,40 | 5,39 | 3,01 | 0,53 | 2,87 | 5,73 |
| 202317810 | 0,63 | 0,63 | 3,33 | 0,48 | 0,30 | 0,61 |
| 202317817 | 0,39 | 0,39 | 3,73 | 0,43 | 0,17 | 0,33 |
| 202317818 | 0,82 | 0,82 | 3,44 | 0,47 | 0,38 | 0,76 |
| 202317886 | 0,58 | 0,58 | 3,17 | 0,50 | 0,29 | 0,58 |
| 202317900 | 1,80 | 1,78 | 2,96 | 0,54 | 0,97 | 1,94 |
| 202317916 | 0,76 | 0,76 | 2,18 | 0,73 | 0,56 | 1,11 |
| 202318097 | 0,47 | 0,47 | 3,42 | 0,47 | 0,22 | 0,44 |
| 202318098 | 0,50 | 0,50 | 3,63 | 0,44 | 0,22 | 0,44 |
| 202318132 | 0,17 | 0,14 | 2,32 | 0,69 | 0,12 | 0,23 |
| 202318180 | 1,35 | 1,35 | 3,42 | 0,47 | 0,63 | 1,26 |
| 202318195 | 0,27 | 0,27 | 2,54 | 0,63 | 0,17 | 0,34 |
| 202318202 | 0,36 | 0,36 | 3,32 | 0,48 | 0,17 | 0,35 |
| 202318334 | 0,49 | 0,49 | 3,38 | 0,47 | 0,23 | 0,46 |