Appendix

- The total activation energy for ionic conduction through a pore is given by:  Ea  = ED + EB

where Ea   is the total activation energy, EB is the Born energy and ED is the energy required for diffusion     through the aqueous medium filling the pore, which is ∼18 KJ/mol.

- Using the experimentally determined activation energy for conduction Ea, the Born energy EB  can be calculated

- The Born energy for inserting an ion into a pore is given by EB= Z2  q2   α4 πε0 ɛhr  , where α = ~ 0.2 for a long cylindrical pore. From this, the average pore radius r was calculated.

- The average pore conductivity can then be calculated via the following equation:

Gp= σ AL= σπr2d

where Gp is the conductivity of the pore, d is the thickness of the bilayer and σ is the specific conductivity                                                   inside the pore.

- The specific conductivity inside the pore (σ) is calculated via the following equation:

σ =σPbs *γ partition

where   σPbs represents the specific conductivity of the PBS solution, which was 17 × 10-3 S/cm, and where γ partition  represents the partition coefficient. Therefore:

Gp= σ AL=σPbs*γpartition× πr2d

- To calculate the partition coefficient   γpartition, the following equation was used:

γpartition= e-EBKT

- The total membrane conductance Gm  is the total number of pores nr multiplied by the specific conductance of each pore Gp:

Gm =  nr×Gp