Supplementary Methods

Supplementary Method 1. Acquisition and analysis of multimodal MRI

MRI data were acquired on a 3T scanner (uMR 890, United Imaging Healthcare, Shanghai, China) with a dedicated 64-channel head coil. As our previous protocol of RJNB-Dementia dataset1,2, the T1-weighted structural images were acquired with a 3D fast spoiled gradient-echo sequence, with 0.75 mm isotropic voxels (field of view read: 240 mm, output images of 360 × 439× 480 slices, repetition time (TR) / inversion time (TI) = 7.5 ms /1100 ms, echo time (TE) = 3.4 ms, flip angle = 7 degree). The 3D FLAIR acquisition parameters were: TR= 6000 ms, TE = 408 ms, and TI = 1841ms, with 0.75 mm isotropic voxels. Diffusion images were acquired with an echo-planar imaging (EPI) sequence, and the parameters were: TR/TE= 3500/77.3 ms, voxel size = 1.5 × 1.5 × 1.5 mm, 100 slices, multiple band = 10, diffusion direction = 32/64, b = 1500/3000, phase encoding direction = PA. A pair of four b0 images with reversed phase-encode blips (AP and PA) was acquired to correct susceptibility-induced distortion. The ASL scans were performed using a 5-delay pCASL protocol with background suppressed 3D GRASE (gradient and spin echo) readout (labeling pulse duration = 1.8 s, post label delay = 0.5-1.0-1.5-2.0-2.5 s, matrix = 64 × 64, voxel size = 3.5 × 3.5 × 3.5 mm, 6 pairs of tag and control for each delay).

CP and WMH segmentation

The region of interest (ROI) of CP was segmented from 3D-T1 weighted images. Both baseline and follow-up 3D-T1 images were segmented automatically based on FreeSurfer 7.1.1 platform as previously described3,4. The segmentation of brain volumes is based on an atlas containing probabilistic information regarding the location of structures. This pre-processing step involves motion correction of 3D T1 images, brain extraction, segmentation of subcortical white matter and deep gray matter volumetric structures, intensity normalization, white matter surface smoothing, topology correction, and optimization of pial and white matter surfaces. The total intracranial volume (TIV) and the CP of the lateral ventricle were determined through this process. In addition, the cortical thickness was determined by measuring the distance between the white matter and pial surfaces. All final automatic segmentations underwent quality checks by a senior neurologist (X.X.) who was blinded to clinical and other image data. The CP ROI were used as mask for analysis after registration with the following MRI modalities. The CP volumes (CPV) were expressed as ratios to TIV.

The WMH was defined as subcortical hyperintensities without cavitation on T2 FLAIR based on the recommendations of Standards for Reporting Vascular Changes on Neuroimaging (STRIVE)5. We utilized FSL's Brain Intensity AbNormality Classification Algorithm (BIANCA, FMRIB Software Library, Oxford, UK), version 6.0, a fully automated, supervised k-nearest neighbor (k-NN) algorithm for WMH segmentation. The binary masks of total hyperintensities were generated using BIANCA and then carefully inspected and manually corrected to produce the final WMH binary masks. These derived WMH masks were further categorized into periventricular WMH (pWMH) and deep WMH (dWMH) by applying a 10 mm distance threshold from the ventricles1. Subsequently, all maps were visually inspected and manually corrected as needed. The volumes of pWMH and dWMH were expressed as ratios to the total volume of white matter.

Diffusion MRI analysis

Pre-processing: The pre-processing pipelines for diffusion MRI generally followed the protocols of an image-processing pipeline developed by UK Biobank6. The original diffusion MRI data were corrected for susceptibility-induced distortion by top-up method7. Then the data were further corrected for head motion and eddy currents using the Eddy tool in FSL. The output data were computed into diffusion tensor maps using the “DTIFIT” function of FSL.

Fitting diffusion tensors: The DTIFIT estimated a diffusion tensor at each voxel of the brain, and outputs a set of images including fractional anisotropy (FA) and mean diffusivity (MD), as well as voxel-wise diffusivity in the directions of the x-axis (Dx), y-axis (Dy), and z-axis (Dz) images. We linearly aligned individual FA/MD maps with the T1-weighted image. The registration process is a combination of affine transformation8. Then, the CP mask in T1-weighted image could be used to extract mean FA and MD value in the CP. Visual examinations were performed to identify any co-registration bias.

DTI-ALPS: The FA map of each subject was registered to the FA map template in the standard MNI space, and the accuracy of the co-registration was visually confirmed. The Dx, Dy, Dz maps of the subject were then warped based on the registration matrix obtained from the FA map. The ROIs corresponding to the left projection fibers (superior and posterior corona radiata) and association fibers (superior longitudinal fasciculus) were extracted using atlas labels. The range of ROIs in the craniocaudal direction was limited to where the x-axis lines were perpendicular to the lateral ventricle bodies. A mask with FA values greater than 0.2 was applied to exclude CSF voxels. Finally, the Dx, Dy, and Dz values in these ROIs on projection fibers and association fibers were determined as Dxproj, Dyproj, Dzproj, Dxassoc, Dyassoc, Dzassoc, respectively. The DTI-ALPS index was automatically computed using the following equation9:

DTI-ALPS index= [(Dxproj + Dxassoc) / (Dyproj + Dzassoc)]

Free-Water image of CP: The FWf can be measured quantitatively by neurite orientation dispersion and density imaging (NODDI) model derived from multi-shell diffusion MRI. NODDI models the voxel contents as neurites (thin pipes, nominally axons), the spaces between them, and freely diffusing water10,11. NODDI estimation for FWf was done using Accelerated Microstructure Imaging via Convex Optimization6.  After registered with T1-weighted image, the averaged Fwf of CP was calculated from the left and right CP ROI (Fig. 1B).

Cerebral blood flow in CP

We used FSL BASIL toolbox to estimate perfusion in the CP. The key steps in BASIL pipeline includes motion correction, registration, calibration using M0 image, and Bayesian kinetic modelling to estimate perfusion maps, with spatial regularization and partial volume correction (PVC). Following these steps, the whole brain perfusion map was registered to the T1-weighted image, and the accuracy of the co-registration was visually confirmed. Subsequently, the cerebral blood flow (CBF) of the choroid plexus was extracted from the CP region of interest segmented from the T1-weighted image.

Supplementary Method 2. Acquisition and analysis of of PET images

We utilized PET scans to evaluate the spatial accumulation of Aβ and Tau, as well as synaptic density in the study. The PET scans were conducted using a 3T whole-body PET/MR scanner (uPMR 790, United Imaging, China). Participants were administered an intravenous injection of 18F-Florbetapir (FBP) at an average dose of 3.7 MBq/kg body weight to visualize Aβ accumulation. Static FBP-PET data were acquired in sinogram mode 50 minutes after the injection. Additionally, all participants underwent a static 18F-MK-6240 PET scan to image Tau. Furthermore, a 18F-SynVesT-1 PET scan was performed to visualize synaptic vesicle glycoprotein 2A (SV2A) as previously described2. Participants refrained from taking drugs targeting SV2A for at least 24 hours before the 18F-SynVesT-1 PET scan. A 30-minute static PET scan was initiated at 60 minutes post-injection of 18F-SynVesT-1 (~3.7 MBq/kg body weight). Each participant had the three PET scans on different days within two weeks; the scans were scheduled at least three days apart from each other.

An automated pipeline was utilized to derive cortical standardized uptake value ratios (SUVR) using the PETSurfer toolbox in Freesurfer, with the cerebellum cortex serving as the reference region12. Initially, high-resolution segmentation was generated from structural T1 images to facilitate PVC techniques. Subsequently, registration of PET and anatomical images was conducted and visually validated. To mitigate partial volume effects resulting from cortical atrophy in AD, the extended Muller–Gartner method was employed as a PVC approach13. Surface-based maps of Aβ, Tau, and SV2A were smoothed on a two-dimensional surface using a Gaussian kernel with a full width at half maximum (FWHM) of 5mm. These biomarker maps were then utilized for vertex-wise analysis and mean cortical SUVRs were extracted for spatial correlation analyses.

Supplementary Method 3. Peripheral blood biomarkers

We collected a 20mL venous blood sample from participants at the visits. Samples were centrifuged to isolate serum and plasma before being stored at -80℃. Protein concentrations in ethylenediaminetetraacetic acid (EDTA) plasma were assessed using the ultra-sensitive single-molecule array (Simoa) platform. Specifically, GFAP and NFL levels were quantified utilizing the Neurology 4-Plex E Advantage kit (QTX-103670, Quanterix, Billerica, Massachusetts, United States). Serum concentrations of NRGN and TNFα were determined using the Luminex 200 system (Luminex, Austin, TX, USA) with the Human ProcartaPlex Mix & Match 3-plex kit (PPX-03-MXMFYR9, Thermo Fisher Scientific).

References for supplementary materials

1.	Zhang J, Chen H, Wang J, et al. Linking white matter hyperintensities to regional cortical thinning, amyloid deposition, and synaptic density loss in Alzheimer’s disease. Alzheimers Dement. Published online April 22, 2024. doi:10.1002/alz.13845

2.	Zhang J, Wang J, Xu X, et al. In vivo synaptic density loss correlates with impaired functional and related structural connectivity in Alzheimer’s disease. J Cereb Blood Flow Metab. 2023;43(6):977-988. doi:10.1177/0271678X231153730

3.	Li B, Jang I, Riphagen J, et al. Identifying individuals with Alzheimer’s disease-like brains based on structural imaging in the Human Connectome Project Aging cohort. Hum Brain Mapp. 2021;42(17):5535-5546. doi:10.1002/hbm.25626

4.	Li B, Zhang M, Riphagen J, et al. Prediction of clinical and biomarker conformed Alzheimer’s disease and mild cognitive impairment from multi-feature brain structural MRI using age-correction from a large independent lifespan sample. Neuroimage Clin. 2020;28:102387. doi:10.1016/j.nicl.2020.102387

5.	Duering M, Biessels GJ, Brodtmann A, et al. Neuroimaging standards for research into small vessel disease-advances since 2013. Lancet Neurol. Published online May 23, 2023:S1474-4422(23)00131-X. doi:10.1016/S1474-4422(23)00131-X

6.	Daducci A, Canales-Rodríguez EJ, Zhang H, Dyrby TB, Alexander DC, Thiran JP. Accelerated Microstructure Imaging via Convex Optimization (AMICO) from diffusion MRI data. Neuroimage. 2015;105:32-44. doi:10.1016/j.neuroimage.2014.10.026

7.	Andersson JLR, Skare S, Ashburner J. How to correct susceptibility distortions in spin-echo echo-planar images: application to diffusion tensor imaging. Neuroimage. 2003;20(2):870-888. doi:10.1016/S1053-8119(03)00336-7

8.	Smith SM, Jenkinson M, Woolrich MW, et al. Advances in functional and structural MR image analysis and implementation as FSL. Neuroimage. 2004;23 Suppl 1:S208-219. doi:10.1016/j.neuroimage.2004.07.051

9.	Taoka T, Masutani Y, Kawai H, et al. Evaluation of glymphatic system activity with the diffusion MR technique: diffusion tensor image analysis along the perivascular space (DTI-ALPS) in Alzheimer’s disease cases. Jpn J Radiol. 2017;35(4):172-178. doi:10.1007/s11604-017-0617-z

10.	Kamiya K, Hori M, Aoki S. NODDI in clinical research. J Neurosci Methods. 2020;346:108908. doi:10.1016/j.jneumeth.2020.108908

11.	Zhang H, Schneider T, Wheeler-Kingshott CA, Alexander DC. NODDI: practical in vivo neurite orientation dispersion and density imaging of the human brain. Neuroimage. 2012;61(4):1000-1016. doi:10.1016/j.neuroimage.2012.03.072

12.	Greve DN, Svarer C, Fisher PM, et al. Cortical surface-based analysis reduces bias and variance in kinetic modeling of brain PET data. Neuroimage. 2014;92:225-236. doi:10.1016/j.neuroimage.2013.12.021

13.	Greve DN, Salat DH, Bowen SL, et al. Different partial volume correction methods lead to different conclusions: An (18)F-FDG-PET study of aging. Neuroimage. 2016;132:334-343. doi:10.1016/j.neuroimage.2016.02.042