| Raw material |  |  | Ref |
| --- | --- | --- | --- |
| Corn Stover | 0,01 | USD/Ton | this research |
| supplies |  |  |  |
| water | 0,33 | USD/Ton | [1] |
| enzymes | 49,30 | USD/kg |  |
| H2SO4 | 0,17 | USD/Ton |  |
| Lime | 0,13 | USD/Ton |  |
| Ethanol (solvent) | 1,84 | USD/L |  |
| CO2 | 0,48 | USD/kg | [2] |
| NaOH(solid) | 251 | USD/Ton | [3] |
| Activated Carbon | 600 | USD/ton | [4] |
| Products |  |  |  |
| Biogas | 0,15 | USD/m3 | [1] |
| Solid digestate | 7,36 | USD/Ton |  |
| Liquid digestate | 4,16 | USD/Ton |  |
| Butanol | 1,23 | USD/L | [5] |
| Acetone | 1,2 | USD/kg |  |
| Ethanol | 1,3 | USD/L |  |
| Xylitol | 3000 | USD/ton | [4] |
| kraft lignin | 350 | USD/ton | [6] |
| organosolv lignin | 450 | USD/ton |  |
| Biomethane | 0,45 | USD/ton | [7] |
| Utilities |  |  |  |
| Ethylene glycol | 0,85 | USD/L | [5] |
| low Pressure steam | 7,89 | USD/ton | [1] |
| Medium Pressure steam | 8,07 | USD/ton |  |
| Electricity | 0,25 | USD/Kwh |  |
| Cooling Water | 0,042 | USD/m3 |  |
| supervisors | 2,21 | USD/h | this research |
| operators | 1,42 | USD/h |  |

sample for the calculation of impacts following the PB-LCA methodology:

The base case (B.C) will be taken together with the results of the climate change category presented in Table 7. First, it starts by annualizing the functional unit (1kg corn) considered in the LCA analysis according to the limits considered in the analysis. flow of 22,080 tons/year was considered. production can be estimated at a 1:1.2 ratio (corn:CS).

Cornyear=22,080*11.2=18,400 ton/year

Using the characterization factors provided by Ryberg et al. (Ryberg et al. 2018a) the impact of the climate change category on the PB control variables is calculated annualizing the result.

| ICVn=CFn.LCIn | Eq. (2) |
| --- | --- |
| ICVclimate change=3.53*10-13w.yrm2.kg CO2*0.49 kg CO2kg corn*1.84e7 kg cornyr |  |
| ICVclimate change=1.8636 e-5wm2 |  |

From the equations in Table 6, the SOS was calculated according to the criteria used. We took a world population of 8000 million people, a population in Colombia of 52 million and 1 million in the region of Sucre. Data provided by the International Monetary Fund for world GDP were used (104,476,432 mUSD) and for the GDP of Colombia (363,835 mUSD) [9]. The GDP of the Sucre region was taken to represent 0.79% of Colombia's GDP. and that corn farming accounts for 11.4% of the sucre region's GDP. [10]

SOSn=SOST*PaPworld

SOSa=1wm2*18,000=0.000125 wm2

SOSn=SOST*GDPnGDPworld

SOSb=1wm2*363,835 mUSD*0,79%*11,4%104,476,432 mUSD=3.1363e-6 wm2

SOSn=SOST*PcountryPworld*GDPnGDPcountry

SOSc=1wm2*528,000*0,79%*11,4%=5.85e-6 wm2

Once the SOS has been calculated, we proceed to calculate the TL and determine the impact of the activity in terms of the PBs.

TLn=ICVclimate changeSOSn

TLa climate change=1.86 e-5wm20.000125 wm2=0,149

TLb climate change=1.86 e-5wm23.13e-6 wm2=5.943

TLc climate change=1.86 e-5wm25.85e-6 wm2=3.183

Simulation conditions

Organosolvents: The modeling of the organosolv pretreatment followed the results obtained in the experimental work of Park et al. [11] With a solvent mixture 56% self-catalyzed ethanol. Temperature 185 °C. The modeling consisted of the separation stage of the lignocellulosic fractions with the respective fiber washes, followed by the recovery of the solvent [12]. Followed a 4:1 w/w ratio of solvents/CS [13].

Enzymatic Hydrolysis: The simulation followed the methodology proposed by [14]. Where a reaction temperature of 50° C was taken. A ratio of 20 mg of enzyme per gram was used. A solid/liquid ratio of 1:20 was applied [15]. For the yields for CS with Organsolv pretreatment, were used the experimental results of [11]. The yield of the CS subjected to the dilute acid was taken from the yield of the experimental tests of [16]. For CS that has not been subjected to pretreatment, experimental results of [17].

Alkaline pretreatment: The main function of alkaline pretreatment is to remove lignin from lignocellulosic material. For the reaction conditions as well as for the pretreatment efficiency, we used as reference the experimental results obtained by [18]. A 1:8 w/w ratio of alkali/CS was used. The temperature was set at 120 °C for a reaction time of 30 min. For each kg of CS, 6 kg of water is added to the solution.

Acid pretreatment: Acid treatment seeks the release of hemicellulose by degrading it into xylose C5 sugars. During this process, secondary reactions take place where other substances are formed, mainly furfural. For the pretreatment modeling, we followed the kinetic model proposed by [19] Formulated from experimental tests with CS. A concentration of 2% by weight of sulfuric acid was taken. With a solid/liquid ratio of 1:20. The operating temperature was set at 100 °C.

ABE: The results of the fermentation process followed the model presented by [5] and [20], The operating conditions for the fermentation also followed the results of this author, setting the reactor operating temperature at 37 °C. For the simulation in Aspen Plus of the separation zone of acetone, butanol, ethanol, the model and conditions presented by  [21].[22]

Xylitol: Experimental results are taken from [23] the simulation was guided by the process proposed by [24]. As well as the crystallization yields for detoxification with CA, we used the results obtained in [25]. Reactions involved were taken from [26], the hydrolysate was concentrated to 90 g/l xylose and fermentation was carried out at 30 °C.

Lignina Kraft: The LignoBoost process will be used for lignin purification, the amount of CO2 used will be 200kg per ton of lignin [27] the general process was simulated by following the process diagram presented by [28]

Anaerobic Digestion: For the simulation of anaerobic digestion, were followed the model proposed by [29] Following a stoichiometric model, the digestion conditions are taken from [30], furthermore, the conditions as the general scheme for methane production by pressure water washing were mainly based on [31]

Ethanol: Ethanol fermentation followed a classical scheme with two distillation towers for ethanol concentration until a point close to the azeotropic concentration, then dried to anhydrous point with a molecular sieve stage in conjunction with a fermentation using saccharomyces cerevisiae [32].

[1]	M. Ortiz-Sanchez, J. C. Solarte-Toro, P. J. Inocencio-García, and C. A. Cardona Alzate, “Sustainability analysis of orange peel biorefineries,” Enzyme Microb Technol, vol. 172, p. 110327, Jan. 2024, doi: 10.1016/J.ENZMICTEC.2023.110327.

[2]	“Mild Steel Industrial Carbon Dioxide Cylinder, 150 Bar at best price in Coimbatore.” Accessed: Aug. 03, 2024. [Online]. Available: https://www.indiamart.com/proddetail/co2-cylinder-27218850333.html

[3]	“Caustic Soda Price in Colombia - 2022 - Charts and Tables - IndexBox.” Accessed: Aug. 03, 2024. [Online]. Available: https://www.indexbox.io/search/caustic-soda-price-colombia/

[4]	H. M. R. Özüdoğru, M. Nieder-Heitmann, K. F. Haigh, and J. F. Görgens, “Techno-economic analysis of product biorefineries utilizing sugarcane lignocelluloses: Xylitol, citric acid and glutamic acid scenarios annexed to sugar mills with electricity co-production,” Ind Crops Prod, vol. 133, pp. 259–268, Jul. 2019, doi: 10.1016/J.INDCROP.2019.03.015.

[5]	S. I. Meramo-Hurtado, Á. González-Delgado, L. Rehmann, E. Quinones-Bolanos, and M. Mehvar, “Comparative analysis of biorefinery designs based on acetone-butanol-ethanol fermentation under exergetic, techno-economic, and sensitivity analyses towards a sustainability perspective,” J Clean Prod, vol. 298, p. 126761, May 2021, doi: 10.1016/J.JCLEPRO.2021.126761.

[6]	J. A. Poveda-Giraldo, J. C. Solarte-Toro, and C. A. Cardona Alzate, “The potential use of lignin as a platform product in biorefineries: A review,” Renewable and Sustainable Energy Reviews, vol. 138, p. 110688, Mar. 2021, doi: 10.1016/J.RSER.2020.110688.

[7]	“EU Natural Gas TTF - Price - Chart - Historical Data - News.” Accessed: Aug. 03, 2024. [Online]. Available: https://tradingeconomics.com/commodity/eu-natural-gas

[8]	M. W. Ryberg et al., “How to bring absolute sustainability into decision-making: An industry case study using a Planetary Boundary-based methodology,” Science of The Total Environment, vol. 634, pp. 1406–1416, Sep. 2018, doi: 10.1016/J.SCITOTENV.2018.04.075.

[9]	“Report for Selected Countries and Subjects.” Accessed: Aug. 03, 2024. [Online]. Available: https://www.imf.org/en/Publications/WEO/weo-database/2023/October/weo-report?c=512,914,612,171,614,311,213,911,314,193,122,912,313,419,513,316,913,124,339,638,514,218,963,616,223,516,918,748,618,624,522,622,156,626,628,228,924,233,632,636,634,238,662,960,423,935,128,611,321,243,248,469,253,642,643,939,734,644,819,172,132,646,648,915,134,652,174,328,258,656,654,336,263,268,532,944,176,534,536,429,433,178,436,136,343,158,439,916,664,826,542,967,443,917,544,941,446,666,668,672,946,137,546,674,676,548,556,678,181,867,682,684,273,868,921,948,943,686,688,518,728,836,558,138,196,278,692,694,962,142,449,564,565,283,853,288,293,566,964,182,359,453,968,922,714,862,135,716,456,722,942,718,724,576,936,961,813,726,199,733,184,524,361,362,364,732,366,144,146,463,528,923,738,578,537,742,866,369,744,186,925,869,746,926,466,112,111,298,927,846,299,582,487,474,754,698,&s=NGDPD,&sy=2021&ey=2028&ssm=0&scsm=1&scc=0&ssd=1&ssc=0&sic=0&sort=country&ds=.&br=1

[10]	MinCIT, “Departmental Economic Profiles Department of Sucre Economic Profile: Department of Sucre Contents,(in Spanish)” 2024.

[11]	Y. C. Park, T. H. Kim, and J. S. Kim, “Flow-Through Pretreatment of Corn Stover by Recycling Organosolv to Reduce Waste Solvent,” Energies 2018, Vol. 11, Page 879, vol. 11, no. 4, p. 879, Apr. 2018, doi: 10.3390/EN11040879.

[12]	A. García, M. G. Alriols, R. Llano-Ponte, and J. Labidi, “Energy and economic assessment of soda and organosolv biorefinery processes,” Biomass Bioenergy, vol. 35, no. 1, pp. 516–525, Jan. 2011, doi: 10.1016/J.BIOMBIOE.2010.10.002.

[13]	R. Nitzsche, M. Budzinski, and A. Gröngröft, “Techno-economic assessment of a wood-based biorefinery concept for the production of polymer-grade ethylene, organosolv lignin and fuel,” Bioresour Technol, vol. 200, pp. 928–939, Jan. 2016, doi: 10.1016/J.BIORTECH.2015.11.008.

[14]	M. T. Ashraf and J. E. Schmidt, “Process simulation and economic assessment of hydrothermal pretreatment and enzymatic hydrolysis of multi-feedstock lignocellulose – Separate vs combined processing,” Bioresour Technol, vol. 249, pp. 835–843, Feb. 2018, doi: 10.1016/J.BIORTECH.2017.10.088.

[15]	M. Ortiz-Sanchez, J. C. Solarte-Toro, P. J. Inocencio-García, and C. A. Cardona Alzate, “Sustainability analysis of orange peel biorefineries,” Enzyme Microb Technol, vol. 172, p. 110327, Jan. 2024, doi: 10.1016/J.ENZMICTEC.2023.110327.

[16]	J. Li, H. Zhang, M. Lu, and L. Han, “Comparison and intrinsic correlation analysis based on composition, microstructure and enzymatic hydrolysis of corn stover after different types of pretreatments,” Bioresour Technol, vol. 293, p. 122016, Dec. 2019, doi: 10.1016/J.BIORTECH.2019.122016.

[17]	Z. Wang et al., “Enhancing enzymatic hydrolysis of corn stover by twin-screw extrusion pretreatment,” Ind Crops Prod, vol. 143, p. 111960, Jan. 2020, doi: 10.1016/J.INDCROP.2019.111960.

[18]	M. Chen, J. Zhao, and L. Xia, “Comparison of four different chemical pretreatments of corn stover for enhancing enzymatic digestibility,” Biomass Bioenergy, vol. 33, no. 10, pp. 1381–1385, Oct. 2009, doi: 10.1016/J.BIOMBIOE.2009.05.025.

[19]	Q. Jin, H. Zhang, L. Yan, L. Qu, and H. Huang, “Kinetic characterization for hemicellulose hydrolysis of corn stover in a dilute acid cycle spray flow-through reactor at moderate conditions,” Biomass Bioenergy, vol. 35, no. 10, pp. 4158–4164, Oct. 2011, doi: 10.1016/J.BIOMBIOE.2011.06.050.

[20]	K. Darkwah, S. E. Nokes, J. R. Seay, and B. L. Knutson, “Mechanistic simulation of batch acetone-butanol-ethanol (ABE) fermentation with in situ gas stripping using Aspen PlusTM,” Bioprocess Biosyst Eng, vol. 41, no. 9, pp. 1283–1294, Sep. 2018, doi: 10.1007/S00449-018-1956-6.

[21]	K. F. Haigh, A. M. Petersen, L. Gottumukkala, M. Mandegari, K. Naleli, and J. F. Görgens, “Simulation and comparison of processes for biobutanol production from lignocellulose via ABE fermentation,” Biofuels, Bioproducts and Biorefining, vol. 12, no. 6, pp. 1023–1036, Nov. 2018, doi: 10.1002/BBB.1917.

[22]	T. C. Ezeji, P. M. Karcher, N. Qureshi, and H. P. Blaschek, “Improving performance of a gas stripping-based recovery system to remove butanol from Clostridium beijerinckii fermentation,” Bioprocess Biosyst Eng, vol. 27, no. 3, pp. 207–214, May 2005, doi: 10.1007/S00449-005-0403-7/TABLES/1.

[23]	S. I. Mussatto and I. C. Roberto, “Kinetic behavior of Candida guilliermondii yeast during xylitol production from highly concentrated hydrolysate,” Process Biochemistry, vol. 39, no. 11, pp. 1433–1439, Jul. 2004, doi: 10.1016/S0032-9592(03)00261-9.

[24]	D. Dasgupta et al., “Energy and life cycle impact assessment for xylitol production from corncob,” J Clean Prod, vol. 278, p. 123217, Jan. 2021, doi: 10.1016/J.JCLEPRO.2020.123217.

[25]	D. B. Hodge, C. Andersson, K. A. Berglund, and U. Rova, “Detoxification requirements for bioconversion of softwood dilute acid hydrolyzates to succinic acid,” Enzyme Microb Technol, vol. 44, no. 5, pp. 309–316, May 2009, doi: 10.1016/J.ENZMICTEC.2008.11.007.

[26]	K. L. Gerbrandt, “The Impacts of Xylitol Production from Hemicellulose Residues: Process Design, Life Cycle, and Techno-Economic Assessment,” 2014.

[27]	M. Kienberger, S. Maitz, T. Pichler, and P. Demmelmayer, “Systematic Review on Isolation Processes for Technical Lignin,” Processes 2021, Vol. 9, Page 804, vol. 9, no. 5, p. 804, May 2021, doi: 10.3390/PR9050804.

[28]	P. Tomani, “THE LIGNOBOOST PROCESS,” CELLULOSE CHEMISTRY AND TECHNOLOGY Cellulose Chem. Technol, vol. 44, no. 3, pp. 53–58, 2010.

[29]	E. Hilby, “Modeling of Biogas Formation and Utilization using Aspen Plus”, Accessed: Apr. 07, 2024. [Online]. Available: http://www.ub.tuwien.ac.athttp://www.ub.tuwien.ac.at/eng

[30]	C. M. Liu et al., “Biogas production and microbial community properties during anaerobic digestion of corn stover at different temperatures,” Bioresour Technol, vol. 261, pp. 93–103, Aug. 2018, doi: 10.1016/J.BIORTECH.2017.12.076.

[31]	E. Barbera, S. Menegon, D. Banzato, C. D’Alpaos, and A. Bertucco, “From biogas to biomethane: A process simulation-based techno-economic comparison of different upgrading technologies in the Italian context,” Renew Energy, vol. 135, pp. 663–673, May 2019, doi: 10.1016/J.RENENE.2018.12.052.

[32]	Y. Zhao, A. Damgaard, and T. H. Christensen, “Bioethanol from corn stover – a review and technical assessment of alternative biotechnologies,” Prog Energy Combust Sci, vol. 67, pp. 275–291, Jul. 2018, doi: 10.1016/J.PECS.2018.03.004.