Specific parameters: Gansu Provincial People's Hospital used the IQon spectral CT (Philips Healthcare, Netherlands) for enhanced scanning, administering the contrast agent Iohexol (320 mgI/mL) at a dosage of 1.5 mL/kg with a flow rate of 3.5 mL/s. The scanning parameters were as follows: slice thickness of 5 mm, constant tube voltage of 120 kV, automatic milliampere-seconds technology to optimize radiation dose and image quality, scanning matrix of 512 x 512, coverage of 100 slices, pitch of 0.953, and rotation speed of 0.5 s/revolution.
Gansu Second People's Hospital employed a Siemens 16-slice spiral CT scanner for enhanced imaging, using the contrast agent Iopamidol 370 at a dosage of 1 mL/kg with a flow rate of 4.0 mL/s. The scanning parameters included a tube voltage of 120 kV, current ranging from 180 to 200 mA, slice thickness of 10 mm, inter-slice spacing of 5 mm, a field of view of 40-50 cm, pitch of 1.0 mm, and rotation speed of 0.5 s/revolution.