SUPPLEMENTARY INFORMATION

High Spatiotemporal Availability of Hydrogen by Electrolysis of Municipal Reclaimed Water in China

Weixiang Chao, Xiuling Yan, Lu Lu*

State Key Laboratory of Urban Water Resource and Environment, School of Civil and Environmental Engineering, Harbin Institute of Technology, Shenzhen, Shenzhen 518055, China

*Corresponding author E-mail: lulu@hit.edu.cn

Table of Contents

1. Definition of the life-cycle assessment aims and boundary.	1

1.1 The assessment aims and boundary of electrolytic hydrogen (H2) from different water resources.	1

Figure S1 | Comprehensive calculation procedures employed for assessing H2 production scenarios using various electrolytic water resources.	1

1.2 Life-cycle inventories (LCI) of the wastewater/water/seawater treatment, electrolysis, and H2 transport.	2

Method S1 | The process of reclaimed water production in a typical municipal wastewater treatment plant.	2

Table S1 | LCI of wastewater treatment plants producing 1m3 reclaimed water.	6

Table S2 | LCI of water treatment plants producing 1m3 tap water.	10

Table S3 | LCI of reverse-osmosis facilities purifying 1m3 feedstock water for electrolytic H2 production	12

Table S4 | LCI of industrial alkaline electrolyzers producing 1kg H2.	15

Table S5 | LCI of pipes and ships transporting 1tkm H2.	18

Table S6 | Assumptions for pipes and ships transporting H2 with different amounts, and delivery distances.	19

Figure S2 | Schematic of typical wastewater treatment plants producing reclaimed water.	20

Figure S3 | Schematic of typical water treatment plants producing tap water.	20

Figure S4 | Schematic of a 3-stage reverse-osmosis system equipped with zero-liquid discharging polishing for the purification of feedstock water.	21

Figure S5 | Schematic of an industrial alkaline electrolyzer.	21

Figure S6 | Mechanism and configuration of alkaline electrolyzers’ stacks.	22

Figure S7 | Cost of H2 transport by pipes (gaseous) and ships (liquid) in hierarchical transport distance (200-3000 km) and transport amount.	22

1.3 Calibrations of the water and electricity consumption by electrolytic H2 production.	23

Table S7 | Calibrations of the water and electricity consumption by RO facilities.	23

Table S8 | Calibration of the installed capacity of industrial alkaline electrolyzer determining the LCI of electrolyzer.	25

Table S9 | Calibration of the water and electricity consumption by electrolyzers electrolyzing water with pollutants.	26

1.4 Water qualities used for calibrating the efficiency of electrolysis and water treatment.	27

Table S10 | Typical qualities of reclaimed water in China.	27

Table S11 | Typical qualities of terrestrial freshwater in China.	28

Table S12 | Typical qualities of tap water in China.	29

Table S13 | Typical qualities of inshore seawater in China.	30

1.5 Economic assumptions of the assessment projects and weighting of the power grids.	31

Table S14 | Major economic assumptions evaluating the composite cost of H2 production.	31

Table S15 | Structure of power grids in China, 2021.	32

Table S16 | Feed-in tariff prices and transmission prices of electricity in China, 2021.	33

Table S17 | Carbon emission factors for different types of electric power.	35

1.6 Calculation of the carbon emissions and composite cost of electrolytic H2.	36

2. Evaluation of the available water resources and electricity.	37

2.1 Calculation of available water resources and electricity.	37

2.2 Statistical data supporting the calculation of available reclaimed water.	39

Table S18 | Data calculating the available reclaimed water in China, 2021.	39

2.3 Statistical data supporting the calculation of available purified tap water.	40

Table S19 | Data calculating the additional capacities of water treatment plants supplying tap water besides water demand of basic industrial and domestic activities.	40

Table S20 | Data calculating the available per capita water resource after deducting the water demand of basic activities.	41

Figure S8 | Water resources per capita in China from 2003 to 2021.	42

2.4 Statistical data supporting the calculation of available desalinated sweater and offshore wind power.	43

Table S21 | Data calculating the production amount of offshore wind power in coastal provinces.	43

Figure S9 | Installed capacity and provincial distribution of seawater desalination facilities.	43

Figure S10 | Present water quality of seawater in China’s coastal provinces at 2021..	44

Figure S11 | Average wind atlas in China and its coastal areas at 2021..	44

3. Evaluation of the electrolysis efficiency and determination of the H2 equilibrium.	45

3.1 Demonstrations of the purified water production from reclaimed water, tap water, and raw seawater.	45

Method S2 | The production process of purified water provided by the RO facilities.	45

Table S22 | Required qualities of pure water for feedwater into alkaline electrolyzers.	45

Table S23 | Major technical parameters of commercial RO elements.	46

Figure S12 | Schematic of feedwater purification for alkaline electrolysis producing H2.	46

Figure S13 | Specific resources’ consumption, treatment cost and carbon emission for feedstock pre-purification using different water resources.	47

3.2 Demonstration of the alkaline electrolysis.	48

Method S3 | Model of industrial alkaline electrolysis.	48

Table S24 | Technical parameters of industrial alkaline electrolyzer.	48

3.3 Calibration of the electrolysis efficiency.	49

Method S4 | Current efficiency (E, %) influenced by competitive reactions in alkaline electrolysis.	49

Method S5 | Laboratory measurements of alkaline water electrolysis.	50

Table S25 | Previous research data and assumed data for calibrating the water and electricity consumption by electrolyzers directly electrolyzing water with interfering pollutants.	52

Figure S14 | Procedures for calibrating the specific consumption of water and electricity for the production of electrolytic H2 using water resources containing certain pollutants.	53

Figure S15 | Schematic of cross-validation processes.	54

Figure S16 | Multi-parameter linear regression fitting for cross-validation of the assumption that pollutants (mainly organics and salt ions) in water linearly reduce the current efficiency of electrolysis.	55

Figure S17 | Contribution of multiple parameters to the current efficiency of water electrolysis.	56

Figure S18 | Mutual relationship between factors with major contribution and the resultant current efficiency of electrolysis.	57

Figure S19 | Laboratory measurements of alkaline water electrolysis using various water resources.	58

Figure S20 | Specific resources’ consumption for electrolytic H2 using different water sources.	60

3.4 Calculations of the H2 equilibrium nationwide.	61

Table S26 | Primary energy consumption (104 ton coal-eq) in China, 2021 used for predicting the present and future H2 demand.	61

Figure S21 | Available amount of water and electricity resources for the production of electrolytic H2.	62

Figure S22 | H2 equilibrium, including H2 production and demand in provinces across China at 2021.	64

3.5 Influence of the competitive pollutants in water resources on electrolysis and the recent strategies for direct electrolysis of polluted water.	65

Table S27 | Potential competitive redox reactions in direct electrolysis of reclaimed water and their electrochemical standard potential (E0).	65

Table S28 | Current strategies for direct electrolysis of water with pollutants (e.g., raw seawater) and their specific electricity consumption for 1kg H2 production.	66

3.6 Calculations of the negative-carbon effects of electrolysis.	68

Method S6 | Calculation of anodic oxygen in electrolyzers as the by-product for energy-saving aeration in WWTPs’ aerobic tank.	68

Figure S23 | Volume of gas fed into the aerobic tank using either air or anodic oxygen.	70

4. Prediction of the electrolytic H2 production in the future.	71

4.1 Prediction of the future electrolytic H2 production.	71

4.2 Future technical level of the electrolysis and the future social development.	73

Table S29 | Prediction of technical progress in alkaline water electrolysis.	73

Table S30 | Prediction of parameters used for Kaya identity in the future.	73

4.3 Prediction framework of the future scenarios.	74

Figure S24 | Reduction schematic of national carbon emission (EMCO2) based on the Kaya identity.	74

Figure S25 | Structure of LEAP’s calculations for the prediction of future structure and cost of electricity.	74

Figure S26 | Future electricity cost of different types’ power.	75

Figure S27 | Future cost and carbon emissions of onshore and offshore electric grids.	75

4.4 Prediction of the future H2 demand, electricity prices, and the resource consumption by electrolytic H2.	76

Figure S28 | Variation of H2 production and consumption demand from 2021 to 2060.	76

Figure S29 | Consumption of water and electricity for the electrolytic H2 production from 2021 to 2060.	77

Figure S30 | Percentage of resources used for electrolytic H2 production in the corresponding available resources from 2021 to 2060.	78

Supplementary References	79

# 1. Definition of the life-cycle assessment aims and boundary.

## 1.1 The assessment aims and boundary of electrolytic hydrogen (H2) from different water resources.

### Figure S1 | Comprehensive calculation procedures employed for assessing H2 production scenarios using various electrolytic water resources. The primary input models and data encompass: (1) Models (highlighted in light green) calibrating specific water and electricity consumption for H2 production, installed capacity of alkaline water electrolyzers, decarbonization of electric grids for sustainable energy transition in China, and H2 transport based on hierarchical transport distance and amount. (2) Data (depicted in gray blue) evaluating national/provincial resources availability (including reclaimed water, tap water, desalinated seawater, and offshore wind power) for electrolytic H2 production, life-cycle inventory (accounting for chemicals, materials, and energy consumed in production) assessing environmental impacts such as carbon emissions, and economic assumptions evaluating the composite cost of electrolytic H2 from different technical routes using various water resources as per the designed scenarios (represented in dark green). Uncertainty distribution analysis is conducted on all results, based on input parameter uncertainty, using a Monte Carlo simulation with 100,000 trials. Results include H2 equilibrium considering production and demand, levelized carbon emissions and composite cost of electrolytic H2, their provincial distribution across China, and temporal variation from the present towards carbon peak at 2030 and carbon neutrality at 2060, are all analyzed based on the aforementioned procedures.

## 1.2 Life-cycle inventories (LCI) of the wastewater/water/seawater treatment, electrolysis, and H2 transport.

### Method S1 | The process of reclaimed water production in a typical municipal wastewater treatment plant.

The schematic in Figure S2, coupled with data provided in Data S1, serves as the basis for the wastewater treatment model producing reclaimed water. This model offers mathematical representations of the treatment processes. Some outcomes are subsequently employed in calculating the levelized carbon emissions and the composite cost of H2 via direct electrolyzing reclaimed water.

| Data S1 ¦ Typical water qualities of wastewater discharged into municipal wastewater treatment plants 1-4. |  |  |
| --- | --- | --- |
| Number | Items (mg/L) a | Value range (Distribution) |
| 1 | Temperature (°C) | 10-40 (triangular) c |
| 2 | Total suspended solids (TSS) | 250-400 (triangular) |
| 3 | Total dissolved solids (TDS) | 150-1000 (triangular) |
| 4 | Oil, grease | 50-100 (triangular) |
| 5 | Petroleum | 10-15 (triangular) |
| 6 | pH | 6.5-9.5 (triangular) |
| 7 | Biological oxygen demand in 5 days (BOD5) | 80-300 (triangular) |
| 8 | Chemical oxygen demand (CODCr) b | 200-500 (triangular) |
| 9 | Ammonia nitrogen (NH3-N) | 25-45 (triangular) |
| 10 | Total nitrogen (TN) | 30-70 (triangular) |
| 11 | Total phosphorus | 5-8 (triangular) |
| 12 | Anionic surfactant | 10-20 (triangular) |
| 13 | Residual chlorine | 8 (uniform) d |
| 14 | Sulfide | 1 (uniform) |
| 15 | Fluoride | 20 (uniform) |
| 16 | Chloride | 500-800 (triangular) |
| 17 | Sulfate | 400-600 (triangular) |
| 18 | Cu | 1.0-2.5 (triangular) |
| 19 | Zn | 1-5 (triangular) |
| 20 | Mn | 2-5 (triangular) |
| 21 | Fe | 5-10 (triangular) |
| 22 | Volatile phenol | 0.5-1.0 (triangular) |
| 23 | Benzene | 1.0-2.5 (triangular) |
| Notes: a, Unless otherwise specified, the default unit for listed items is mg/L. Specific units, such as (°C) for temperature, are indicated alongside the items. b, The chemical oxygen demand is determined by spectrophotometry following digestion with potassium dichromate (K2Cr2O7). c, The triangular distribution of parameters is characterized by the sequential order of minimum, baseline, and maximum values. d, The uniform distribution of parameters signifies an absence of significant fluctuations. |  |  |

For a typical medium-sized wastewater treatment plants (WWTPs), key assumptions are based on data reported in literatures 5-14, as outlined below. WWTPs are assumed to conduct primary treatment (bar screen, grit remover, primary settler), biological secondary treatment (A2O (anaerobic, anoxic, aerobic) tank, secondary settler), tertiary advanced treatment (coagulation, flocculation, pre-/post-chlorination, sand filtering, UV oxidation), and sludge treatment (centrifugation thickening, mechanical dewatering, mesophilic anaerobic digestion). WWTPs are assumed to have a 30,000-50,000 m3/day treatment capacity. Wastewater influents with hierarchical contaminants’ (total suspended solids (TSS), volatile suspended solids (VSS), biological oxygen demand in five days (BOD5), chemical oxygen demand in K2Cr2O7 index (CODCr), total nitrogen (TN), organic nitrogen (Organic-N), ammonia nitrogen (NH3-N), total phosphorus (TP)) strength from low to high level are represented in Data S1. This data represents the hierarchical quality level of wastewater discharged into municipal pipes and WWTPs. Hierarchical total removal of contaminants by sequential primary, secondary, and tertiary treatments satisfying national discharging standards are assumed based on the reported data from references (90-97% TSS, 91-98% BOD5, 80-95% CODCr, 50-88% TN, 88-97% TP). The primary settler is capable of settling 20-30% BOD5, 50-60% TSS, 20-40% CODCr, 15-25% TN, and 25-35% TP to primary sludges (70-74% VSS/TSS). The hydraulic retention time (HRT) and effective volume of anaerobic, anoxic, aerobic tanks are set at 1:1:3. Sequential tanks can remove 9-16-68% CODCr, 0-10-67% BOD5, 8-38-55% NH3-N besides generating secondary sludges (79-81% VSS/TSS) according to sludge yields (0.07-0.25 kg VSS/kg BOD5 removal in anaerobic, anoxic tanks while 0.4-0.5 kg VSS/kg BOD5 removal in aerobic tank). The reflux ratio of secondary sludges to biological tanks is set at 50-70%. Mixed sludges (75-85% VSS/TSS) including primary and secondary sludges are centrifugated to thickened sludges (5% solids content) with 85% solids capture efficiency, which is fed into the anaerobic digestion (AD) for recovery of biogas. The composition of biogas from AD is assumed with 50-75% methane (CH4), 25-50% carbon dioxide (CO2), trace H2 and H2S. This biogas can be purified by a water scrubber to obtain bio-methane or can be in-situ combusted for thermal generation. The AD digester operates at a VSS loading rate of 1.4 kg VSS m3/day, yielding 0.4-0.8 m3 biogas/kg VSS destroyed, wherein 15-20% volume for headspace biogas accumulation is reserved. Calculations of other parameters including total AD digester volume, solid retention time of sludges, biogas yields, heat consumption, yields of liquid/solid digestates after AD, yields of mixed sludges involving dewatering solid digestate, FeCl3, and polymers (polyacrylamide), are demonstrated in the following part. A mass balance involving water, solids, and gases in the above WWTPs is also provided in the following part.

Input parameters’ median values and distribution range are detailed above, with all parameter distribution patterns set as triangular. Equations S1 and S2 provide the mass flow of primary and secondary sludges.

mps=Q⋅CinfluentsTSS⋅ηprimary equation S1

mss=(1-R)⋅(∆BOD5AA⋅γAA+∆BOD5aerobic⋅γaerobic) equation S2

where the mass production of primary sludges (mps, kg/day) is determined by the treatment capacity of WWTPs (Q, 50,000 m3/day), TSS concentration in wastewater influents (CinfluentsTSS, kg/m3), and TSS removal by the primary settler (ηprimary, %). The mass production of secondary sludges (mss, kg/day) derives from the removal amounts of BOD5 in anaerobic-anoxic (∆BOD5AA, kg) and aerobic (∆BOD5aerobic, kg) tanks and the reflux ratio (R, %). Different metabolisms determine the yields of sludges in anaerobic-anoxic (γAA, kg VSS/kg BOD5) and aerobic (γaerobic, kg VSS/kg BOD5) tanks.

Equations S3-S7 provide the volume and mass flow of mixed sludges after thickening, the volume of AD digester, the SRT of mixed sludges in AD digester, the heat requirement of AD digester, and the yields of biogas from AD digester.

Vms=mmsρms=ηcapture-thickening⋅(mps+mss)ρms equation S3

VAD=(1+ηheadspace)⋅(mms⋅ηms-vss/LRms-vss) equation S4

SRTms=(mms⋅ηms-vss/LRms-vss)/Vms equation S5

HeatAD=Vms⋅ρms⋅Hmsspecific⋅(Toperation-Tambient)⋅(1+ηambient-loss) equation S6

YieldbiogasAD=mms⋅ηms-vss⋅ηvssAD, destory⋅ϵms-vss-biogas equation S7

where the mass (mms, kg/day) or volume (Vms, m3/day) of mixed sludges are determined by the solids capture efficiency of thickening (ηcapture-thickening, %) and its own density (ρms, 1030 kg/m3). The volume of AD digester (VAD, m3) and solids retention time (SRTms, days) of mixed sludges are determined by the VSS content in mixed sludges (ηms-vss, %) and the daily loading rate of AD digester (LRms-vss, kg VSS m3/day), with an additional ratio (ηheadspace, %) assumed for headspace allowing for biogas accumulation. The heat requirement for maintaining the performance of AD digester depends on the specific heat of mixed sludges (Hmsspecific, kJ/kg), the operation (Toperation, °C) and ambient temperature (Tambient, °C), and the ratio of ambient heat loss (ηambient-loss, %). The yields of biogas from AD digester (YieldbiogasAD, m3/day) depend more on the ratio of destroyed VSS (ηvssAD, destory, %) in mixed sludges during AD and the conversion factor of VSS to biogas (ϵms-vss-biogas, m3 biogas/kg VSS destroyed).

Equations S8-S13 calculate the mass and volume flow of residual solid digestates (for dewatering), dewatered digestates (for mechanical drying and transport for landfilling or pyrolysis in municipal waste incineration), and liquid digestates (for backflow into the primary settler).

Masssd=mms⋅(1-ηms-vss)+mms⋅ηms-vss⋅(1-ηms-vssAD, destory) equation S8

Masscs=Q⋅δFeCl3primary-secondary+Masssd⋅δPAM equation S9

Vos=mos/(ηsolids-os⋅ρos)=(Masssd+Masscs)(ηsolids-os⋅ρos) equation S10

Vos-dewater=mos-dewater(ηsolids-os-dewater⋅ρos-dewater) equation S11

mos-dewater=mos⋅ηcapture-dewatering equation S12

Vliquid digestate=Vos-Vos-dewater equation S13

where the mass of solid digestates after AD (Masssd, kg/day) is the sum of fixed suspended solids (FSS) and residual VSS. The mass of chemical sludges (Masscs, kg/day) includes FeCl3 added into primary and secondary settlers for phosphorus removal and polyacrylamide (PAM) added into solid digestates for enhanced dewatering, with different addition ratios set for FeCl3 (δFeCl3primary-secondary, kg/m3) and PAM (δPAM, kg/ton solid digestate). The volume of overall sludges (Vos, m3/day) containing solid digestates and chemical sludges is calculated from its mass (mos, kg/day), solids content (ηsolids-os, %), and own density (ρos, kg/m3). The volume of overall sludges after dewatering (Vos-dewater, m3/day) is determined by its mass (mos-dewater, kg/day), own density (ρos-dewater, 1060 kg/m3), and solids content (ηsolids-os-dewater, %) depending on the solids capture efficiency of dewatering (ηsolids-os-dewater, %). The volume of liquid digestates (Vliquid digestate, m3/day) derives from Vos subtracting Vos-dewater. Liquid digestates from AD effluents after dewatering return to the influents into the primary settler, while residual solid digestates are fed into the treatment of mechanical drying and transport for landfilling or pyrolysis in municipal waste incineration.

| ### Table S1 ¦ LCI of wastewater treatment plants producing 1m3 reclaimed water 5-14. |  |  |  |
| --- | --- | --- | --- |
| Wastewater (WW) collection (1 m3) |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Municipal WW collection grid | 2.80E-09 | p | Below |
| Domestic WW collection grid | 3.92E-06 | p | Below |
| Pumping stations | 2.24E-09 | p | Pump station {GLO}¦ market for ¦ Cut-off, S |
| Electricity | 0.21 | kWh | Electricity, low voltage {GLO}¦ market group for ¦ Cut-off, S |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| WW leakage to groundwater | 0.1 | m3 | Wastewater, from residence {RoW}¦ market for wastewater, from residence ¦ Cut-off, S |
| Municipal/domestic WW collection grid (1 p) |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| PVC pipes | 1.26E+03 | ton | Polyethylene, high density, granulate {RoW}¦ production ¦ Cut-off, S |
| Steel pipes | 1077 | ton | Steel, chromium steel 18/8 {RoW}¦ steel production, electric, chromium steel 18/8 ¦ Cut-off, S |
| PE pipes | 29 | ton | Ethylene, pipeline system {RoW}¦ production ¦ Cut-off, S |
| Pumping stations | 1 | p | Pump station {GLO}¦ market for ¦ Cut-off, S |
| Excavation for pipes installation | 1.80E+05 | m3 | Excavation, hydraulic digger {RER}¦ processing ¦ Cut-off, S |
| Sand for pipes padding | 4.67E+04 | ton | Sand {RoW}¦ gravel and quarry operation ¦ Cut-off, S |
| Transport of pipes | 1.18E+05 | tkm | Transport, freight, lorry 16-32 metric ton, EURO6 {RoW}¦ transport, freight, lorry 16-32 metric ton, EURO6 ¦ Cut-off, S |
| Transport of sand | 2.34E+06 | tkm | Same with above description |
| Transport of excavated soil | 2.36E+06 | tkm | Same with above description |

| Table S1 ¦ LCI of wastewater treatment plants producing 1m3 reclaimed water (continue). |  |  |  |
| --- | --- | --- | --- |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| PVC pipes to recycling | 1.26E+03 | ton | PVC (waste treatment) {GLO}¦ recycling of PVC ¦ Cut-off, S |
| Municipal/domestic WW collection grid (1 p) |  |  |  |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Cement coated steel pipes to landfill | 9.38E+03 | ton | Waste cement in concrete and mortar {RoW}¦ treatment of, collection for final disposal ¦ Cut-off, S Waste reinforcement steel {RoW}¦ treatment of waste reinforcement steel, collection for final disposal ¦ Cut-off, S |
| PE pipes to recycling | 29 | ton | PE (waste treatment) {GLO}¦ recycling of PE ¦ Cut-off, S |
| Wastewater treatment (1 m3) – primary and secondary treatments |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Collected wastewater influents | 1.04 | m3 | Above |
| Iron chloride | 64 | g | Iron (III) chloride, without water, in 40% solution state {GLO}¦ market for ¦ Cut-off, S |
| Iron sulfate | 47 | g | Iron sulfate {RoW}¦ market for iron sulfate ¦ Cut-off, S |
| Aluminum sulfate | 13 | g | Aluminium sulfate, powder {RoW}¦ market for aluminium sulfate, powder ¦ Cut-off, S |
| Transport of pre-treated waste to landfill | 10.2 | kgkm | Transport, freight, lorry 16-32 metric ton, EURO6 {RoW}¦ transport, freight, lorry 16-32 metric ton, EURO6 ¦ Cut-off, S |
| Electricity | 0.59 | kWh | Electricity, medium voltage {GLO}¦ market group for ¦ Cut-off, S |
| WWTPs (A2O as secondary treatment) | 3.74E-09 | p | Wastewater treatment facility, capacity 4.7E10l/year {GLO}¦ market for ¦ Cut-off, S |
| Wastewater treatment (1 m3) – tertiary treatment |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Aluminum sulfate | 10.8 | g | Aluminium sulfate, powder {RoW}¦ market for aluminium sulfate, powder ¦ Cut-off, S |
| UV lamps | 2.40E-05 | p | Ultraviolet lamp {GLO}¦ market for ¦ Cut-off, S |
| Transport of UV lamps | 0.0024 | kgkm | Transport, freight, lorry 16-32 metric ton, EURO6 {RoW}¦ transport, freight, lorry 16-32 metric ton, EURO6 ¦ Cut-off, S |

| Table S1 ¦ LCI of wastewater treatment plants producing 1m3 reclaimed water (continue). |  |  |  |
| --- | --- | --- | --- |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Sodium hypochlorite | 8.01E-03 | L | Sodium hypochlorite, without water, in 15% solution state {RoW}¦ market for sodium hypochlorite, without water, in 15% solution state ¦ Cut-off, S |
| Wastewater treatment (1 m3) – tertiary treatment |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Electricity | 0.197 | kWh | Electricity, medium voltage {GLO}¦ market group for ¦ Cut-off, S |
| Tertiary treatment facility | 3.91E-09 | p | Below |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Water vapor to air | 0.04 | m3 | Water/m3 |
| Solids waste to landfill | 0.014 | kg | Municipal solid waste {RoW}¦ treatment of, sanitary landfill ¦ Cut-off, S |
| Tertiary treatment facility (1 p) |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Land use | 1000 | m2a | Occupation, industrial area |
| Concrete for sand filters | 146 | m3 | Concrete, normal {GLO}¦ market group for concrete, normal ¦ Cut-off, S |
| Reinforcing steel for sand filters | 35.1 | ton | Reinforcing steel {RoW}¦ production ¦ Cut-off, S |
| Silica sand for filters media | 611 | ton | Silica sand {RoW}¦ production ¦ Cut-off, S |
| Gravel for base media of sand filters | 115 | ton | Gravel, round {RoW}¦ gravel and sand quarry operation ¦ Cut-off, S |
| Transport of building materials | 5.56E+04 | tkm | Transport, freight, lorry 16-32 metric ton, EURO6 {RoW}¦ transport, freight, lorry 16-32 metric ton, EURO6 ¦ Cut-off, S |
| Sludges treatment (1ton dry weight) – centrifugation thickening |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Electricity | 13.2 | kWh | Electricity, medium voltage {GLO}¦ market group for ¦ Cut-off, S |

| Table S1 ¦ LCI of wastewater treatment plants producing 1m3 reclaimed water (continue). |  |  |  |
| --- | --- | --- | --- |
| Sludges treatment (1ton dry weight) – anaerobic digestion |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Electricity | 89.9 | kWh | Electricity, medium voltage {GLO}¦ market group for ¦ Cut-off, S |
| Diesel | 7.5 | kg | Diesel, low-sulfur {RoW}¦ market for ¦ Cut-off, S |
| Anaerobic digestion plants | 1.00E-09 | p | Anaerobic digestion plant, for sewage sludge {GLO}¦ market for ¦ Cut-off, S |
| Sludges/solid digestate treatment (1ton dry weight) – mechanical dewatering |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Electricity | 48.8 | kWh | Electricity, medium voltage {GLO}¦ market group for ¦ Cut-off, S |
| Polyacrylamide | 3.5 | kg | Polyacrylamide {GLO}¦ market for ¦ Cut-off, S |
| Sludges/solid digestate treatment (1ton dry weight) – thermal drying |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Steam heat | 0.82 | MJ | Heat, from steam, in chemical industry {RoW}¦ market for heat, from steam, in chemical industry ¦ Cut-off, U |
| Electricity | 0.0144 | kWh | Electricity, medium voltage {GLO}¦ market group for ¦ Cut-off, S |
| Transport and landfilling (1ton dry weight) |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Sanitary landfill facility | 3.30E-06 | p | Sanitary landfill facility {GLO}¦ market for ¦ Cut-off, S |
| Transport of waste solids | 400 | tkm | Transport, freight, lorry 16-32 metric ton, EURO6 {RoW}¦ transport, freight, lorry 16-32 metric ton, EURO6 ¦ Cut-off, S |
| Notes: The LCI includes wastewater collection (unit process: 1 m3 wastewater influents discharged into WWTPs), primary-secondary-tertiary treatments (unit process: 1 m3 treated wastewater), and sludge treatment (unit process: 1 ton dry sludges or solid digestates) involving centrifugation thickening, anaerobic digestion, mechanical dewatering, thermal drying, and transport to landfilling sites. The detailed LCI amounts are scenario-based as described in Method S1. The LCI for wastewater collection and primary-secondary-tertiary treatment are based on typical WWTPs with a daily treatment capacity of 50,000-100,000 m3. The LCI for sludge and solid digestate treatment is based on a typical system with a daily treatment capacity of 15-20 tons. |  |  |  |

| ### Table S2 ¦ LCI of water treatment plants producing 1m3 tap water 15-18. |  |  |  |
| --- | --- | --- | --- |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Lake water | 8.84E-05 | m3 | Water, lake, RoW (in water) |
| River water | 9.44E-04 | m3 | Water, river, RoW (in water) |
| Activated carbon | 6.61E-07 | kg | Activated carbon, granular {GLO}¦ market for activated carbon, granular ¦ Cut-off, U |
| Aluminium sulfate | 4.02E-05 | kg | Aluminium sulfate, powder {RoW}¦ market for aluminium sulfate, powder ¦ Cut-off, U |
| Chlorine dioxide | 1.11E-07 | kg | Chlorine dioxide {RoW}¦ market for chlorine dioxide ¦ Cut-off, U |
| Chlorine | 2.00E-06 | kg | Chlorine, liquid {RoW}¦ market for chlorine, liquid ¦ Cut-off, U |
| Iron (III) sulfate | 7.39E-06 | kg | Iron (III) sulfate, without water, in 12.5% iron solution state {GLO}¦ market for ¦ Cut-off, U |
| Lime | 6.20E-06 | kg | Lime, hydrated, loose weight {RoW}¦ market for lime, hydrated, loose weight ¦ Cut-off, U |
| Oxygen | 4.13E-05 | kg | Oxygen, liquid {RoW}¦ market for ¦ Cut-off, U |
| Polyacrylamide | 1.58E-07 | kg | Polyacrylamide {GLO}¦ market for ¦ Cut-off, U |
| Silica sand | 1.50E-06 | kg | Silica sand {GLO}¦ market for ¦ Cut-off, U |
| Sodium hydroxide | 7.86E-07 | kg | Sodium hydroxide, without water, in 50% solution state {GLO}¦ market for ¦ Cut-off, U |
| Sodium hypochlorite | 4.21E-07 | kg | Sodium hypochlorite, without water, in 15% solution state {RoW}¦ market for sodium hypochlorite, without water, in 15% solution state ¦ Cut-off, U |
| Sodium silicate | 1.44E-06 | kg | Sodium silicate, solid {RoW}¦ market for sodium silicate, solid ¦ Cut-off, U |
| Sodium tripolyphosphate | 1.11E-07 | kg | Sodium tripolyphosphate {GLO}¦ market for ¦ Cut-off, U |
| Spent activated carbon | 6.41E-07 | kg | Spent activated carbon, granular {GLO}¦ spent activated carbon, granular, Recycled Content cut-off ¦ Cut-off, U |
| Tap water | 2.10E-05 | kg | Tap water {RoW}¦ market for ¦ Cut-off, U |
| Ultraviolet lamp | 1.75E-10 | p | Ultraviolet lamp {GLO}¦ market for ¦ Cut-off, U |
| Water treatment plant | 1.52 | p | Water works, capacity 1.1E+10 L/year {RoW}¦ water works construction, capacity 1.1E+10 L/year, conventional treatment ¦ Cut-off, U |
| Diesel | 1.60 | MJ | Diesel, burned in building machine {GLO}¦ market for ¦ Cut-off, U |
| Electricity | 4.79-9.27 | kWh | Electricity, medium voltage {GLO}¦ market for electricity, medium voltage ¦ Cut-off, U |

| Table S2 ¦ LCI of water treatment plants producing 1m3 tap water (continue). |  |  |  |
| --- | --- | --- | --- |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Heat | 1.78E-04 | MJ | Heat, district or industrial, natural gas {RoW}¦ market for heat, district or industrial, natural gas ¦ Cut-off, U |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Wastewater | 3.26E-05 | m3 | Wastewater, unpolluted {RoW}¦ market for wastewater, unpolluted ¦ Cut-off, U |
| Notes: The LCI of 1 m3 tap water is based on typical water treatment plants with an annual treatment capacity of 0.8-1.1E+7 m3. |  |  |  |

| ### Table S3 ¦ LCI of reverse-osmosis facilities purifying 1m3 feedstock water for electrolytic H2 production 19-26. |  |  |  |
| --- | --- | --- | --- |
| Production of 1 m3 desalinated seawater |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| RO pre-purification plant | 3.33E-10 | p | Below |
| Seawater | 1.18 | m3 | —— |
| RO membranes | 4.01E-02 | m2 | Seawater reverse osmosis module {GLO}¦ seawater reverse osmosis module production, 8-inch spiral wound, baseline ¦ Cut-off, S |
| UF (ultrafiltration) membranes for pre-treatment | 3.84E-04 | m2 | Same with above description |
| Iron sulfate | 6.70E-04 | kg | Iron sulfate {RoW}¦ production ¦ Cut-off, S |
| Sodium hydroxide | 0.0267 | kg | Sodium hydroxide, without water, in 50% solution state {RoW}¦ chlor-alkali electrolysis, membrane cell ¦ Cut-off, S |
| Sodium tripolyphosphate | 0.0033 | kg | Sodium tripolyphosphate {RoW}¦ production ¦ Cut-off, S |
| Sulfuric acid | 0.039 | kg | Sulfuric acid {RoW}¦ zinc sulfide production ¦ Cut-off, S |
| Carbon dioxide | 0.021 | kg | Carbon dioxide, in chemical industry {RoW}¦ ammonia production, steam reforming, liquid ¦ Cut-off, S |
| Sodium hypochlorite | 0.0034 | kg | Sodium hypochlorite, without water, in 15% solution state {RoW}¦ sodium hypochlorite production, product in 15% solution state ¦ Cut-off, S |
| Lime | 0.083 | kg | Lime {RoW}¦ production, milled, loose ¦ Cut-off, S |
| Electricity | 3.49 | kWh | Transport, freight, lorry 16-32 metric ton, EURO6 {RoW}¦ transport, freight, lorry 16-32 metric ton, EURO6 ¦ Cut-off, S |
| Transport of chemicals | 8.86 | kgkm | Same with above description |
| Transport of membranes | 0.032 | kgkm | Same with above description |
| Transport of waste to landfill | 5.77E-07 | kgkm | Same with above description |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| NH3 | 5.34E-12 | kg | Ammonia |
| NOx | 7.21E-12 | kg | Nitrogen oxides |
| Table S3 ¦ LCI of reverse-osmosis facilities purifying 1m3 feedstock water for electrolytic H2 production (continue). |  |  |  |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Cr | 2.25E-14 | kg | Chromium compounds |
| Production of 1 m3 desalinated seawater |  |  |  |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Ni | 1.58E-13 | kg | Nickel compounds |
| Cu | 1.54E-14 | kg | Copper compounds |
| Zn | 1.28E-13 | kg | Zinc compounds |
| Phosphorus | 2.67E-10 | kg | Phosphorus, total |
| Total organic carbon | 4.01E-10 | kg | TOC, Total Organic Carbon |
| Na | 5.07E-09 | kg | Sodium |
| Nitrogen | 6.38E-11 | kg | Nitrogen, total |
| Hg | 6.19E-16 | kg | Mercury compounds |
| Plastics to landfill | 1.15E-08 | kg | Waste polyvinylchloride product {RoW}¦ treatment of, collection for final disposal ¦ Cut-off, S |
| Other materials to landfill | 2.58E-11 | kg | Waste cement, hydrated {RoW}¦ treatment of, residual material landfill ¦ Cut-off, S |
| RO pre-purification plant (1 p) |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Land use | 2.55E+06 | m2a | Occupation, industrial area |
| Concrete | 225 | m3 | Concrete, normal {RoW}¦ concrete, all types to generic market for concrete, normal strength ¦ Cut-off, S |
| Steel | 241 | ton | Steel, low-alloyed, hot rolled {RoW}¦ production ¦ Cut-off, S |
| Pumping Stations | 1 | p | Pump station {GLO}¦ market for ¦ Cut-off, S |
| Reinforcing steel | 5.46 | ton | Reinforcing steel {RoW}¦ production ¦ Cut-off, S |
| Cast iron | 1.82 | ton | Cast iron removed by turning, primarily roughing, conventional {RoW}¦ cast iron turning, primarily roughing, conventional ¦ Cut-off, S |
| Transport of building materials | 4.18E+04 | tkm | Transport, freight, lorry 16-32 metric ton, EURO6 {RoW}¦ transport, freight, lorry 16-32 metric ton, EURO6 ¦ Cut-off, S |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Concrete to landfill | 0.541 | ton | Waste concrete {RoW}¦ treatment of, inert material landfill ¦ Cut-off, S |
| Table S3 ¦ LCI of reverse-osmosis facilities purifying 1m3 feedstock water for electrolytic H2 production (continue). |  |  |  |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Steel to landfilling | 248 | ton | Scrap steel {RoW}¦ treatment of, inert material landfill ¦ Cut-off, S |
| Notes: The detailed LCI amounts are scenario-based as described in Method S2. The LCI for the reverse-osmosis module is based on a small-scale plant with a production rate of 20 m3/h of desalinated seawater. Other water types including purified reclaimed water and purified tap water besides desalinated seawater required purification by RO facilities also follow the LCI listed herein. |  |  |  |

| ### Table S4 ¦ LCI of industrial alkaline electrolyzers producing 1kg H2 27-33. |  |  |  |
| --- | --- | --- | --- |
| Alkaline water electrolyzer’s stack |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Copper | 1.93E-04 | kg | Copper {GLO}¦ market for ¦ Cut-off, S |
| Unalloyed steel | 1.93E-02 | kg | Steel, unalloyed {GLO}¦ market for ¦ Cut-off, S |
| Nickel | 1.83E-03 | kg | Nickel, class 1 {GLO}¦ market for nickel, class 1 ¦ Cut-off, S |
| Primary Aluminum | 4.34E-05 | kg | Aluminum, primary, ingot, at plant/RNA |
| Bulk polyvinylchloride | 7.52E-05 | kg | Polyvinylchloride, bulk polymerised {GLO}¦ market for ¦ Cut-off, S |
| Tetrafluoroethylene | 7.52E-06 | kg | Tetrafluoroethylene {GLO}¦ market for ¦ Cut-off, S |
| Acrylonitrile-butadiene-styrene copolymer | 1.54E-05 | kg | Acrylonitrile-butadiene-styrene copolymer {GLO}¦ market for ¦ Cut-off, S |
| Polyphenylene sulfide | 3.28E-05 | kg | Polyphenylene sulfide {GLO}¦ market for ¦ Cut-off, S |
| Polysulfone | 2.51E-05 | kg | Polysulfone {GLO}¦ polysulfone production, for membrane filtration production ¦ Cut-off, S |
| N-methyl-2-pyrrolidone | 1.25E-04 | kg | N-methyl-2-pyrrolidone {GLO}¦ market for ¦ Cut-off, S |
| Aniline | 4.72E-06 | kg | Aniline {GLO}¦ market for ¦ Cut-off, S |
| Acetic anhydride | 5.20E-06 | kg | Acetic anhydride {GLO}¦ market for acetic anhydride ¦ Cut-off, S |
| Purified terephthalic acid | 8.48E-06 | kg | Purified terephthalic acid {GLO}¦ market for ¦ Cut-off, S |
| Nitric acid | 3.18E-06 | kg | Nitric acid, without water, in 50% solution state {GLO}¦ market for ¦ Cut-off, S |
| Hydrochloric acid | 1.25E-05 | kg | Hydrochloric acid, without water, in 30% solution state {GLO}¦ tetrafluoroethane production ¦ Cut-off, S |
| Graphite | 4.14E-05 | kg | Graphite {GLO}¦ market for ¦ Cut-off, S |
| Lubricating oil | 4.63E-08 | kg | Lubricating oil {GLO}¦ market for ¦ Cut-off, S |
| Zirconium oxide | 1.06E-04 | kg | Zirconium oxide {GLO}¦ market for ¦ Cut-off, S |
| Carbon monoxide | 1.45E-05 | kg | Carbon monoxide {RoW}¦ market for ¦ Cut-off, S |
| Decarbonised water | 1.06E-03 | kg | Water, decarbonised {RoW}¦ market for water, decarbonised ¦ Cut-off, S |
| Deionised water | 8.29E-03 | kg | Water, deionised {RoW}¦ market for water, deionised ¦ Cut-off, S |
| Industrial machine | 1.54E-08 | kg | Industrial machine, heavy, unspecified {GLO}¦ market for ¦ Cut-off, S |
| Plaster mixing | 7.52E-05 | kg | Plaster mixing {GLO}¦ market for ¦ Cut-off, S |

| Table S4 ¦ LCI of industrial alkaline electrolyzers producing 1kg H2 (continue). |  |  |  |
| --- | --- | --- | --- |
| Alkaline water electrolyzer’s stack |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Liquid nitrogen | 2.90E-04 | kg | Nitrogen, liquid {RoW}¦ air separation, cryogenic ¦ Cut-off, S |
| Potassium hydroxide | 1.90E-02 | kg | Potassium hydroxide {GLO}¦ market for ¦ Cut-off, S |
| Steam from chemical industry | 1.10E-01 | kg | Steam, in chemical industry {GLO}¦ market for ¦ Cut-off, S |
| Electricity (high-voltage) | 3.47E+00 | kJ | Electricity, high voltage {RoW}¦ market for ¦ Cut-off, S |
| District or industrial heat from combustion of natural gas | 8.48E+00 | kJ | Heat, district or industrial, natural gas {GLO}¦ market group for ¦ Cut-off, S |
| Heat from steam in chemical industry | 6.74E-02 | kJ | Heat, from steam, in chemical industry {RoW}¦ market for heat, from steam, in chemical industry ¦ Cut-off, S |
| Balance of the plants for alkaline water electrolysis |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Occupation area | 2.97E+02 | m2 | Occupation, industrial area |
| Transformation natural area | 1.49E+01 | m2 | Transformation, from grassland, natural (non-use) |
| Transformation industrial area | 1.49E+01 | m2a | Transformation, to industrial area |
| Wrought aluminum alloy | 1.00E+02 | kg | Aluminium, wrought alloy {GLO}¦ market for ¦ Cut-off, S |
| Organic chemicals | 2.00E+02 | kg | Chemical, organic {GLO}¦ market for ¦ Cut-off, S |
| Anodic class copper | 1.00E+02 | kg | Copper, anode {GLO}¦ market for copper, anode ¦ Cut-off, S |
| Electronics for control units | 1.10E+03 | kg | Electronics, for control units {GLO}¦ market for ¦ Cut-off, S |
| Granulate polypropylene | 3.00E+02 | kg | Polypropylene, granulate {GLO}¦ market for ¦ Cut-off, S |
| Chromium steel | 1.90E+03 | kg | Steel, chromium steel 18/8, hot rolled {GLO}¦ market for ¦ Cut-off, S |
| Low-alloyed steel | 4.80E+03 | kg | Steel, low-alloyed, hot rolled {GLO}¦ market for ¦ Cut-off, S |
| Normal concrete | 2.33E+00 | m3 | Concrete, normal {GLO}¦ market group for concrete, normal ¦ Cut-off, S |

Table S4 ¦ LCI of industrial alkaline electrolyzers producing 1kg H2 (continue).

Notes: This LCI excludes the consumption of electricity and water for electrolysis. The detailed LCI amounts are scenario-based as described in Method S3 and Method S4. The installed capacity listed in this table is derived from the results presented in Table S8.

| ### Table S5 ¦ LCI of pipes and ships transporting 1tkm H2 18. |  |  |  |
| --- | --- | --- | --- |
| 1 km H2 transport pipe |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Construction land | 3330 | m2a | Occupation, construction site |
| Industrial land | 49.7 | m2a | Occupation, industrial area |
| Land transformation | 2000 | m2 | Transformation, from forest, unspecified |
|  | 2.49 | m2 | Transformation, from unspecified |
|  | 2000 | m2 | Transformation, to annual crop |
|  | 2.49 | m2 | Transformation, to industrial area |
| Hall | 0.2 | m2 | Building, hall, steel construction {GLO}¦ market for ¦ Cut-off, U |
| Multi-storey | 16 | m3 | Building, multi-storey {GLO}¦ market for ¦ Cut-off, U |
| Cast iron | 343.14 | kg | Cast iron {GLO}¦ market for ¦ Cut-off, U |
| Cement | 2824.52 | kg | Cement, unspecified {RoW}¦ market for cement, unspecified ¦ Cut-off, U |
| Concrete | 1.22 | m3 | Concrete, normal {RNA}¦ market for concrete, normal ¦ Cut-off, U |
| Drawing of pipe | 13988.27 | kg | Drawing of pipe, steel {GLO}¦ market for ¦ Cut-off, U |
| Hydraulic digger | 1200 | m3 | Excavation, hydraulic digger {GLO}¦ market for ¦ Cut-off, U |
| Skid-steer loader | 19000 | m3 | Excavation, skid-steer loader {GLO}¦ market for ¦ Cut-off, U |
| Pitch | 989.94 | kg | Pitch {RoW}¦ market for pitch ¦ Cut-off, U |
| Polyethylene (high density) | 2377.79 | kg | Polyethylene, high density, granulate {GLO}¦ market for ¦ Cut-off, U |
| Polyethylene (low density) | 760.79 | kg | Polyethylene, low density, granulate {GLO}¦ market for ¦ Cut-off, U |
| Reinforcing steel | 13682.39 | kg | Reinforcing steel {GLO}¦ market for ¦ Cut-off, U |
| Transport | 10.35 | hr | Transport, helicopter {GLO}¦ market for ¦ Cut-off, U |
| Transport | 4.14 | p | Transport, helicopter, LTO cycle {GLO}¦ market for ¦ Cut-off, U |
| Transport | 8.07 | km | Transport, passenger car {RoW}¦ market for ¦ Cut-off, U |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Decommissioned pipeline | 7073.10 | kg | Decommissioned pipeline, natural gas {RoW}¦ market for decommissioned pipeline, natural gas ¦ Cut-off, U |
| Waste bitumen | 629.83 | kg | Waste bitumen {RoW}¦ market for waste bitumen ¦ Cut-off, U |
| Waste concrete | 3276 | kg | Waste concrete, not reinforced {RoW}¦ market for waste concrete, not reinforced ¦ Cut-off, U |
| Waste plastic | 1351.66 | kg | Waste plastic, mixture {RoW}¦ market for waste plastic, mixture ¦ Cut-off, U |
| Table S5 ¦ LCI of pipes and ships transporting 1tkm H2 (continue). |  |  |  |
| 1 km H2 transport pipe |  |  |  |
| Outputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Waste reinforcement steel | 10209.63 | kg | Waste reinforcement steel {RoW}¦ market for waste reinforcement steel ¦ Cut-off, U |
| 1 tkm H2 transport by ship |  |  |  |
| Inputs | Amount per unit process | Unit | Description in Ecoinvent database |
| Freezing operation | 1.60 | kg day | Operation, reefer, freezing {GLO}¦ market for ¦ Cut-off, U |
| Transport | 1 | tkm | Transport, freight, sea, container ship {GLO}¦ market for transport, freight, sea, container ship ¦ Cut-off, U |
| Notes: The unit process for H2 transport pipes is 1 km, and for H2 transport ships, it is 1 tkm. The LCI of H2 transport pipes is adapted from that of natural-gas pipes. The ship used for H2 transport is equipped with cooling facilities to maintain the compressed state of H2 in the ship’s storage tank. |  |  |  |

| ### Table S6 ¦ Assumptions for pipes and ships transporting H2 with different amounts, and delivery distances. |  |  |
| --- | --- | --- |
| Items | Transport methods |  |
|  | Pipes | Ships |
| Pipes’ length (km) | 100-1000-4000 a | —— |
| Pipes’ transport amount (104 ton/year) | 500-800-1500 b | —— |
| Pipes’ lifespan (year) | 20-30-40 c | —— |
| Shipping length (km) | —— | 50-100-200 d |
| Shipping transport amount (104 ton/year) | —— | 500-800-1500 b |
| Notes: a, The length of the pipes is determined by the geographical distance range between different provinces. b, The transport amount of the pipes is determined based on the calculation of H2 equilibrium as shown in Figure S22. c, The lifespan of the pipes is determined based on the current technical maturity of high-pressure gas transport. d, The shipping length is determined based on the average distance between offshore wind power plants and coastal ports. |  |  |

### Figure S2 | Schematic of typical wastewater treatment plants producing reclaimed water. Key processes include (i) primary treatment by bar screen, grit removal, and primary settler to eliminate most bulk solid waste and insoluble substances. (ii) Secondary treatment by a biological tank with A2O (sequential anaerobic-anoxic-aerobic processes) technique and secondary settler to remove most soluble pollutants, especially for thorough removal of nitrogen and phosphorus elements. (iii) Advanced tertiary treatment by coagulation, flocculation, filtering, advanced oxidation, and chlorination. (iv) Residual sludges from the side streams are thickened, dewatered, and finally transported to disposal sites. Arrows of different colors exhibit separate matter flows.

### Figure S3 | Schematic of typical water treatment plants producing tap water. Key processes include pre-treatment by flashing mixing with polymers for coagulation and sedimentation of total dissolved solids, filtering, and disinfection treatment, which produce the purified non-potable/tap water into the distribution pipelines to the customer end-use. Sludges from the side streams are thickened, dewatered, and finally transported to disposal sites. Arrows of different colors exhibit separate matter flows.

### Figure S4 | Schematic of a 3-stage reverse-osmosis system equipped with zero-liquid discharging polishing for the purification of feedstock water containing various pollutants, primarily salt ions. The feedstock water is propelled into the first stage of the reverse-osmosis module by a high-pressure pump. The brine from the residual flows of the first stage reverse-osmosis module is subsequently fed into the next stage of the reverse-osmosis module to enhance the overall water recovery ratio. This process employs 3 stages of reverse-osmosis modules for optimal water recovery. All permeate water from the 3 stages of the reverse-osmosis module is transported to the end-use, including water treatment plants, distributing pipelines, or in-situ end-use such as water electrolyzers. The final residual brine from these stages of the reverse-osmosis module is concentrated and crystallized by thermal evaporation into solid waste for harmless disposal. Arrows of different colors exhibit separate matter flows.

### Figure S5 | Schematic of an industrial alkaline electrolyzer. The schematic describes the demonstration in Table S8. Key modules include the supply of feedstock water, KOH lye as the addition into electrolyte, cooling water in the circulating chamber of electrolysis stacks to avoid overloading, gas separator for purification of anodic oxygen and cathodic H2, and electrolysis rectifier calibrating the alternating current (AC) electricity from grids into direct current (DC) electricity. Arrows of different colors exhibit separate flows of matters/energy.

### Figure S6 | Mechanism and configuration of alkaline electrolyzers’ stacks. a, Mechanisms of alkaline electrolysis, where hydroxide ion, as the ions’ carrier, transfers across the diaphragm (anion-exchange membrane) driven by the applied voltage between the anode producing oxygen (left, linked to the positive pole of the power source) and the cathode producing H2 (right, linked to the negative pole of the power source). b, Stacks’ configuration of the alkaline electrolyzer containing 4 series’ chambers in parallel connection to the power source, where each parallel contains 210 single chambers in series connection. The number of parallel series and series chambers are determined as the demonstration in Table S8, which also determines the installed power capacity of the electrolyzer.

### Figure S7 | Cost of H2 transport by pipes (gaseous) and ships (liquid) in hierarchical transport distance (200-3000 km) and transport amount. Data is obtained based on the basic scenario reported by the International Energy Agency 34,35. The left axis is displayed with a linear function.

## 1.3 Calibrations of the water and electricity consumption by electrolytic H2 production.

| ### Table S7 ¦ Calibrations of the water and electricity consumption by RO facilities 36. a |  |  |
| --- | --- | --- |
| Parameters Description (Abbreviation) | Unit (range) | Definition of Calculations |
| Concentration of TDS in Seawater (CTDS-sw) | mg/L | Conversion of Units: 1 atm = 1.01325 bar 1 L = 1/1000 m3 1 kg = 1000 g = 1000,000 mg % = 1/100 dimensionless unit Marked Abbreviation: sw (Seawater) Calculation of Parameters: (1) i = αn+(1-α) = 1+α(n-1) The index of van't Hoff (i) is determined based on (1), where dissociation of solute (α) is the proportion of solute molecules dissolved in solution, n is the number of ions obtained by dissociation of solute. α is assumed as 1 since NaCl is assumed as the only solute in feedstocks (seawater). Value of n is 2 based on the subsequent ionization of NaCl in water as shown in (2): (2) NaCl⇋Na++Cl- (3) OsPj = i × RUGC × (Tj + 273) × CM-NaCl-j (4) CM-NaCl-j = CTDS-j / MNaCl where the subscript of j in (3) and (4) represents parameters from the seawater. |
| Temperature of Seawater (Tsw) | °C |  |
| van't Hoff Index (NaCl as TDS) (i) | —— (2) |  |
| Molar Mass of NaCl (MNaCl) | —— (58.44) |  |
| Universal Gas Constant (RUGC) | L·atm/(mol·K) (0.082) |  |
| Osmotic Pressure of Seawater (OsPsw) | bar |  |
| Operation Pressure of Commercial RO Module (OpPRO) | bar |  |
| Pressure Drop across RO Memebrane (Pdrop-a-mem) | bar |  |
| Capacity Factor of RO Facilities (CFRO) | (0.75-0.90) |  |
| Back Pressure of RO Membrane from Permeates (Pback-permeate) | bar |  |
| Actual Pressure Differences in RO Treating Seawater (Pactual diff-sw) | bar |  |
| Ideal Water Recovery of 3-Stage RO Module (RecoverStage-3-RO-Idea) | % |  |
| Actual Water Recovery of 3-Stage RO Module Treating Seawater (RecoverRO-3-sw) | % |  |
| Rate of Water Production by 3-Stage RO Module (RateStage-3-RO) | m3/day |  |
| Rate of Feedstock of Seawater into 3-Stage RO Module (RateFeed-sw-3-RO) | m3/day |  |
| Specific Seawater Consumption by 3-Stage RO Module (SCsw-3-RO) | m3 SW/m3 effluents |  |
| Rate of Brine from Seawater into 3-Stage RO Module (RateBrine-sw-3-RO) | m3/day |  |
| Efficiency of High-pressure Pump (ηpump) | % |  |
| Efficiency of Mortor(ηmotor) | % |  |
| Specific Electricity Consumption of 3-Stage RO Treating Seawater (SECsw-3-RO) | kWh/m3 |  |
| Concentration of TDS in Permeates from RO Module (CTDS-product) | mg/L |  |
| Rate of Salt Generation of 3-Stage RO Treating Seawater (RateSalt-sw-3-RO) | kg/day |  |
| Specific Electricity Consumption by Brine Concentrator (SECBC) | kWh/m3 brine |  |
| Specific Electricity Consumption by Brine Crystallizer (SECBCr) | kWh/m3 concentrated brine |  |

Table S7 ¦ Calibrations of the water and electricity consumption by RO facilities (continue).

Notes: RO element type is Eco Pro 400i as shown in Table S23, other parameters can be given by the following equations (1-1) to (1-9): (1-1) Pactual diff-j = OpPRO - (OsPj + Pdrop-a-mem + 0.5 × Pback-permeate) (1-2) RecoverRO-3-j = (Pactual diff-j / OpPRO) × RecoverStage-3-RO-Idea (1-3) RateStage-3-RO = 6 (RO Stacks) × 4 (Pressure Vessels) × 6 (RO Membrane Element) × 43 (Daily Production per RO Element, m3/day) × CFRO (1-4) RateFeed-j-3-RO = RateStage-3-RO / RecoverRO-3-j (1-5) RateBrine-LLs-3-RO = RateFeed-j-3-RO - RateStage-3-RO (1-6) RateSalt-j-3-RO = (RateFeed-j-3-RO × CTDS-j) - (RateStage-3-RO × CTDS-product) (1-7) SECj-3-RO = RateFeed-j-3-RO × OpPRO / (ηpump × ηmotor × 36 × RateStage-3-RO) + (SECBC + SECBCr) × RateBrine-sw-3-RO (1-8) SCj-3-RO = RateFeed-j-3-RO / RateStage-3-RO where the subscript of j in (1-1) to (1-8) represents parameters from seawater. Equation (1-2) calculating actual recovery of RO module is obtained based on the law that water flux (Jw) transporting through the unit area of RO membrane per unit time (L/(m2·h)) is proportional to the pressure difference across membranes (Pactual diff-j, bar), which can be exhibited as follows: (1-9) Jw = KA × ΔP = KA × Pactual diff-j where KA (~1.23×10-11 m/(Pa·s)) is the transport coefficient of water across RO membrane. Notes: a, This table pertains to desalinated seawater treated by a 3-stage reverse osmosis process.

| ### Table S8 ¦ Calibration of the installed capacity of industrial alkaline electrolyzer determining the LCI of electrolyzer 37,38. |  |  |
| --- | --- | --- |
| Parameters Description (Abbreviation) | Unit (range) | Definition of Calculations |
| Number of Transported Electrons during Water Electrolysis (NTE-WE) | — (2) a | Conversion of Units: 1 L = 1 / 1000 m3 1 kg = 1000 g = 1000,000 mg % = 1 / 100 dimensionless unit 1 h = 3600 s 1 day = 24 h 1 year = 365 days 1 MW = 1 kW 1 m3 water = 1000 kg water Water Electrolysis: AWE anode: 2OH-→H2O+0.5O2+2e- AWE cathode: 2H2O+2e-→H2+2OH- Marked Abbreviation: AWE (alkaline water electrolysis) |
| Quantity of Electric Chargs per Electron (QECE) | C/mol (96485) |  |
| Volume of 1 mol H2 at Standard Condition (VH2-Std) | Nm3 (0.0224) |  |
| Electricity Demand for 1 Nm3 H2 Production by Electrolysis at Standard Condition (EDH2-Std-WE) | A·h/Nm3 H2 (2392.98) |  |
| Radius of Avtive Areas for AWE (RAA-AWE) | m (1) |  |
| Current Density of AWE (CDAWE) | A/m2 (2000-5000) |  |
| Total Electric Current in AWE Stacks (TECAWE) | A |  |
| H2 Production from per Single Chamber in AWE Stacks (PH2-SC-AWE) | Nm3/h |  |
| Net Production Rate of AWE per Set (NPRAWE) | Nm3/h (2400-3880) |  |
| Ratio of Safe Margin of Single Chamber for AWE (RSMSC-AWE) | % (8-12) |  |
| Number of Required Single Chamber in AWE Stacks (NR-SC-AWE) | p |  |
| Number of Parallel Lines for AWE (NPL-AWE) | — (4) |  |
| Number of Single Chamber in AWE Stacks in per Parallel Line (NSC-AWE-PL) | p |  |
| Applied Voltage of AWE Stacks (AVAWE) | V (1.8-2.4) |  |
| Total Voltage of AWE Stacks (TVAWE) | V |  |
| Total Current of AWE Stacks (TCAWE) | A |  |
| Total Power of AWE Stacks (TPAWE) | MW |  |
| Notes: The number of single chambers in AWE (alkaline water electrolysis) stacks per parallel line is rounded up to the nearest integer based on the principle of numerical approximation. The total power capacity of AWE stacks can be calculated using equations (1-1) to (1-8): (1-1) EDH2-Std-WE = (NTE-WE × QECE / 3600) / VH2-Std (1-2) TECAWE = CDAWE × π × RAA-AWE^2 (1-3) PH2-SC-AWE = TECAWE × t / EDH2-Std-WE, t = 1 h (1-4) NR-SC-AWE = (NPRAWE / PH2-SC-AWE) × (1 + RSMSC-AWE) (1-5) NSC-AWE-PL = NR-SC-AWE / NPL-AWE (1-6) TVAWE = NSC-AWE-PL × AVAWE (1-7) TCAWE = TECAWE × NPL-AWE (1-8) TPAWE = TVAWE × TCAWE Notes: a, The number of electrons transported during water electrolysis is determined by the reactions outlined in the calculation definitions. |  |  |

| ### Table S9 ¦ Calibration of the water and electricity consumption by electrolyzers electrolyzing water with pollutants. |  |
| --- | --- |
| Parameters Description (Abbreviation) | Unit (range) |
| Decrease Ratio of Current Density of Electrolysis using Simulated Seawater (DRCD-SSW) a | % (7-15) |
| Concentration of TDS in Simulated Seawater (CTDS-SSW) | mg/L (29.22) |
| Concentration of TDS in Reclaimed Water (CTDS-RW) | mg/L |
| Concentration of CODCr in Reclaimed Water (CCOD-RW) | mg/L |
| Concentration of TN in Reclaimed Water (CTN-RW) | mg/L |
| Concentration of NH4-N in Reclaimed Water (CNH4-N-RW) | mg/L |
| Concentration of NO3-N in Reclaimed Water (CNO3-N-RW) | mg/L |
| Concentration of Cl- in Reclaimed Water (CCl-RW) | mg/L |
| Concentration of Protein in Reclaimed Water (CProtein-RW) | mg/L |
| Concentration of Carbohydrate in Reclaimed Water (Ccarbohydrate-RW) | mg/L |
| Concentration of Organic Acids in Reclaimed Water (CO-acids-RW) | mg/L |
| Basic Electricity Consumption by AWE (BECAWE) 34,35 | kWh/kg H2 (42.27-48.94) |
| Basic Water Consumption by AWE (BWCAWE) 34,35 | kg H2O/kg H2 (9-13) |
| Specific Electricity Consumption by AWE using Reclaimed Water (SECAWE) | kWh/kg H2 |
| Specific Water Consumption by AWE using Reclaimed Water (SWCAWE) | kg H2O/kg H2 |
| Predicted Decrease Ratio of Current Density of Electrolysis using Reclaimed Water (PDRCD-RW) | % |
| Notes: a, Reference values are set based on previous research using seawater as feedstock water for calibrating electricity and water consumption when reclaimed water is used as feedstock water. Definition of calculations: (1-1) PDRCD-RW=DRCD-SSW×(i=1nCi)/CTDS-SSW (1-2) SECAWE = BECAWE × (1 + PDRCD-RW) (1-3) SWCAWE = BWCAWE × (1 + PDRCD-RW) where the subscript i represents different pollutants including TDS (total dissolved solids), CODCr (chemical oxygen demand by indexes of potassium permanganate), TN (total nitrogen), NH4-N (ammonia nitrogen), NO3-N (nitrate nitrogen), Cl-, protein, carbohydrate, and organic acids as shown in Table S9. This calculation strategy can also be applied when seawater is directly electrolyzed in an alkaline electrolyzer. The pollutant concentrations of typical seawater can be found in Table S13. |  |

## 1.4 Water qualities used for calibrating the efficiency of electrolysis and water treatment.

| ### Table S10 ¦ Typical qualities of reclaimed water in China 39. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number | Items (mg/L) a | Grade 1 standards |  | Grade 2 standards | Grade 3 standards |
|  |  | Level A | Level B |  |  |
| 1 | Chemical oxygen demand (CODCr) b | 50 | 60 | 100 | 120 c |
| 2 | Biological oxygen demand in 5 days (BOD5) | 10 | 20 | 30 | 60 d |
| 3 | Total Suspended solids (TSS) | 10 | 20 | 30 | 50 |
| 4 | Oil, grease | 1 | 3 | 5 | 20 |
| 5 | Petroleum | 1 | 3 | 5 | 15 |
| 6 | Total nitrogen (TN) | 15 | 20 | —— | —— |
| 7 | Ammonia nitrogen (NH3-N) | 5 (8) e | 8 (15) | 25 (30) | —— |
| 8 | Total phosphorus | 1 (0.5) f | 1.5 (1) | 3 (3) | 5 (5) |
| 9 | pH | 6-9 |  |  |  |
| Notes: The limits of water quality indexes are daily mean values. a, The default unit for listed items is mg/L, with specific units, such as (MPN/L) for the number of fecal coliforms, indicated alongside the items. b, The chemical oxygen demand is determined by spectrophotometry following digestion with potassium dichromate (K2Cr2O7). c, When the CODCr of wastewater exceeds 350 mg/L, the removal ratio of CODCr should exceed 60%. d, When the BOD5 of wastewater exceeds 160 mg/L, the removal ratio of BOD5 should exceed 50%. e, The value outside the bracket is the index limit at temperatures above 12 °C, while the value inside the bracket is the index limit at temperatures below 12 °C. f, The value outside the bracket is the index limit for wastewater treatment plants (WWTPs) built before 31.12.2005, while the value inside the bracket is the index limit for WWTPs built after 01.01.2006. |  |  |  |  |  |

| ### Table S11 ¦ Typical qualities of terrestrial freshwater in China 40. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number | Items (mg/L) a | Classification of water quality |  |  |  |  |
|  |  | Level I | Level II | Level III | Level IV | Level V |
| 1 | Temperature (°C) | Maximum weekly increase≤1 maximum weekly decrease≤2 |  |  |  |  |
| 2 | pH | 6-9 |  |  |  |  |
| 3 | Dissolved oxygen, ≥ | 7.5 | 6 | 5 | 3 | 2 |
| 4 | Chemical oxygen demand (CODMn) b, ≤ d | 2 | 4 | 6 | 10 | 15 |
| 5 | Chemical oxygen demand (CODCr) c, ≤ | 15 | 15 | 20 | 30 | 40 |
| 6 | Biological oxygen demand in 5 days (BOD5), ≤ | 3 | 3 | 4 | 6 | 10 |
| 7 | Ammonia nitrogen (NH3-N), ≤ | 0.15 | 0.5 | 1.0 | 1.5 | 2.0 |
| 8 | Total phosphorus, ≤ | 0.02 (0.01) e | 0.1 (0.025) | 0.2 (0.05) | 0.3 (0.1) | 0.4 (0.2) |
| 9 | Total nitrogen (TN), ≤ | 0.2 | 0.5 | 1.0 | 1.5 | 2.0 |
| 10 | Cu, ≤ | 0.01 | 1.0 | 1.0 | 1.0 | 1.0 |
| 11 | Zn, ≤ | 0.05 | 1.0 | 1.0 | 2.0 | 2.0 |
| 12 | Fluoride (based on F-), ≤ | 1.0 | 1.0 | 1.0 | 1.5 | 1.5 |
| 13 | Volatile phenol, ≤ | 0.002 | 0.002 | 0.005 | 0.01 | 0.1 |
| 14 | Petroleum, ≤ | 0.05 | 0.05 | 0.05 | 0.5 | 1.0 |
| 15 | Sulfide, ≤ | 0.05 | 0.1 | 0.2 | 0.5 | 1.0 |
| 16 | Sulfate (based on SO42-), ≤ | 250 |  |  |  |  |
| 17 | Chloride (based on Cl-), ≤ | 250 |  |  |  |  |
| 18 | Nitrate (based on N), ≤ | 10 |  |  |  |  |
| 19 | Fe, ≤ | 0.3 |  |  |  |  |
| 20 | Mn, ≤ | 0.1 |  |  |  |  |
| Notes: a, The default unit for listed items is mg/L, with specific units, such as (°C) for temperature, indicated alongside the items. b,c, The chemical oxygen demand is determined by spectrophotometry following digestion with either potassium permanganate (KMnO4, CODMn) or potassium dichromate (K2Cr2O7, CODCr). d, The label of ≤ indicates that water quality indexes should not exceed the provided values in Table S11. e, The value outside the bracket is the index limit for common freshwater, while the value inside the bracket is the index limit for freshwater in lakes or reservoirs. |  |  |  |  |  |  |

| ### Table S12 ¦ Typical qualities of tap water in China 41. |  |  |
| --- | --- | --- |
| Number | Items (mg/L) a | Limits |
| 1 | Fluoride | 1.0 |
| 2 | Nitrate | 10 |
| 3 | Bromate | 0.01 |
| 4 | Chlorite | 0.7 |
| 5 | Chlorate | 0.7 |
| 6 | pH | 6.5-8.5 |
| 7 | Al | 0.2 |
| 8 | Fe | 0.3 |
| 9 | Mn | 0.1 |
| 10 | Cu | 1.0 |
| 11 | Zn | 1.0 |
| 12 | Chloride | 250 |
| 13 | Sulfate (based on SO42-) | 250 |
| 14 | Total dissolved solids (TDS) | 1000 |
| 15 | Hardness (based on CaCO3) | 450 |
| 16 | Chemical oxygen demand (CODMn) b | 3 |
| 17 | Ammonia nitrogen (NH3-N) | 0.5 |
| 18 | Na | 200 |
| 19 | Volatile phenol | 0.002 |
| Notes: a, The default unit for listed items is mg/L, with specific units, such as (NTU) for turbidity, indicated alongside the items. b, The chemical oxygen demand is determined by spectrophotometry following digestion with potassium permanganate (KMnO4, CODMn). |  |  |

| ### Table S13 ¦ Typical qualities of inshore seawater in China 42. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number | Items (mg/L) a | Class I | Class II | Class III | Class IV |
| 3 | Total suspended solids (TDS) b, ≤ c | 10 | 10 | 100 | 150 |
| 7 | Temperature | ≤1°C (summer), ≤2°C (other seasons) |  | ≤4°C based on local temperature |  |
| 8 | pH | 7.8-8.5 |  | 6.8-8.8 |  |
| 9 | Dissolved oxygen, > d | 6 | 5 | 4 | 3 |
| 10 | Chemical oxygen demand (CODCr) e, ≤ | 2 | 3 | 4 | 5 |
| 11 | Biological oxygen demand in 5 days (BOD5), ≤ | 1 | 3 | 4 | 5 |
| 12 | Inorganic nitrogen, ≤ | 0.20 | 0.30 | 0.40 | 0.50 |
| 13 | Nonionic ammonia, ≤ | 0.020 |  |  |  |
| 14 | Active phosphate, ≤ | 0.015 | 0.030 |  | 0.045 |
| 21 | Cu, ≤ | 0.005 | 0.010 | 0.050 |  |
| 22 | Zn, ≤ | 0.020 | 0.050 | 0.10 | 0.50 |
| 26 | Sulfide, ≤ | 0.02 | 0.05 | 0.10 | 0.25 |
| 27 | Volatile phenol, ≤ | 0.005 |  | 0.010 | 0.050 |
| 28 | Petroleum, ≤ | 0.05 |  | 0.30 | 0.50 |
| Notes: Seawater with quality lower than class IV can be defined as the inferior class IV. a, The default unit for listed items is mg/L, with specific units, such as (MPN/L) for the total Coliform, indicated alongside the items. b, The value is on the matter with artificially added into the seawater. c, The label ‘≤’ indicates that water quality indexes should not exceed the provided values in Table S13. d, The label ‘>’ indicates that water quality indexes should exceed the provided values in Table S13. e, The chemical oxygen demand is determined by the spectrophotometry via the digestion by potassium dichromate (K2Cr2O7). |  |  |  |  |  |

## 1.5 Economic assumptions of the assessment projects and weighting of the power grids.

| ### Table S14 ¦ Major economic assumptions evaluating the composite cost of H2 production. |  |  |
| --- | --- | --- |
| Items (description of parameters, abbreviation) | Unit | Numerical range |
| Electricity prices (Pelec) | 2021$/kWh | equation 6 |
| Change of US dollars into CNY (CUSD-CNY) | ￥/2021 $ | 6.4515 a |
| Depreciation rate of fixed assets (DRfa) | % | 4-6-8 |
| Ratio of annual cost of operation and maintenance in CAPEX units (RACO&M-CAPEX) b | % | 3-5-8 |
| Ratio of annual cost of staffs' salary in CAPEX units (RACSS-CAPEX) | % | 1-3-5 |
| Carbon tax at 2021 (Taxcarbon) | 2021 $/kg CO2-eq | 7.98±2.97 c |
| H2 transport cost by pipes (Ptrans-pipes) | 2021 $/tkm | Figure S7 |
| H2 transport cost by ships (Ptrans-ships) | 2021 $/tkm | Figure S7 |
| Capital expenditure of wastewater treatment plants (CAPEXWWTPs) | 2021 $/ton capacity | 235.1-236.9-238.3 |
| Capital expenditure of anaerobic digester (CAPEXAD) | 2021 $/ton dry sludge | 22.8-34.7-46.6 |
| Life-span of WWTPs and BoPs and AD facilities (LSWWTPs-AD) | years | 20-30-40 |
| Specific cost per RO element (37 m2 active area) (SCRO-E) | 2021 $/element | 370 |
| Specific cost per pressure vessel (SCvessel) | 2021 $/vessel | 1150 |
| Specific cost of other RO equipment (BoPs) per train (SCBoP-RO-Train) | 2021 $/train (4000 production-m3/day) | 940000 |
| Rate of annual rplacement of RO eement (RRO-year-replace) | % | 13.1-14.3-15.5 |
| Specific construction cost of concentrate disposal facility (SCCCDF) | 2021 $/(m3·day) | 77.5 |
| Specific construction cost of waste and solid handling facility (SCCWSHF) | 2021 $/(m3·day) | 100 |
| Specific cost for disposal of waste streams from 3-stage RO (SCDWS-RO) | 2021 $/(m3·day) | 0.025 |
| Life-span of RO and BoPs and facilities treating wastes (LSRO-BoPs-Waste) | years | 20-25-30 |
| Specific constructure cost of AWE and BoPs at 2021 (SCCAWE-Pre) | 2021 $/kW | 500-1100-1400 |
| Life-span of AWE Stacks (LSAWE) | years | 8-10-15 |
| Levelized Treatment Cost of Tap Water from water treatment plants (LTCTW-WTPs) | $/m3 | 0.337-0.472-0.752 d |

Table S14 ¦ Major economic assumptions evaluating the composite cost of H2 production (continue).

Notes: a, The value represents the annual mean change in 2021. b, CAPEX denotes capital expenditure. c, The value represents the mean ± standard deviation (5-95% confidence interval) across national carbon markets in China at 2021. d, The value is displayed as minimum-median-maximum and represents the average prices for industrial water supply in China’s provinces.

| ### Table S15 ¦ Structure of power grids in China, 2021 43. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Province | Power generation (108 kWh) a |  |  |  |  |  |
|  | Thermal power | Nuclear power | Wind power | Solar power | Hydro power | Total power |
| Beijing | 448.68 |  | 4.01 | 6.23 | 13.66 | 472.58 |
| Tianjin | 761.85 |  | 17.69 | 19.98 | 0.21 | 799.73 |
| Hebei | 2698.74 |  | 511.36 | 279.32 | 24.00 | 3513.42 |
| Shanxi | 3229.00 |  | 469.11 | 189.5 | 38.54 | 3926.15 |
| Inner Mongolia | 4891.80 |  | 967.21 | 211.9 | 49.02 | 6119.93 |
| Liaoning | 1496.78 | 400.07 | 227.22 | 55.09 | 78.42 | 2257.58 |
| Jilin | 730.69 |  | 137.94 | 52.26 | 104.86 | 1025.75 |
| Heilongjiang | 948.66 |  | 161.55 | 51.2 | 39.13 | 1200.54 |
| Shanghai | 969.37 |  | 18.34 | 15.35 |  | 1003.06 |
| Jiangsu | 4841.31 | 485.16 | 415.66 | 195.32 | 31.44 | 5968.89 |
| Zhejiang | 3048.54 | 732.74 | 48.96 | 154.57 | 237.68 | 4222.49 |
| Anhui | 2741.26 |  | 106.91 | 154.5 | 80.72 | 3083.39 |
| Fujian | 1722.42 | 777.2 | 151.87 | 25.04 | 274.28 | 2950.81 |
| Jiangxi | 1243.82 |  | 103.59 | 80.31 | 135.55 | 1563.27 |
| Shandong | 5281.04 | 197.08 | 409.39 | 310.45 | 12.36 | 6210.32 |
| Henan | 2458.77 |  | 328.32 | 135.69 | 116.29 | 3039.07 |
| Hubei | 1476.01 |  | 134.33 | 83.14 | 1598.89 | 3292.37 |
| Hunan | 1016.70 |  | 149.64 | 37.93 | 537.59 | 1741.86 |
| Guangdong | 4638.57 | 1204.1 | 136.51 | 102.64 | 224.41 | 6306.23 |
| Guangxi | 1194.62 | 181.26 | 160.62 | 28.08 | 517.36 | 2081.94 |
| Hainan | 254.39 | 97.63 | 5.04 | 16.1 | 18.07 | 391.23 |
| Chongqing | 681.21 |  | 22.61 | 4.83 | 282.78 | 991.43 |
| Sichuan | 666.79 |  | 109.43 | 29.65 | 3724.46 | 4530.33 |
| Guizhou | 1446.58 |  | 104.77 | 82.59 | 734.46 | 2368.40 |
| Yunnan | 455.46 |  | 235.48 | 51.14 | 3028.16 | 3770.24 |
| Shaanxi | 2281.76 |  | 175.93 | 140.71 | 141.35 | 2739.75 |
| Gansu | 1006.78 |  | 288.44 | 149.8 | 451.80 | 1896.82 |
| Ningxia | 1597.68 |  | 281.16 | 183.33 | 20.72 | 2082.89 |
| Notes: a, The statistic power generation refers to the electricity generated and linked to the national grids (national grids and south grids). |  |  |  |  |  |  |

| ### Table S16 ¦ Feed-in tariff prices and transmission prices of electricity in China, 2021 44. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provinces | Feed-in tariff prices ($/kWh) |  |  |  |  | Electric transmission prices ($/kWh) |  |  |
|  | Thermal power | Nuclear power | Wind power | Solar power | Hydro power | 35 kV | 110 kV | ≥220 kV |
| Beijing | 0.053-0.070 |  | 0.076 | 0.124 | 0.038 | 0.030 | 0.024 | 0.022 |
| Tianjin | 0.051-0.087 |  | 0.075 | 0.124 | 0.038 | 0.021 | 0.019 | 0.016 |
| Hebei | 0.046 |  | 0.067 | 0.124 | 0.038 | 0.019 | 0.016 | 0.013 |
| Shanxi | 0.046-0.100 |  | 0.088 | 0.133 | 0.049 | 0.011 | 0.007 | 0.004 |
| Inner Mongolia | 0.039-0.105 |  | 0.064 | 0.116 | 0.044 | 0.015 | 0.011 | 0.009 |
| Liaoning | 0.053-0.141 | 0.053 | 0.079 | 0.100 | 0.051 | 0.015 | 0.012 | 0.008 |
| Jilin | 0.053-0.103 |  | 0.069 | 0.133 | 0.067 | 0.022 | 0.017 | 0.016 |
| Heilongjiang | 0.054-0.081 |  | 0.072 | 0.096 | 0.066 | 0.016 | 0.015 | 0.011 |
| Shanghai | 0.059-0.103 |  | 0.107 | 0.172 | 0.000 | 0.022 | 0.018 | 0.016 |
| Jiangsu | 0.056-0.103 | 0.058 | 0.099 | 0.160 | 0.038 | 0.016 | 0.012 | 0.009 |
| Zhejiang | 0.060-0.134 | 0.061 | 0.106 | 0.163 | 0.082 | 0.014 | 0.011 | 0.010 |
| Anhui | 0.055 |  | 0.087 | 0.134 | 0.058 | 0.017 | 0.013 | 0.010 |
| Fujian | 0.056-0.076 | 0.054 | 0.085 | 0.141 | 0.054 | 0.016 | 0.012 | 0.009 |
| Jiangxi | 0.060 |  | 0.097 | 0.151 | 0.050 | 0.019 | 0.017 | 0.016 |
| Shandong | 0.059-0.105 | 0.059 | 0.088 | 0.115 | 0.038 | 0.019 | 0.017 | 0.015 |
| Henan | 0.051-0.108 |  | 0.088 | 0.123 | 0.045 | 0.021 | 0.017 | 0.015 |
| Hubei | 0.038-0.082 |  | 0.088 | 0.146 | 0.040 | 0.015 | 0.013 | 0.010 |
| Hunan | 0.066-0.107 |  | 0.090 | 0.141 | 0.050 | 0.020 | 0.016 | 0.012 |
| Guangdong | 0.063-0.107 | 0.060 | 0.087 | 0.141 | 0.040 | 0.014 | 0.011 | 0.007 |
| Guangxi | 0.057 | 0.054 | 0.087 | 0.124 | 0.037 | 0.015 | 0.011 | 0.004 |
| Hainan | 0.063-0.095 | 0.062 | 0.087 | 0.064 | 0.059 | 0.012 | 0.011 | 0.010 |

| Table S16 ¦ Feed-in tariff prices and transmission prices of electricity in China, 2021 (continue). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provinces | Feed-in tariff prices ($/kWh) |  |  |  |  | Electric transmission prices ($/kWh) |  |  |
|  | Thermal power | Nuclear power | Wind power | Solar power | Hydro power | 35 kV | 110 kV | ≥220 kV |
| Chongqing | 0.059-0.107 |  | 0.078 | 0.057 | 0.046 | 0.018 | 0.016 | 0.013 |
| Sichuan | 0.063-0.101 |  | 0.081 | 0.113 | 0.040 | 0.016 | 0.010 | 0.007 |
| Guizhou | 0.048-0.108 |  | 0.077 | 0.136 | 0.042 | 0.016 | 0.011 | 0.008 |
| Yunnan | 0.059-0.088 |  | 0.066 | 0.106 | 0.029 | 0.015 | 0.011 | 0.008 |
| Shaanxi | 0.047 |  | 0.080 | 0.098 | 0.047 | 0.015 | 0.012 | 0.011 |
| Gansu | 0.042-0.095 |  | 0.062 | 0.123 | 0.034 | 0.013 | 0.011 | 0.009 |
| Ningxia | 0.040-0.103 |  | 0.074 | 0.129 | 0.030 | 0.011 | 0.009 | 0.007 |
| Xinjiang | 0.031 |  | 0.066 | 0.126 | 0.035 | 0.016 | 0.012 | 0.007 |
| Notes: a, The statistic power generation refers to the electricity generated and linked to the national grids (national grids and south grids). |  |  |  |  |  |  |  |  |

| ### Table S17 ¦ Carbon emission factors for different types of electric power 18. |  |  |
| --- | --- | --- |
| Power Types | Carbon Emission Factor (kg CO2-eq/kWh) | Description in Ecoinvent database |
| Thermal power | 0.729-1.422 | Electricity, high voltage {CN}¦ electricity production, natural gas, at conventional power plant ¦ Cut-off, S Electricity, high voltage {CN}¦ electricity production, hard coal ¦ Cut-off, S |
| Nuclear power | 0.014±10 % | Electricity, high voltage {CN}¦ electricity production, nuclear, pressure water reactor ¦ Cut-off, S |
| Wind power | 0.015-0.044 | Electricity, high voltage {CN}¦ electricity production, wind, <1MW turbine, onshore ¦ Cut-off, S Electricity, high voltage {CN}¦ electricity production, wind, >3MW turbine, onshore ¦ Cut-off, S Electricity, high voltage {CN}¦ electricity production, wind, 1-3MW turbine, offshore ¦ Cut-off, S Electricity, high voltage {CN}¦ electricity production, wind, 1-3MW turbine, onshore ¦ Cut-off, S |
| Solar power | 0.080-0.093 | Electricity, low voltage {CN}¦ electricity production, photovoltaic, 3kWp slanted-roof installation, multi-Si, panel, mounted ¦ Cut-off, S Electricity, low voltage {CN}¦ electricity production, photovoltaic, 3kWp slanted-roof installation, single-Si, panel, mounted ¦ Cut-off, S |
| Hydro power | 0.005-0.006 | Electricity, high voltage {CN}¦ electricity production, hydro, reservoir, non-alpine region ¦ Cut-off, S Electricity, high voltage {CN}¦ electricity production, hydro, run-of-river ¦ Cut-off, S |
| Notes: The carbon emission factor (2020) listed in this table is used to approximate the provincial level of carbon emissions in China at 2021. |  |  |

## 1.6 Calculation of the carbon emissions and composite cost of electrolytic H2.

Environmental emissions such as carbon emissions are calculated as follows:

EIi=j=1n(CAj×EFj) equation 13

where EIi represents the emission impacts (EI, kg CO2-eq per process) of process i, which includes the procedural process of 1 kg H2. CAj indicates the consumption amount (CA, mass or energy unit per 1 kg H2) of items j (e.g., materials, chemicals, energy) as shown in specific life-cycle inventories (LCI). EFj refers to the emission factor (EF, kg CO2-eq per consumable items) of j consumed for H2 production, which is assessed by IPCC-2021-GWP100, while the EF of electricity (kg CO2-eq/kWh) is separately calculated as the following demonstration.

Techno-economic analyses, following a similar perspective of life-cycle assessment, estimate the composite cost of H2 as follows:

Costoverallan=AF×Costcapex+CostO&Man+CostSalaryan+Costtaxan+Costtransan equation 15

AF=r(1+r)n(1+rn-1) equation 16

LCH2=CostoverallanYH2an equation 17

where the annual overall cost (Costoverallan, $) is the summary of annualized capital expenditures (AF⋅Costcapex, AF is the annual factor, r is the depreciation rate ranging 4-8%), operation and maintenance expenditure (CostO&Man, $), salary (CostSalaryan, $), carbon tax (Costtaxan, $), and transport expenditure (Costtransan, $). The levelized cost of H2 (LCH2, $/kg H2) is the normalization of Costoverallan by the annual H2 production amount (YH2an, kg). Most consumables’ price except electricity are consulted based on the current commodity market.

The carbon emissions of grids’ electricity are calculated as follows, which is determined via weighting methods:

EFelec=i=1nEFC-i⋅Wi equation 18

where the carbon emissions of electricity (EFelec, kg CO2-eq/kWh) are determined by the emission factor of different power types (EFC-i, kg CO2-eq/kWh), and their corresponding weighting (Wi, %) in the national/provincial electric grids.

Similarly, the composite cost of grids’ electricity (CCelec, $/kWh) is calculated based on the weighting of different power types in the national/provincial electric grids. This calculation considers the feed-in tariff prices (FTp-i, $/kWh) and electric transmission prices (ETp-i, $/kWh) delivering at hierarchical voltages as follows:

CCelec=i=1n(Wi⋅FTp-i+ETp-i) equation 19

Above carbon emissions and composite cost of grids’ electricity are then employed in the previously mentioned life-cycle assessment and techno-economic analyses. Specifically, the data and demonstrations in Supplementary Section 1 determine the carbon emissions and composite cost of H2 within its cradle-to-gate range (Figure S1).

# 2. Evaluation of the available water resources and electricity.

## 2.1 Calculation of available water resources and electricity.

The available reclaimed water (ARW, 108 m3) is calculated as follows:

ARW=i=1nAIW,i+ADW,i×rwc,i×rwt,i×1-rreuse,i-AEC,i×rwd,i equation 20

where the subscript i represents China’s provinces, AIW,i (108 m3) and ADW,i (108 m3) are the annually discharged amount of industrial/domestic wastewater, respectively. rwc,i (%) is the wastewater collection ratio, rwt,i (%) is the advanced wastewater treatment ratio, the treated effluents are reclaimed water or have similar water quality with those of reclaimed water, rreuse,i (%) is the reuse ratio of reclaimed water. AEC,i (108 m3) is the amount of reclaimed water for ecological compensation. rwd,i (dimensionless) is the ratio of reclaimed water deployed for regional water balance, especially for areas lacking water resources, which ranges from 0.50 to 0.53 (the value is estimated based on reports of South-to-North Water Diversion from the State Council of China) 45.

The value of available freshwater per capita (AFW-cap, m3/capita) determines whether a region can use purified tap water for H2 production as follows:

AFW-cap=ATFW,iApop,i-j=1nAwater use,i,jApop,i equation 21

where the subscript i represents China’s provinces, and the subscript j represents multiple water demands. Apop,i (104 capita) is the value of population, ATFW,i (108 m3) is the total amount of freshwater, Awater use,i,j (108 m3) is the amount of water usage including water used for H2 production. ATFW,iApop,i (m3/capita) represents the water resources per capita, and j=1nAwater use,i,jApop,i (m3/capita) represents the water usage per capita, when their difference is larger than 3000 m3/capita (the definition of moderate water stress by The United Nations) 46, the province potentially has excessive freshwater for electrolysis. Otherwise, the province would hardly have available freshwater for electrolysis since it is probably suffering water stress.

Then, water treatment plants (WTPs) with additional capacities supplying tap water besides water demand of basic industrial and domestic activities, provide the tap water from freshwater. The installed capacity of RO facilities in analyzed regions finally determine the available purified tap water (ATFW, 108 m3) for H2 production. Notably, the installed capacity of RO facilities exceeds the amount of tap water, which demonstrates that all tap water provided by WTPs can be purified for H2 production. ATFW can be calculated as follows:

ATFW=i=1n(PCWTPs,i×Fcap,i×YPW,i×365) equation 22

where the subscript i represents China’s provinces. PCWTPs,i (104 m3/day) is the additional capacity of water treatment plants, Fcap,i (dimensionless) ranging from 0.7 to 0.9 is the average capacity factor of water treatment plants nationwide 47, YPW,i (m3 pure water/m3 feedstock) is the yield of purified tap water when RO facilities purify the tap water.

The available desalinated seawater (ADSW, 108 m3) is calculated as follows:

ADSW=i=1n(Csd,i×Csd-ro,ij=1nCsd,j×Fcap,i×365) equation 23

where the subscript i represents China’s provinces, and the subscript j represents different technologies of seawater desalination. Csd,i (104 ton/day) is the total installed capacity of seawater desalination in coastal regions, Csd-ro,i (104 ton/day) is the installed capacity of reverse-osmosis facilities in coastal regions, Csd,j (104 ton/day) is the installed capacity of different desalination technologies in coastal regions, Fcap,i (dimensionless) ranging from 0.6 to 0.8 is the average capacity factor of reverse-osmosis facilities nationwide (the value derives from The 2021 National Seawater Utilization Report published by the China’s Ministry of Natural Resources) 48.

The available amount of offshore wind electricity (AOWE, 108 kWh) is calculated as follows:

AOWE=i=1n(COWE,i×EDOWE,i) equation 24

where the subscript i represents China’s provinces. COWE,i (104 kWh) is the installed capacity of offshore wind engines, EDOWE,i (h) is the annually effective operation time of offshore wind power.

## 2.2 Statistical data supporting the calculation of available reclaimed water.

| ### Table S18 ¦ Data calculating the available reclaimed water in China, 2021 49. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Province a | Industrial wastewater (108 ton) | Domestic wastewater (108 ton) | Wastewater collection (%) | Wastewater treatment (%) | Reuse of reclaimed water (%) | Ecological compensation (108 ton) |
| Beijing | 19.4 | 2.9 | 86.49 | 100.00 | 4.77 | 15.7 |
| Tianjin | 7.0 | 4.8 | 87.30 | 100.00 | 96.35 | 11.3 |
| Hebei | 27.8 | 17.7 | 72.05 | 98.58 | 81.83 | 39.3 |
| Shanxi | 15.1 | 12.3 | 71.14 | 88.36 | 91.28 | 4.5 |
| Inner Mongolia | 11.7 | 13.4 | 76.99 | 100.00 | 81.60 | 29.1 |
| Liaoning | 26.6 | 16.5 | 65.72 | 79.50 | 91.53 | 8.7 |
| Jilin | 13.1 | 9.2 | 72.74 | 81.70 | 79.04 | 8.1 |
| Heilongjiang | 15.9 | 17.8 | 68.80 | 100.00 | 49.54 | 1.6 |
| Shanghai | 24.7 | 65.0 | 95.03 | 100.00 | 0.00 | 0.9 |
| Jiangsu | 66.1 | 250.2 | 72.98 | 98.36 | 84.67 | 5.1 |
| Zhejiang | 50.6 | 35.8 | 66.84 | 100.00 | 79.31 | 6.8 |
| Anhui | 36.5 | 82.1 | 60.00 | 100.00 | 86.81 | 9.0 |
| Fujian | 32.4 | 35.4 | 61.27 | 98.52 | 74.17 | 14.9 |
| Jiangxi | 28.8 | 48.7 | 35.71 | 80.17 | 73.62 | 4.6 |
| Shandong | 40.3 | 32.6 | 71.72 | 100.00 | 84.62 | 21.4 |
| Henan | 45.1 | 28.0 | 75.15 | 92.18 | 89.76 | 34.8 |
| Hubei | 51.9 | 85.6 | 53.29 | 96.72 | 86.79 | 21.0 |
| Hunan | 48.4 | 62.1 | 58.54 | 90.14 | 61.78 | 11.9 |
| Guangdong | 117.9 | 78.2 | 71.51 | 96.22 | 83.68 | 6.7 |
| Guangxi | 36.1 | 36.5 | 48.36 | 99.33 | 77.90 | 6.3 |
| Hainan | 8.5 | 1.5 | 55.80 | 93.77 | 17.20 | 1.0 |
| Chongqing | 22.5 | 19.3 | 64.36 | 94.51 | 31.79 | 1.6 |
| Sichuan | 57.0 | 21.8 | 50.38 | 98.56 | 61.25 | 6.9 |
| Guizhou | 20.0 | 20.0 | 49.19 | 100.00 | 58.25 | 2.0 |
| Yunnan | 27.5 | 15.7 | 62.66 | 96.46 | 75.88 | 5.0 |
| Shaanxi | 20.3 | 10.9 | 84.56 | 90.93 | 22.02 | 5.9 |
| Gansu | 9.7 | 6.5 | 83.12 | 98.05 | 90.75 | 11.3 |
| Ningxia | 3.7 | 4.2 | 73.79 | 91.37 | 86.43 | 3.3 |
| Xinjiang | 18.7 | 11.2 | 87.62 | 77.64 | 5.24 | 16.2 |
| Notes: a, Provincial data for Tibet and Qinghai are excluded as these provinces have water resource conservation zones. Considering water security in these provinces, almost no exploitation of water resources is suggested. |  |  |  |  |  |  |

## 2.3 Statistical data supporting the calculation of available purified tap water.

| ### Table S19 ¦ Data calculating the additional capacities of water treatment plants supplying tap water besides water demand of basic industrial and domestic activities 49. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Province a | Production capacity (104 m3/day) b | Water consumption (104 m3/year) |  |  |  |
|  |  | Manufacturing operation | Public services | Domestic households | Other accesses |
| Beijing | 699.29 | 11370.02 | 45718.93 | 67094.02 | 2712.74 |
| Tianjin | 480.60 | 33384.08 | 13948.83 | 38251.02 | 2891.44 |
| Hebei | 794.69 | 46259.24 | 24814.05 | 68428.20 | 4095.14 |
| Shanxi | 411.90 | 17370.17 | 16604.34 | 45637.40 | 2692.45 |
| Inner Mongolia | 394.07 | 26095.08 | 9278.92 | 26730.99 | 5532.85 |
| Liaoning | 1329.53 | 70488.90 | 37917.09 | 86387.11 | 15770.02 |
| Jilin | 654.81 | 23920.85 | 15956.37 | 37335.80 | 4049.96 |
| Heilongjiang | 787.41 | 34040.54 | 18031.94 | 42542.18 | 7514.93 |
| Shanghai | 1221.00 | 42927.37 | 76482.44 | 115705.46 | 12581.80 |
| Jiangsu | 3608.17 | 209128.01 | 77998.42 | 220347.47 | 51422.34 |
| Zhejiang | 2109.32 | 157686.49 | 62769.99 | 187403.14 | 11741.92 |
| Anhui | 1122.41 | 78932.69 | 33083.80 | 101661.82 | 8890.20 |
| Fujian | 966.47 | 43750.99 | 36130.86 | 88577.35 | 10601.43 |
| Jiangxi | 691.98 | 33423.39 | 19330.64 | 70887.41 | 5681.83 |
| Shandong | 1957.19 | 159235.82 | 49194.33 | 138376.52 | 12186.85 |
| Henan | 1255.37 | 52865.32 | 32765.61 | 107177.55 | 8391.91 |
| Hubei | 1652.64 | 80062.06 | 27142.73 | 143573.58 | 10018.13 |
| Hunan | 1086.26 | 41301.28 | 34839.24 | 110201.46 | 11728.92 |
| Guangdong | 4231.74 | 258615.83 | 174566.63 | 421865.03 | 48421.94 |
| Guangxi | 750.60 | 39335.19 | 28171.45 | 100039.85 | 2530.01 |
| Hainan | 195.88 | 2695.17 | 7093.81 | 29015.74 | 6814.70 |
| Chongqing | 769.76 | 39302.26 | 25696.44 | 77428.36 | 8700.07 |
| Sichuan | 1521.34 | 49257.84 | 50596.09 | 168291.38 | 20727.37 |
| Guizhou | 454.02 | 20339.17 | 7536.17 | 48535.26 | 2109.94 |
| Yunnan | 484.71 | 26244.03 | 8168.49 | 56075.31 | 3933.27 |
| Shaanxi | 645.93 | 35128.60 | 6822.25 | 73794.09 | 6727.03 |
| Gansu | 363.86 | 15928.73 | 8756.29 | 26142.26 | 4749.53 |
| Ningxia | 273.46 | 8254.46 | 6005.77 | 13097.32 | 4756.55 |
| Xinjiang | 541.67 | 21353.78 | 15671.77 | 33350.84 | 13112.01 |
| Notes: a, The total water resources amount excludes the overlapping part of surface water and underground water. Provincial data for Tibet and Qinghai are excluded as these provinces have water resource conservation zones. Considering water security in these provinces, almost no exploitation of water resources is suggested. b, The production capacity of tap water is determined by the installed capacity of membrane-based treatment units in water treatment plants, since the final polishing of tap water is normally purified by nano-filtration, ultra-filtration, and reverse-osmosis processes. |  |  |  |  |  |

| ### Table S20 ¦ Data calculating the available per capita water resource after deducting the water demand of basic activities 50,51. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Province a | Water resources (108 m3) a | Population (104 capita) b | Water usage (108 m3) |  |  |  |
|  |  |  | Domestic | Industrial c | Agriculture | Ecological compensation |
| Beijing | 61.3 | 2189 | 19.4 | 2.9 | 2.8 | 15.7 |
| Tianjin | 39.8 | 1373 | 7.0 | 4.8 | 9.3 | 11.3 |
| Hebei | 376.6 | 7448 | 27.8 | 17.7 | 97.1 | 39.3 |
| Shanxi | 207.9 | 3480 | 15.1 | 12.3 | 40.8 | 4.5 |
| Inner Mongolia | 942.9 | 2400 | 11.7 | 13.4 | 137.5 | 29.1 |
| Liaoning | 511.7 | 4229 | 26.6 | 16.5 | 77.2 | 8.7 |
| Jilin | 459.2 | 2375 | 13.1 | 9.2 | 79.9 | 8.1 |
| Heilongjiang | 1196.3 | 3125 | 15.9 | 17.8 | 289.2 | 1.6 |
| Shanghai | 53.9 | 2489 | 24.7 | 65.0 | 15.3 | 0.9 |
| Jiangsu | 500.8 | 8505 | 66.1 | 250.2 | 246.2 | 5.1 |
| Zhejiang | 1344.7 | 6540 | 50.6 | 35.8 | 73.3 | 6.8 |
| Anhui | 883.3 | 6113 | 36.5 | 82.1 | 144.1 | 9.0 |
| Fujian | 758.7 | 4187 | 32.4 | 35.4 | 99.8 | 14.9 |
| Jiangxi | 1419.7 | 4517 | 28.8 | 48.7 | 167.3 | 4.6 |
| Shandong | 525.3 | 10170 | 40.3 | 32.6 | 115.8 | 21.4 |
| Henan | 689.2 | 9883 | 45.1 | 28.0 | 115.0 | 34.8 |
| Hubei | 1188.8 | 5830 | 51.9 | 85.6 | 177.7 | 21.0 |
| Hunan | 1790.6 | 6622 | 48.4 | 62.1 | 199.9 | 11.9 |
| Guangdong | 1221.2 | 12684 | 117.9 | 78.2 | 204.2 | 6.7 |
| Guangxi | 1541.2 | 5037 | 36.1 | 36.5 | 189.6 | 6.3 |
| Hainan | 341.6 | 1020 | 8.5 | 1.5 | 34.0 | 1.0 |
| Chongqing | 750.8 | 3212 | 22.5 | 19.3 | 28.7 | 1.6 |
| Sichuan | 2924.5 | 8372 | 57.0 | 21.8 | 158.6 | 6.9 |
| Guizhou | 1091.4 | 3852 | 20.0 | 20.0 | 62.1 | 2.0 |
| Yunnan | 1615.8 | 4690 | 27.5 | 15.7 | 112.1 | 5.0 |
| Shaanxi | 852.5 | 3954 | 20.3 | 10.9 | 54.6 | 5.9 |
| Gansu | 279 | 2490 | 9.7 | 6.5 | 82.6 | 11.3 |
| Ningxia | 9.3 | 725 | 3.7 | 4.2 | 56.9 | 3.3 |
| Xinjiang | 809 | 2589 | 18.7 | 11.2 | 527.9 | 16.2 |
| Notes: a, The total water resources amount excludes the overlapping part of surface water and underground water. Provincial data for Tibet and Qinghai are excluded as these provinces have water resource conservation zones. Considering water security in these provinces, almost no exploitation of water resources is suggested. b, The population is the registered number at the end of the calendar year. c, The value includes the water amount consumed for thermal or nuclear power (e.g., cooling water). |  |  |  |  |  |  |

### Figure S8 | Water resources per capita in China from 2003 to 2021. The national population is the statistical value at the end of each civil year. Total water resources comprise surface water, including its overlapping part with underground water. A 1% derivation of water resources is defined as the input uncertainty for per capita water resources, and the results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The lines represent the mean value, while the expanded filling shade around these lines represents their uncertainty range. Hierarchical levels of water stress (water usage shortage at 1700 m3 per capita, medium water stress at 2000 m3 per capita, and moderate water stress at 3000 m3 per capita) defined by the United Nations 52 are also provided in the figure to illustrate China’s water shortage situations. Results indicate that China is a typical country facing severe water shortage. Since the water resources in Tibet and Qinghai provinces are precious natural reserves, which cannot be exploited, scenarios including or excluding the statistical value of Tibet and Qinghai provinces are compared. The left axis is displayed with a linear function.

## 2.4 Statistical data supporting the calculation of available desalinated sweater and offshore wind power.

| ### Table S21 ¦ Data calculating the production amount of offshore wind power in coastal provinces 53. |  |  |  |
| --- | --- | --- | --- |
| Province | Installed capacity (104 kW) a | Territorial sea area | Annual effective duration (h) |
| Liaoning | 105 | Bo Sea | 3000-3500 |
| Hebei | 30 | Bo Sea | 3000-3500 |
| Tianjin | 9 | Bo Sea | 3000-3500 |
| Shandong | 261 | Bo, Yellow Sea | 3000-3500 |
| Jiangsu | 1183 | Yellow Sea | 3000-3500 |
| Shanghai | 67 | East Sea | 3000-3500 |
| Zhejiang | 279 | East Sea | 3000-3500 |
| Fujian | 321 | East, South Sea | 4000-4500 |
| Guangdong | 791 | South Sea | 4000-4500 |
| Notes: a, The value represents the electricity capacity of offshore wind power linked to the onshore electric grids nearby. |  |  |  |

### Figure S9 | Installed capacity and provincial distribution of seawater desalination facilities. a, Provincial installed capacity of seawater desalination facilities in China in 2021. b, Engineering scale capacity of different forms of seawater desalination facilities in China in 2021. The capacity of multi-stage distillation, forward osmosis, and electro-dialysis is negligible compared to the mainstream reverse-osmosis, thus their capacity is marked with the detailed value in the bar chart column. The values are based on the base scenario provided by the government report (The 2021 National Seawater Utilization Report from the Ministry of Natural Resources) 54. The left axis is displayed with a linear function.

### Figure S10 | Present water quality of seawater in China’s coastal provinces at 2021. a, Area percentage of seawater with hierarchical water quality in China’s coastal regions during spring, summer, autumn seasons and average scenarios in 2021. b, Area percentage of seawater with favorable water quality (Class I and II) in China’s coastal provinces. Data is obtained based on the basic scenario in the government report (The 2021 Communique on the State of China’s Marine Ecological Environment provided by the Ministry of Ecology and Environment) 55. The left axis is displayed with a linear function.

### Figure S11 | Average wind atlas in China and its coastal areas at 2021. a, Wind speed and b, wind power potential at China and its coastal regions in China at 2021. The values are measured at the height of 100 m relative to sea level. Average data for available collection in 2021 is exhibited here. The source data is obtained from the data-catalog provided by the World Bank Organization 56.

# 3. Evaluation of the electrolysis efficiency and determination of the H2 equilibrium.

## 3.1 Demonstrations of the purified water production from reclaimed water, tap water, and raw seawater.

### Method S2 | The production process of purified water provided by the RO facilities.

Herein, the pure water produced from reclaimed water, tap water and seawater by RO modules (Table S22) is assumed to meet the quality of electrolysis-grade water. Taking seawater desalination producing purified water as an example, a model considering seawater temperature (10-40 °C), salinity (30-32 g/L TDS), and the basic technical parameters of a 3-stage reverse osmosis (RO) module (as shown in Table S23 and Figure S4) provided by Dupont (75-85% ideal water recovery, 0.75-0.90 capacity factor, operation pressure at 41 bar, pressure drop across RO membrane at 1 bar, back pressure of RO membrane from permeates at 1 bar) is employed here to evaluate seawater desalination operations 19-26. Details of the modules and calculations of desalinated seawater generation can be found in Table S7. Other water types including purified reclaimed water and purified tap water required purification by RO modules follow the above similar procedures.

| ### Table S22 ¦ Required qualities of pure water for feedwater into alkaline electrolyzers 57. |  |  |
| --- | --- | --- |
| Items | Unit | Water quality indexes |
| Electric conductivity a | Ω·cm | ≥ 1.0×105 |
| Fe2+/Fe3+ a | mg/L | < 1.0 |
| Cl- a | mg/L | < 2.0 |
| Total suspended solids a | mg/L | < 1.0 |
| Alkaline concentration b | % | 27-32 |
| CO32- b | mg/L | < 100 |
| Fe2+/Fe3+ b | mg/L | < 3 |
| Cl- b | mg/L | < 800 |
| Notes: a, These values are required when common alkali like sodium hydroxide is used as electrolyte. b, These values are required when caustic alkali like potassium hydroxide is used as electrolyte. Optimal electrolysis systems like the one shown in Table S24 can slightly decrease the alkaline concentration employed in the electrolyzer. |  |  |

| ### Table S23 ¦ Major technical parameters of commercial RO elements 58-61. |  |  |  |
| --- | --- | --- | --- |
| Parameters (Units) | Element type |  |  |
|  | Eco Pro 400i | BW 30-400 | XLE 440 |
| Active area (m2) | 37 | 37 | 41 |
| Permeate flow rate (m3/day) | 43 | 40 | 53 |
| Typical stabilized salt rejection (%) | 99.7 | 99.5 | 99.0 |
| Minimum salt rejection (%) | 99.4 | 99.0 | 97.0 |
| Maximum operation temperature (°C) | 45 | 45 | 45 |
| Maximum operation pressure (bar) | 41 | 41 | 41 |
| Maximum element pressure-drop (bar) | 1.0 | 1.0 | 1.0 |
| pH range (continuous operation) | 2-11 | 2-11 | 2-11 |
| Maximum feed slit density index (—) | 5 | 5 | 5 |
| RO element prices ($) a | 370 | 370 | 410 |
| Average water flux coefficient (10-11 m/Pa·s) b | 1.23 | 0.69 | 1.50 |
| Average salt flux coefficient (10-7 m/s) b | 1.02 | 0.98 | 3.45 |
| Notes: a, The element price is estimated based on $10/m2 active area RO membrane. b, The values are calculated using equation (1-9) in Table S7. |  |  |  |

### Figure S12 | Schematic of feedwater purification for alkaline electrolysis producing H2. Electrolytic water requires high standards as demonstrated in Table S22. Sequential purification of feedstock water including pre-filtering by multi-stage reverse-osmosis, cation/anion exchange removing any potential competitive ions, ultrafiltration, and advanced ultraviolet disinfection is conducted before the electrolytic water is pumped into the electrolyzers. Lines marked with light blue color demonstrate the flow direction of feedstock water.

### Figure S13 | Specific resources’ consumption, treatment cost and carbon emission for feedstock pre-purification using different water resources. Specific water and electricity consumption for feedstock pre-purification by 3-stage reverse osmosis using different water sources including reclaimed water, tap water, and raw seawater. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The left axis is displayed with a linear function.

## 3.2 Demonstration of the alkaline electrolysis.

### Method S3 | Model of industrial alkaline electrolysis.

Based on the schematic shown in Figure S5 and data in Table S24, a calculation model for the installed capacity of alkaline electrolyzer is demonstrated here to mathematically describe H2 production 27-33. The calculated results can be employed in subsequent calculations for levelized carbon emission and composite cost of H2, as the life-cycle inventory (LCI) and construction cost per megawatt are determined by these calculations. A model based on the commercial alkaline electrolyzer shown in Table S24 is developed. Details of the modules and calculations of the electrolyzer’s capacity can be found in Table S8.

| ### Table S24 ¦ Technical parameters of industrial alkaline electrolyzer 38. |  |  |
| --- | --- | --- |
| Parameters | Unit | Value |
| Net production rate (Nm3/h at 0 °C, 1 bar) | Nm3/h a | 2400-3880 |
| Net production rate (kg/24 h) | kg/24 h | 5.180-8.374 |
| Production capacity dynamic range | % | 15-100 b |
| Power consumption at stack | kWh/kg H2 | 44.0-49.3 |
| Water consumption at stack | kg H2O/kg H2 | 10.0-15.0 |
| H2 purity | % | 99.9 ± 0.1 |
| H2 purity (with optional purification) | % | 99.99-99.999 |
| O2 content in H2 | < ppm (v) | 2 |
| H2O content in H2 | < ppm (v) | 2 |
| O2 purity | % | 99.5 ± 0.2 |
| Delivery pressure | barg | 1-200 |
| Dimensions | m2 | ~770 |
| Ambient temperature | °C | 5-35 |
| Electrolyte | 25% KOH aqueous solution |  |
| Notes: a, Nm3/h denotes normal m3/h, with the normal condition set at 0 °C, 1 bar. b, The value is the variation range based on the net production rate as shown in Table S24. |  |  |

## 3.3 Calibration of the electrolysis efficiency.

### Method S4 | Current efficiency (E, %) influenced by competitive reactions in alkaline electrolysis.

Organics and other potential pollutants in reclaimed water or seawater, in addition to ions, can affect electrolysis processes. These pollutants, such as glucose and methanol, which have lower oxidation potential than that of water, can contribute to the current efficiency. According to previous research investigating the influence of co-existing pollutants on current efficiency, the contribution of these pollutants to the current efficiency can be approximately linear 62-67, as shown in equations S25-S28.

CECOD=-4VFd[COD]Idt equation S25

CEN2=-3VFd[TN]Idt equation S26

CENO3-=8VFd[NO3-]Idt equation S27

CEClO3-=6VFd[ClO3-]Idt equation S28

where [COD] is expressed in molar concentration (mM O2), N2 is assumed to account entirely for gaseous nitrogen, and the CE for chlorate production is expressed based on 6-electron transferred from chloride. V (m3) is the volume of electrolyte, F is Faraday’s constant (96500 C/mol), I (A/m3) is the volume current density, and t (s) is the time duration of electrolysis. With significant time variations, the differential concentrations of COD, TN, nitrate, and chlorate appear to be almost linearly correlated with the specific charges passed. Thus, the pseudo first-order rate constants for pollutant degradation can support that a series of linear regressions of the differential concentrations on the specific charge passed can be used to estimate the mean current efficiency. In other words, based on the concentration of salt ions and pollutants, a preliminary linear estimation can be made of how much the current efficiency of H2 production in an alkaline electrolyzer will decrease when water containing these matters is used as feedstock water. This analysis employs electrolysis data from previous research (Table S25) to evaluate the probable current efficiency of electrolyzer using different water resources (Figures S14-S18) with various water quality indexes.

### Method S5 | Laboratory measurements of alkaline water electrolysis.

This analysis also validates the hypothesis proposed in Method S4, that the current efficiency of alkaline electrolysis decreases linearly with the presence of certain pollutants during the direct electrolysis of reclaimed water or seawater. Several types of water resources, including reclaimed water from wastewater treatment plants, actual seawater, and ultrapure water (freshwater or seawater post multi-stage reverse osmosis purification), are considered. The following content details laboratory measurement steps. As large-scale industrial alkaline electrolysis stacks are assembled from multiple small single-chambers in series (Table S8), data from a 50 mL laboratory-scale electrolysis cell may also be able to provide reference values for evaluating the performance of industrial-scale electrolysis stacks. Pt@C (Macklin, 20% wt) and IrO2 (Sinero, 99.9% purity with >85% wt Ir content), the most widely used cathode and anode catalysts, are loaded on hydrophilic carbon-fiber paper (CFB, Toray, unmodified). These serve as working electrodes in alkaline electrolyte (1M KOH, Macklin, 90% purity) for measurements of half-reaction (cathode hydrogen evolution and anode oxygen evolution reactions) and full water splitting. Detailed procedures include the preparation of the working electrodes and the measurement of electrochemical performance.

The working electrodes are prepared by dispersing 10 mg electrocatalysts into 1 mL ethanol (Mackin, 99.7%) containing 50μL Nafion solution (Dupont, 5 wt%) using bath sonication (15 min). This forms a homogeneous ink suspension. Subsequently, 25μL ink suspension is loaded dropwise onto a carbon-fiber paper with a surface area of 1.0×2.0 cm (repeated four times) and dried at room temperature for approximately 2 hours. The loading amount of electrocatalysts on carbon-fiber paper is about 1.0 mg·cm-2. Specifically, to evaluate the HER activity of the working electrodes electrolyzing various water resources, 10 mg commercial Pt@C powders (20% wt) are dispersed into 1 mL ethanol plus 50μL Nafion solution using bath sonication (15 min), and then 25μL Pt@C ink suspension is loaded dropwise onto 1.0×2.0 cm carbon-fiber paper. The same process is followed to evaluate the OER activity of the working electrodes, using IrO2 powders instead.

Electrochemical measurements, utilizing Gamry Interface 1010E, were conducted in a 1 M KOH solution saturated with N2. A standard three-electrode configuration was employed for both anodic oxygen evolution reaction (OER) and anodic hydrogen evolution reaction (HER). Carbon-fiber paper served as the counter electrode, while a saturated calomel electrode (SCE) was used as the reference electrode. All reported potentials (V) in this analysis were converted to the potential of the reversible hydrogen electrode (RHE) using the following equation:

ERHE=Eapply+0.24+0.0591×pH equation S29

Prior to formal electrochemical measurements, continuous cyclic voltammetry (CV) scans were performed until a stable and reproducible CV curve was achieved. Polarization curves (linear sweep voltammograms, LSV) were recorded at a scanning rate of 5 mV·s-1. The corresponding Tafel slope (b) was calculated using the following equation:

η=a+b×log10(j) equation S30

where η is the overpotential (mV) and j is the current density (mA·cm-2).

Electrochemical impedance spectra (EIS) were measured at an overpotential of 100 mV for HER and 300 mV for OER within the frequency range of 0.1 to 105 Hz. EIS results were fitted using equivalent circuits as shown in Figure S19. The electrochemical active surface area (ECSA) was evaluated by calculating the electrochemical double-layer capacitance (Cdl, mF) derived from CV results, which can be determined using the following equation:

ECSA=Cdl/Cs equation S31

where Cdl is the double electric layer capacitance (mF) obtained by obtaining cyclic voltammetric curves at different sweep rates in the non-Faraday region and Cs is the specific capacitance (mF·cm-2). CV was measured at varying scanning rates of 40, 60, and 80 mV·s-1 within the potential range of -0.1 to 0.2 V (vs RHE’s potential) for HER and 1.2 to 1.7 V (vs RHE’s potential) for OER.

For overall water splitting, commercial IrO2 and Pt@C were used to assemble the IrO2||Pt@C electrolyzer. An anion-exchange membrane served as the diaphragm of the H-type water-electrolysis cell. The polarization curve of overall water splitting was obtained by applying a cell voltage from 0 to 4 V at a scanning rate of 5 mV·s-1. Stability was evaluated by applying a constant current density of 100 mA·cm-2 to the assembled electrolyzer for 24 hours. Electrochemical measurement results are presented in the following Figure S19. Electrolysis data from various water resources facilitate the basis for a correlation analysis, integrating major water quality indexes (CODCr, TN, and total dissolved solids (TDS) demonstrating the overall concentration of salt ions) that influence current efficiency of electrolysis as shown in Figure S17. Ultrapure water without any pollutants, is assumed to be the ideal reference level. Conversely, salt ions and organic matter in reclaimed water or actual seawater could induce competitive side reactions. This leads to an elevated current density for these water types under identical applied potential in half-reactions, but resulting in less hydrogen evolution due to the occurrence of side reactions. Additionally, during the corresponding overall water-splitting measurements, the applied voltage necessary for electrolysis of these water types also increases under the same current density (Figure S19). This suggests that to achieve equivalent hydrogen production, these water resources, containing certain pollutants, necessitate higher electrical energy consumption, thereby reducing current efficiency. Consequently, laboratory measurements confirm the hypothesis demonstrated in Method S4 that the current efficiency of alkaline electrolysis decreases linearly with the presence of certain pollutants during the direct electrolysis of reclaimed water or seawater.

| ### Table S25 ¦ Previous research data and assumed data for calibrating the water and electricity consumption by electrolyzers directly electrolyzing water with interfering pollutants 62-67. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number | pH | TDS a (mg/L) | CODCr b (mg/L) | TN c (mg/L) | NH4+ (mg/L) | NO3- (mg/L) |
| 1 e | 8.9 | 4.6 | 310 | 182 | 216 | 0.62 |
| 2 e | 8.9 | 4.6 | 310 | 182 | 216 | 0.62 |
| 3 e | 9 | 7 | 480 | 350 | 432 | 6.2 |
| 4 e | 9 | 7 | 480 | 350 | 432 | 6.2 |
| 5 e | 9.1 | 14 | 900 | 854 | 684 | 0.62 |
| 6 e | 9.1 | 14 | 900 | 854 | 684 | 0.62 |
| 7 e | 9 | 17.6 | 460 | 924 | 792 | 6.2 |
| 8 e | 9 | 17.6 | 460 | 924 | 792 | 6.2 |
| 9 f | 7 | 10 | 50 | 15 | 5 | 4 |
| 10 f | 7.5 | 20 | 60 | 20 | 8 | 6 |
| 11 f | 6 | 30 | 100 | 25 | 25 | 8 |
| 12 f | 9 | 50 | 120 | 30 | 30 | 10 |
| Number | Cl- (mg/L) | Protein (mg BSA L-1) d | Carbohydrate (mg glucose L-1) | Organic acids (mg CH3COOH/L) | Current density (A/m2) | Current efficiency (%) |
| 1 | 237.515 | 160 | 16 | 1.2 | 104.5 | 50.88 |
| 2 | 237.515 | 160 | 16 | 1.2 | 128.02 | 49.07 |
| 3 | 354.5 | 250 | 39 | 96 | 150.65 | 75.06 |
| 4 | 354.5 | 250 | 39 | 96 | 185.52 | 89.45 |
| 5 | 744.45 | 520 | 63 | 186 | 227.89 | 86.74 |
| 6 | 744.45 | 520 | 63 | 186 | 273.09 | 83.19 |
| 7 | 1169.85 | 280 | 23 | 5.4 | 291.94 | 87.64 |
| 8 | 1169.85 | 280 | 23 | 5.4 | 370.99 | 93.06 |
| 9 | 100.0 | 0.1 | 1 | 1 | 300 g | 99.94 h |
| 10 | 150.0 | 0.3 | 3 | 3 | 300 g | 99.91 h |
| 11 | 200.0 | 0.5 | 5 | 5 | 300 g | 99.86 h |
| 12 | 250.0 | 0.7 | 15 | 20 | 300 g | 99.82 h |
| Notes: a, TDS denotes total dissolved solids. b, CODCr denotes chemical oxygen demand by index of potassium dichromate. c, TN denotes total nitrogen. d, BSA denotes bovine serum albumin as the representative matter of protein. e, These data are derived from previous research. f, These data are assumed based on the water quality indexes of reclaimed water as shown in Table S10. g, This value is the typical current density employed in commercial alkaline water electrolysis. h, These values are assumed based on the assumption that linear regressions of the differential concentrations on the specific charge passed can be used to estimate the mean current efficiency, namely the increasing ions or pollutants in feedwater could linearly decrease the mean current efficiency. |  |  |  |  |  |  |

### Figure S14 | Procedures for calibrating the specific consumption of water and electricity for the production of electrolytic H2 using water resources containing certain pollutants. Major procedures include the input of water quality indexes, current density, and corresponding current efficiency in alkaline water electrolysis collected from references, determined as literature data. Hypothetical data based on the water quality indexes of reclaimed water and seawater in China is merged with the literature data to form a dataset for the subsequent cross-validation of the assumption that pollutants (mainly organics and salt ions) in water linearly reduce the current efficiency of electrolysis with their increase at a constant level of current density (300 mA/m2), as demonstrated in Method S4. Following the verified assumption, the decrease in current efficiency can be obtained, which determines the specific consumption of water and electricity for electrolytic H2 production.

### Figure S15 | Schematic of cross-validation processes. The primary procedures involve merging reported data from literature and assumed data based on the water quality indexes of reclaimed water or seawater, normalizing the merged data with different numerical ranges and specific units, and conducting four successive cross-validations. In each validation, a different set of assumed data is used for validation, while the remaining data supports the linear regression as fitting data. All results are collected and synthetically considered to verify the assumption that pollutants (mainly organics and salt ions) in water linearly reduce the current efficiency of electrolysis with their increase at a constant level of current density (300 mA/m2).

### Figure S16 | Multi-parameter linear regression fitting for cross-validation of the assumption that pollutants (mainly organics and salt ions) in water linearly reduce the current efficiency of electrolysis with their increase at a constant level of current density (300 mA/m2). a, First, b, second, c, third, and d, fourth cross-validation. All four fittings of merged data exhibit favorable linear regression between the fitted and true current efficiency with a relative coefficient R2=0.97. The first, second, and fourth validation follow the fitting pattern of the dataset, but the third validation exhibits certain deviation due to improper weighting for the input parameters as shown in Figure S17. Data points with lower applied current density are gathered around the left bottom of the linear regression line, while points with higher applied current density are gathered around the median or top right of the linear regression line. The left axis is displayed with a linear function.

### Figure S17 | Contribution of multiple parameters to the current efficiency of water electrolysis. Results of the a, first, b, second, c, third, and d, fourth cross-validation are shown. In most validation scenarios except the third, concentration of organic acids (e.g., acetic acid), CODCr in easily biodegradable composition, total nitrogen mainly in ammonia and biodegradable Kjeldahl nitrogen significantly influence the current efficiency of electrolysis using water containing pollutants. These substrates normally have a lower oxidation potential than water and would thus compete with the oxygen evolution reaction. Additionally, in most validation scenarios, chloride ions also have a notable influence on the current efficiency due to their faster reaction kinetics involving 2 electrons in alkaline electrolysis, despite their oxygen potential being higher than that of the oxygen evolution reaction (involving 4 electrons). Notably, the third validation overly intensifies the contribution of pH of feedstock water and led to deviation as shown in Figure S16c, since the pH of electrolyte from different water sources with hierarchical pH level should all be modified to alkaline (pH~14) in actual industrial production of electrolytic H2. The bottom axis is displayed with a linear function.

### Figure S18 | Mutual relationship between factors with major contribution and the resultant current efficiency of electrolysis. The relationship of current efficiency with a, concentration of organic acids in water, b, concentration of CODCr in water, c, concentration of total nitrogen in water, and d, applied current density. Except for the data points gathering at the left bottom with lower applied current density, the current efficiency of the remaining data points generally follows the principle that increasing concentrations of organic acids, CODCr, and total nitrogen result in decreasing current efficiency. Additionally, data points with increasing applied current density do not linearly increase their current efficiency as shown in Figure S18d, which potentially demonstrates the deviation influence of pollutants in water for the electrolysis efficiency. The left axis is displayed with a linear function.

### Figure S19 | Laboratory measurements of alkaline water electrolysis using various water resources. All electrochemical measurement results are tested under alkaline conditions with a 1M KOH solution as the electrolyte, but different water resources are consumed. Data is automatically and continuously recorded by a multi-functional potentiostat (Interface 1010E, Gamry, U.S.A). Digital graphs of a, cells for half-reaction measurements, and b, the IrO2||Pt@C electrolyzer at the cell voltage with a current density of 100 mA·cm-2 are provided here. c, Major water quality indexes influencing current efficiency, as shown in Figure S17, including CODCr, TN, and total dissolved solids (TDS) which demonstrate the overall concentration of salt ions, are listed for reclaimed water, raw seawater, and ultrapure water. The left axis is displayed using a log10 function. These water quality index results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on three repetitive measurements. Digital graphs of reclaimed water, raw seawater, and ultrapure water are embedded in the figure. Notably, the raw seawater originates from a densely populated bay located in Shenzhen, Guangdong Province, China, inhabited by a variety of birds, resulting in more pronounced organic pollution compared to the other two water types. No obvious pollutants can be detected in the ultrapure water as the concentration level of CODCr, TN, and TDS approaches non-detectable. Cyclic voltammograms, measured at scanning rates of 40, 60, and 80 mV·s-1, are presented for d, reclaimed water, e, raw seawater, and f, ultrapure water. The same charts exhibit cyclic voltammograms of cathodic hydrogen evolution reaction (HER) and anodic oxygen evolution reaction (OER) with median breakpoints. The left axis is displayed using a linear function, and the breakpoints at the bottom axis in Figure S19d-S19f demonstrate the data ranging from 0.4 to 1.2. The figures inserted in (d) to (f) demonstrate the electrochemical active surface area (ECSA) of anode and cathode electrodes. These are determined at a potential of 0.1V for HER, and 1.5V, for OER, versus the potential of RHE. g, Polarization curves, measured at a scanning rate of 5 mV·s-1, are shown for reclaimed water, raw seawater, and ultrapure water. Both HER and OER polarization curves are displayed in the same charts with median breakpoints. The left axis is displayed using a linear function, and the breakpoint at the bottom axis in Figure S19g demonstrates the data ranging from 0.5 to 1.0. Tafel slopes of h, HER and i, OER, calculated from the previous polarization curves, are provided for reclaimed water, raw seawater, and ultrapure water. The left axis is displayed using a linear function. j, Electrochemical impedance spectroscopy (EIS) spectra, obtained at an overpotential of 100 mV for HER and 300 mV for OER, are fitted with the equivalent circuit models shown in the embedded circuit. The left axis is displayed using a linear function, and the breakpoint at the bottom axis in Figure S19j demonstrates the data ranging from 0.5 to 2.0. k, Polarization curves of the IrO2||Pt@C electrolyzer for overall water splitting are also displayed besides the charts of chronopotentiometry spectra. l, Continuous 24-hour chronopotentiometry measurements of the IrO2||Pt@C electrolyzer at 100 mA·cm-2 are conducted for reclaimed water, raw seawater, and ultrapure water. Electrodes and diaphragms after 24 hours of electrolysis using different water resources are also recorded in the embedded figures. The left axis of the polarization curve and chronopotentiometry measurements is displayed using a linear function. It can be observed that accumulated pollutants occur around the diaphragms approaching the anode chamber when reclaimed water and raw seawater are electrolyzed, thus increasing the applied voltage. This results in a lower current efficiency due to the increased transfer resistance of OH- when other pollutants co-exist in the feedstock water.

### Figure S20 | Specific resources’ consumption for electrolytic H2 using different water sources. a, Specific electricity and b, specific water consumption for electrolytic H2 using different water sources including direct usage of reclaimed water, usage of purified reclaimed water, usage of purified tap water, desalinated seawater, and direct usage of raw seawater. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The value of the base scenario is obtained from the report published by the International Energy Agency, while the values of the other technical routes are obtained by calibrating the base scenario according to Method S4, Figures S14-S18, and the calculation method of specific consumption of water and electricity for electrolytic H2. The left axis is displayed with a linear function.

## 3.4 Calculations of the H2 equilibrium nationwide.

The H2 equilibrium (HEi, 104 ton) can be calculated as follows:

HEi=Ai,jSWCAWE,j-(k=1nPEk)×RH2-PE×ConverH2-coal equation 32

HEi=Ai,jSECAWE,j-(k=1nPEk)×RH2-PE×ConverH2-coal equation 33

where the subscript i represents China’s provinces, the subscript j represents different technical routes for electrolytic H2 production, the subscript k represents different types of primary energy consumption. Ai,j (108 m3 water or 108 kWh electricity) is the available resources for H2 production. PEk (104 ton) is the amount of primary energy consumption. RH2-PE (%) ranging from 4.0 to 4.5 is the substitution ratio of H2 in the primary energy consumption at 2021 68,69. ConverH2-coal (1 kg coal-eq equals 0.2061 kg H2) is the conversion relation of coal-equivalent to H2 based on the higher thermal enthalpy of H2. SWCAWE,j (kg H2O/kg H2) and SECAWE,j (kWh/kg H2) are the specific water and electricity consumption.

| ### Table S26 ¦ Primary energy consumption (104 ton coal-eq) in China, 2021 used for predicting the present and future H2 demand 43,70. a |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Province | Coal | Petroleum | Natural gas | Electricity | Heat | Others |
| Beijing | 35.84 | 1970.52 | 825.47 | 1460.19 | 684.21 | 126.63 |
| Tianjin | 1485.91 | 2087.99 | 756.02 | 1213.96 | 749.82 | 76.49 |
| Hebei | 15925.64 | 2069.41 | 2298.64 | 5335.84 | 2187.96 | 273.72 |
| Shanxi | 8514.42 | 1019.70 | 872.16 | 3170.82 | 1097.75 | 5.54 |
| Inner Mongolia | 8797.12 | 1862.93 | 942.76 | 4863.72 | 2432.02 | 4.35 |
| Liaoning | 7306.37 | 6822.28 | 945.25 | 3161.84 | 2640.87 | 277.88 |
| Jilin | 1788.58 | 1351.29 | 423.17 | 974.90 | 1125.19 | 36.79 |
| Heilongjiang | 2840.77 | 2082.75 | 520.51 | 1333.02 | 2070.63 | 271.51 |
| Shanghai | 1308.95 | 4572.07 | 670.39 | 2069.45 | 334.17 | 52.39 |
| Jiangsu | 8292.45 | 4429.82 | 2437.30 | 8471.77 | 2311.28 | 57.20 |
| Zhejiang | 2155.59 | 4877.28 | 1603.42 | 6555.56 | 2123.47 | 83.80 |
| Anhui | 4547.54 | 2412.39 | 733.85 | 3169.81 | 546.73 | 47.49 |
| Fujian | 2552.60 | 3120.10 | 527.16 | 3394.08 | 705.85 | 93.71 |
| Jiangxi | 3076.58 | 1510.87 | 456.43 | 2213.05 | 219.67 | 71.16 |
| Shandong | 9863.91 | 5997.67 | 2710.38 | 9091.81 | 6033.48 | 71.55 |
| Henan | 5389.56 | 3618.53 | 1356 40 | 4615.49 | 1406.90 | 136.17 |
| Hubei | 4979.83 | 4355.06 | 759.51 | 2897.55 | 403.64 | 194.17 |
| Hunan | 4367.01 | 4072.79 | 598.73 | 2449.61 | 237.09 | 673.40 |
| Guangdong | 3225.11 | 7813.03 | 1903.62 | 9370.31 | 1114.23 | 116.36 |
| Guangxi | 4023.49 | 1545.34 | 424.71 | 2748.33 | 388.17 | 192.10 |
| Hainan | 165.91 | 730.22 | 399.91 | 475.30 | 102.03 | 4.14 |

| Table S26 ¦ Primary energy consumption (104 ton coal-eq) in China, 2021 used for predicting the present and future H2 demand (continue). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Province | Coal | Petroleum | Natural gas | Electricity | Heat | Others |
| Chongqing | 1854.17 | 1385.39 | 1539.72 | 1576.50 | 261.04 | 31.53 |
| Sichuan | 3873.05 | 3405.23 | 3277.69 | 3759.51 | 342.42 | 236.29 |
| Guizhou | 2895.14 | 2113.75 | 547.67 | 2035.47 | 65.89 | 11.12 |
| Yunnan | 4256.30 | 2139.56 | 230.09 | 2477.82 | 87.22 | 143.29 |
| Shaanxi | 4024.40 | 1184.98 | 1323.14 | 2687.55 | 880.19 | 10.72 |
| Gansu | 1902.83 | 1018.49 | 446.93 | 1753.67 | 628.18 | 12.31 |
| Ningxia | 3179.13 | 292.14 | 284.70 | 1499.07 | 613.43 | 0.00 |
| Xinjiang | 3887.75 | 2136.67 | 1635.23 | 4176.15 | 2425.37 | 35.29 |
| Notes: a, Based on the density of H2 at normal condition (0 °C, 1 atmospheric pressure, 0.0899 kg/m3 H2) and the higher thermal enthalpy of H2 (286 kJ/mol), 4.361 kg coal-eq equals 104 m3 H2, namely, 1 kg coal-eq equals 0.2061 kg H2, the conversion ratio is employed in the calculation of H2 equilibrium. |  |  |  |  |  |  |

### Figure S21 | Available amount of water and electricity resources for the production of electrolytic H2. a, Available reclaimed water, tap water, desalinated seawater (left axis) and offshore wind power (right axis) for electrolytic H2 from 2021 to 2060. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The lines represent the mean value, while the expanded filling shade around these lines represents their uncertainty range. Both left and right axes are displayed with a linear function. b, Electricity production from 2021 to 2060, which primarily includes electric types of coal, natural gas, biomass, solar, wind, hydro, nuclear towards the scenarios of carbon peak at 2030 and carbon neutrality at 2060. The results are calculated based on the LEAP model and scenarios of energy transition of China 71. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The filling area represents the mean value, while the expanded dashed lines around these filling areas represent their uncertainty range. The results of predicted scenarios for future H2 production undergo smoothing processing to illustrate the trend of continuous change between decades. The left axis is displayed with a linear function.

### Figure S22 | H2 equilibrium, including H2 production and demand in provinces across China at 2021. H2 equilibrium of technical routes using a, reclaimed water, b, purified reclaimed water, c, purified tap water, d, desalinated seawater, and e, raw seawater. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. Exclusions apply to Hong Kong, Macao, and Taiwan due to data unavailability, and Qinghai and Tibet to preserve precious water resources in nature reserves. The negative value of H2 equilibrium demonstrates the H2 demand while the positive value demonstrates the H2 production. The breakpoint at the left axis in Figure S22 demonstrates the data ranging -900 to -400. The left axis is displayed with a linear function.

## 3.5 Influence of the competitive pollutants in water resources on electrolysis and the recent strategies for direct electrolysis of polluted water.

| ### Table S27 ¦ Potential competitive redox reactions in direct electrolysis of reclaimed water and their electrochemical standard potential (E0) 72. |  |  |
| --- | --- | --- |
| Species | Reaction Red. ⇌ Ox at pH 14 | E0 [VSHE] d |
| H2O | H2+2OH-⇌2H2O+2e- | -0.828 |
| Cl- | Cl-+4OH-⇌ClO2-+2H2O+4e- | 0.760 |
|  | Cl-+2OH-⇌ClO-+H2O+2e- | 0.890 |
|  | 2Cl-⇌Cl2g+2e- | 1.358 |
| SO42- | SO32-+2OH-⇌SO42-+H2O+2e- | -0.930 |
|  | 2SO42-⇌S2O82-+2e- | 2.010 |
| Br- | Br-+6OH-⇌BrO3-+3H2O+6e- | 0.61 |
|  | Br-+2OH-⇌BrO-+H2O+2e- | 0.761 |
|  | 2Br-⇌Br2aq+2e- | 1.0873 |
| F- | 2F-⇌F2+2e- | 2.866 |
| Na+ | Na⇌Na++e- | -2.71 |
| Mg2+ | Mg⇌Mg2++2e- | -2.372 |
| Ca2+ | Ca⇌Ca2++2e- | -2.868 |
| K+ | K⇌K++e- | -2.931 |
| Sr2+ | Sr⇌Sr2++2e- | -2.899 |
| B(OH)3 |  |  |
| B(OH)4- |  |  |
| CO2* a |  |  |
| HCO3- b |  |  |
| CO32- c |  |  |
| OH- | 4OH-⇌O2+2H2O+4e- | 0.401 |
| H+ |  |  |
| Notes: a, b, c, Different forms of CO2 existing in impurified water cannot be reduced or oxidized under mild conditions due to their chemical durability. d, E0 [VSHE] denotes the standard potential of reactions relative to the potential of the standard H2 electrode (0 V). Not all impurified species have intensive concentrations in impurified water like seawater or unbuffered wastewater, except Cl-, Br-, Na+, Ca2+, Mg2+, which normally interfere with water electrolysis as competitive ions since they restrict the mass transfer and redox of protons (H+) or hydroxide ions (OH-). |  |  |

| ### Table S28 ¦ Current strategies for direct electrolysis of water with pollutants (e.g., raw seawater) and their specific electricity consumption for 1kg H2 production. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Strategy (Catalysts/configurations) | Current density (mA·cm-2) | Applied voltage (V) | Hydrogen yield (Nm3·m-2·h-1) | Electricity consumption (kWh·kg-1 H2) |
| a 1 73 | Forward-osmosis membrane, driven by concentrated alkaline electrolytes. | 950 | 1.79-1.82 | —— b | 44.05 c |
| 2 74 | Sodium-ion conducting exchange membrane and Pt atom anchored on Ni-Fe-P nanowires. | 400 | 1.66 | —— | 43.87 |
| 3 75 | Hydrophobic porous polytetrafluoroethylene (PTFE)-based waterproof breathable membrane with concentrated KOH solutions serving as self-dampening electrolytes. | 250-400 | 1.95-2.30 | —— | 50.05-61.18 |
| 4 76 | Decoupled water electrolysis employing nearly neutral NaBr electrolytes. | 1000 | 2.40 | 3.75-5.00 d | 57.43-64.61 e |
| 5 77 | Lewis-acid-modified electrodes (Cr2O3–CoOx) featuring a modified local reaction environment in acid electrolytes. | 1000 | 1.87 | 3.75-5.00 d | 44.74-50.34 |
| 6 78 | Ir-HxWO3 catalyst with a tailored acid-like microenvironment in neutral electrolytes. | 10 | 1.25 | 0.02-0.04 d | 39.65-53.05 |
| 7 79 | Catalyst involving an amorphous molybdenum oxide layer constructed on the beaded-like cobalt oxide interface via atomic layer deposition. | 1000 | 1.93 | 4.19 | 51.39 |
| 8 80 | Earth-abundant layered double hydroxide electrocatalyst (i.e., CoFe-Ci@GQDs/NF). | 1250 | 2.00 | 3.75-5.00 d | 59.81-67.31 |
| 9 81 | Microscopic bubble/precipitate traffic system (MBPTS) constructed with honeycomb-type 3D cathodes for robust anti-precipitation. | 500 | 3.30-3.40 | —— | 90.6 f |
| 10 82 | Atomic Ir catalyst on cobalt iron layered double hydroxide (Ir/CoFe-LDH) tailoring Cl- adsorption and modulating the electronic structure of the Ir active centers. | 800 | 2.20-2.30 | 3.75-5.00 d | 43.04-48.50 |
| 11 83 | Corrosion-resistant RuMoNi electrocatalyst with in situ-formed molybdate ions on its surface to repel chloride ions. | 500 | 1.62 | 2.09 | 43.00 |
| 12 84 | Bifunctional catalyst (Ru-Ni (Fe) P2/NF) with partial substitution of Fe to Ni atoms in Ni (Fe) P2. | 1000 | 2.16 | 3.75-5.00 d | 51.75-58.23 |

| Table S28 ¦ Current strategies for direct electrolysis of water with pollutants (e.g., raw seawater) and their specific electricity consumption for 1kg H2 production (continue). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Strategy (Catalysts/configurations) | Current density (mA·cm-2) | Applied voltage (V) | Hydrogen yield (Nm3·m-2·h-1) | Electricity consumption (kWh·kg-1 H2) |
| 13 85 | Binder-free electrode, designed and constructed by in-situ growth of homogeneous CoP on rGO nanosheets wrapped around the surface of Ti fiber felt. | 10 | 1.26 | 0.02-0.04 d | 39.90-53.38 |
| 14 86 | Ni-based electrodes with catalysts (e.g., NiFe-LDH). | 400 | 2.50-2.60 | 1.5-2.5 d | 50.73-62.73 |
| Notes: a, References are provided alongside the number series. b, The mark “——” indicates that values of H2 yield in references are not reported, whereas values of c, electricity consumption for H2 production, are directly reported, as demonstrated by No. 1, 2, 3, 9. d, These values of H2 yield are assumed based on the industrial electrolysis, as demonstrated in Table S8 and Table S24, due to the lack of results reported in related references. Specifically, values of current density and applied voltage are used to obtain the assumed values by inserting them into regression lines, which yield the median values as the assumed values. e, Results of electricity consumption (ECSW, kWh·kg-1 H2) are calculated using the following equation: ECSW=(10×CD×AV×∆t×S)/1000(HY/ρH2) where CD (mA·cm-2) is the current density of electrolyzers, AV (V) is the applied voltage between anodes and cathodes, ∆t (1h as default value) is the electrolysis duration, S (1m2 as default value) is the active area of electrolysis, HY (Nm3·m-2·h-1) is the yield of electrolytic H2 (with Nm3 representing the gas volume at normal conditions), and ρH2 (kg/Nm3) is the density of H2 at normal conditions. f, Values deviating significantly from the most probable range (~ 43.0 to 67.3 kWh·kg-1 H2) are not considered in subsequent analyses. |  |  |  |  |  |

## 3.6 Calculations of the negative-carbon effects of electrolysis.

### Method S6 | Calculation of anodic oxygen in electrolyzers as the by-product for energy-saving aeration in WWTPs’ aerobic tank.

The energy saving of aeration methods (ESA, %) when the anodic oxygen produced from electrolyzers is fed into the aerobic tank can be estimated based on the required gas volume when conventional air and anodic oxygen are respectively fed. This estimation assumes that the energy consumption of blasting aeration pump is linearly related with the transport amount of gas 87-91. The volume of required gas (V, L) for both air and anodic oxygen can be calculated as follows:

V=mO2/(CPO2×ρ) equation S34

where mO2 (kg) is the mass of 1 kg oxygen (O2), CPO2 (%) is the composition purity of O2 in air or anodic oxygen, ρ (kg/L) is the gas density of air or anodic oxygen at temperature ranging from 10 °C to 30 °C, which is the typical operation temperature of wastewater treatment plants. Related definition of parameters with uncertainty is provided in the Data S2, while the CPO2 of anodic oxygen (CPO2,anode, %) is calibrated according to the following equation based on the commercial alkaline electrolysis.

CPO2,anode=CPO2,mark-100×(mH2O×CCl-/2)(mH2O-mH2) equation S35

where CPO2,mark (%) is the ideal marked oxygen purity of commercial alkaline electrolyzers when pure water is electrolyzed, mH2O (kg H2O/kg H2) is the calibrated specific water consumption when reclaimed water is directly electrolyzed to produce H2, CCl- (kg/kg H2O) is the typical concentration of chloride ions in reclaimed water, while (mH2O×CCl-/2) (kg Cl2) demonstrate the by-production amount of chlorine (Cl2) in anodes besides oxygen evolution, which is defined as the major impurities during alkaline electrolysis here since other pollutants in reclaimed water is less obvious and cannot significantly influence the water electrolysis as shown in Table S10. mH2 (kg) is the production amount of H2 (1 kg) consuming specific reclaimed water, while (mH2O-mH2) demonstrates the mass summary of anodic oxygen and chlorine. The energy saving of aeration methods (ESA, %) thus can be calculated as follows:

ESA=100×VO2,air-VO2,anode O2VO2,air equation S36

where VO2,air (L) is the volume required for air with 1kg O2 content, while VO2,anode O2 (L) is the volume required for anodic oxygen with 1kg O2 content. Results are also shown in Figure S23.

| Data S2 ¦ Definition of parameters for calculating the energy saving of aeration methods and formation amount of NaClO3 in electrolytes. |  |  |
| --- | --- | --- |
| Items | Unit | Value Range |
| Temperature | °C | 10-30 a |
| Density of air | 10-3 kg/L | 1.164-1.247 b |
| O2 content in air | % | 20.8-21.0 c |
| Density of O2 | 10-3 kg/L | 1.308-1.430 d |
| Anodic O2 content electrolyzing pure water | % | 99.3-99.7 e |
| Specific consumption of reclaimed water | kg H2O/kg H2 | 9.47-12.59 f |
| Concentration of chloride ions in reclaimed water | kg/kg H2O | 0.5-0.8 g |
| Ratio of chlorine evolution in anodic chamber | —— | 0.0-1.0 h |
| Molecular mass of NaClO3 | g/mol | 106.44 |
| Molecular mass of Cl2 | g/mol | 70.9 |
| Notes: a, The temperature is determined based on the typical operation scenarios of wastewater treatment plants through hierarchical seasons. b, d, The density is determined based on the variation range of temperature. c, The O2 content is determined based on a common scenario in domestic area with regular features. e, The O2 content is determined based on the ideal scenario when alkaline electrolyzers electrolyze pure water without any impurities, while partial water vapor exists in the output of anodic oxygen. f, The value is calibrated based on the water quality indexes and industrial models as demonstrated in Tables S10, S11. g, The value is determined based on the water quality indexes of typical reclaimed water in China. h, The ratio is determined based on scenarios that chlorine evolution in anodic chamber is completely restricted (ratio=0.0) or completely not restricted (ratio=1.0) to cover the most probable interfering scenarios from chloride ions. |  |  |

Notably, the by-production of anodic Cl2 is traditionally fed into concentrated alkaline (sodium hydroxide) solutions, potentially in-situ adsorbed by the electrolyte based on the following reactions:

2NaOH+Cl2=NaCl+NaClO+H2O

6NaOH+3Cl2=5NaCl+NaClO3+3H2O

where the first reaction normally occurs at ambient temperature, the second reaction would occur when the solution is heated to temperature exceeding 70 °C, which is the typical operation temperature of commercial alkaline electrolyzers. Thus, NaClO3 should be the major by-products in the electrolytes. Based on their stoichiometric relationship and molecular mass provided in Data S2, the production amount based on per kg of O2 fed into aerobic tank can be obtained with value of 0.14±0.12 kg NaClO3/kg O2. Since NaClO3 has lower or negligible capacity of oxidation corrosion in (weak) alkaline solutions, its existence in the electrolytes should not significantly influence the stability of electrolysis.

### Figure S23 | Volume of gas fed into the aerobic tank using either air or anodic oxygen. An energy saving of 76.36±4.94 % for the aeration blasting pump can be achieved when anodic oxygen is used as a replacement for air. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. Detailed calculations and parameters for this part can be found in Method S6 and Data S2.

# 4. Prediction of the electrolytic H2 production in the future.

## 4.1 Prediction of the future electrolytic H2 production.

To elucidate the impacts of H2 production in the future, this section forecasts the future scenarios for H2 production, carbon emission, and composite cost of each technical route. The spatial scale predicting future scenarios is the entire China. The time span is set from 2021 to 2060, encompassing the process of achieving carbon peak and neutrality that China is currently transiting.

Significantly, the structure of electric grids profoundly impacts the cleanliness and cost of electrolytic H2. Scenarios of carbon peak and neutrality for China are determined with the following assumptions, that consumption of non-fossil fuels reaches ~25% and reduce 65% carbon emission based on that of 2005 for carbon peak at 2030, while consumption of non-fossil fuels reaches >80% and achieves zero carbon emission nationwide for carbon neutrality at 2060. The interval years (2021-2030 and 2030-2060) are assumed with uniformly linear progress to 2030 and 2060. A method based on the Kaya Identity (EMCO2) (Figure S24) and LEAP model (the Low Emissions Analysis Platform, https://leap.sei.org/, Figure S25), is utilized to predict the future structure and cost of electric grids 92, which is primarily driven by carbon emissions from fossil-fuels consumption, population, and national economies as follows:

EMCO2=CO2E×EGDP×GDPP×P equation37

where CO2 (Mt CO2-eq) represents the total carbon emission from energy consumption, E (Mt coal-eq) denotes the total primary energy consumption, GDP (2021 104 $) signifies the national gross domestic products, and P (capita) indicates the national population. The influence of these parameters on the total carbon emission (EMCO2, Mt CO2-eq) is depicted in Figure S24. Consequently, based on the predicted variation of these parameters, the structure of future electric grids can be evaluated. Using the method of lowest levelized cost of energy, the future electricity cost of different power types is also evaluated (Figure S26). Notably, the carbon emission factors of different electricity types are assumed to approximate the 2021 scenarios, according to the estimation guidelines provided by the Intergovernmental Panel on Climate Change 93.

Population is a significant factor affecting future available water resources (Aw,future, 108 m3/year), particularly reclaimed water and tap water, for water electrolysis. This can be evaluated as follows.

Aw,future=Pi,futurePpresent×Aw,present equation 38

where the subscript i represents decennial years in the future: 2030, 2040, 2050, 2060. Pi,future (104 capita) is the future population, Ppresent (104 capita) is the present population, and Aw,present (108 m3/year) is the present available reclaimed water or tap water. Additionally, future available desalinated seawater and offshore wind power are predicted based on infrastructure planning published by the Chinese government, which are defined with the current growth rate and trend. Based on socio-economic predictions from The United Nations, future available resources are predicted.

Advancements of alkaline electrolysis also influence specific electricity (SCEfuture, kWh/kg H2) and water consumption (SCWfuture, kg H2O/kg H2). Future scenarios can be modified based on data and equations 39-40, as improvements in current efficiency (CC, %) would decrease actual electricity and water consumption.

SCEfuture=(1-CCi-CCpresent)×SCEpresent equation 39

SCWfuture=(1-CCi-CCpresent)×SCWpresent equation 40

where the subscript i represents decennial years in the future: 2030, 2040, 2050, 2060. SCEpresent is the specific consumption of electricity (kWh/kg H2) at present while SCWpresent is the specific consumption of water (kg H2O/kg H2) at present.

## 4.2 Future technical level of the electrolysis and the future social development.

| ### Table S29 ¦ Prediction of technical progress in alkaline water electrolysis 34. |  |  |  |
| --- | --- | --- | --- |
| Parameters | Periods |  |  |
|  | Present (2021) | 2030 | Long-term (2040-2060) |
| Electrical efficiency (%, LHV) a | 63-70 | 65-71 | 70-80 |
| Operation pressure (bar) | 1-30 | —— | —— |
| Operation temperature (°C) | 60-80 | —— | —— |
| Stack lifetime (operating hours) | 60,000-90,000 | 90,000-100,000 | 100,000-150,000 |
| Load range (% relative to nominal load) | 10-110 | —— | —— |
| Plant footprint (m2/kW) | 0.095 | —— | —— |
| CAPEX (2021 $/kW) b | 500-1400 | 400-850 | 200-700 |
| Notes: a, LHV denotes the lower heat value of H2 (242 kJ/mol). b, CAPEX denotes capital expenditure. |  |  |  |

| ### Table S30 ¦ Prediction of parameters used for Kaya identity in the future 94,95. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters | Year |  |  |  |
|  | Present (2021) | 2030 | 2050 | 2060 |
| Population (108 capita) a | 14.00 | 14.64 | 14.02 | 13.33 |
| GDP (1012 CNY) | 99 | 169 | 338 | 435 |
| Industrial structure (%) b | 7:39:54 | 6:37:57 | 4:33:63 | 4:30:66 |
| Urbanization (%) | 61 | 66-68 | 78-81 | 83-85 |
| Consumption of primary energy (108 ton coal-eq) | 48.6 | 60 | 60 | 59 |
| Consumption of electricity (1012 kWh) | 7.25 | 10.7 | 16 | 17 |
| Notes: a, Data is obtained from the sustainable development report provided by the United Nations. b, The industrial structure is composed of the first (agriculture), second (industry), and third (commerce and service) industries. |  |  |  |  |

## 4.3 Prediction framework of the future scenarios.

### Figure S24 | Reduction schematic of national carbon emission (EMCO2) based on the Kaya identity. The major future scenarios for the energy transition of China towards carbon peak at 2030 and carbon neutrality at 2060 include the increase in population (P) and gross domestic product per capita (GDP/P), decrease in carbon emission per capita (CO2/P) and energy consumption intensity per unit of output value (E/GDP), synchronous progress of social decarbonization and electrification in all industries at the national level.

### Figure S25 | Structure of LEAP’s calculations for the prediction of future structure and cost of electricity based on the scenarios towards carbon peak at 2030 and carbon neutrality at 2060 in China 96. Variation in national population and macro-economics (e.g., industrial structure, gross domestic product, urbanization ratio) is the dominant driving force towards energy transition with the defined limitation of fossil-fuels energy consumption and renewable energy consumption.

### Figure S26 | Future electricity cost of different types’ power including coal, natural-gas, biomass, solar, wind, hydro, nuclear and additional electricity transmission based on the prediction by the LEAP model with defined input parameters and limitation of energy consumption. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The filling area represents the mean value, while the expanded solid lines around these filling areas represent their uncertainty range, but the uncertainty range is not obvious. The results of predicted scenarios for future electricity prices undergo smoothing processing to illustrate the trend of continuous change between decades. The left axis is displayed with a linear function.

### Figure S27 | Future cost and carbon emissions of onshore and offshore electric grids. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. a, Levelized cost and b, levelized carbon emissions of onshore and offshore electric grids, which are calculated according to the data from the Figure S21, Figure S26, and Table S17. The solid lines represent the mean values, while the expanded shades around these lines represent their uncertainty range. The results of predicted scenarios for future electric grids undergo smoothing processing to illustrate the trend of continuous change between decades. The left axis is displayed with a linear function.

## 4.4 Prediction of the future H2 demand, electricity prices, and the resource consumption by electrolytic H2.

### Figure S28 | Variation of H2 production and consumption demand from 2021 to 2060. a, Structure of H2 production including refining of fossil-fuels (e.g., coal gasification, natural-gas reforming), industrial by-production, and water electrolysis from 2021 to 2060. With the progress of energy transition in China, the contribution of water electrolysis for H2 production would significantly increase and dominate the national H2 supply. A 1% variation of H2 production is defined here as the uncertainty. The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The filling area represents the mean value, while the expanded dashed lines around these filling areas represent their uncertainty range. The results of predicted scenarios for future H2 production undergo smoothing processing to illustrate the trend of continuous change between decades. The left axis is displayed with a log10 function. b, Structure of H2 consumption in various industries including conventional industries (e.g., ammonia synthesis), fuels/feedstocks for traffic, power, heating, and grids’ adjustment, etc., which is exhibited in the basic scenario here. Above data is obtained based on the scenario in China’s governments’ planning and China Hydrogen Council 97,98. The left axis is displayed with a linear function.

### Figure S29 | Consumption of water and electricity for the electrolytic H2 production from 2021 to 2060. Comparisons of the consumed and available a, water and b, electricity for producing electrolytic hydrogen to meet the national hydrogen demand. The available amount of raw seawater is not demonstrated herein since this route is only restricted by its access to the available offshore wind power. All sub-figures in a use the same range of Y-axis (1-103 exhibited with a logarithm (base 10) function), and all sub-figures in b use the same range of Y-axis (1-107 exhibited with a logarithm (base 10) function). The results are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials.

### Figure S30 | Percentage of resources used for electrolytic H2 production in the corresponding available resources from 2021 to 2060 (electricity in solid lines and water in dashed lines) when resources are only used to supply a, the national H2 produced by water electrolysis and b, the whole national H2 demand, displayed on an axis with a log10 function. All data clusters are displayed as mean ± standard deviation within a 5-95% confidence interval, based on 100,000 Monte Carlo simulation trials. The lines represent the mean value, while the expanded filling shade around these lines, whether solid or dashed, represents their uncertainty range. The results of predicted scenarios for future H2 production undergo smoothing processing to illustrate the trend of continuous change between decades.

# Supplementary References

1	Measures for the administration of permits for discharge of urban wastewater into municipal collection networks. (Ministry of Housing and Urban-Rural Development of China, 2023); https://www.mohurd.gov.cn/gongkai/zhengce/zhengceguizhang/202212/20221212_769349.html.

2	Discharge standards for water pollutants. (Ministry of Ecology and Environment of China, 2023); https://www.mee.gov.cn/ywgz/fgbz/bz/bzwb/shjbh/swrwpfbz/.

3	Discharge standard of pollutants for municipal waste-water treatment plant. (Ministry of Ecology and Environment of China, 2002); https://www.mee.gov.cn/ywgz/fgbz/bz/bzwb/shjbh/swrwpfbz/200307/t20030701_66529.shtml.

4	Guo, H., Li, X., Ren, Q. et al. Water quality features of influents discharged into typical municipal wastewater treatment plants in China. Water & Wastewater Engineering 54, 12-15 (2018).

5	Alvarado, V. I., Hsu, S. C., Lam, C. M. & Lee, P. H. Beyond Energy Balance: Environmental Trade-Offs of Organics Capture and Low Carbon-to-Nitrogen Ratio Sewage Treatment Systems. Environmental Science & Technology 54, 4746-4757 (2020).

6	de Faria, A. B. B., Spérandio, M., Ahmadi, A. & Tiruta-Bama, L. Evaluation of new alternatives in wastewater treatment plants based on dynamic modelling and life cycle assessment (DM-LCA). Water Research 84, 99-111 (2015).

7	Li, Y., Luo, X. Y., Huang, X. W., Wang, D. W. & Zhang, W. L. Life Cycle Assessment of a municipal wastewater treatment plant: a case study in Suzhou, China. Journal of Cleaner Production 57, 221-227 (2013).

8	Medina-Martos, E. et al. Techno-economic and life cycle assessment of an integrated hydrothermal carbonization system for sewage sludge. Journal of Cleaner Production 277, 122930 (2020).

9	Petit-Boix, A. et al. Addressing the Life Cycle of Sewers in Contrasting Cities through an Eco-Efficiency Approach. Journal of Industrial Ecology 22, 1092-1104 (2018).

10	Pfluger, A. R. et al. Lifecycle Comparison of Mainstream Anaerobic Baffled Reactor and Conventional Activated Sludge Systems for Domestic Wastewater Treatment. Environmental Science & Technology 52, 10500-10510 (2018).

11	Pintilie, L., Torres, C. M., Teodosiu, C. & Castells, F. Urban wastewater reclamation for industrial reuse: An LCA case study. Journal of Cleaner Production 139, 1-14 (2016).

12	Rodriguez-Garcia, G. et al. Environmental and economic profile of six typologies of wastewater treatment plants. Water Research 45, 5997-6010 (2011).

13	Singh, A. D., Upadhyay, A., Shrivastava, S. & Vivekanand, V. Life-cycle assessment of sewage sludge-based large-scale biogas plant. Bioresource Technology 309, 123373 (2020).

14	Zhang, Y. Z. et al. Wastewater treatment technology selection under various influent conditions and effluent standards based on life cycle assessment. Resources Conservation and Recycling 154, 104562 (2020).

15	Del Borghi, A., Strazza, C., Gallo, M., Messineo, S. & Naso, M. Water supply and sustainability: life cycle assessment of water collection, treatment and distribution service. International Journal of Life Cycle Assessment 18, 1158-1168 (2013).

16	Fayyaz, S. et al. Life cycle assessment of reverse osmosis for high-salinity seawater desalination process: Potable and industrial water production. Journal of Cleaner Production 382, 135299 (2023).

17	Staub, M. et al. Aquifer recharge with reclaimed water: life-cycle assessment of hybrid concepts for non-potable reuse. Journal of Water Reuse and Desalination 5, 142-148 (2015).

18	Ecoinvent 3.8 Database. (PRé Sustainability, 2023); https://simapro.com/.

19	Elsaid, K. et al. Environmental impact of emerging desalination technologies: A preliminary evaluation. Journal of Environmental Chemical Engineering 8, 104099 (2020).

20	Harris, S., Tsalidis, G., Corbera, J. B., Gallart, J. J. E. & Tegstedt, F. Application of LCA and LCC in the early stages of wastewater treatment design: A multiple case study of brine effluents. Journal of Cleaner Production 307, 127298 (2021).

21	Ibrahim, Y., Arafat, H. A., Mezher, T. & AlMarzooqi, F. An integrated framework for sustainability assessment of seawater desalination. Desalination 447, 1-17 (2018).

22	Najjar, E., Al-Hindi, M., Massoud, M. & Saad, W. Life cycle assessment and cost of a seawater reverse osmosis plant operated with different energy sources. Energy Conversion and Management 268, 115964 (2022).

23	Panagopoulos, A. & Haralambous, K. J. Environmental impacts of desalination and brine treatment - Challenges and mitigation measures. Marine Pollution Bulletin 161, 111773 (2020).

24	Razman, K. K., Hanafiah, M. M. & Mohammad, A. W. An overview of LCA applied to various membrane technologies: Progress, challenges, and harmonization. Environmental Technology & Innovation 27, 102803 (2022).

25	Tarpani, R. R. Z., Lapolli, F. R., Recio, M. A. L. & Gallego-Schmid, A. Comparative life cycle assessment of three alternative techniques for increasing potable water supply in cities in the Global South. Journal of Cleaner Production 290, 125871 (2021).

26	Zhou, J., Chang, V. W. C. & Fane, A. G. An improved life cycle impact assessment (LCIA) approach for assessing aquatic eco-toxic impact of brine disposal from seawater desalination plants. Desalination 308, 233-241 (2013).

27	Akhtar, M. S., Dickson, R. & Liu, J. J. Life Cycle Assessment of Inland Green Hydrogen Supply Chain Networks with Current Challenges and Future Prospects. Acs Sustainable Chemistry & Engineering 9, 17152-17163 (2021).

28	Kuckshinrichs, W. & Koj, J. C. Levelized cost of energy from private and social perspectives: The case of improved alkaline water electrolysis. Journal of Cleaner Production 203, 619-632 (2018).

29	Lee, B. et al. Integrative techno-economic and environmental assessment for green H2 production by alkaline water electrolysis based on experimental data. Journal of Environmental Chemical Engineering 9, 106349 (2021).

30	Palmer, G., Roberts, A., Hoadley, A., Dargaville, R. & Honnery, D. Life-cycle greenhouse gas emissions and net energy assessment of large-scale hydrogen production via electrolysis and solar PV. Energy & Environmental Science 14, 5113-5131 (2021).

31	Parra, D., Zhang, X. J., Bauer, C. & Patel, M. K. An integrated techno-economic and life cycle environmental assessment of power-to-gas systems. Applied Energy 193, 440-454 (2017).

32	Terlouw, T., Bauer, C., McKenna, R. & Mazzotti, M. Large-scale hydrogen production via water electrolysis: a techno-economic and environmental assessment. Energy & Environmental Science 15, 3583-3602 (2022).

33	Zhao, G. L. et al. Life cycle assessment of H2O electrolysis technologies. International Journal of Hydrogen Energy 45, 23765-23781 (2020).

34	Global Renewables Outlook: Energy transformation 2050. (IRENA, 2020); https://www.irena.org/publications/2020/Apr/Global-Renewables-Outlook-2020.

35	Global Hydrogen Review 2022 (International Energy Agency, 2022); https://www.iea.org/reports/global-hydrogen-review-2022.

36	Wu, Z., Zhai, H., Grol, E.J. et al. Treatment of brackish water for fossil power plant cooling. Nature Water 1, 471–483 (2023).

37	Shih, A. J. et al. Water electrolysis. Nature Reviews Methods Primers 2, 84 (2022).

38	Technical parameters of atmospheric alkaline electrolyser. (NEL, 2023); https://nelhydrogen.com/product/atmospheric-alkaline-electrolyser-a-series/.

39	Integrated wastewater discharge standard. (Ministry of Ecology and Environment of China, 1996); https://www.mee.gov.cn/ywgz/fgbz/bz/bzwb/shjbh/swrwpfbz/199801/t19980101_66568.shtml.

40	Environmental quality standards for surface water. (Ministry of Ecology and Environment of China, 2002); https://www.mee.gov.cn/ywgz/fgbz/bz/bzwb/shjbh/shjzlbz/200206/t20020601_66497.shtml.

41	Standards for drinking water quality. (National Health Commission of China, 2022); https://openstd.samr.gov.cn/bzgk/gb/newGbInfo?hcno=99E9C17E3547A3C0CE2FD1FFD9F2F7BE.

42	Sea water quality standard. (Ministry of Ecology and Environment of China, 1997); https://www.mee.gov.cn/ywgz/fgbz/bz/bzwb/shjbh/shjzlbz/199807/t19980701_66499.shtml.

43	China Energy Statistical Yearbook. (State Statistics Bureau of China, 2021); http://www.stats.gov.cn/sj/ndsj/.

44	The third price regulation of provincial electricity transmission and distribution. (National Development and Reform Commission of China, 2021); https://www.ndrc.gov.cn/xxgk/zcfb/tz/202305/t20230515_1355747.html.

45	Outline of the national water network construction plan. (State Council of China, 2023); https://www.gov.cn/zhengce/202305/content_6876214.htm.

46	UN World Water Development Report. (The United Nations, 2023); https://www.unwater.org/publications/un-world-water-development-report-2023.

47	The Urban Constructure Statistical Yearbook (Ministry of Housing and Urban-Rural Development of China, 2021); https://www.mohurd.gov.cn/gongkai/fdzdgknr/sjfb/tjxx/index.html.

48	The National Seawater Utilization Report. (Ministry of Natural Resources of China, 2021); https://www.gov.cn/xinwen/2022-09/28/content_5713053.htm.

49	The Urban Constructure Statistical Yearbook (Ministry of Housing and Urban-Rural Development of China, 2021); https://www.mohurd.gov.cn/gongkai/fdzdgknr/sjfb/tjxx/index.html.

50	The Water Resources Bulletin. (Ministry of Water Resources of China, 2021); http://www.mwr.gov.cn/sj/tjgb/szygb/202206/t20220615_1579315.html.

51	The Statistical Yearbook. (State Statistics Bureau of China, 2021); http://www.stats.gov.cn/sj/ndsj/.

52	UN World Water Development Report. (The United Nations, 2023); https://www.unwater.org/publications/un-world-water-development-report-2023.

53	Report of “The development of renewable energy in China reached a new level at 2021”. (National Energy Administration of China, 2021); https://www.gov.cn/xinwen/2022-01/29/content_5671076.htm.

54	The National Seawater Utilization Report. (Ministry of Natural Resources of China, 2021); https://www.gov.cn/xinwen/2022-09/28/content_5713053.htm.

55	The Communique on the State of China's Marine Ecological Environment. (Ministry of Ecology and Environment of China, 2021); https://www.mee.gov.cn/hjzl/sthjzk/jagb/.

56	Wind Speed and Wind Power Potential Maps. (THE WORLD BANK Data Catalog, 2023); https://datacatalog.worldbank.org/search/dataset/0039490/World---Wind-Speed-and-Wind-Power-Potential-Maps.

57	Technical conditions of pressurized water electrolysis system for hydrogen production. (Standardization Administration of China, 2019); https://openstd.samr.gov.cn/bzgk/gb/newGbInfo?hcno=D011EE625481BD0F5F33AA77BEF841D8.

58	Product Data Sheet. FilmTec™ Eco Pro-440i. Membranes. Form No. 45-D01715-en, Rev. 2. (DuPont™, 2020); https://www.dupont.com/products/filmtececopro440i.html.

59	DuPont™. Product data sheet. FilmTec™ BW30-400 Membranes. Form No. 45-D01505-en, Rev. 4. (DuPont™, 2020); https://www.dupont.com/content/dam/dupont/amer/us/en/water-solutions/public/documents/en/RO-FilmTec-BW30-400-PDS-45-D01505-en.pdf.

60	Product data sheet. FilmTec™ XLE-440 Membranes. Form No. 45-D01513-en, Rev. 4. (DuPont™, 2020); https://www.dupont.com/content/dam/dupont/amer/us/en/water-solutions/public/documents/en/RO-FilmTec-XLE-440-PDS-45-D01513-en.pdf.

61	Water Treatment for Industrial and Power Markets. Packed Bed Ion Exchange Systems. (Dupont™, 2019); https://www.dupont.com/resource-center.html?BU=water-solutions.

62	Cho, K. & Hoffmann, M. R. Molecular hydrogen production from wastewater electrolysis cell with multi-junction BiOx/TiO2 anode and stainless steel cathode: Current and energy efficiency. Applied Catalysis B-Environmental 202, 671-682 (2017).

63	Cho, K., Kwon, D. & Hoffmann, M. R. Electrochemical treatment of human waste coupled with molecular hydrogen production. Rsc Advances 4, 4596-4608 (2014).

64	Cho, K. et al. Effects of Anodic Potential and Chloride Ion on Overall Reactivity in Electrochemical Reactors Designed for Solar-Powered Wastewater Treatment. Environmental Science & Technology 48, 2377-2384 (2014).

65	Chung, C. M., Hong, S. W., Cho, K. & Hoffmann, M. R. Degradation of organic compounds in wastewater matrix by electrochemically generated reactive chlorine species: Kinetics and selectivity. Catalysis Today 313, 189-195 (2018).

66	Huang, X. et al. Electrochemical disinfection of toilet wastewater using wastewater electrolysis cell. Water Research 92, 164-172 (2016).

67	Martínez-Huitle, C. A. & Panizza, M. Electrochemical oxidation of organic pollutants for wastewater treatment. Current Opinion in Electrochemistry 11, 62-71 (2018).

68	Guo, Y., Peng, L., Tian, J. & Mauzerall, D. L. Deploying green hydrogen to decarbonize China's coal chemical sector. Nature Communications 14, 8104 (2023).

69	Hauglustaine, D. et al. Climate benefit of a future hydrogen economy. Communications Earth & Environment 3, 295 (2022).

70	Energy statistical method. (State Statistics Bureau of China, 2021); http://www.stats.gov.cn/sj/zbjs/202302/t20230202_1897102.html.

71	Zhang, S., Chen, W. Assessing the energy transition in China towards carbon neutrality with a probabilistic framework. Nature Communications 13, 87 (2022).

72	Dresp, S., Dionigi, F., Klingenhof, M. & Strasser, P. Direct Electrolytic Splitting of Seawater: Opportunities and Challenges. Acs Energy Letters 4, 933-942 (2019).

73	Cassol, G. S. et al. Ultra-fast green hydrogen production from municipal wastewater by an integrated forward osmosis-alkaline water electrolysis system. Nature Communications 15, 2617 (2024).

74	Shi, H. et al. A sodium-ion-conducted asymmetric electrolyzer to lower the operation voltage for direct seawater electrolysis. Nature Communications 14, 3934 (2023).

75	Xie, H. et al. A membrane-based seawater electrolyser for hydrogen generation. Nature 612, 673-678 (2022).

76	Slobodkin, I., Davydova, E., Sananis, M., Breytus, A. & Rothschild, A. Electrochemical and chemical cycle for high-efficiency decoupled water splitting in a near-neutral electrolyte. Nature Materials 23, 398-405 (2024).

77	Guo, J. et al. Direct seawater electrolysis by adjusting the local reaction environment of a catalyst. Nature Energy 8, 264-272 (2023).

78	Zheng, X. et al. Tailoring a local acid-like microenvironment for efficient neutral hydrogen evolution. Nature Communications 14, 4209 (2023).

79	Zhou, L. et al. A restricted dynamic surface self-reconstruction toward high-performance of direct seawater oxidation. Nature Communications 15, 2481 (2024).

80	Fan, R. et al. Ultrastable electrocatalytic seawater splitting at ampere-level current density. Nature Sustainability 7, 158-167 (2024).

81	Liang, J. et al. Efficient bubble/precipitate traffic enables stable seawater reduction electrocatalysis at industrial-level current densities. Nature Communications 15, 2950 (2024).

82	Duan, X. et al. Dynamic chloride ion adsorption on single iridium atom boosts seawater oxidation catalysis. Nature Communications 15, 1973 (2024).

83	Kang, X. et al. A corrosion-resistant RuMoNi catalyst for efficient and long-lasting seawater oxidation and anion exchange membrane electrolyzer. Nature Communications 14, 3607 (2023).

84	Wu, D. et al. Fe‐Regulated Amorphous‐Crystal Ni(Fe)P2 Nanosheets Coupled with Ru Powerfully Drive Seawater Splitting at Large Current Density. Small 19, 2300030 (2023).

85	Xu, X. et al. Corrosion-resistant cobalt phosphide electrocatalysts for salinity tolerance hydrogen evolution. Nature Communications 14, 7708 (2023).

86	Zhang, S. et al. Concerning the stability of seawater electrolysis: a corrosion mechanism study of halide on Ni-based anode. Nature Communications 14, 4822 (2023).

87	Kang, K. H., Lee, G. & Kim, J. K. Efficient aerobic denitrification in the treatment of leather industry wastewater containing high nitrogen concentration. Environmental Engineering Research 20, 79-87 (2015).

88	Lee, I., Lim, H., Jung, B., Colosimo, M. F. & Kim, H. Evaluation of aeration energy saving in two modified activated sludge processes. Chemosphere 140, 72-78 (2015).

89	Song, K. et al. Effects of aeration and internal recycle flow on nitrous oxide emissions from a modified Ludzak-Ettinger process fed with glycerol. Environmental Science and Pollution Research 22, 19562-19570 (2015).

90	Terashima, M., So, M., Goel, R. & Yasui, H. Determination of diffuser bubble size in computational fluid dynamics models to predict oxygen transfer in spiral roll aeration tanks. Journal of Water Process Engineering 12, 120-126 (2016).

91	Zhao, Y. G., Guo, X. M., She, Z. L., Gao, M. C. & Guo, L. Effect of gradual-increasing aeration mode in an aerobic tank on nutrients' removal and functional microbial communities. Environmental Technology 38, 2621-2628 (2017).

92	Wang, X., Lu, Z., Li, T. & Zhang, P. Carbon-neutral power system transition pathways for coal-dominant and renewable Resource-abundant regions: Inner Mongolia as a case study. Energy Conversion and Management 285, 117013 (2023).

93	2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC, 2019); https://www.ipcc.ch/report/2019-refinement-to-the-2006-ipcc-guidelines-for-national-greenhouse-gas-inventories/.

94	Medium-Long Term Trends of Population Change in China (2021-2050). (United Nations Fund for Population Activities, 2022); https://china.unfpa.org/zh-Hans/publications/22070101.

95	World economic situation and prospects. (The United Nations, 2023); https://www.ndrc.gov.cn/wsdwhfz/202110/t20211012_1299485.html.

96	LEAP, the Low Emissions Analysis Platform. (Stockholm Environment Institute, 2023); https://leap.sei.org/default.asp?action=introduction.

97	Development of China hydrogen energy and fuel cell industry. (China Hydrogen Council, 2020); https://www.zjkhreri.org.cn/Report/1978.html.

98	Medium and Long-Term Plan for the Development of Hydrogen Energy Industry (2021-2035). (National Development and Reform Commission of China, National Energy Administration, 2022); http://zfxxgk.nea.gov.cn/2022-03/23/c_1310525630.htm.