Supporting Information

ANALYSIS OF MIXED COMPOSITION COLD PLASMA BY OPTICAL EMISSION

DIAGNOSTICS AND SIMULATIONS

Shani Har Lavana, Sharona Atlasa,b, Amir Kaplanb, Avi Lehrera, Illya Rozenberga, Hao Zhaoc,  Joshua H. Barabana

(a) Department of Chemistry, Ben-Gurion University of the Negev, P.O. Box 653, Beer-Sheva, 84105, ISRAEL

(b) Department of Chemical Engineering, Nuclear Research Center Negev, P.O. Box 9001, Beer-Sheva, 84190, ISRAEL

(c) College of Engineering, Peking University, Beijing 100871, PR China

Table of Contents

OES spectra	2

Nitrogen	2

Argon	3

Simulation examples of the SPS	3

Model outputs	4

Calculated temperature as a function of power	4

Reactor properties	5

Nitrogen plasma properties	6

Ar/N2 Mixtures	7

Rotational temperature	7

Comparison between simulated N2 and Ar plasma densities	9

Model description	10

Reaction set	10

Cross section data base	13

Model results	15

Calculating rotational temperature	15

Calculating vibrational temperature	16

References	16

# OES spectra

## Nitrogen

Fig. S1 Spectrum of nitrogen plasma obtained at 50 SCCM and 50 W, using the Flame-S spectrometer. The figure displays two nitrogen band systems, the First Positive System (FPS) and the Second Positive System (SPS), as well as the A2Σ+-X2Π transition of NO [1].

Fig. S2 A zoom in of the SPS emission at 50 SCCM and 50 W, recorded with a Horiba spectrometer 2400 lines/mm. The corresponding vibrational transitions are indicated with the notation (v'→v'').

## Argon

Fig. S3 Emission lines of Argon in its first ionization state (Ar I) at 100 SCCM and 100 W, recorded using a Flame-S spectrometer.

# Simulation examples of the SPS

Fig. S4 Example of PGOPHER simulation and fit for the rotational temperature of the (0,1) band, at 100 SCCM and 50W, with the rotational temperature determined  TRot=410 ±7 K.

Fig.  S5 Example of PGOPHER simulation and fit for the vibrational temperature, at 100 SCCM and 50W, with the vibrational temperature determined  TVib=6100 ±150 K. The 353 nm band corresponds to the (1,2) transition.

# Model outputs

## Calculated temperature as a function of power

Fig. S6-S7 The calculated (a) rotational and (b) vibrational temperatures as a function of plasma power, compared to our experimental data. The original calculation is marked in red, and the actual power scaling considering the reflected power is marked in blue.

## Reactor properties

Fig. S8 Calculated electron temperatures a function of plasma power. An increase in electron temperatures is observed, while the temperature hierarchy Te>Tvib> Trot  indicates a non-thermal equilibrium environment.

Fig. S9 Electron density as a function of plasma power. Increased plasma power, results in higher electron density, facilitating greater ionization and excitation processes.

## Nitrogen plasma properties

Fig. S10 Nitrogen ions plasma density as a function of power, at 100 SCCM.

Fig. S11 Increase in the reduced electric field in an N2 plasma as a function of power at 100 SCCM. Higher reduced electric field enhance the effectiveness of vibrational excitations.

# Ar/N2 Mixtures

## Rotational temperature

Fig. S12 Rotational temperatures obtained vs. Ar pressure with the addition of 4 SCCM of nitrogen.

Fig. S13 Rotational temperatures obtained vs. Ar pressure with the addition of 8 SCCM of nitrogen.

Fig. S14-S15: The (0,1) and (1,0) transitions at different concentrations, converge to a single rotational temperature at high Ar pressures.

## Comparison between simulated N2 and Ar plasma densities

Fig. S16 Variation in simulated nitrogen ions plasma density as a function of nitrogen pressure at 50 W in a pure nitrogen plasma.

Fig. S17 The simulated reduced electric field of Ar plasma density increasing as a function of pressure at 50 W in a pure Ar plasma.

# Model description

## Reaction set

| # | Reaction | Rate coefficient (m3/s) | Ref. |
| --- | --- | --- | --- |
|  | Elastic |  |  |
| 1 | e + N2 e + N2 | K(ε) | [2,3] |
|  | Excitation |  |  |
| 2 | e + N2 e + N2(A) | K(ε) | [2,3] |
| 3 | e + N2 e + N2(B) | K(ε) | [2,3] |
| 4 | e + N2 e + N2(B’) | K(ε) | [2,3] |
| 5 | e + N2 e + N2(a’) | K(ε) | [2,3] |
| 6 | e + N2 e + N2(C) | K(ε) | [2,3] |
|  | Dissociation |  |  |
| 7 | e + N2 e + N(4S) + N(2D) | K(ε) | [2,3] |
|  | Rotational excitation |  |  |
| 8 | e + N2 (X1∑g+, J=0) ⇄ e +N2 (X1∑g+, J=1-10) | K(ε) | [4] |
|  | Vibration (Excitation) |  |  |
| 9 | e + N2(X1∑g+, 0 ≤υ ≤5) ⇄ e + N2(X1∑g+, υ +1) | K(ε) | [5,6,7] |
|  | Ionization |  |  |
| 10 | e + N2 2e + N2+ | K(ε) | [7] |
| 11 | e + N2 2e + N(2D) + N+ | 4.95 x 10-16(Te)1.22exp(-21.66/Te) | [8] |
| 12 | e + N2 3e + 2N+ | 9.95 x 10-16(Te)0.56exp(-43.62/Te) | [8] |
| 13 | e + N(2D) + N(2P) e + N2+ | 1.00 x 10-19 | [9] |
| 14 | e + N(2P) + N(2P) e + N2+ | 1.00 x 10-19 | [10] |
| 15 | N2(A) + N2(A) e + N4+ | 1.00 x 10-19 | [8] |
| 16 | N2(A) + N2(a’) e + N4+ | 1.50 x 10-17 | [12] |
| 17 | N2(a’) + N2(a’) e + N4+ | 5.00 x 10-17 | [9] |
|  | Excitation |  |  |
| 18 | e + N2(X1∑g+, υ =1) e + N2(A) | 1.52 x 10-14(Te)-0.49exp(-8.44/Te) | [8] |
| 19 | e + N2(X1∑g+, υ =2) e + N2(A) | 1.42x 10-14(Te)-0.47exp(-8.09/Te) | [8] |
| 20 | e + N2(X1∑g+, υ =3) e + N2(A) | 1.37 x 10-14(Te)-0.46exp(-7.81/Te) | [8] |
| 21 | e + N2(X1∑g+, υ =4) e + N2(A) | 1.34 x 10-14(Te)-0.46 exp(-7.55/Te) | [8] |
| 22 | e + N2(X1∑g+, υ =5) e + N2(A) | 1.30 x 10-14(Te)-0.46 exp(-7.30/Te) | [8] |
| 23 | e + N2(X1∑g+, υ =6) e + N2(A) | 1.27 x 10-14(Te)-0.45 exp(-7.05/Te) | [8] |
|  | Dissociation of vibrational states |  |  |
| 24 | e + N2(X1∑g+, υ =1) e + N(4S) + N(2D) | 1.08 x 10-14(Te)0.53exp(-13.20/Te) | [8] |
| 25 | e + N2(X1∑g+, υ =2) e + N(4S) + N(2D) | 1.07 x 10-14(Te)0.53exp(-12.94/Te) | [8] |
| 26 | e + N2(X1∑g+, υ =3) e + N(4S) + N(2D) | 7.62 x 10-15(Te)0.64exp(-11.90/Te) | [8] |
| 27 | e + N2(X1∑g+, υ =4) e + N(4S) + N(2D) | 7.56 x 10-15(Te)0.65exp(-11.67/Te) | [8] |
| 28 | e + N2(X1∑g+, υ =5) e + N(4S) + N(2D) | 1.04 x 10-14(Te)0.54exp(-12.20/Te) | [8] |
| 29 | e + N2(X1∑g+, υ =6) e + N(4S) + N(2D) | 8.82 x 10-15(Te)0.59exp(-11.58/Te) | [8] |
|  | Ion recombination |  |  |
| 30 | e + N2+ 2e + N+ + N+ | 3.74 x 10-16(Te)1.48exp(-25.0/Te) | [8] |
| 31 | e + N2+ N(4S) + N(2D) | 8.93 x 10-15Te-0.30 | [11] |
| 32 | e + N2+ N(2D) + N(2D) | 6.46 x 10-15Te-0.30 | [11] |
| 33 | e + N2+ N(4S) + N(2P) | 3.61 x 10-15Te-0.30 | [11] |
| 34 | e + N3+ N2(A) + N(4S) | 3.22 x 10-14Te-0.50 | [11] |
| 35 | e + N3+ N2(X) + N(2P) | 3.22 x 10-14Te-0.50 | [11] |
| 36 | e + N4+ N2(X) + N2(A) | 3.20 x 10-13Te-0.50 | [11] |
| 37 | N2(A) +N2+ N(4S) + N3+ | 5.50 x 10-17 | [11] |
| 38 | N +N3+ N2(A) + N2+ | 6.60 x 10-17 | [11] |
| 39 | N +N2+ N2 +N+ | 7.20 x 10-19(300/Tg)-1 | [11] |
| 40 | N2(X, υ =4-6) +N+ N(4S) +N2+ | 2.00 x 10-17 | [11] |
| 41 | N2(A) +N+ N(2P) + N2+ | 2.00 x 10-17 | [11] |
| 42 | N +N4+ N2 + N2+ N+ | 1.00 x 10-17 | [11] |
| 43 | N2 +N4+ N2 + N2+ N2+ | 8.67 x 10-23(300/Tg)-6.45exp(900/Tg) | [11] |
|  | Quenching of metastable N atom and N2 |  |  |
| 44 | N2(A) + N2(A) N2(B) + N2(X) | 7.70×10-17 | [12] |
| 45 | N2(A) + N2(A) N2(C) + N2(X) | 1.50×10-16 | [12] |
| 46 | N2(A) + N2(X) N2(X) + N2(X) | 3.00×10-22 | [13] |
| 47 | N2(B) + N2(X) N2(A) + N2(X) | 2.85×10-17 | [9] |
| 48 | N2(B) + N2(X) N2(X) + N2(X) | 1.50×10-18 | [9] |
| 49 | N2(a’) + N2(X) N2(B) + N2(X) | 1.90×10-19 | [9] |
| 50 | N2(C) + N2(X) N2(a’) + N2(X) | 1.32×10-17 | [9] |
| 51 | N2(X) + N(4S) N2(A) + N(S) | 1.2×10-18Te-1.5exp⁡(-6.2Te) | [14] |
| 52 | N2(X) + N(2D) + N2 + N(4S) | 1.00 x 10-19exp(-510/Tg) | [9] |
| 53 | N2(X) + N(2P) + N2 + N(4S) | 3.00 x 10-23 | [9] |
| 54 | N(4S) + N(2P) N(4S) + N(4S) | 6.20 x 10-19 | [9] |
| 55 | N2(A) + N(4S) N2(X) + N(2P) | 4.00 x 10-17 | [8] |
|  | N atom excitation and de-excitation |  |  |
| 56 | e + N(4S) e + N(2D) | 2.74 x 10-14(Te)-0.40exp(-3.35/Te) | [8] |
| 57 | e + N(4S) e + N(2P) | 9.11 x 10-15(Te)-0.45exp(-4.80/Te) | [8] |
| 58 | e + N(4D) e + N(2P) | 1.01 x 10-14(Te)-0.18exp(-3.94/Te) | [8] |
| 59 | e + N(2P) e + N(2D) | 1.64 x 10-14(Te)-0.17exp(-2.69/Te) | [8] |
| 60 | e + N(2D) e + N(4S) | 1.00 x 10-14(Te)-0.36exp(-0.83/Te) | [8] |
| 61 | e + N(2P) e + N(4S) | 5.45 x 10-15(Te)-0.41exp(-1.05/Te) | [8] |
|  | V-T relaxation |  |  |
| 62 | N2(υ =5-6) + N2(A) N2(υ =0) + N2(A) | 3.00 x 10-17 | [11] |
| 63 | N2(υ =1) + N2 N2(υ =0) + N2 | 5.19 x 10-28(300/Tg)-8.51 | [11] |
| 64 | N2(υ =2) + N2 N2(υ =1) + N2 | 1.15 x 10-27(300/Tg)-8.54 | [11] |
| 65 | N2(υ =3) + N2 N2(υ =2) + N2 | 2.18 x 10-27(300/Tg)-8.53 | [11] |
| 66 | N2(υ =4) + N2 N2(υ =3) + N2 | 4.32 x 10-27(300/Tg)-8.30 | [11] |
| 67 | N2(υ =5) + N2 N2(υ =4) + N2 | 6.87 x 10-27(300/Tg)-8.27 | [11] |
| 68 | N2(υ =6) + N2 N2(υ =5) + N2 | 1.10 x 10-26(300/Tg)-8.15 | [11] |
| 69 | N2(υ =1) + N2(υ =1) N2(υ =0) + N2(υ =2) | 1.73 x 10-20(300/Tg)-1.42 | [11] |
| 70 | N2(υ =1) + N2(υ =2) N2(υ =0) + N2(υ =3) | 2.18 x 10-20(300/Tg)-1.54 | [11] |
| 71 | N2(υ =1) + N2(υ =3) N2(υ =0) + N2(υ =4) | 2.60 x 10-20(300/Tg)-1.55 | [11] |
| 72 | N2(υ =1) + N2(v=4) N2(υ =0) + N2(υ =5) | 2.80 x 10-20(300/Tg)-1.63 | [11] |
| 73 | N2(υ =1) + N2(υ =5) N2(υ =0) + N2(υ =6) | 3.09 x 10-20(300/Tg)-1.63 | [11] |
|  | 3-body recombination a |  |  |
| 74 | N(4S) + N(4S) + N2 N2 + N2 | 8.27 x 10-46exp(500/Tg) | [9,11,15] |
| 75 | N(4S) + N(4S) + N2 N2(A) + N2 | 1.70 x 10-45 | [9,13] |
| 76 | N(4S) + N(4S) + N2 N2(B) + N2 | 2.40 x 10-45 | [9,13] |
| 77 | N(4S) + N(4S) + N(4S) N2(A) + N2 | 1.40 x 10-44 | [9] |
| 78 | N(4S) + N(4S) + N(4S) N2(B) + N2 | 1.40 x 10-44 | [9] |
| 79 | N2 + N(4S) + N+ N2 + N2+ | 1.00 x 10-41 | [11] |
| 80 | N2+ + N2 + N2 N4+ + N2 | 5.20 x 10-41(300/Tg)2.20 | [11] |
| 81 | N+ + N2 + N2 N3+ + N2 | 1.70 x 10-41(300/Tg)2.10 | [11] |
| 82 | N2+ + N2 + N(4S) N3+ + N2 | 9.00 x 10-42exp(400/Tg) | [11] |
|  | Radiation |  |  |
| 83 | N2(B) N2(A) + hν (FPS) | 2.0×105 b | [12,15] |
| 84 | N2(C) N2(B) + hν (SPS) | 2.74×107 b | [9,15] |
| 85 | N2(B’) N2(X) + hν | 2.56×104 b | [9] |
| 86 | N2(B’) N2(B) + hν | 3.81×104 b | [15] |
| 87 | e + N+ N(S) + hν | 1.0×10-12(TeTg)-0.7 | [14] |

a Rate constant is in cm6/s

b Rate constant is in 1/s

Te in eV, Tg in K

## Cross section data base

The model includes the following species:

| N2(X) | N(4S) | N+(3P) | e- |
| --- | --- | --- | --- |
| N2(X, J = 1 – 10) | N(2D) | N2+(X) |  |
| N2(X, υ = 1 – 6) | N(2P) | N3+(X) |  |
| N2(A) |  | N4+(X) |  |
| N2(B) |  |  |  |
| N2(B’) |  |  |  |
| N2(C) |  |  |  |
| N2(a’) |  |  |  |

N2(a), N2(w) and N2(a’) are included in the model as effective lumped states, because they are strongly coupled; N2(a) and N2(w) are converted into N2(a′) through collisional quenching with N2 with a high reaction rate coefficient. N2(a’) is the metastable state. Similarly, N2(B) and N2(W) are considered as a lumped state and N2(C) and N2(E) triplet states are also considered as a single state [9].

Vibrational levels of N2 molecule at the ground state:

| υ | ϵυ, eV |
| --- | --- |
| 0 | 0.000 |
| 1 | 0.288 |
| 2 | 0.573 |
| 3 | 0.855 |
| 4 | 1.133 |
| 5 | 1.408 |
| 6 | 1.679 |

There are 30 surface reactions:

Diffusional losses of neutral species to the reactor walls are estimated by an effective diffusion length, given by [11] :

Λn= πL2+ 2.405R2-1/2

Diffusional losses of neutral species to the reactor walls are calculated according to Bohm velocity.

Kinetics of vibrational states of N2(X 1g+ , υ = 0 – 6):

| e - V | e + N2(X , υ) ⇌ e + N2(X , ω) |
| --- | --- |
| V - V | N2(X , υ) + N2(X , ω) ⇌ N2(X , υ-1) + N2(X , ω+1) |
| V – T N2-N2 | N2(X , υ) + N2 ⇌ N2(X , υ-1) + N2 |
| V – T N2-N | N2(X , υ) + N ⇌ N2(X , υ-1) + N |
| e - D | e + N2(X , υ) ⟶ e + N + N |
| V - D | N2(X , υ) + N2(X , ω=6) ⟶ N2(X , υ-1) + N + N |
| Wall | N2(X , υ) + wall ⟶ N2(X , υ-1) |

The model is global, meaning that  there is no spatial solution and the entire plasma volume is considered homogenous.

The model was run for over 240,000  RF cycles to reach steady state for all species (some species reach to steady state before). Densities are taken at steady state, after 1800 µs.

## Model results

### Calculating rotational temperature

NJN0=exp-EjkBT= exp-BhcJ(J+1)kBT

Plotting ln[P(j)] vs. J(J+1)

Trot=BhckBSlope  = 2.875Slope

Nitrogen rotational constant [16]:

B=1.99824 cm-1

BhckB  =2.875 K

### Calculating vibrational temperature

Using a harmonic oscillator model Eυ= υ+12hν and

Nυ=expEυkBT

with a vibrational frequency for nitrogen [15] of ω = 2358.57 cm-1 , followed by plotting ln[P(Nv)] vs. (υ+1/2) we have Tvib=hνkBSlope  = 3394Slope or plotting ln[(Nv)] vs. energy levels,

in that case Tvib is calculating according to:

Tvib=1kBSlope

Densities and temperatures were calculated for 100 sccm N2 (295 mTorr) for 40, 50, 60, 80 and 100 W. Additional series of runs was performed where the absorbed power was assumed to be 95% of the applied power. This is a realistic assumption since there are some heat losses and reflected power that can be summed to 5%. A similar assumption was made in [5], for ICP case Pabs ≈ 75%Prf.

# References

- Piper, L. G., & Cowles, L. M. (1986). Einstein coefficients and transition moment variation for the NO (A 2Σ+–X 2Π) transition. The Journal of Chemical Physics, 85(5), 2419-2422. http://doi.org/10.1063/1.451098

- http://lxcat.net/Itikawa. , Itikawa, Y. database

- Itikawa, Y. (2006). Cross sections for electron collisions with nitrogen molecules. Journal of Physical and Chemical Reference Data, 35(1), 31-53. https://doi.org/10.1063/1.1937426‏

- https://lxcat.net/IAA,  IAA database, Schmalzried. A., Luque. A., & Lehtinen. N. (2023).

- http://lxcat.net/Biagi, Biagi database, transcription of data from SF Biagi's Fortran code, Magboltz. versions 8.9.

- Song, M. Y., Cho, H., Karwasz, G. P., Kokoouline, V., & Tennyson, J. (2023). Electron scattering on molecular nitrogen: common gas, uncommon cross sections. The European Physical Journal D, 77(6), 105. https://doi.org/10.1140/epjd/s10053-023-00687-5

- www.lxcat.net/, LIST-Lisbon database, FLINDERS database

- Thorsteinsson, E. G. (2008). The nitrogen discharge: a global (volume averaged) model study (Master’s Thesis), Department of Electrical and Computer Engineering, University of Iceland.

- Davies, H. L., Guerra, V., van der Woude, M., Gans, T., O’Connell, D., & Gibson, A. R. (2023). Vibrational kinetics in repetitively pulsed atmospheric pressure nitrogen discharges: average-power-dependent switching behaviour. Plasma Sources Science and Technology, 32(1), 014003.‏ https://doi.org/10.1088/1361-6595/aca9f4

- ‏Vereshchagin, K. A., Smirnov, V. V., & Shakhatov, V. A. (1997). CARS study of the vibrational kinetics of nitrogen molecules in the burning and afterglow stages of a pulsed discharge. Technical Physics, 42, 487-494.‏ https://doi.org/10.1134/1.1258658

- Thorsteinsson, E. G., Gudmundsson, J. T. (2009). A global (volume averaged) model of a nitrogen discharge: I. Steady state. Plasma Sources Science and Technology, 18(4), 045001.‏ https://doi.org/10.1088/0963-0252/18/4/045001

- Alves, L. L., Marques, L., Pintassilgo, C. D., Wattieaux, G., Es-Sebbar, E. T., Berndt, J., Kovacević, E., Carrasco. N., Boufendi. L., & Cernogora, G. (2012). Capacitively coupled radio-frequency discharges in nitrogen at low pressures. Plasma Sources Science and Technology, 21(4), 045008.‏ https://doi.org/10.1088/0963-0252/21/4/045008

- Gordiets, B., Ferreira, C. M., Pinheiro, M. J., & Ricard, A. (1998). Self-consistent kinetic model of low-pressure-flowing discharges: I. Volume processes. Plasma Sources Science and Technology, 7(3), 363.‏ https://doi.org/10.1088/0963-0252/7/3/015

- Moravej, M., Yang, X., Barankin, M., Penelon, J., Babayan, S. E., & Hicks, R. F. (2006). Properties of an atmospheric pressure radio-frequency argon and nitrogen plasma. Plasma Sources Science and Technology, 15(2), 204. https://doi.org/10.1088/0963-0252/15/2/005

- Guerra, V., Tejero-del-Caz, A., Pintassilgo, C. D., & Alves, L. L. (2019). Modelling N2–O2 plasmas: volume and surface kinetics. Plasma Sources Science and Technology, 28(7), 073001.‏ https://doi.org/10.1088/1361-6595/ab252c

- Bendtsen, J. (1974). The rotational and rotation‐vibrational Raman spectra of 14N2, 14N15N and 15N2. Journal of Raman Spectroscopy, 2(2), 133-145.‏ https://doi.org/10.1002/jrs.1250020204