# Supplementary materials

# Flow velocity and nutrients affect CO2 emissions from agricultural drainage channels in the North China Plain

Peifang Leng1,2,3, Fadong Li1,3, Kun Du1,3, Zhao Li1,3, Congke Gu1,3, and Matthias Koschorreck2

1 Key Laboratory of Ecosystem Network Observation and Modeling, Institute of Geographic and Natural Resources Research, Chinese Academy of Sciences, Beijing, China,

2 Department of Lake Research, Helmholtz Centre for Environmental Research-UFZ, Magdeburg, Germany,

3 College of Resources and Environment, University of Chinese Academy of Sciences, Beijing, China

Corresponding author: Fadong Li (lifadong@igsnrr.ac.cn)

This supporting information contains additional presentation of the controlled experiments methods and results, including three figures.

Methods of chamber flux measurements

The fluxes of carbon dioxide were measured by a floating chamber connected to a cavity ring-down spectrometer (CRDS) (Picarro G2508, USA) with high precision in real-time (Martin & Moseman-Valtierra, 2015). A cylindrical static chamber of fiber reinforced plastic (20 cm diameter × 13 cm height) was placed at the water surface, penetrating 1-2 cm into the water. The chamber was kept floating by polyethylene (PE) foam. For each measurement, the chamber was placed at the water surface for up to 3 min for CO2 measurement. The CO2 flux (mmol m-2 d-1) was calculated from the observed change rate of  S (ppm s-1) in the chamber using equation (1) (see Campeau and Giorgio (2014) for more details):

|  | F=(S×V/A)×t1×t2 | (1) |
| --- | --- | --- |

where V is the chamber gas volume (m3), A is the chamber area (m2),

|  | t1=8.64×104 s d-1 | (2) |
| --- | --- | --- |

denotes the conversion factor from seconds to days, and t2 is a conversion factor from mole fraction (ppm) to concentration (mmol m-3) at in-situ air temperature (T in K) and atmospheric pressure (p in Pa):

|  | t2=P8.314×T×1000 | (3) |
| --- | --- | --- |

The cumulative CO2 evasion at a certain time was calculated as the integral of CO2 flux over time. The CO2 flux can be delineated according to the fitting result from the relationship between time and CO2 flux:

|  | ft=f0∙exp⁡(-kt) | (4) |
| --- | --- | --- |

where f0 and ft  is the CO2 flux (mmol m-2 h-1) at the initial time and a certain reaction time (t; h), respectively. k (h-1) is the reaction constant. When the CO2 flux is integrated by t, the total CO2 evasion amount (nt) in each container at a certain time t can be defined as:

|  | nt=S×0tftdt=S∙f0k∙[1-exp-kt] | (4) |
| --- | --- | --- |

where S is the surface area of the water (m2) in each container.

Figure S1. The illustration of the 16 experimental mesocosms arrangement (a), a photo showing the floating chamber used for direct flux measurement (b), and a photo showing the mesocosm system for the controlling experiments (c).

Figure S2. pCO2 changing over time. pCO2 could be calculated using alkalinity and DIC by the CO2SYS program. Alkalinity was measured at the beginning and in the end with a linear decrease over time. DIC is from Figure 3 (a-d). The intersection of the horizontal red dashed line (pCO2air = 395 μatm) and the pCO2 changing line indicates the time when pCO2 under different treatments was equal to pCO2 in the air.

Figure S3. Stacked area of DIC, TOC, CO2, and CaCO3 over time under different C:N ratio (C:N = 5, Redfield Ratio, 10, and 12) among four treatments.

References

Campeau, A., & Giorgio, P. A. (2014). Patterns in CH4 and CO2 concentrations across boreal rivers: Major drivers and implications for fluvial greenhouse emissions under climate change scenarios. Global change biology, 20(4), 1075-1088. doi:10.1111/gcb.12479

Martin, R. M., & Moseman-Valtierra, S. (2015). Greenhouse Gas Fluxes Vary Between Phragmites Australis and Native Vegetation Zones in Coastal Wetlands Along a Salinity Gradient. Wetlands, 35(6), 1021-1031. doi:10.1007/s13157-015-0690-y