Supplementary Table 1.  Small molecule screening data

| Category | Parameter | Description |
| --- | --- | --- |
| Assay | Type of assay | Bioluminescence Resonance Energy Transfer (BRET)- and cell-based assays in HEK293 cells |
|  | Target | Receptor (AT1R) targeting to endosomes |
|  | Primary measurement | Detection of fluorescence/proximity between receptor and an endosomal marker |
|  | Key reagents | Coelenterazine 400a (Nanolight) |
|  | Assay protocol | Namkung, Y. et al. Monitoring G protein-coupled receptor and beta-arrestin trafficking in live cells using enhanced bystander BRET. Nat Commun 7, 12178, doi:10.1038/ncomms12178 (2016). |
|  | Additional comments |  |
| Library | Library size | 115,303 compounds |
|  | Library composition | Known bioactives, natural products, drug-like and scaffold molecules |
|  | Source | Institute for Research in Immunology and Cancer (IRIC): https://www.iric.ca/en/research/platforms-and-infrastructures/high-throughput-screening/?section=technologies |
|  | Additional comments | https://www.iric.ca/en/research/platforms-and-infrastructures/high-throughput-screening |
| Screen | Format | 384-well, flat white-bottom plates |
|  | Concentration(s) tested | 10 M compound, 0.5% DMSO |
|  | Plate controls | 32 wells/plate; column 1 and 23 |
|  | Reagent/ compound dispensing system | Biomek FX (Beckman) with pintool (V&P scientific), Multidrop 384 (Thermo) |
|  | Detection instrument and software | SpectraMax L with SoftMax Pro software and Envision (Perkin-Elmer) |
|  | Assay validation/QC | Z’ score=0.732 |
|  | Correction factors | Quench -/+ 50% of fluorescence |
|  | Normalization | % AngII-promoted BRET compared to DMSO |
|  | Additional comments |  |
| Post-HTS analysis | Hit criteria | Inhibiting >40% or potentiating >100% AT1R and B2R internalization |
|  | Hit rate | Final: 40/115,303 compounds |
|  | Additional assay(s) | Retesting of initial hits in original assay (AT1R internalization) and secondary assay (B2R internalization). Validation of internalization by microscopy |
|  | Confirmation of hit purity and structure | Hit compounds were repurchased from original supplier |
|  | Additional comments |  |