Understanding your biases in collecting organismal VOCs

Journal of Chemical Ecology

Lucas Seybert1, Christophe Duplais1*

1Department of Entomology, Cornell AgriTech, Cornell University, Geneva 14464 NY, USA

*Corresponding author: c.duplais@cornell.edu

Appendix S1: Statistical analysis.3

Table S1. List of adsorbent characteristics commercially available as sorbent tubes.  4

Table S2. Compounds selected for the VOC standard mixture. 5

Table S3. VOC recovery by adsorbent cartridges.6

Table S4. Total number of features identified in chromatograms of rosemary samples.7

Figure S1. Comparison of VOC recovery by different vendors and lots of Porapak® Q. 8

Figure S2. Comparison of sample chromatograms of rosemary VOCs.9

Appendix S1: Statistical Analysis. All statistical analyses were performed in R (v. 4.3.3) with RStudio (2024.09.1) as an environment, using the following packages: multcompView, car, nlme, magrittr, dplyr, tidyr, readxl, emmeans, multcomp, and ggplot2. Code was provided by K. T. Wilcox (Cornell Statistical Consulting Unit) and modified by LS.

Each compound within a dataset was analyzed individually, with adsorbent type (or adsorbent vendor) as the independent variable and compound area/concentration as the dependent variable. KTW advised that the transformation of the data and the type of ANOVA performed should be determined on an compound-by-compound basis based on whether the data fit the assumptions of the statistical test. The variance (SD2) within each adsorbent was calculated, and the smallest and largest variance within a given compound were compared. A traditional ANOVA assumes that the variance is the same for all independent variables; KTW advised that the ratio between the highest and lowest variance should not be much greater than four. For variables where the ratio of variances was >>4, the data was log-transformed and the comparison of the variances was repeated. If the log-transformed variances fit the assumptions of a traditional ANOVA, the ANOVA was performed on the log-transformed data. In many cases, neither the raw nor the transformed data fit the assumption of equal variance, so a Welch’s ANOVA—which does not assume equal variance—was performed.

For all compounds in the VOC standard trials, a Welch’s ANOVA was used. In the rosemary trials, a traditional ANOVA was performed on the raw area counts for d-limonene, β-myrcene, cineole, and terpinolene; an ANOVA was performed on the log-transformed area counts for α-pinene, β-pinene, camphene, and camphor; and a Welch’s ANOVA was performed on the raw area counts for γ-terpinene, endo-borneol, and bornyl acetate. For the vendor comparisons, a traditional ANOVA was used for all compounds, while in the comparisons between lots from a specific vendor, a traditional ANOVA could only be used for hexanal and R-(+)-limonene and a Welch’s ANOVA was required for the remaining compounds.

Regardless of the type of ANOVA used, Tukey’s HSD (p<0.05) was used to determine whether adsorbents differed from one another for a given compound.

Table S1. List of adsorbent characteristics commercially available as sorbent tubes.\

| Matrix | Vendor Reference | Name (Composition) | Polarity | Pore diameter (nm) | Surface area (m2/g) |
| --- | --- | --- | --- | --- | --- |
| Charcoal | Orbo 32a SKC 226-01 | Activated coconut charcoal | Low | - | 800–1500 |
| Carbon | Orbo 90a SKC-80 SKC 226-81 | Carboxen-564 Anasorb 747 (carbon molecular sieves) | Low | 0.9 | 400 |
|  | Orbo 101a | Carbotrap (graphitized carbon black) | Low | 10 | 240 |
| Porous polymer | Orbo 403a SKC 226-35-03 | Tenax® TA (poly(2,6-diphenyl-p-phenylene oxide) polymer) | 2-Low | 200 | 35 |
|  | Orbo 43 SKC 226-30-04 | Amberlite XAD-2, supelpak 20 (styrene-divinylbenzene copolymer) | 2-Low | 9 | 300 |
|  | Orbo 47a SKC 226-95 ZST-090 | Amberlite XAD-7, supelpak 70 (aliphatic acrylic polymer) | 5-Moderate | 55 | 75 |
|  | SKC 226-107 ZST-020 | Chromosorb 102 (Benzene, diethenyl-, ethenylbenzene copolymer) | 3-Low | 300-400 | 50 |
|  | ZST-120a | Porapak® N, HayeSep N (DVB-EVB Ethyleneglycol dimethacrylate copolymer) | 7-High | - | 250-350 |
|  | SKC 226-114 ZST-125a | Porapak® P, HayeSep P (DVB/Styrene copolymer) | 2-Low | 91 | 100-200 |
|  | Orbo 1103a SKC 226-115a ZST-130a | Porapak® Q, SuperQb (DVB/Ethylvinylbenzene copolymer) | 2-Low | 75 | 500-600 |
|  | SKC 226-116A ZST-140 | Porapak® T, HayeSep T (Ethyleneglycol dimethacrylate copolymer) | 10-High | - | 225-350 |
|  | ZST-135 | Porapak® R, HayeSep R (DVB/Vinyl Methylpyrolidone copolymer) | 5-Moderate | 76 | 450-600 |

aSorbent tubes used in this study

bSuper Q is Porapak® Q extensively washed with organic solvents and acids to remove residual monomers and impurities. http://www.axialscientific.com/files/alltech_gc_packed.pdf

Table S2. Compounds selected for the VOC standard mixture.

| Compound | Structure | Functional group(s) | Category |
| --- | --- | --- | --- |
| Hexanal |  | Aldehyde | C6 volatile |
| 1-Hexanol |  | Alcohol | C6 volatile |
| 3-Hexenyl acetate |  | Este, alkene | C6 volatile |
| R-(+)-Limonene |  | Alkene | Monoterpene |
| Linalool oxide |  | Alcohol, ether, alkene | Monoterpenoid |
| Linalool |  | Alcohol, alkene | Monoterpenoid |
| 2-Phenylethyl alcohol |  | Phenyl, alcohol | Benzenoid23 |
| Methyl salicylate |  | Alcohol, ketone | Benzenoid |
| β-Caryophyllene |  | n/a | Sesquiterpene |

Table S3.  Mean and standard error of VOC percentage recovery by adsorbent cartridges. Superscripts indicate means that are significantly different from one another (Tukey’s HSD, p < 0.05).

| Adsorbent | Hexanal | 1-Hexanol | 3-Hexenyl acetate | R-(+)- Limonene | Linalool oxides | Linalool | 2-Phenylethyl alcohol | Methyl salicylate | Caryophyllene |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbo 101 | 67% ± 5%A | 52% ± 4%AB | 73% ± 7%AB | 70% ± 7% AB | 73% ± 7% AB | 54% ± 6% AB | 46% ± 5%BC | 50% ± 6% BC | 44% ± 5% BC |
| Orbo 32 | 51% ± 2%A | 57% ± 1%AB | 60% ± 1%B | 31% ± 1% BC | 55% ± 6%ABC | 51% ± 1% B | 32% ± 1% C | 9% ± 0.4%C | 38% ± 2% B |
| Orbo 47 | 55% ± 2%A | 63% ± 2%AB | 61% ± 2% B | 65% ± 2% AB | 61% ± 2%B | 49% ± 2%B | 47% ± 2%B | 50% ± 3%B | 38% ± 1%B |
| Orbo 90 | 23% ± 5%BC | 18% ± 4%D | 21% ± 5%C | 15% ± 3%C | 22% ± 5%CD | 17% ± 4%C | 9% ± 2%D | 5% ± 1%C | 18% ± 4%C |
| Porapak® N | 59% ± 3%A | 57% ± 8%ABC | 51% ± 10%ABC | 56% ± 8%AB | 52% ± 13%ABCD | 40% ± 11%ABC | 39% ± 12%ABCD | 42% ± 12%ABC | 25% ± 6%BC |
| Porapak® P | 44% ± 3%AB | 70% ± 3%AB | 81% ± 3%A | 80% ± 4%A | 89% ± 4%A | 81% ± 4%A | 80% ± 4%A | 82% ± 4%A | 79% ± 4%A |
| Porapak® Q | 43% ± 6%AB | 39% ± 7%BCD | 27% ± 7%C | 31% ± 7%C | 24% ± 7%CD | 20% ± 7%C | 20% ± 6%CD | 19% ± 6%C | 19% ± 5%C |
| Porapak® R | 54% ± 7%AB | 57% ± 7%AB | 54% ± 7%ABC | 57% ± 6%AB | 53% ± 9%ABCD | 43% ± 9%ABC | 33% ± 7%BCD | 43% ± 9%ABC | 32% ± 7%BC |
| Tenax® | 6.3% ± 0.4%C | 20.9% ± 0.8%CD | 66% ± 1%B | 20.3%± 0.5%C | 4.0% ± 0.1%D | 23.2%± 0.5%C | 47.0% ± 0.8%B | 63.1% ± 1%AB | 15.8% ± 0.8%C |

Table S4. Total number of features identified in chromatograms of rosemary samples collected via adsorbent and TF-SPME. Features were identified in MZmine using parameters described in methods and counted prior to annotation. Features appearing in only one sample from a given treatment were not included.

| Material | Number of feature |
| --- | --- |
| Porapak® P | 14 |
| Porapak® Q | 8 |
| Tenax® | 8 |
| DVB-TF-SPME | 95 |

Figure S1. (A) Comparison of VOC recovery using pre-packed Porapak®  Q tubes from three different vendors (N=4-10 replicates per vendor). No statistically significant differences were observed between vendors. (B) Comparison of VOC recovery using two different lots of Porapak®  Q tubes from MilliporeSigma (N=4-6 replicates per lot). Significantly different recoveries are indicated with a *.

Figure S2. Comparison of sample chromatograms of rosemary VOCs collected using TF-SPME (black) and Porapak® P (red).