The United States Government retains and the publisher, by accepting the article for publication, acknowledges that the United States Government retains a nonexclusive, paid-up, irrevocable, worldwide license to publish or reproduce the published form of this manuscript, or allow others to do so, for the United States Government purposes. The Department of Energy will provide public access to these results of federally sponsored research in accordance with the DOE Public Access Plan (http://energy.gov/downloads/doe-public-access-plan).

# Supplementary information

Ruben Millan‐Solsona,1* Spenser R. Brown,2 Lance Zhang,2 Sita Sirisha Madugula,1 HuanHuan Zhao,3 Blythe Dumerer, 1 Amber N. Bible,2 Nickolay V. Lavrik,1 Rama K. Vasudevan,1 Arpan Biswas,4 Jennifer L. Morrell-Falvey,2 Scott Retterer, 1,2 Martí Checa,1 Liam Collins.1*

1	Center for Nanophase Materials Sciences, Oak Ridge National Laboratory, Oak Ridge, Tennessee 37831, USA.

2	Biosciences Division, Oak Ridge National Laboratory, Oak Ridge, Tennessee 37831, USA.

3Bredesen Center for Interdisciplinary Research, University of Tennessee, Knoxville, USA

4University of Tennessee-Oak Ridge Innovation Institute, Knoxville, TN 37996, USA.

Email: solsonarm@ornl.gov & collinslf@ornl.gov

S1: Model Training and Data Augmentation

To enhance the accuracy and robustness of bacterial detection in AFM images, we trained a YOLOv8 model using experimental images with a size of 15 x 15 μm². The training process was performed on the Roboflow platform, where a curated dataset of experimentally acquired images was used. To expand this dataset, we employed data augmentation techniques, essential for improving the model’s ability to generalize across various bacterial orientations and positions. In Figure S1, an example of the data augmentation process is presented, specifically showcasing image rotation. By rotating the original images, we created variations that simulate different bacterial orientations, thus helping the model recognize bacteria more reliably in diverse scenarios. This augmentation is a key step in maximizing the training data without needing additional experimental images, enhancing the model’s detection performance.

Figure S1. Data augmentation steps are ±90° rotations, ±10° shear (vertical and/or horizontal) and ±25% saturation. In (a) an original training image is shown and in (b) a rotated image for data augmentation.

S2: Detection Accuracy and Confidence Parameter

The detection precision of bacteria using YOLOv8 is highly influenced by the confidence threshold, a hyperparameter that determines the model’s certainty in identifying an object as a bacterium. Adjusting this threshold allows us to balance between high precision and the risk of false positives (detecting non-bacterial structures as bacteria). Figure S2 illustrates the relationship between the confidence threshold and detection accuracy. It highlights that while a lower confidence threshold may capture more bacteria, it can also increase the likelihood of erroneous detections. Conversely, a higher confidence threshold can yield higher precision but may miss some bacteria. Therefore, optimizing the confidence parameter is crucial to ensure adequate precision without compromising specificity.

Figure S2. (a) Single AFM scan and precision of bacteria detection with the confidence hyperparameter, for three different values (b) 0.5, (c) 0.25 and (d) 0.015.

S3: Quantitative segmentation

As an example data set, in Figure S3, the larger stiched AFM image is shown as well as zoomed-in region reveals more clearly the individual bacterial cells targeted for segmentation and analysis. We find that using YOLOv8, the segmentation process accurately identifies the boundaries of each bacterium, as seen in the green-colored segmented image (Figure S3.b), where the bacterial surface coverage is measured at 85.23%. The corresponding data table (Figure S3.c) presents quantitative metrics for each bacterium, including key features such as area, perimeter, orientation, centroid coordinates, eccentricity, and cell count. Overall, this workflow enables automated and comprehensive analysis of community structure, providing high-resolution metrics at the cellular level and facilitating a deeper understanding of biofilm morphology and growth patterns. 4

Figure S3. An example dataset is shown. (a) Original AFM image. (b) Detection and counting of bacteria. (c) table obtained with the quantification of various characteristics extracted from the segmentation, such as area, eccentricity, major axis, minor axis, etc. In (d) a large stitched area is shown. From (e-g) are the area, eccentricity and orientation maps for the region shown in (a). Below in (e-g) the distribution is shown for the individual maps and for the entire stitched area shown in (d).

S4: Automated analysis of bacterial surface attachment using Large-Area AFM data

Figure S4. (a) Large area stitched topographic image of Pantoea sp. YR343 cells grown on PFOTS-treated glass surfaces for 60 minutes. (b) A zoom of the region marked in (a) and in (c) a zoom of the dotted red box marked in (b) showing a very long bacterial cells likely in the process of cell division. (d-f) The Area map, statistics and a zoom of the area map of the same region marked in (a). (g-i) The eccentricity map, the statistics and a zoom of the eccentricity map of the same region marked in (a).

S5: Cluster Analysis of cell Area

The spatial analysis of cell clustering employed two key techniques namely, Moran's I and K-means clustering, along with calculating the radially averaged 2D autocorrelations. Moran's I is a statistical measure of spatial autocorrelation that quantifies whether similar values (e.g., cell area) cluster together in space. A positive Moran's I indicates clustering of similar values, while a negative value suggests dissimilarity (e.g., large cells near small cells). In this study, we observed a weak but statistically significant positive Moran's I (0.0714, p-value = 0.001), indicating slight clustering of similarly sized cells across the analyzed region. To investigate the possibility of local clustering, K-means clustering, a machine learning technique, was used to group cells based on their sizes into distinct clusters. The results suggested some grouping of cells by area, reinforcing the presence of localized clustering patterns. The radial autocorrelation function further showed that spatial dependency diminishes rapidly with distance, revealing that clustering is limited to small scales, while patterns become random at larger distances. The radially averaged 2D autocorrelation, which integrates directional correlations into a single radial metric, supports these findings by highlighting the rapid decay of spatial dependency beyond small scales.

Although the observed correlations were weak, this type of spatial analysis can be highly informative in other contexts, such as identifying microbial colony development patterns, analyzing how resource gradients affect biofilm structure, or studying the effects of surface engineering on bacterial attachment for the design of antibacterial materials.

Figure S5. (a) Mapa de la distribucion de clusterbasados en el area mediante k-means para k=4, se observa claramente como la distribucion no es aleatoria. (b) Comparacion de autocorrelacion obserbada para la distribucion en (a) versus una distribucion random. (c) Se muestra la autocorrelacion de (a) debil pero estadisticamente significativa (p-value = 0.001).

S6: Flagella analysis using Large-Area AFM data

To create the stitched topographic image highlighting the flagella shown in Figure S6.a, it was necessary to perform a specific flattening of each region of Figure 5.a. To do this, a mask was made using a 150 nm threshold and the bacteria-free areas were selected. Then a segmentation by area has been carried out, for example in figure S6.b the segmentation of regions 1 to 19 is shown (each color represents a different region) corresponding to the dotted white box in figure S6.a. With the regions well delimited, we have proceeded to cut each of the regions and carry out a specific flattening for each region, greatly improving the flattening. For example, figure S6.c shows region 3 of figure S6.b (light pink) where we can see the original cropped region, the cropped mask showing the area of ​​the gap, the flattened region, the mask to highlight the flagella and calculate the area covered by them and the area of ​​the gap of each of the regions. Furthermore, as highlighted in the main text, it allows calculating the statistical dependence on height (figure S6.e) and the relationship between the area covered by the flagella and the area of​​ the gap.

Figure S6. (a) Stitched topographic image highlighting the flagella. (b) Example of segmentation by regions corresponding to the dotted white box in (a). In (c) the cropped original region, the region mask area, the flattened region and the mask used to select the area covered by the flagella are shown. In (d) a zoom of the flattened regions corresponding to the dotted white box in (a) is shown and finally in (e) the height statistics corresponding to the entire image (a) (not counting the bacteria) and the flagella areas are shown.

# S7: Automated analysis of community structure using Large-Area AFM data

Figure S7. (a) Large area stitched image of bacterial cells grown on a patterned surface. (b) Area map and (c) eccentricity map are shown.