# Additional file 8:

# Preparation of the sample pad and others and assembling of the ICA

The sample pad, absorbent pad and PVC board were provided by Jieyi Biotechnology Co., Ltd. (Shanghai, China). The sample pad was saturated with PBS solution (pH 9.0) containing 1.0% (w/v) BSA, 0.5% (w/v) PVA-205 and 0.5% (v/v) Tween-20 and dried before use. H5072 cellulose was used as the absorbent pad without treatment. The PVC board served as the bottom plate of the test strip.

The pre-treated conjugate pad, NC membrane, sample pad, and absorbent pad were assembled and immobilized on the PVC board as described in Fig. 3, overlapping 5 mm between each membrane. Then these strips were cut into 4 mm wide and stored in sealing bag with desiccant. The control line should always show a visible red line. The appearance of both two lines indicated a positive result.