### Supplementary Information

Table S 1 UHT-PBR setup for a 20 ha cultivation system for biobutanol production (Guerra, T., personal communication, September 19, 2018)

|  | Amount | Volume [m3] | Area [m2] |
| --- | --- | --- | --- |
| Pre inoc. UHT-PBR | 12 | 1 | 16 |
| Inoc. UHT-PBR | 4 | 21 | 590 |
| UHT PBR | 72 | 98 | 2,769 |
| Total |  | 7,115 | 201,924 |

Table S 2 Main components and specifications for the biobutanol production system based on Guerra, T. (personal communication, January 23, 2019)

| Item | Material | UHT PBR | Inoc UHT PBR | Pre-inoc. UHT PBR | Assumption |
| --- | --- | --- | --- | --- | --- |
|  |  | [kg Material /a] | [kg Material/a] | [kg Material] |  |
| Tubing | PMMA | 15,577 | 541 | 122 |  |
| Circulation pump 1 | Stainless steel | - | - | 5 | Lowara CO(M) 350/05 |
| Circulation pump 2 | Stainless steel | - | 9 | - | Lowara ESHS 50-160/75 |
| Circulation pump 3 | Stainless steel | 2,036 | - | - | Lowara FHS-FHF 125-200/450 45 KW |
| Harvesting pump | Stainless steel | 113 |  |  | Lowara FHS-FHF 125-200/450 45 KW |
| Nutrient pump | Stainless steel | 0.4 | - | - | Lowara CO (M) 350/03 |
| Blowers | Stainless steel | 6,143 | 73 | 11 | MANVAC 220V Single Phase Blower |
| U-Bends | Borosilicate glass | 2,808 | 33 | 5 |  |
| Support | Cast Iron | 61,776 | 732 | 106 |  |
| Fittings | Rubber | 3,510 | 42 | 6 |  |
| Manifolds | Stainless steel | 9,477 | 112 | 16 |  |
| Tank | Stainless steel | 111,618 | 1,323 | 191 |  |
| Thermal regulation coil | Stainless steel | 24,921 | 295 | 43 |  |
| Tube connectors | PP | 2,106 | 25 | 4 |  |
| CO2 circuit | Polyurethane | 228 | 3 | 0.4 |  |

Table S 3 Baseline assumptions for pumping energy inputs

| Item | Amount | Capacity [kW] | Operating scheme [h/d] | Energy consumption [kWh/a] | Assumption |
| --- | --- | --- | --- | --- | --- |
| Blower | 1 | 1.5 | 24 | 18,442,909 | upscaled by volume a |
| Circulation pump 1 | 12 | 0.55 | 24 | 57,024 | Selected by flow, Lowara CO(M) 350/05 0.55 KW |
| Circulation pump 2 | 4 | 7.5 | 24 | 259,200 | Selected by flow, Lowara ESHS 50-160/75 7.5KW |
| Circulation pump 3 | 72 | 45 | 24 | 27,993,600 | Selected by flow, Lowara FHS-FHF 125-200/450 45 KW |
| Harvesting pump | 4 | 45 | 1 | 64,800 | Selected by flow, Lowara FHS-FHF 125-200/450 45 KW |
| Nutrient pump | 1 | 0.37 | 1 | 133.20 | upscaled by nutrient flow specification b |

a Based on  Guerra, T. (personal communication, January 12, 2016). b Based on  Guerra, T. (personal communication, April 18, 2019)

Table S 4 Main equipment for the pervaporation process to harvest biobutanol [17]

| Item | Material | [kg Material/a] | Assumption |
| --- | --- | --- | --- |
| Pre-Heater | Stainless steel | 338 | Tranter HE |
| Cooler | Stainless steel | 338 | Tranter HE |
| Heater 1 | Stainless steel | 338 | Tranter HE |
| Condenser 1 | Stainless steel | 338 | Tranter HE |
| Heater 2 | Stainless steel | 338 | Tranter HE |
| Condenser 2 | Stainless steel | 338 | Tranter HE |
| Pervaporator Filtration 1 | Polysulfone | 115 | Own calculation |
| Pervaporator Filtration 2 | Polysulfone | 115 | Own calculation |
| Pervaporator pump 1 | Stainless steel | 13,234 | Lowara FHS-FHF 125-200/450 45 KW |
| Pervaporator pump 2 | Stainless steel | 713 | Lowara FHS-FHF 125-200/450 45 KW |
| Distillation Boiler | Stainless steel | 338 | Tranter HE |
| Distillation Condenser | Stainless steel | 338 | Tranter HE |

Table S 5 Energy inputs for the pervaporation process to harvest biobutanol

| Item | Energy consumption [kWh/a] | Assumption |
| --- | --- | --- |
| Net of pre Phase (heating + condensing] | 295,542,140 | Upscaled based on Wagner et al. (2019) acc. to Liu et al. (2013) |
| Net of pervaporation 1 (heating + condensing) | 144,255,103 | Upscaled based on Wagner et al. (2019) acc. to Liu et al. (2013) |
| Net of pervaporation 2 (heating + condensing) | 7,835,584 | Upscaled based on Wagner et al. (2019) acc. to Liu et al. (2013) |
| Pervaporator pumps | 45,767,845 | Linearly upscaled based on Wagner et al. (2019) |
| Net Distillation | 4,620,985 | Upscaled based on Wagner et al. (2019) acc. to Liu et al. (2013) |

Table S 6 Components for the hydrothermal liquefaction (HTL) process

| Item | Material | [kg material/a] | Assumption |
| --- | --- | --- | --- |
| Reactor | Stainless steel | 3.85 | Dimensioning a, own estimation on materials |
| Principal pump | Stainless steel | 10.91 | Selected a, Dosing pump LDE3_LEWA |
| Solvent pump | Stainless steel | 11.6 | Selected a, Syringe pump ISCO 1820 |
| Solvent tank | Stainless steel | 32.5 | own estimation |
| Heat exchanger | Stainless steel | 1.47 | Tranter HE selected a |
| 3-Phase Separator | Stainless steel | 13.65 | Flottweg Separator selected based on [30] |
| Ceramic filter | Ceramic Al2O3 | 0.00002 | own estimation a |

a Based on Boatman, T. (personal communication, January 7, 2019).

Table S 7 Energy inputs for HTL process based on Jones et al. (2014) and Boatman, T. (personal communication, January 7, 2019).

| Item | Energy consumption [kWh/a] | Assumption |
| --- | --- | --- |
| Principal pump | 47,520 | own estimation |
| Solvent pump | 864 | own estimation |
| Heat exchanger | 1,598,903 | own estimation |
| 3-Phase Separator | 47,520 | own estimation |