Supplementary Information for

Development of a chiral HPLC method for the separation and quantification of hydroxychloroquine enantiomers

Xisheng Xiong#, Kun Wang#, Tao Tang, Jinzhi Fang, Yijun Chen1*

1State Key Laboratory of Natural Medicines and Laboratory of Chemical Biology, China Pharmaceutical University, 639 Longmian Avenue, Jiangning District, Nanjing, 211198, China

* Corresponding author. yjchen@cpu.edu.cn

# Equal contribution

This file includes:

I. Supplementary Figure (with legends)

II. Supplementary Equation

SUPPLEMENTARY RESULTS

I. SUPPLEMENTARY FIGURE

Supplementary Figure 1 | Effects of DEA concentration on the enantiomeric resolution of Rac-HCQ. Chromatographic conditions: Chiralpak AD-H (4.6 mm×150 mm, 5 μm particle size); n-hexane-isopropanol (93:7, v/v) with various concentrations of DEA as mobile phase at a flow rate of 1.0 ml/min with UV detection at 343 nm at 25℃.

II. SUPPLEMENTARY EQUATIONS

The linear regression curves between peak area and sample concentration were obtained by utilizing Rac-HCQ samples ranging from 1 to25 μg/mL. Three standard curves of Rac-, S- and R-HCQ were plotted respectively, and the linear equation and the square value of the correlation coefficient (R2) were obtained. The equation for Racemate was y=21801x-1927.9 (R2 = 0.9955); the equation for S-enantiomer was y=13173x-4205 (R2 = 0.9968); the he equation for R-enantiomer was y=13331x-3378 (R2 = 0.9973).