Supplementary Figure 1. Measurement and analysis of patient off-center distance using axial CT images. (A) Measurement of off-center distance: (ⅰ) select a transverse image containing manubrium and draw a horizontal line (green dashed line) that passes through both armpits. (ⅱ) locate the center of the display field of view (DFOV) of image. (ⅲ) record the vertical distance from the center of DFOV (blue dot) to the axillary line (red dot). The vertical distance (red line) represented the off-center distance. Images are displayed at window width of 670 HU and window level of 80 HU. (B) The off-center distances were comparable between MP and AP group. Statistical p-values were calculated by Wilcoxon signed-rank tests. Bars present the mean ± standard deviation (SD). ** denote p < 0.001 n = 127 each.