2. MATHEMATICAL FORMULATION

Mathematics is an essential component of any simulation. This chapter is dedicated to describing the mathematical modeling of the subject under investigation. Throughout this work, it is noted that Darcy's flow model serves as the foundation for most recent research. The focus of this study is on the porous material confined within a simple geometric surface. The governing equations, which vary depending on the geometry, are presented in their respective forms. In order to minimize the heat content in the bottom portion of the physical domain, the lower wall of a non-square cavity is heated with both uniform and non-uniform temperatures in this study. Additionally, the impact of radiation is examined. While keeping the boundary conditions, the lower surface of the cavity is heated differentially using a power law coefficient (λ). The top horizontal surface of the chamber is cold, and its two angled walls are adopted isentropic process.

Figure. 1. Working model

- The fundamental equations for steady, two-dimensional buoyant heat transfer distribution in a porous cavity, built on the principles of mass, momentum, and energy conservation, are typically derived from the Darcy-Brinkman model or a similar model that incorporates the effects of porosity. These equations generally include.

Continuity Equation (Mass Conservation):

The mass conservation for porous medium is given by:

(1)

Where u and v are the velocity components in the x- and  y-directions, respectively.

Momentum Equations (Darcy  Model):

The momentum equations consider for the forces acting on the fluid. In a porous medium, these correlations are modified to include permeability effects. The x- and y-momentum equations are:

X - Direction:

(2)

Where K   is the permeability of the porous medium, and  p is the pressure.

Y-direction:

(3)

Where ρ is the fluid density, g is the gravitational acceleration,.

Energy Equation (Thermal Energy Conservation):

The energy equation for buoyant energy transfer in a working medium is governed by the balance between conduction and convection. It is given by:

(4)

Where T is the temperature, and α is the thermal diffusivity of the fluid.

The expressions can be made dimensionless using suitable scaling variables for velocity, temperature, and coordinates. Typically, the dimensionless parameters involved include:

- Rayleigh number (Ra): characterizing the buoyancy force.

- Prandtl number (Pr): characterizing the ratio of kinematic viscosity to thermal diffusivity.

The non-dimensional forms of the governing equations are then more commonly used in the analysis of natural convection problems in porous cavities.

- (5)

NuLocal =                                        							                        (6)

## 3. GRID REFINEMENT STUDY

A grid refinement test was exemined for a buoyant heat transfer in a porous cavity at a Ra = 1000. Different mesh sizes were used to discretize the computing domain, ranging from a coarse grid of 30 × 30, 40 × 40, 50 × 50, to 60 × 60.The results showed that as the mesh was refined, the solution gradually stabilized, with noticeable improvements in accuracy. However, beyond a mesh size of 40×40, no significant changes in the flow or temperature fields were observed, indicating that the solution had converged. Therefore, a mesh size of 40×40 was deemed sufficient to capture the key flow characteristics while maintaining computational efficiency.

Table 1: Mesh convergence Test for Ra = 103

| Mesh size | [Present] | [1] |
| --- | --- | --- |
| 10 x 10 | 4.683 | 6.935 |
| 20 x 20 | 6.543 |  |
| 30 x 30 | 6.862 |  |
| 40 x 40 | 6.935 |  |
| 50 x 50 | 6.936 |  |
| 60 x 60 | 6.936 |  |

## 4. NUMERICAL VALIDATION

The validation of the present numerical model can be performed by comparing the obtained results with the benchmark data provided by Varol et al [16]. For buoyant energy transfer in a non-square  chamber. In their study, they considered a chamber with an aspect ratio (AR) of 0.5, tilted at a 60-degree angle, with uniform heating applied to the lower surface.

The figures provided by Varol et al [16]. Display isotherms and stream functions for this configuration. The isotherms, representing the temperature distribution, show a clear thermal stratification with contour lines indicating heat transfer within the chamber. The fluid patterns plot illustrates the flow patterns generated by natural convection, where the circulation cells are characterized by streamlines enclosing a central vortex.

In the current work, the computed isotherms and stream functions were compared against those of Varol et al. The temperature distribution and flow patterns show good agreement, particularly in the locations and shapes of the isothermal layers and the circulation regions. The central vortex in the stream function plot is well captured, with its magnitude aligning closely with the reference value (-5.84). Similarly, the thermal gradients along the heated surface match the reference isotherms, confirming the accuracy of the thermal flow simulation.

Any slight deviations observed in the comparison could stem from differences in grid resolution, numerical schemes, or boundary condition implementations. Nonetheless, the overall accuracy between the present results and those of Varol et al. [16]. Confirms the reliability of the model for simulating buoyant energy transfer in a non-square chamber under the specified conditions.

.

Figure. 2:  Varol et al. [16] provided isotherms and velocities for AR = 0.5 for a trapezoidal chamber tilted at 600 uniform heating of the lower surface

5. RESULTS AND DISCUSSIONS.

## 5.1 Effect of   λ When Rd = 1

The diagram represented the effect of the heat source length parameter λ on the temperature contours and flow patterns in a trapezoidal cavity with a uniform heating condition at the lower surface. where the Rayleigh number Rd=1
When (λ=0) the heat source length is zero, the isotherms are concentrated near the lower boundary, indicating that the heat input is localized at the base of the trapezoidal cavity. The temperature gradients are steepest near the heated surface, and thermal diffusion extends towards the top of the cavity.
As λ increases to 0.5, the isotherms begin to spread out more evenly, with a broader region of the lower surface contributing to the heating. The flow-induced convection results in smoother temperature gradients throughout the cavity, with the isotherms bending upward more noticeably, were signifying stronger natural convection effects.
At λ=0.75 the isotherms further expand upwards, showing a more pronounced distribution of heat within the cavity. The isotherms exhibit less curvature near the bottom, implying more uniform heating, and there is greater thermal mixing as convection cells become more established.
When the entire lower surface is uniformly heated (λ=1), the isotherms are distributed uniformly from the bottom to the top of the cavity, showing an even temperature profile. The heating extends the thermal stratification across the cavity, indicating well-developed natural convection that facilitates thorough heat transfer. For λ=0, the stream function contours show weak convection with two distinct circulation cells near the heated boundary. The flow is primarily confined to the lower region, and the rest of the cavity remains relatively stagnant.
With λ=0.5, a larger central circulation cell forms in the cavity, indicating stronger natural convection. The streamlines reveal a more intense circulation pattern compared to λ=0, suggesting that more heat is being convected throughout the cavity. At λ=0.75, the circulation strengthens further, and a single dominant vortex emerges, covering most of the cavity. The streamline pattern is denser, reflecting a higher velocity and more efficient fluid movement. When the entire bottom side is heated (λ=1), the fluid flow is dominated by a strong, large central vortex, with higher stream function values, indicating intense convective motion. The direction of flow indicates that convection predominates in the heat transfer process, resulting in effective heat distribution throughout the cavity.

As λ increases, the heating becomes more uniform, resulting in better-developed thermal fields (isotherms) and more vigorous convective motion. The flow transitions from weak convection with small circulation cells at low λ to strong and dominant buoyant heat transfer with large vortices at λ=1 indicating efficient energy transfer entire  the trapezoidal cavity.

Figure. 3: For constant Ra, different power model and Rd=1, fluid flows and fluctuations in temperature.

## 5.2 Effect of Power Law Model When Rd = 2

Figure. 4 flow directions and isotherms contours for Ra=500 with different power models and Rd=2

The image displays a series of contour plots illustrating temperature distribution on the left and fluid motion on the right, within a trapezoidal chamber for different magnitutes of the parameter λ. The parameter λ likely represents a geometric or physical variable affecting the system, such as the inclination of the chamber or the H/L ratio. when λ=0,The temperature distribution is mostly stratified in horizontal layers, with temperature contours remaining relatively parallel near the hot surface (bottom) and cooler towards the top. This indicates that heat transfer is dominated by conduction, as the contours are closely packed near the top corners of the chamber .When λ=0.5,the isotherms begin to curve near the sides of the enclosure, suggesting an increase in density effect as the contours bend upwards along the sidewalls. There is still a mix of conduction and convection occurring. When λ=0.75, the isotherms show more pronounced curvature, indicating stronger convection effects. The buoyant force is increasingly dominated by convection as the flow within the chamber becomes more vigorous and λ=1 The isotherms display significant curvature, especially near the mid-point of the cavity, showing that convective energy transfer is now the pre dominant mode of heat transfer. The scatter of temperature contours becomes more influenced by the circular flow patterns.

### On Right Column, When λ=0 the streamlines indicate two symmetric circulation cells in the cavity, with fluid flowing in a circular pattern. The flow is relatively weak, with streamlines showing that convection is limited and λ=0.5 The circulation cells become more distinct, showing stronger fluid motion. The fluid circulates around the cavity more actively, which enhances convective heat transfer. When λ=0.75,A single strong circulation cell forms, indicating more vigorous flow. The streamlines become more compact, suggesting increased convective activity. When λ=1 The single circulation cell is well-developed, with tight streamlines near the mid-point of the cavity. This indicates the highest level of convection, with a strong rotating flow pattern that dominates convective energy transfer in the chamber.

As λ changes  from 0 to 1, the mode of heat exchange shifts from conduction-dominated (λ=0) to convection-dominated (λ=1).The stream functions show the evolution of flow structures as convection strengthens, transitioning from weak circulations to a strong central vortex .The isotherms reflect this shift, with temperature contours becoming more distorted as convective effects grow, showing that fluid motion enhances thermal mixing in the chamber. This diagram illustrates the complex interaction between fluid flow and free convective energy transfer in a non-square chamber under varying conditions, likely influenced by geometric changes or fluid properties.

## 5.3   Impact of Power Law Coefficient on different walls

- Figure 5: Variation of local Nusselt number with respect to λ for heated wall

Figure 5 depicts the effect of the power-law coefficient λ on the local Nu allocation along the non-dimensional length X of the heated wall in a non-square cavity. The local Nu represents the buoyant energy transfer rate at specific points along the wall. The four curves correspond to various values of the power-law coefficient (λ), which governs the different heat input across the heated surface.

For λ=0, where the heating is uniform, the local Nu reaches a apex at approximately X ≈ 0.5, indicating that the mid-point of the heated surface experiences the highest buoyant energy transfer. The Nu drops significantly near the edges, with negative values at the ends, indicating heat loss in those regions.As the power-law coefficient increases to λ=0.5, the peak heat transfer shifts slightly to the left (toward X≈0.45), and the maximum Nusselt number decreases compared to λ=0. This suggests that the power-law heating redistributes the buoyant energy transfer more evenly across the surface, reducing the sharp peaks in heat transfer. With further increases in λ, as seen in the λ=0.75 and λ=1 cases, the buoyant energy transfer becomes more uniform along the heated wall. The peak local Nu continues to decrease, indicating a more gradual distribution of heat, with the differences in heat transfer between the central and edge regions diminishing. The profiles for λ=0.75 and λ=1 demonstrate smoother curves, showing less pronounced peaks and more stable buoyant energy transfer rates across the wall.

As the power-law coefficient λ increases, the heat transfer distribution becomes more uniform, reducing the apex values of the local Nu and spreading the free convective energy transfer more evenly along the heated surface. The diagram clearly shows that applying a power-law heating condition reduces the intensity of localized heat transfer, creating a more balanced thermal environment across the wall.

- Figure 6: Variation of local Nu with power law coefficient for cold wall

## 5.4 Impact of Power Law Coefficient on different Radiation factors

- The Figure.7 shows the relationship between the average Nu and the power law coefficient for different values of Rd (Radiation parameter).   When Rd = 0, this curve shows the average Nu when radiation is absent. As the power law coefficient increases, the Nu decreases, indicating a reduction in heat transfer efficiency. Rd = 1 this curve shows the Nu when the radiation parameter is moderate. The same trend of decreasing Nu with an higher power law coefficient is observed, but the overall Nu is higher than for Rd = 0.  Rd = 2, this curve shows the Nu when the radiation parameter is higher. Once again, the Nu lower value with enhancing power law coefficient, but for any given power law coefficient, the Nu is peak magnitute than for Rd = 0 and Rd = 1.

- Figure 7: Effect of Mean Nusselt number with various power law coefficients

Table 4.2: Power law models for various radiation effects

| Radiation Parameter | Power-law Correlations | Regression Coefficients |
| --- | --- | --- |
| Rd = 2 |  | R2= 0.996 |
| Rd = 1 |  | R2= 0.997 |
| Rd = 0 |  | R2= 0.995 |

The table highlights the strong relationship between the radiation parameter and the power-law correlations across different values of Rd. All three R2 values are close to 1, representing that the power-law correlations provide an excellent fit to the data regardless of the radiation parameter value. This indicates that the model is robust and reliable for predicting system behavior across varying levels of radiation influence.  The regression coefficient (R2) represents how well the power-law correlation fits the data. An R2 value of 1.0 would indicate a perfect fit, while values close to 1, like those in the table, show a very strong correlation between the model and the data.

(7)

R2= 0.9785

Fig. 8: Differentiation of present data with equation (7).

The diagram presents a comparison between the normalized Nu obtained from the present simulation data and the corresponding values calculated using Equation (7). The purpose of this comparison is to validate the accuracy of the current numerical model by differentiating the present data across a range of Radiation factors (Rd).The x-axis represents the normalized actual Nusselt number obtained from the numerical simulations.The y-axis shows the normalized Nusselt number computed from Equation (7).The solid black line represents a perfect correlation (slope = 1) between the actual values and those predicted by Equation (7). It serves as a reference line, showing ideal agreement.All the data points, regardless of the value of Rd, fall very close to the reference line (solid black line), indicating a strong agreement between the simulated Nusselt number and the values predicted by Equation (7).The data points for Rd=0, Rd=1, and Rd=2 all exhibit a similar trend, with minimal deviation from the reference line across the full range of normalized Nu showing that the model performs consistently across different conditions.

The close alignment of the data points with the reference line validates the accuracy of the present numerical model. It shows that the relationship between the normalized actual Nusselt number and the predicted values from Equation (7) is highly consistent, indicating reliable performance of the model for various Rayleigh-Darcy numbers (0, 1, and 2). This confirms the robustness of the model in predicting buoyant energy transfer characteristics in the studied system.

- 6. CONCLUSIONS

In this study, buoyant energy transfer within a non-square chamber with an aspect ratio (AR) of 0.5 was investigated, focusing on the combined effects of radiation and power-law heating applied to the lower heated wall. The cavity's slant walls were adiabatic, and the upper wall was maintained at a low temperature, creating a thermal gradient that drives the flow. The analysis revealed significant influences of radiation and the power-law parameter on the buoyant energy transfer characteristics and flow behavior within the chamber.

- The temperature dispersion and flow characteristics changed significantly when the power-law parameter increased strengthening convective motion and improving the transfer of heat.

- The presence of radiation further intensified these effects, especially near the heated surface, where thermal radiation contributed to an overall rise in the rate of buoyant energy transfer. The results also highlighted the importance of the boundary conditions at the slant walls, with the adiabatic nature of these walls restricting heat exchange and shaping the thermal convection cells.

- Overall, the study demonstrated that the combination of radiation and power-law heating plays a critical role in controlling the thermal and flow dynamics in trapezoidal cavities. These results provide valuable insights into the design and optimization of systems involving buoyant energy transfer in geometrically complex enclosures, particularly in applications where radiation and non-uniform heating are prominent factors.

- 7. NOMENCLATURE

| Nu | Local heat transfer co-efficient | γ | Inclination Angle |
| --- | --- | --- | --- |
| Ra | Rayleigh Number | V | Velocity Along Y Axis |
| Rd | Radiation factor | λ | Power Law Coefficient |
| K | Permeability ,m2 |  | Non Dimensional Temperature |
| x y | Dimensional Coordinates Along Two Directions | ψ | Stream Function |
| Th | Heated Wall Temperature,0 C | ψ | Dimensionless Stream Function |
| Tc | Cooled Wall Temperature,0 C | U | Velocity Along X Axis |

# 8. REFERENCES

- K. Aparna and K.N. Seetharamu, "Investigations on the Effect of Non-Uniform Temperature on Fluid Flow and Heat Transfer in a Trapezoidal Cavity Filled with Porous Media," International Journal of Heat and Mass Transfer, vol. 108, pp. 63–78, 2017.

- Nield, D. A., & Bejan, A. (2006). Convection in Porous Media. Springer.

- Ingham, D. B., & Pop, I. (2005). Transport Phenomena in Porous Media III. Pergamon Press.

- Das, S. K., & Mahmud, H. (2011). "Effect of Aspect Ratio on Heat Transfer and Fluid Flow in Trapezoidal Cavities." International Journal of Heat and Fluid Flow, 32(4), 642-652.

- Basak, T., Roy, S., & Pop, I. (2013). "Natural Convection Heat Transfer in Trapezoidal Enclosures." International Journal of Heat and Mass Transfer, 56(1-2), 267-279.

- Gupta, S., & Das, P. K. (2012). "Effect of Non-Uniform Heating on Natural Convection in Enclosures." Journal of Heat Transfer, 134(3), 032501.

- Mahmud, S., & Fraser, R. A. (2015). "Natural Convection in a Trapezoidal Cavity with Non-Uniform Heating." Journal of Thermophysics and Heat Transfer, 29(2), 324-332.

- Vafai, K., & Tien, C. L. (1981). "Boundary and Inertia Effects on Flow and Heat Transfer in Porous Media." International Journal of Heat and Mass Transfer, 24(2), 195-203.

- Lauriat, G., & Prasad, V. (1989). "Non-Darcy Convection in Porous Media." Journal of Thermophysics and Heat Transfer, 3(1), 62-66.

- Beckermann, C., & Viskanta, R. (1993). "Natural Convection in Enclosures with Porous Media." Applied Mechanics Reviews, 46(1), 1-27.

- Bilgen, E., & Oztop, H. F. (2005). Natural convection in inclined trapezoidal enclosures. International Journal of Heat and Mass Transfer, 48(1), 147–155.

- Chen, C. H., Chang, W. J., & Wang, C. C. (2012). Forced convection of power-law fluids in porous media. International Journal of Heat and Fluid Flow, 33(1), 165–175.

- Kumar, A., Singh, S., & Gupta, P. (2022). Parametric study on energy transfer characteristics of non-Newtonian fluids in porous trapezoidal cavities. Journal of Fluid Mechanics, 920, 113-129.

- Mahmoudi, A., Shahi, M., & Pop, I. (2010). Natural convection of non-Newtonian fluids in enclosures: A review. International Journal of Thermal Sciences, 49(6), 1305–1315.

- Nield, D. A., & Bejan, A. (2006). Convection in porous media (3rd ed.). Springer.

- Yasin Varol, Hakan F.Oztop,Ioan Pop, “ Maximum density effects on buoyancy-drivenconvection in a porous trapezoidal cavity”, Int. Communication in Heat and MassTransfer, Vol. 37, 2010, pp 401-409.