Supplementary Material

S_1 Chaotic Maps

Logistic Map

The Logistic equation, popularly known as the Logistic map, is widely used in population dynamics. It is the simplest first-order nonlinear difference equation, whose dynamic characteristics takes the form

Xt+1=aXt1-Xt									(1)

where Xt+1 and Xt are the values of X at the (t + 1)th and tth iterations, respectively, and a is the fertility rate. The initial value of X can take any value randomly from (0, 1).

Fig. S_1(a) presents the variation of the Logistic time series, and Fig. S_1(b) presents the two-dimensional (2D) phase space diagram constructed with a delay time value of 1. The Logistic map’s quadratic nonlinearity results in the growth curve with a hump, as shown in its phase space diagram. The steepness of this 2D trajectory is dictated by the parameter a (fertility rate), whose value should be in the range of (1, 4) for nontrivial dynamic behavior (May 1976). The fertility rate, a, is also called the bifurcation value as the Logistic equation transforms into different behaviors qualitatively with the shift in this value. For a, ranging between “3.8 and 4.0”, the Logistic map exhibits purely chaotic behavior (May 1976). Many studies (Yuan et al. 2002; Cheng et al. 2008; Arunkumar and Jothiprakash 2013) have used the Logistic sequence with a = 4 in the optimization of water resources systems, stating that it exhibits a purely chaotic nature. Therefore, in the present study, to choose the best-fit fertility rate, the Logistic time series with the value of a, varying from 3.8 to 4 and with a step size of 0.02 is considered.

Though the Logistic equation is germane in various social and biological contexts, it bounds its physical applicability as X is limited in the range of (0, 1) (May 1976). However, in the present context, as GA works on probabilistic rules, the fusion of the Logistic map with GA is novel. The resulting chaos-directed genetic algorithm (CDGA) model with the Logistic map is named CDGA-I.

Henon Map

Henon (1976) conducted a numerical experiment to show that a simple two-dimensional map that may be given as

xi+1=Byi+A-xi2	;	yi+1=xi						(2)

exhibits chaotic behavior depending on its parameters A and B and the initial values of (x, y). Parameters A and B control the stretching and folding behavior of the Henon attractor. For A = 1.4, B = 0.3, and with the initial values of x0 = 0.13 and y0 = 0.5, the above equations give a chaotic time series. The values of the time series generated from these ranges between (-1.79853, 1.78216). To be compatible with the range required in the optimization models (0, 1), they are normalized using a simple min-max normalization method. Although the time series appears to be random (see Fig. S_1(c) for the normalized Henon series), the map evolves in the form of a strange attractor, as seen from the two-dimensional phase space diagram (Fig. S_1(d)). At this particular initial state and parametric values, the Henon map does not diverge to infinity with an increase in its data length. It confines its dynamic evolution within the attractor region with trajectories apparently wandering in a highly erratic fashion. The resulting CDGA model formulated using the Henon map is named CDGA-II.

Lorenz Map

Lorenz (1963) developed a simple mathematical continuous model in the form of ordinary differential equations that represents atmospheric convection. It is the earliest deterministic model notable in the context of chaos theory. It is a set of three ordinary differential equations, popularly known as the Lorenz map, and takes the form

S= σ (T-S);	T=rS-T-SZ;	Z=ST-bZ				(3)

where S is the convection current intensity, T is the temperature difference between the hot and cold currents, Z is proportional to the distortion from linearity of the vertical temperature profile, σ is the Prandtl number, r is the Rayleigh number, and b is the thickness of the fluid layer. The ‘dot’ represents the derivative for the non-dimensional time. The Lorenz map is sensitive to initial conditions and its parametric values, based on which it exhibits various behavioral regimes. In the present study, the initial values for this three-dimensional system are considered as (1, 1, 1), and a magnitude of 10, 28, and 8/3 is considered for σ, r, and b, respectively. In the present study, the Lorenz sequence is also normalized to have values in the range (0, 1). Fig. S_1(e) shows the one-dimensional variation of the normalized Lorenz S sequence and Fig. S_1(f) shows its attractor in three-dimensional phase space. The resulting CDGA model formulated using the Lorenz map is named CDGA-III.

Fig. S_1 Time series and phase space plots of Logistic, Henon, and Lorenz Maps

S_2 Computational Results for Bakryan Network

The computational results on the application of the CDGA models to the Bakryan network (BRN) are discussed as follows:

Results of CDGA-I Model

From the sensitivity analysis of the CDGA-I model, it is found that, unlike the GA model, the CDGA-I requires comparatively small population size Ps and generation size Gs for convergence. Fixing Ps, Gs, and crossover probability Pc as 53, 100, and 0.06, mutation probability Pm is varied from 0.05 to 0.1, with a step size of 0.01. For different fertility rate a, values, the computational results of the CDGA-I model for both the non-sequential approach (NSA) and sequential approach (SA) are presented in Fig. S_2(a) to Fig. S_2(d). It is observed that the CDGA-I model is successful in reaching the optimal solution with less number of minimum functional evaluations (MFEs) (as compared to that of the GA model, 2,200), at every a value and Pm combinations, except for a = 3.96 at Pm = {0.05, 0.06} of the NSA and at Pm = 0.08 of the SA (see Fig. S_2 (b)); see Figs. S_2(a), (c), and (d) for results at a = 3.94, 3.98, and 4.0, respectively. From the overall computational results, it is observed that the CDGA-I model requires very fewer MFEs (728), exploring a minimal percentage of the solution space (3.087 × 10-5), with an average computation time of 8.003 s, which is very less when compared to that of the GA model (which explores 9.330 × 10-5 percentage of the solution space requiring 2,200 MFEs, with an average computation time of 74.988 s).

Results of CDGA-II Model

For the CDGA-II model, by fixing Ps, Gs, and Pc as 53, 100, and 0.7, respectively, Pm is varied from 0.03 to 0.1. Both the NSA and SA with the x sequence excel in attaining the optimal cost of $903,620 for every combination of Pm considered, except for the SA with Pm = {0.03, 0.04}, as shown in Fig. S_2(e). With the y sequence, the optimal solution is not obtained at Pm = {0.03, 0.04, 0.05} using NSA and at Pm = {0.03, 0.04, 0.07} using SA (see Fig. S_2(f)). With the x sequence, for both the approaches at Pm = {0.06, 0.1}, and with the NSA at Pm = {0.03, 0.04, 0.07}, the optimal solution is located with a fewer functional evaluation when compared to the GA model (See Fig. S_2(e)). With the y sequence, at Pm = {0.06, 0.09, 0.1} for both the approaches and at Pm = 0.08 of the SA, the optimal solution is obtained with a fewer functional evaluation when compared to that of the GA model (Fig. S_2(f)). Of all the combinations with the CDGA-II model, the sequential x approach outperforms the others with much less computational effort requiring only 1,040 MFEs by exploring a very less percentage of the solution space, i.e., 4.411 × 10-5 in an average computation time of 8.034 s.

Results of CDGA-III Model

From the results of the sensitivity analysis using the CDGA-III model, only the S sequence with the SA is successful, in converging to the optimal cost of $903,620. The optimal parameter set ascertained is Ps = 204, Gs = 100, Pm = 0.09, and Pc = 0.7. For the optimal convergence, the MFEs required are 6,600 with an average computation time of 20.690 s. Thus, similar to the results obtained for TLN, the CDGA-III model excels in converging to the optimal solution for BRN with less average computation time, but with more MFEs. It explores 2.799 × 10-5 percent of the solution space for converging to the optimal solution, which is less in comparison with the total available solution space.

Fig. S_2 Computational results for BRN using CDGA-I and CDGA-II models

S_3 Computational Results for Goyang Network

The computational results on the application of the CDGA models to the Goyang network (GYN) are discussed as follows:

Results of CDGA-I Model

For GYN, from the results of the sensitivity analysis using the CDGA-I model, it is found that the Gs is the same as that of the GA model (50) with a slightly higher Pc and Pm values (0.7, 0.05). Fixing these parameters, Ps is varied from 50 to 300. The results from the sensitivity analysis using NSA and SA with different a values are presented in Fig. S_3(a) to (d). For every value of a considered, for both the NSA and SA, the MFEs required to reach the optimal cost of 177,009,557 KRW falls below 10,088 (i.e., less than GA model) when Ps = 53 (see Fig. S_3(a) to (d)). For any Ps value above 53, the MFEs required for locating the optimal solution is high, except for Ps = 102 at a = 4.0 using the SA (Fig. S_3(d); see Figs. S_3 (a), (b), and (c) for results obtained using a = 3.94, 3.96, and 3.98, respectively). The results demonstrate that the CDGA-I model outperforms the GA model in decreasing the search space and, thus, the computational effort. For the best combination of parameters set at a = 3.98 using the NSA, the percentage of the solution space explored by the CDGA-I model is 5.881 × 10-22 with a far fewer number of function evaluations (7,280) and with an average computation time of 27.002 s.

Results of CDGA-II Model

The application of the CDGA-II model proves its efficacy in converging to the optimal cost of 177,009,557 KRW for GYN. With a fixed Pc of 0.7, the best combination of Ps, Gs, and Pm, for locating the optimal solution is presented in Fig. S_3(e) to (h) for both the approaches using x and y sequences. It is evident, from Fig. S_3(e) to (h), that for all the combinations of parameters and with all the approaches, the MFEs are above 10,088, i.e., greater than the GA model. With the NSA, the lower MFEs required of all combinations are 28,000 and 22,256 using the x and y sequences, respectively. Similarly, with the sequential x and y approaches, the lower MFEs required are 26,400 and 29,000 respectively. From the results, it is clear that the CDGA-II model needs more computational effort when compared to the GA and CDGA-I models. However, with the best combination set of Ps = 53, Gs = 500, Pm = 0.035, and Pc = 0.7, and with the non-sequential y approach, on average, the time needed for executing the algorithm is less (25.985 s) when compared to that of the GA and CDGA-I models. With this optimal parameter set, the percentage of the solution space explored by the CDGA-II model for convergence to the optimal solution is 1.798 × 10-21. And the only variation in the optimal parameter set from the GA model is Pm and Pc, and from the CDGA-I model, it is Pm. Thus, the CDGA-II model excels over the GA and CDGA-I models in terms of the average computation time (low) and success rate (high) in achieving the optimal cost at every possible trial executed.

Results of CDGA-III Model

Using the CDGA-III model, from the sensitivity analysis results, it is found that, both the approaches with the three sequences of the Lorenz map fail to converge to an optimal cost of 177,009,557 KRW. However, the CDGA-III model with the three sequences using NSA and SA is successful in locating an optimal feasible design cost of 177,010,359 KRW, reported by Mora-Melia et al. (2015). And its variation from 177,009,557 KRW is just 0.00045 percent.

Fig. S_3 Computational results for GYN using CDGA-I and CDGA-II models

S_4 Performance Comparison

GA and CDGA Models

The results on the application of the GA and CDGA models on the three benchmark problems, TLN, BRN, and GYN, whose dimensions vary from 8 to 30 suggests that introducing the chaos ergodicity in the GA search phenomenon acts as a carrier wave in decreasing the search space, inducing faster convergence to the optimal solutions. Fig. S_4 presents the convergence plots (left) (i.e., the evolution to the minimum design cost over the generations) and the bar charts (right) (comparing the MFEs) for every model considered in the present study. These plots correspond to the optimal parameter sets that were finalized through the sensitivity analysis. For TLN, the GA model converged to an optimal cost of $419,000 at the 30th generation. Although the minimum cost obtained in the first generation is $1,210,000 with the CDGA-I model, it soon converged to the optimal design cost at the 16th generation, followed by the CDGA-II model and the CDGA-III model at the 32nd and the 327th generations, respectively (see Fig. S_4(a)). The CDGA-II model took two more generations in comparison to the GA model, but its MFEs is less than that of the GA model (due to its low Ps, i.e., 53), as shown in the bar diagram for TLN, Fig. S_4(b). For BRN, as shown in the convergence plot of Fig. S_4(c), the CDGA-I model converged faster to the optimal design cost of $903,620 at the 15th generation, followed by the CDGA-II model at the 20th, the GA model at the 22nd, and the CDGA-III model at the 33rd generations. For GYN, the CDGA-I model converged faster, followed by the GA and CDGA-II models at Gs of 140, 194, and 428, respectively, see Fig. S_4(c). The CDGA-III model failed to converge to the optimal solution of 177,00,9557 KRW.

For all the three benchmark problems, in terms of computational efficiency, the CDGA-I model excelled over the other models (Fig. S_4(b), (d), (f)). Although the CDGA-II model required more function evaluations when compared to the CDGA-I model, it outperformed with a 100% success rate. The same is observed with the CDGA-III model. However, with an increase in the number of decision variables, the CDGA-III model suffered from sub-optimal convergence (for GYN). When the Ps and Gs are further increased to check for the possibility of convergence to the optimal solution, the computational load increased, which negatively influenced the computational efficiency. The same was observed with another higher-dimensional chaotic map, the Mackey Glass equation (results not presented).

GA and CDGA with Other Optimization Models

In the present study, the consideration of four benchmark problems that are widely studied provides a platform for the comparison of the performance efficiency of the developed models with the other optimization models reported in the literature. For the TLN, the GA model developed in the present study itself outperforms some of the improvised GA models and advanced metaheuristic models reported in the literature (see Table S_4). A similar observation is also made for the BRN, as presented in Table S_4. For GYN, an improved feasible cost (177,009,557 KRW) is obtained in comparison with Kim et al. (1994), Geem (2006) among others. When the CDGA models are considered, the CDGA-I model is outstanding with its excellent performance in converging to the optimal design cost (for the respective benchmark problems) with a reduced computational effort when compared to the certain deterministic and metaheuristic optimization techniques, as presented in Table S_4.

Fig. S_4 Performance comparison of GA and CDGA models

Table S_4. GA and CDGA models in comparison to the models reported in the literature

| Authors | Algorithm | Functional evaluations |
| --- | --- | --- |
| Two loop network |  |  |
| Alperovits and Shamir (1977); Krapivka and Ostfeld (2009) | Linear programming (LP); GA-LP | sub-optimal solutions |
| Savic and Walters (1997); Reca and Martinez (2006); Poojitha et al. (2020) | GA | 65,000; 10,000; 3,600 |
| Cunha and Sousa (1999) | Simulated annealing | 25,000 |
| Eusuff and Lansey (2003) | Shuffled frog leaping | 11,155 |
| Geem (2006) | Harmony search (HS) | 5,000 |
| Suribabu and Neelakantan (2006); Sedki and Ouazar (2012) | Particle swarm optimization (PSO) | 5,138; 3,120 |
| Poojitha et al. (2020) | Differential Evolution (DE) | 7,600 |
| Sedki and Ouazar (2012) | PSO-DE | 3,080 |
| Present Study | i) GA ii) CDGA-I | i) 3,000 ii) 1,600 |
| Bakryan network |  |  |
| Lee and Lee (2001) | GA | 15,000 |
| Geem (2006) | HS | 5,000 |
| Poojitha et al. (2020) | GA, DE | 2,600 |
| Present study | i) GA ii) CDGA-I | i) 2,200 ii) 728 |
| Goyang network |  |  |
| Kim et al. (1994); Geem (2006) | GA; HS | sub-optimal solutions |
| Poojitha et al. (2020) | GA, DE | 7,800; 9,550 |
| Present study | i) GA ii) CDGA-I | i) 10,088 ii) 7,280 |

References

Alperovits E, Shamir U (1977) Design of optimal water distribution systems. Water Resources Research 13(6): 885–90

Arunkumar R, Jothiparakash V (2013) Chaotic evolutionary algorithms for multi reservoir optimization. Water Resources Management 27(15): 5207–5222

Cheng C-T, Wang W-C, Xu D-M, Chau K-W (2008) Optimizing hydropower reservoir operation using hybrid genetic algorithm and chaos. Water Resources Management 22(7): 895–909

Cunha MDC, Sousa J (1999) Water distribution network design optimization: simulated annealing approach. Journal of Water Resources Planning and Management 125(4): 215–221

Eusuff MM, Lansey KE (2003) Optimization of water distribution network design using the shuffled frog leaping algorithm. Journal of Water Resources Planning and Management 129(3): 210–225.

Geem ZW (2006) Optimal cost design of water distribution networks using harmony search. Engineering Optimization 38 (3): 259–277

Henon M (1976) A two-dimensional mapping with a strange attractor. Communications in Mathematical Physics 50(1): 69–77

Kim JH, Kim TG, Kim JH, Yoon YN (1994) A study on the pipe network system design using non-linear programming. Journal of Korean Water Resources Association 27(4): 59–67

Krapivka A, Ostfeld A (2009) Coupled genetic algorithm-linear programming scheme for least-cost pipe sizing of water-distribution systems. Journal of Water Resources Planning and Management 135(4): 298–302

Lee SC, Lee SI (2001) Genetic algorithms for optimal augmentation of water distribution networks. Journal of Korean Water Resources Association 34(5): 567–575

Lorenz EN (1963) Deterministic nonperiodic flow. Journal of the Atmospheric Sciences 20: 130–141

May RM, (1976) Simple mathematical models with very complicated dynamics. Nature 261: 459–467

Mora-Melia D, Iglesias-Rey PL, Martinez-Solano FJ, Ballesteros-Perez P (2015) Efficiency of evolutionary algorithms in water network pipe sizing.Water Resources Management 29: 4817-4831

Poojitha SN, Singh G, Jothiprakash V (2020) Improving the optimal solution of Goyang network – using genetic algorithm and differential evolution. Water Supply 20(1): 95–102

Reca J, Martinez J (2006) Genetic algorithms for the design of looped irrigation water distribution networks. Water Resources Research 42(5): W05416

Savic DA, and Walters GA (1997) Genetic algorithms for least-cost design of water distribution networks. Journal of Water Resources Planning and Management 123(2): 67–77

Sedki A, Ouazar D (2012) Hybrid particle swarm optimization and differential evolution for optimal design of water distribution systems.  Advanced Engineering Informatics 26: 582–591

Suribabu CR, Neelakantan TR (2006) Design of water distribution networks using particle swarm optimization. Urban Water Journal 3(2): 111–120

Yuan X, Yuan Y, Zhang Y (2002) A hybrid chaotic genetic algorithm for short-term hydro system scheduling. Mathematics and Computers in Simulation 59: 319–327