Appendix

Appendix A-1.

Since the default values are all expressed in calorific value units, when accounting carbon emissions, it is necessary to convert the physical quantities of various energy consumption into the corresponding average low-level heat generation according to the default values of net heat generation given in the guidelines. The default values of average low-level heat generation, carbon content per unit calorific value and carbon oxidation rate of various energy sources are shown in S-1 Table. The data of various energy consumption in China Energy Statistical Yearbook are adopted, and the following processing is carried out when using the original data: (1) Since no carbon dioxide emission is produced when electricity and heat are consumed, according to the principle of local emission and local accounting, the other energy species in the energy balance can be used to (2) Among the 8 major categories in the "input and output of process conversion", "thermal power generation" and "heat supply" are the most important aspects of carbon dioxide emissions. emissions, while "coal washing", "coking", "oil refining", "gas production", "natural gas liquefaction", and "natural gas production" are the most important components of carbon dioxide emissions. (2) "Natural gas liquefaction" and "coal products processing" mainly involve physical changes and can be ignored in the calculation; (3) in "end-use energy consumption", " (3) In "end-use energy consumption", the energy consumption of "raw materials and materials" does not produce carbon dioxide, so this part of energy consumption is not calculated.

S-1 Table

Combustion quality parameters of various energy sources

| Energy Variety | Average Low Calorific Value/ (KJ/kg or m3) | Carbon Content Per Unit Calorific Value /(t·TJ-1) | Carbon Oxidation Rate/％ | Carbon dioxide emissions Coefficient |
| --- | --- | --- | --- | --- |
| Raw Coal | 20908 | 26.37 | 0.94 | 1.9003kg-CO2kg |
| Cleaned Coal | 26344 | 25.41 | 0.98 | 2.4054 kg-CO2kg |
| Other Washed Coal | 8363-12545 | 33.56 | 0.98 | 1.0085 kg-CO2kg |
| Briquettes | 17608 | 33.56 | 0.90 | 1.9500 kg-CO2kg |
| Coke | 28435 | 29.50 | 0.93 | 2.8604 kg-CO2kg |
| Coke Oven Gas | 16726-17981 | 13.58 | 0.99 | 8.5550 kg-CO2kg |
| Other Gas | 5227-35544 | 13.58 | 0.99 | 9.9680 kg-CO2kg |
| Other Coking Products | 35891 | 29.42 | 0.99 | 3.8330 kg-CO2kg |
| Crude Oil | 41816 | 20.10 | 0.98 | 3.0202 kg-CO2kg |
| Gasoline | 43070 | 18.90 | 0.98 | 2.9251 kg-CO2kg |
| Kerosene | 43070 | 19.50 | 0.98 | 3.0179 kg-CO2kg |
| Diesel Oil | 42652 | 20.20 | 0.98 | 3.0959 kg-CO2kg |
| Fuel Oil | 41816 | 21.10 | 0.98 | 3.1705 kg-CO2kg |
| LPG | 50179 | 17.20 | 0.98 | 3.1013 kg-CO2kg |
| Refinery Gas | 46055 | 18.20 | 0.98 | 3.0119 kg-CO2kg |
| Other Petroleum Products | 35162 | 20.00 | 0.98 | 2.5270 kg-CO2kg |
| Natural Gas | 38931 | 15.32 | 0.99 | 21.6502 kg-CO2kg |
| Other Energy | 29307 | 27.75 | 0.93-0.98 | 2.7730 kg-CO2kg |

Appendix A-2.

The energy-intensive industries are electricity, heat production and supply industry, petroleum processing, coking and nuclear fuel processing industry, ferrous metal smelting and rolling processing industry, non-metallic mineral products industry, coal mining and washing industry, chemical raw materials and chemical products manufacturing industry, non-ferrous metal smelting and rolling processing industry, as well as paper production and paper products industry.

Appendix A-3.

Referring to Zhao et al. [S1], the output is the regional real GDP, and the input factors are the capital stock, employment and energy consumption. The indicator of capital stock is gross fixed capital formation, and the actual gross fixed capital formation from 2006 to 2019 is obtained by deflating the fixed asset investment price index which is based on the year of 2005, and account according to the perpetual inventory method, the fixed asset depreciation rate of 9.6%, and the actual gross fixed capital formation in 2006 is taken as the capital stock in 2006, after which the capital stock of the previous year was depreciated and added to the actual total fixed capital formation of the current year as the capital stock of the current year. The energy consumption was calculated by converting the energy used in each region into standard coal through the conversion factors.

Appendix A-4.

The regression results of the baseline model also indicate that digitalization mainly reduces carbon emissions in local area, and the effect in surrounding regions is insignificant. To verify the results, this paper also conducts the spillover effect tests by using the SDM, SAR and SAC model, estimate the coefficient by ordinary least squared method. The results are shown in the Table A-4.  The result in column (1) shows that the overall reducing effect of digitalization on carbon emissions is significant at the 5% level when the SDM is used. The regression result of the SAR model which is shown in column (2) also indicates that digitalization has a significant reductive effect on carbon emissions in the local area, but not significant in surrounding regions. So does the regression result in the column (3) when adopting SAC model. Thus, the digitalization index is significantly negatively correlated with carbon emissions in the local area and has no significant impact on the carbon emissions in surrounding regions, which verifies the result of the baseline regression.

S-4 Table

Tests of spatial spillover effects of digitalization on carbon emissions

|  | (1) | (2) | (3) |
| --- | --- | --- | --- |
| Effect | SDM | SAR | SAC |
| Direct | -0.0733*** | -0.0712*** | -0.0682*** |
|  | (0.0200) | (0.0206) | (0.0198) |
| Indirect | -0.0173 | -0.0126 | -0.0006 |
|  | (0.0296) | (0.0111) | (0.0073) |
| Total | -0.0906** | -0.0838*** | -0.0689*** |
|  | (0.0369) | (0.0271) | (0.0217) |
| Covariables | Yes | Yes | Yes |
| Province Effect | Fixed | Fixed | Fixed |
| Year Effect | Fixed | Fixed | Fixed |
| R-squared | 0.0750 | 0.3099 | 0.5641 |

Appendix A-5.

S-5 Table

Tests of serial correlation on the baseline regression

| Lags(p) | Q(p)-stat | P-value |
| --- | --- | --- |
| 1 | 7.63 | 0.006 |
| 2 | 7.64 | 0.022 |
| 3 | 13.38 | 0.004 |
| 4 | 13.54 | 0.009 |
| 5 | 14.38 | 0.013 |
| 6 | 15.91 | 0.014 |
| 7 | 37.05 | 0.000 |
| 8 | 40.92 | 0.000 |
| 9 | 41.47 | 0.000 |
| 10 | 46.09 | 0.000 |
| 11 | 46.09 | 0.000 |

In order to calculate the serial correlation lag order for the baseline regression, we also perform the serial correlation test in this paper. The null hypothesis of the serial correlation test is that there is no serial correlation up to order p, and the alternative hypothesis is that there is some serial correlation up to order p.

Table A-5 reports the results of the serial correlation tests for the baseline regression. As can be seen from the table, the number of lag periods from 1 to 11, the corresponded to p-values of the Q(p)-stat are all less than 0.1, so we can reject the null hypothesis and conclude that there is serial correlation of lag order 1 to 11. And the larger the lag order, the larger the Q(p)-stat. Therefore, simply using the current neural network (RNN) model will make that the more recent the time, the greater the impact on the output results, thus ignoring the impact of the variables more previous on the variables we need to predict.

Reference

- Zhao P., Gao Y., Sun X., 2022. How does artificial intelligence affect green economic growth?—Evidence from China. Science of the Total Environment.  834, 155306. http://dx.doi.org/10.1016/j.scitotenv.2022.155306.