List of table

Table 1: Geotechnical properties for Radès la Goulette Project	2

Table 2: Consolidation parameters	3

Table 3: Properties of field and laboratory soil	4

Table 4: Comparison between prediction methods of consolidation settlement	5

Table 1: Geotechnical properties for Radès la Goulette Project

| Layer | Thickness (m) | γ (kN/m3) | Cc/ (1+e0) | Cv (10-8m²/s) |
| --- | --- | --- | --- | --- |
| Clay I | 10 | 16.5 | 0.15 | 3 |
| Sand II | 12 | 20 | 0.11 | 9 |
| Clay III | 5 | 19.3 | 0.14 | 1.5 |
| Sand IV | 5 | 21 | 0.075 | 15 |
| Clay V | 5 | 19.1 | 0.15 | 1.5 |

Table 2: Consolidation parameters

| Consolidation of Tunis soft soil |  | Radès La Goulette Bridge Project (Tunisia) | Experimental study |  |
| --- | --- | --- | --- | --- |
| Technique of consolidation |  | Preloading in presence of geodrains | Preloading in presence of geodrains (Test1) single vertical drain model | Preloading in presence of geodrains (Test 2) multiple vertical drain model |
| Thickness of the improved layer H(m) |  | 10 | 0.24 | 0.24 |
| Vertical drainage path(m) |  | H/2=5 | H/2=0.12 | H/2=0.12 |
| Characteristics of geodrains | Length of geodrains (m) | 10 | 0.24 | 0.24 |
|  | Type of geodrains | Mebradrain 88 (MD88) | Mebradrain 88 (MD88) | Mebradrain 88 (MD88) |
|  | Grid pattern | Triangular | Triangular | Triangular |
|  | Diameter of the drain; dw(mm) | 67 | 6.7 | 2.96(3mm) |
|  | Spacing between geodrains S (m) | 1.2 | 0.12 | 0.053 |
|  | Equivalent diameter De(m)=1.05*S | 1.323 | 0.126 | 0.05565 |
|  | Spacing ratio n=De/dw | 18.8 | 18.8 | 18.8 |
| Preload |  | {21, 24.5, 28, 31.5, 35, 49, 52.5and 56kPa} | {21, 24.5, 28, 31.5, 35, 49, 52.5and 56kPa} | {21, 24.5, 28, 31.5, 35, 49, 52.5and 56kPa} |
| height scale H* |  | - | 0.024 | 0.024 |
| Settlement scale S* |  | - | 0.022 | 0.025 |

Table 3: Properties of laboratory soil

|  | (kN/m3) | Particle size analysis |  |  | Atterberg limits |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | %>5mm | %<2mm | %<8mm | L(%) | Ip(%) |
| Studied soil in laboratory | 18 | 0 | 92 | 89 | 34 | 17 |

Table 4: Comparison between prediction methods of consolidation settlement

| Time (days) | Settlement (m) |  |  | Degree of consolidation (%) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Asaoka | Field measurement | Difference between predicted and measured data (%) | Barron method | Field data | Difference between predicted and measured data (%) |  |
| 10 | 0.16 | 0.075 | 53.13 | 6.95 | 5.65 | 18.7 |  |
| 50 | 0.46 | 0.32 | 30.43 | 33.82 | 24.06 | 28.85 |  |
| 120 | 0.814 | 0.77 | 5.41 | 63.7 | 58 | 8.95 |  |
| 240 | 1.12 | 1.11 | 0.9 | 86.55 | 83.45 | 3.58 |  |
| 300 | 1.22 | 1.235 | 1.23 | 91.7 | 91 | 0.76 |  |
| 440 (220days after preloading) | 1.306 | 1.33 | 1.84 | 97.6 | 100 | 2.45 |  |