Appendix 1:

Python code for determining a pile's stiffness from the load-deformation curve:

(It should be noted that the load-settlement data fields (L, S) should be filled based on the load-settlement diagram.)

import numpy as np

import matplotlib.pyplot as plt

# Sample load-settlement data

load = np.array([0, L1, L2, L3, …])

settlement = np.array([0, S1, S2, S3, …])

# Plot the load-settlement curve

plt.plot(settlement, load, marker='o', linestyle='-')

plt.xlabel('Settlement (mm)')

plt.ylabel('Load (kN)')

plt.title('Load-Settlement Curve')

plt.grid(True)

plt.show()

# Calculate Initial Tangent Stiffness

initial_tangent_stiffness = (load[1] - load[0]) / (settlement[1] - settlement[0])

print(f'Initial Tangent Stiffness: {initial_tangent_stiffness:.2f} kN/mm')

# Calculate Secant Stiffness at the last data point

secant_stiffness = load[-1] / settlement[-1]

print(f'Secant Stiffness at maximum load: {secant_stiffness:.2f} kN/mm')

# Calculate Tangent Stiffness using numerical differentiation

tangent_stiffness = np.diff(load) / np.diff(settlement)

tangent_stiffness_mid_points = (settlement[:-1] + settlement[1:]) / 2

# Plotting Tangent Stiffness

plt.plot(tangent_stiffness_mid_points, tangent_stiffness, marker='x', linestyle='-', color='r')

plt.xlabel('Settlement (mm)')

plt.ylabel('Tangent Stiffness (kN/mm)')

plt.title('Tangent Stiffness vs. Settlement')

plt.grid(True)

plt.show()

# Display Tangent Stiffness values

for i, (deform, stiff) in enumerate(zip(tangent_stiffness_mid_points, tangent_stiffness)):

print(f'Tangent Stiffness at settlement {deform:.2f} mm: {stiff:.2f} kN/mm')

f