Supplementary information

Supercritical carbon dioxide extraction of astaxanthin from Corynebacterium glutamicum

Jan Seeger1, Maximilian Zäh2, Volker F. Wendisch1, Christoph Brandenbusch2 and Nadja A. Henke1,3,*

1 Genetics of Prokaryotes, CeBiTec, Bielefeld University, Bielefeld, Germany

2 Laboratory of Thermodynamics, TU Dortmund University, Dortmund, Germany

3 Current address: Institute for Process Engineering in Life Sciences, Karlsruhe Institute of Technology (KIT), Karlsruhe, Germany

* Correspondence: nadja.henke@kit.edu

Figure S1: Experimental workflow to determine extraction yield. The carotenoid content of the biomass was determined before and after scCO2 extraction. Based on that, the extraction yield was calculated.

Figure S2: HPLC chromatograms of carotenoid analysis. A: Chromatogram of initial carotenoid content. B: Chromatogram after scCO2 extraction at 68 °C, 550 bar, 9% (w/w) cosolvent, 0.5 h. C: Chromatogram after scCO2 extraction at 68 °C, 550 bar, 9% (w/w) cosolvent, 14 h.

Table S1: Parameters for scCO2 extraction. All extraction parameters and the corresponding extraction yield are given. All measurements, except the one at 68 °C, 550 bar, 0.5 h, 9% (w/w) cosolvent (n = 3, mean ± sd), were performed as single replicates.

| Temperature [°C] | Pressure [bar] | Extraction time [h] | Cosolvent [% (w/w)] | Extraction yield [%] |
| --- | --- | --- | --- | --- |
| 55 | 550 | 0.5 | - | 6.6 |
| 55 | 550 | 0.5 | 4 | 11.5 |
| 55 | 550 | 0.5 | 9 | 42.7 |
| 50 | 550 | 0.5 | 9 | 41.3 |
| 55 | 550 | 0.5 | 9 | 42.7 |
| 60 | 550 | 0.5 | 9 | 57.4 |
| 68 | 550 | 0.5 | 9 | 67.5 ± 3.7 |
| 75 | 550 | 0.5 | 9 | 47.1 |
| 50 | 650 | 0.5 | 9 | 61.5 |
| 55 | 650 | 0.5 | 9 | 47.0 |
| 68 | 650 | 0.5 | 9 | 40.3 |
| 75 | 650 | 0.5 | 9 | 44.6 |
| 68 | 500 | 0.5 | 9 | 44.7 |
| 68 | 450 | 0.5 | 9 | 40.5 |
| 68 | 550 | 14 | - | 56.3 |
| 68 | 550 | 14 | 9 | 93.3 |
| 68 | 550 | 0.5 | - | 16.7 |