Supplementary Materials

Microwire Implantation. The extracellular action potentials corresponding to single-neuron activity and local field potentials were recorded from the tips of microwires implanted bilaterally along with a clinical depth electrode used to record clinical field potentials (3, 4). The implantation sites were chosen according to clinical criteria, which limits the potential recording sites. For the patients studied here, however, the sites included the hippocampus, prefrontal cortex, anterior cingulate cortex, and amygdala, all bilaterally. In the hippocampus, the wires were targeted to be in the midbody of the hippocampus, just behind the head of the hippocampus, opposite the apex of the cerebral peduncle. In the prefrontal cortex, the wires were targeted to be in the ventromedial prefrontal cortex, below the anterior cingulate gyrus. In the anterior cingulate, the wires were targeted to be in the parts above and behind the genu of the corpus callosum. In the amygdala, the wires were targeted to be in the center of that structure.

Each recording site received a bundle of nine 38 µm diameter platinum-iridium microwires (California Fine Wire), implanted stereotactically (Medtronic StealthStation) using a 1.5 T structural MRI. Electrodes were placed through a skull bolt with a custom frame to align the depth electrode along the chosen trajectory. The error in tip placement using this technique is estimated to be ± 3 mm based on manual inspection of the pre-operative MRI and post-operative CT and prior work (5, 6).

Pre-onset spike counts were recorded 200 - 1000 ms before the onset of the test stimulus for both the visual and auditory sessions. Post-onset spike counts for each cluster were recorded 200 -1000 ms after the onset of the visual stimulus and 200 - 1000 ms after the offset of the sound file for the auditory stimuli. The test period during which post-stimulus spike counts were recorded was chosen because a previous study (7) found that selective responses of hippocampal neurons began ∼300 ms after stimulus onset and because nearly all behavioral responses occurred after 1 s.

Filtering and Event Detection. Extracellular potentials were recorded from the tips of the microwires using techniques previously described (8) and digitized at 29,412 Hz with 16-bit resolution. Possible action potential events (APs) were detected using digital filtering and thresholding (9). Because more than one neuron may be recorded near any given electrode, APs were sorted into several clusters of similar waveform shape using the open-source clustering program KlustaKwik (Klustakwik.sf.net). After sorting, each cluster was graded as being noise, multiunit activity (MUA), or single-unit activity (SUA) based on criteria such as the waveform shape, size of the waveform relative to noise, evidence of a refractory interval, and lack of powerline interference, using the criteria described previously (8).

In our experience, this technique produces results comparable to prior reports in other laboratories (9) in terms of recorded waveform shapes, interspike intervals, and firing rates. While it is important to note that these and other reports of human single-unit recordings (10) do not achieve the quality of unit separation achievable in animal recordings (11) they nonetheless represent neural activity at a much finer spatial and temporal scale than is achievable using other methods such as fMRI.

Data Analysis. First, we identified neurons that responded differently according to whether words were novel (presented the first time) or repeated (presented a second time). For each neuron in each brain region, raw post-stimulus (200 ms to 1000 ms after stimulus onset) spike counts in response to novel words were compared with those in response to repeated words using a paired t-test. Neurons were classified as familiarity neurons if they spiked more to repeated words than to novel words (p < 0.05), and as novelty neurons if they spiked more to novel words than to repeated words (p < 0.05). Raw spike counts were used here as in earlier studies that identified familiarity and novelty neurons (12-14).

Second, we looked for the sparse coding signal in the four brain regions. For every recorded neuron, we computed post-stimulus (200 ms to 1000 ms after stimulus onset) normalized spike counts for each trial (i), where a “trial” refers to the presentation of a novel or repeated word. For each neuron (j), its baseline spike counts (1000 ms to 200 ms before the onset of the word) were computed across all trials in that session (mean and SD of spike counts, μj and σj, respectively). Normalized post-stimulus spike counts for a given trial (Nij) in which sij raw spike counts were recorded on trial i for neuron j is given by Nij = (sij − μj)/σj. A quantile-quantile (QQ) plot was generated for each brain area for the full distribution of rank-ordered normalized single-unit spike counts for novel vs. repeated items. Each distribution contained the spike count for all recorded neurons in response to all test stimuli across all sessions and all patients. Skewness and kurtosis were also calculated for each distribution. We carried out bootstrap analyses to examine whether the skewness and kurtosis for the distribution of repeated items differed significantly from the corresponding parameters of the distribution of novel items.  For each test (e.g., comparing the skewness for novel and repeated items in the hippocampus), 10,000 bootstrap trials were performed. In each case (i) the normalized post-stimulus spike counts from all repeated words (nRepeated) and all novel words (nNovel) were combined; (ii) n spike counts were randomly sampled with replacement from the combined dataset and labelled as nRepeated bootstrap “targets”, and then n spike counts were randomly sampled with replacement from the combined dataset and labelled as nNovel bootstrap “foils”; and (iii ) the difference between the statistic of interest (e.g., skewness) of those two bootstrap samples was computed. The proportion of the 10,000 bootstrap trials in which the absolute value of the difference was greater than the observed difference yielded the p-value.

References

1.	J. T. Wixted et al. Proc Natl Acad Sci U S A 115, 1093-1098 (2018).

2.	M. Coltheart. Quarterly Journal of Experimental Psychology Section A 33, 497-505 (1981).

3.	A. M. Dymond, T. L. Babb, L. E. Kaechele, P. H. Crandall. Biomed Sci Instrum 9, 1-5 (1972).

4.	I. Fried et al. J Neurosurg 91, 697-705 (1999).

5.	M. Nowell et al. Neurosurgery 10 Suppl 4, 525-533; discussion 533-524 (2014).

6.	K. Roessler et al. World Neurosurg 94, 206-210 (2016).

7.	R. Q. Quiroga, L. Reddy, G. Kreiman, C. Koch, I. Fried. Nature 435, 1102-1107 (2005).

8.	A. B. Valdez, E. N. Hickman, D. M. Treiman, K. A. Smith, P. N. Steinmetz. J Neural Eng 10, 016001 (2013).

9.	I. V. Viskontas, B. J. Knowlton, P. N. Steinmetz, I. Fried. J Cogn Neurosci 18, 1654-1662 (2006).

10.	G. Kreiman, C. Koch, I. Fried. Nat Neurosci 3, 946-953 (2000).

11.	D. N. Hill, S. B. Mehta, D. Kleinfeld. J Neurosci 31, 8699-8705 (2011).

12.	U. Rutishauser, A. N. Mamelak, E. M. Schuman. Neuron 49, 805-813 (2006).

13.	U. Rutishauser, E. M. Schuman, A. N. Mamelak. Proc Natl Acad Sci U S A 105, 329-334 (2008).

14.	U. Rutishauser et al. Nat Neurosci 18, 1041-1050 (2015).