Supplementary information

For

Glacier surface velocities in the Chandrabhaga massif, western Himalaya (India) derived using COSI-Corr from Landsat images

Suresh Das

Centre for the Study of Regional Development (CSRD), Jawaharlal Nehru University, New Delhi 110067, India

Correspondence email: suresh41_ssf@jnu.ac.in

ORCID iD: https://orcid.org/0000-0003-4148-8551

Table S1. List of available image matching software.

| Methods | Software/Instrument | Descriptions | Cost | Advantages | Disadvantages | References |
| --- | --- | --- | --- | --- | --- | --- |
| Feature tracking | IMCORR (Image Correlation) | IMCORR is developed at the National Snow & Ice Data Center (NSIDC) and implemented in open-source SAGA software. | Free | Easy to use | Limited uncertainty/miscorrelation information regarding cross-correlation. | (Scambos et al. 1992) (Orheim and Lucchitta 1987) |
| Feature tracking | PyCORR (Python Correlation) | PyCorr takes an identical approach to the IMCORR software package. GoLIVE velocity datasets are being derived using this tool. | Free | Computationally more efficient. open-source libraries for cross-correlation and geospatial transformations of the data | Lack of uncertainty/miscorrelation information regarding cross-correlation. | (Fahnestock et al. 2016) |
| Feature tracking | CIAS (Correlation Image AnalysiS) | CIAS is a free correlation software to track motion in two precisely co-registered images. It can work with normalized cross-correlation or orientation correlation implemented. CAIS is developed in IDL and integrated with Exelis ENVI. | CIAS: Free. ENVI: Commercial | Simple user-friendly software for matching offsets between two images. Based on normalized cross-correlation. | Normalized cross-correlation (NCC) and orientation correlation implemented (NCC-O), no other algorithms. Window-based. It cannot be scripted to automize the matching of more than one image pair. | (Kääb and Kneisel 2006) (Kääb and Vollmer 2000) (Kääb 2002) (Kääb et al. 2005) |
| Feature tracking | ImGRAFT ((Image GeoRectification and Feature Tracking) | ImGRAFT is developed at the Centre for Ice and Climate, Niels Bohr Institute, at the University of Copenhagen. | ImGRAFT: Free. Matlab: Commercial. | It is an open-source image geo-rectification and feature tracking toolbox for Matlab. | The camera setup is difficult for high-altitude glaciers. | (Messerli and Grinsted 2015) |
| Feature tracking | COSI-Corr (Co-registration of Optically Sensed Images and Correlation) | Cosi-Corr is written at the California Institute of Technology. It is used in many fields, such as coseismic deformation, sand dune migrations, slow landslides, and glacier flows. Cosi-Corr is developed in IDL and integrated with Exelis ENVI. | COSI-Corr: Free. ENVI: Commercial. | Automated workflows. Dense velocity field coverage. Glacier-scale spatial coverage. Fast computation. | Imagery choice: optical or SAR Imagery spatial and temporal resolution. Cloud covered scenes poor image contrast/shadow on optical imagery | (Ayoub et al. 2009b) (Ayoub et al. 2009a) (Bridges et al. 2012) (Leprince et al. 2007) |
| Feature tracking | autoRIFT | This effort was funded by the NASA MEaSUREs program in contribution to the Inter-mission Time Series of Land Ice Velocity and Elevation (ITSLIVE) project. | Free | Dense feature tracking. | A limited number of uncertainty/miscorrelation information regarding cross-correlation. | (Gardner et al. 2018) |
| Feature tracking | MicMac (Multi-Image Matches for Auto Correlation Methods) | MicMac is a French Linux-based open-source software package developed by Marc Pierrot-Desseilligny. The matching process has a pyramidal approach and derives a dense point cloud using an energy minimization function and regularization. | Free | MicMac is capable of carrying out the complete process from the raw UAV data to structure from motion and create a deformation map. | Much of its documentation is in French. Mostly applicable for UAV-derived images. | (Ahn and Howat 2011) |
| SAR Interferometry | GAMMA Software | GAMMA Software supports the entire processing chain from SAR raw data to products such as digital elevation models, displacement maps, and land use maps. | Commercial | Modular SAR Processor (MSP) Interferometry, Differential Interferometry and Geocoding (ISP/DIFF&GEO), Land Application Tools (LAT) | High cost. Limited of uncertainty/miscorrelation information regarding cross-correlation. | (Werner et al. 2000) |
| SAR Interferometry | ENVI Sarscape | ENVI SARscape allows processing and analyzing SAR data easily and generating products like DEMs or surface deformation maps while giving the option to integrate this information with other geospatial products. | Commercial | Easy to use. Nearly complete SAR data assessments. | High cost. | (Yan et al. 2015) |
| SAR Interferometry | SNAP | A typical architecture for all Sentinel Toolboxes is jointly developed by Brockmann Consult, Array Systems Computing, and C-S called the Sentinel Application Platform (SNAP). This software extensively supports Sentinel 1 SAR dataset and post-processing. | Free | –Extensibility, Portability, Modular Rich Client Platform, Generic EO Data Abstraction, Tiled Memory Management, and a Graph Processing Framework. | –Only applicable for Sentinel series. –Limited number of uncertainty/miscorrelation information regarding cross-correlation. | (Winsvold et al. 2018) |
| dGPS/Total station | stakes survey | Glacier velocity is traditionally measured by placing stakes on the glacier surface grid formation and monitoring their displacement at regular intervals with field surveys. With the GPS method, the position can be determined with an accuracy of several meters. | Expensive to conduct field surveys, and equipments are costly. | Sub-metre accuracy (sub-centimeter with certain dGPS setups) • Horizontal and vertical displacement information • Point measurements of velocity, rather than an average for a pixel/ cell (e.g., feature tracking) • Sub-daily temporal resolution possible | Site accessibility • Selecting appropriate visible and persistent markers on a debris-covered surface • Spatial coverage limited to only several km² depending upon terrain. - Expensive | (Schneeberger et al. 2000) (Hooke et al. 1989) (Fujita et al. 2001) (Macgregor et al. 2005) (Wagnon et al. 2007) |
| Manual | Boulder movement | The velocity of an individual glacier can be determined from specific boulder movement (location marked with high precision GPS) in different time periods. | Field-based expensive | Sub-metre accuracy (sub-centimetre with certain dGPS setups) • Point measurements of velocity, rather than an average for a pixel/ cell (e.g. feature tracking) | Site accessibility. Costly equipment. Field-based mapping is not possible for all glaciers in the Himalayas. | (Kirkbride 2012) (Haeberli et al. 1979) |

Table S2. Error assessment of derived surface velocity datasets. In the present study, 10000 random points were derived from off-glacier area. The entire data processing was categorized into four steps, and the mean and standard deviation after those procedures were evaluated. The four steps represent the raw results of the original COSI-Corr process (R0), after applying the SNR and magnitude filter (R1), after removing the orbital error (R2), and after removing the satellite artifacts and attitude effects using destripe tool (R3). Here, σoff is the error of the off-glacier velocity, MEAN is the mean stack velocity of random points, SE is the standard error of the mean velocity, STD is the standard deviation of the off-glacier area velocity. Noff is the number of measurements in the off-glacier area.

| Image pair | Time [day] | Time [year] | Direction | Mean [m] |  |  |  | St.dev. [m] |  |  |  | Velocity in stable terrain (m) |  | Error assessment of final velocity product based on stable terrain (m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | (Based on 10000 random points) |  |  |  |
|  |  |  |  | R0 | R1 | R2 | R3 | R0 | R1 | R2 | R3 | Mean | stdev | SE= STDNoff. | σoff=SE2+MEAN2. |
| 14Aug1992-17Aug1993 | 368 | 1.008219 | E/W | 2.447 | 20.447 | 0.124 |  | 4.765 | 4.765 | 4.733 |  | 4.915 | 3.645 | 0.0364 | 4.9156 |
|  |  |  | N/S | 2.419 | 2.419 | 0.445 |  | 6.304 | 6.304 | 5.7332 |  |  |  |  |  |
| 17Aug1993-20Aug1994 | 371 | 1.016438 | E/W | -1.474 | -1.474 | 0.664 |  | 3.914 | 3.914 | 3.782 |  | 3.896 | 2.639 | 0.0263 | 3.8965 |
|  |  |  | N/S | -2.534 | -2.534 | -0.555 |  | 3.594 | 3.594 | 3.325 |  |  |  |  |  |
| 28Aug2000-02Aug2002 | 704 | 1.928767 | E/W | 1.356 | 1.356 | -0.312 |  | 4.001 | 4.001 | 3.556 |  | 2.4069 | 1.8763 | 0.0187 | 2.406 |
|  |  |  | N/S | -0.003 | -0.003 | 1.102 |  | 6.283 | 6.283 | 4.736 |  |  |  |  |  |
| 30Sept2009-17Sept2010 | 352 | 0.964384 | E/W | -0.608 | -0.608 | -0.245 |  | 3.5033 | 3.5033 | 3.465 |  | 4.125 | 3.445 | 0.0344 | 4.126 |
|  |  |  | N/S | 0.659 | 0.659 | 0.892 |  | 3.6429 | 3.6429 | 3.65 |  |  |  |  |  |
| 17Sept2010-22Oct2011 | 400 | 1.09589 | E/W | 0.402 | 0.402 | 0.203 |  | 4.216 | 4.216 | 3.716 |  | 3.845 | 3.9065 | 0.039 | 3.856 |
|  |  |  | N/S | -2.296 | -2.296 | -1.012 |  | 4.7 | 4.7 | 4.586 |  |  |  |  |  |
| 23July2013-28Sep2014 | 432 | 1.184 | E/W | -0.219 | -0.219 | 0.192 | 0.202 | 1.528 | 1.538 | 1.433 | 1.451 | 2.043 | 3.419 | 0.034 | 2.043 |
|  |  |  | N/S | 2.762 | 2.762 | -0.901 | -1.062 | 2.132 | 2.132 | 2.024 | 1.956 |  |  |  |  |
| 28Sept2014-15Sept2015 | 352 | 0.964 | E/W | -0.071 | -0.071 | -0.095 | -0.106 | 0.97 | 0.97 | 0.946 | 0.948 | 1.272 | 3.118 | 0.032 | 1.272 |
|  |  |  | N/S | -0.093 | -0.093 | 0.394 | 0.401 | 1.163 | 1.163 | 1.098 | 1.098 |  |  |  |  |
| 15Sept2015-03Oct2016 | 384 | 1.052 | E/W | 0.146 | 0.146 | 0.085 | 0.104 | 0.888 | 0.888 | 0.876 | 0.876 | 1.308 | 4.019 | 0.041 | 1.304 |
|  |  |  | N/S | 0.2 | 0.2 | -0.469 | -0.506 | 1.247 | 1.247 | 1.092 | 1.081 |  |  |  |  |
| 03Oct2016-30Sept2017 | 362 | 0.992 | E/W | -0.523 | -0.523 | -0.125 | -0.123 | 0.846 | 0.846 | 0.778 | 0.781 | 0.985 | 0.739 | 0.008 | 0.985 |
|  |  |  | N/S | 0.017 | 0.017 | 0.317 | 0.406 | 0.934 | 0.934 | 0.913 | 0.884 |  |  |  |  |
| 30Sept2017-22Aug2018 | 326 | 0.893 | E/W | 0.398 | 0.398 | 0.096 | 0.079 | 1.371 | 1.371 | 1.017 | 1.026 | 1.448 | 0.967 | 0.01 | 1.456 |
|  |  |  | N/S | -1.766 | -1.766 | 0.396 | 0.407 | 1.474 | 1.474 | 1.273 | 1.295 |  |  |  |  |
| 22Aug2018-10Sept2019 | 384 | 1.052 | E/W | -0.667 | -0.667 | -0.132 | -0.125 | 1.324 | 1.324 | 0.855 | 0.857 | 0.996 | 0.614 | 0.006 | 0.996 |
|  |  |  | N/S | 1.246 | 1.246 | -0.193 | -0.226 | 1.082 | 1.082 | 1.015 | 1.006 |  |  |  |  |

# Additional references

Ahn Y, Howat IM (2011) Efficient Automated Glacier Surface Velocity Measurement From Repeat Images Using Exclusion Feature Tracking. IEEE Trans Geosci Remote Sens 49:2838–2846. https://doi.org/10.1109/TGRS.2011.2114891

Ayoub F, Leprince S, Avouac JP (2009a) Co-registration and correlation of aerial photographs for ground deformation measurements. ISPRS J Photogramm Remote Sens 64:551–560. https://doi.org/10.1016/j.isprsjprs.2009.03.005

Ayoub F, Leprince S, Keene L (2009b) User ’ s Guide to COSI-CORR Co-registration of Optically Sensed Images and Correlation. California Institute of Technology: Pasadena, CA, USA

Bridges NT, Ayoub F, Avouac JP, et al (2012) Earth-like sand fluxes on Mars. Nature 485:339–342. https://doi.org/10.1038/nature11022

Fahnestock M, Scambos T, Moon T, et al (2016) Rapid large-area mapping of ice flow using Landsat 8. Remote Sens Environ 185:84–94. https://doi.org/10.1016/j.rse.2015.11.023

Fujita K, Nakazawa F, Rana B (2001) Glaciological observations on Rikha Sambha Glacier in Hidden Valley, nepal Himalayas, 1998 and 1999. Bull Glaciol Res 18:31–35

Gardner AS, Moholdt G, Scambos T, et al (2018) Increased West Antarctic and unchanged East Antarctic ice discharge over the last 7 years. Cryosphere 12:521–547. https://doi.org/10.5194/tc-12-521-2018

Haeberli W, King L, Flotron A (1979) Surface movement and lichen-cover studies at the active rock glacier near the Grubengletscher, Wallis, Swiss Alps. Arct Alp Res 11:421–441. https://doi.org/10.2307/1550561

Hooke RL, Calla P, Holmlund P, et al (1989) A 3 year record of seasonal variations in surface velocity, Storglaciaren, Sweden. J Glaciol 35:235–247. https://doi.org/10.3189/s0022143000004561

Kääb A (2002) Monitoring high-mountain terrain deformation from repeated air- and spaceborne optical data: Examples using digital aerial imagery and ASTER data. ISPRS J Photogramm Remote Sens 57:39–52. https://doi.org/10.1016/S0924-2716(02)00114-4

Kääb A, Kneisel C (2006) Permafrost creep within a recently deglaciated glacier forefield: Muragl, Swiss Alps. Permafr Periglac Process 17:79–85. https://doi.org/10.1002/ppp.540

Kääb A, Lefauconnier B, Melvold K (2005) Flow field of Kronebreen, Svalbard, using repeated Landsat 7 and ASTER data. Ann Glaciol 42:7–13. https://doi.org/10.3189/172756405781812916

Kääb A, Vollmer M (2000) Surface geometry, thickness changes and flow fields on creeping mountain permafrost: Automatic extraction by digital image analysis. Permafr Periglac Process 11:315–326. https://doi.org/10.1002/1099-1530(200012)11:4<315::AID-PPP365>3.0.CO;2-J

Kirkbride M (2012) Ice Flow Vectors on the Debris-Mantled Tasman Glacier , 1957-1986. Geogr Ann Ser A Phys Geogr 77:147–157. https://doi.org/10.1080/04353676.1995.11880435

Leprince S, Barbot S, Ayoub F, Avouac JP (2007) Automatic, Precise, Ortho-rectification and Coregistration for satellite Image Correlation, Application to Ground Deformation Measurement. IEEE J Geosci Rem Sens 45:1529–1558. https://doi.org/10.1109/TGRS.2006.888937

Macgregor KR, Riihimaki CA, Anderson RS (2005) Spatial and temporal evolution of rapid basal sliding on Bench Glacier, Alaska, USA. J Glaciol 51:49–63. https://doi.org/10.3189/172756505781829485

Messerli A, Grinsted A (2015) Image georectification and feature tracking toolbox: ImGRAFT. Geosci Instrumentation, Methods Data Syst 4:23–34. https://doi.org/10.5194/gi-4-23-2015

Orheim O, Lucchitta B (1987) Snow and ice studies by Thematic Mapper and Multispectral Scanner Landsat images. Ann Glaciol 9:109–118

Scambos TA, Dutkiewicz MJ, Wilson JC, Bindschadler RA (1992) Application of image cross-correlation to the measurement of glacier velocity using satellite image data. Remote Sens Environ 42:177–186. https://doi.org/10.1016/0034-4257(92)90101-O

Schneeberger C, Short N, Landl B (2000) DGPS Measurements of Glacier Surface Velocity on Storglaciren, Northern Sweden

Wagnon P, Linda A, Arnaud Y, et al (2007) Four years of mass balance on Chhota Shigri Glacier, Himachal Pradesh, India, a new benchmark glacier in the western Himalaya. J Glaciol 53:603–611. https://doi.org/10.3189/002214307784409306

Werner C, Wegmüller U, Strozzi T, Wiesmann A (2000) GAMMA SAR and interferometric processing software. Eur Sp Agency, (Special Publ ESA SP 211–219

Winsvold SH, Kääb A, Nuth C, et al (2018) Using SAR satellite data time series for regional glacier mapping. Cryosphere 12:867–890. https://doi.org/10.5194/tc-12-867-2018

Yan S, Liu G, Wang Y, et al (2015) Glacier surface motion pattern in the Eastern part of West Kunlun Shan estimation using pixel-tracking with PALSAR imagery. Environ Earth Sci 74:1871–1881. https://doi.org/10.1007/s12665-015-4645-7