Supplementary Information

Strain-enabled ultra-broadband Ge-based photonic devices for low-cost integration in PICs

Yiding Lin1,2,5,†, Danhao Ma3,†, Rui-Tao Wen4,6, Kwang Hong Lee2, Govindo Syaranamual2, Lionel C. Kimerling3, Chuan Seng Tan1,2 and Jurgen Michel2,3,4,*

1School of Electrical and Electronic Engineering, Nanyang Technological University, Singapore, Singapore

2Low Energy Electronic Systems (LEES), Singapore-MIT Alliance for Research and Technology, Singapore, Singapore

3Department of Materials Science and Engineering, Massachusetts Institute of Technology, Cambridge, MA, USA

4Materials Research Laboratories, Massachusetts Institute of Technology, Cambridge, MA, USA

5Currently with Max Planck Institute of Microstructure Physics, Halle (Saale), Germany

6Currently with Southern University of Science and Technology, Shenzhen, China

†Equal contribution

*Correspondence to: jmichel@mit.edu

Table of Contents

S1. Ge-OI fabrication	3

S2. Finite element method simulation of Ge mechanical strain in w-Ge-OI and w-Ge-on-SOI structures	3

S3. Determination of Ge longitudinal (εyy) strain	5

S4. SiNx stress characterization and optimization	7

S5. Fabrication and supplementary characterization of SiNx-strained w-Ge-OI	9

S6. Key process development	12

I.	Effect of top SiNx removal on Ge strain	12

II.	Stressor adhesion enhancement to Ge sidewalls	13

S7. Ge strain with respect to SiNx stressor width	14

S8. Fabrication of SiNx-strained w-Ge-OI MSM photodetectors	15

S9. Supplementary characterization of SiNx-strained w-Ge-OI MSM photodetectors	16

I.	Ti/Al2O3/Ge metal-insulator-semiconductor (MIS) contact	16

II.	IQE and dark current conduction mechanism	18

III.	Gain mechanism	20

S10. Extraction of Ge effective absorption coefficient from photocurrent spectra	21

S11. Bandgap edge wavelength calculation	22

S12. Fabrication and characterization of tensile SiNx-strained Ge0.99Si0.01 EA modulator arrays	23

S13. Optical transmission and ER measurement set-up	24

S14. Ge strain and bandgap extraction from optical transmission spectra using Franz-Keldysh model	24

# S1. Ge-OI fabrication

The Ge-OI platform was fabricated via Ge-on-Si epitaxy, direct wafer bonding and layer transfer process. The schematic process flow is shown in Fig. S1. An un-intentionally doped Ge layer was epitaxial grown on a 200-mm Si (100) carrier wafer in a metal-organic chemical vapour deposition (MOCVD) chamber. A SiO2 layer was then deposited on both the surfaces of Ge and another Si (100) handle wafer by plasma-enhanced chemical vapour deposition (PECVD). Both the SiO2 surfaces were then treated with oxygen (O2) plasma for 15 s, followed by rinsing in de-ionized (DI) water and spin-drying, to heavily populate the surfaces with silanol (Si–OH) groups for a stronger bonding. Immediately afterwards, the two wafers were bonded together by contacting the treated SiO2 surfaces, followed by annealing in N2 ambient at 300 ºC for 3 hrs to enhance the bonding strength. The bonded pair went through grinding and tetramethylammonium hydroxide (TMAH) etching to remove the Si carrier wafer. The exposed Ge layer was then thinned down to a desired thickness via chemical-mechanical polishing (CMP). Specifically, the residual Ge thickness was ~ 100 and ~ 400 nm, respectively, in the micro-Raman study (Fig. 2) and the metal-semiconductor-metal (MSM) photodetectors (Fig. 3).

Fig. S1 A schematic of the process flow for Ge-OI fabrication.

# S2. Finite element method simulation of Ge mechanical strain in w-Ge-OI and w-Ge-on-SOI structures

The finite element method simulation was constructed by mimicking the practical experimental conditions. This was realized by adopting multiple solid mechanics modules in COMSOL Multiphysics, each corresponding to an experimental process, and solving them in sequence according to the experimental procedures. The stress output from the previous module was imported as the initial stress to the subsequent module, and the output from the last module was converted to the final mechanical strain. All materials are set as linear elastic.

The modelling of SiNx-strained w-Ge-OIs firstly followed the temperature variation during the Ge-OI fabrication discussed in Supplementary S1, consisting of the Ge-on-Si epitaxy by metal-organic chemical vapour deposition (MOCVD) at 600ºC, followed by the direct wafer bonding and post-bonding annealing at 300ºC, and finally cooling down to room temperature (300 K). Subsequently, the Ge-OI was patterned into a shape of waveguide and SiNx stressor was then introduced on its sidewalls with an intrinsic stress, both of which are temperature-independent due to their identical initial and end states at room temperature and the linear elastic setting of materials. Geometrical symmetry was applied penetrating the structure via the y-z and x-z planes (Fig. 1c, d). The stress obtained from the final module was converted to strain via below relations:

εxx=σxx-v•(σyy+σzz)E				               (S.1)

εyy=σyy-v•(σxx+σzz)E 				               (S.2)

εzz=σzz-v•(σxx+σyy)E 				               (S.3)

where εxx, εyy and εzz are the mechanical strain in Ge along the transverse (x-), longitudinal (y-) and vertical (z-) directions, respectively. σxx, σyy and σzz are the corresponding mechanical stress values. v=0.273 and E=103 GPa are the Poisson’s ratio and Young’s modulus of Ge, respectively. Detailed dimensions of the structure can be found in the caption of Fig. 1. The longitudinal dimension of w-Ge-OI is 100 µm. The Yong’s modulus of SiNx in the simulation is 200 GPa.

The strain in Ge0.99Si0.01 on SOI structure was modelled in a similar manner by mimicking the sequence of Ge-on-Si growth at 730ºC, cooling down to room temperature, Ge0.99Si0.01 waveguide formation and SiNx deposition.

# S3. Determination of Ge longitudinal (εyy) strain

The Ge mechanical strain in Ge-OI and w-Ge-OI structures without SiNx stressor was firstly studied using the finite element simulation discussed in S2. Fig. S2a shows the strain profile in Ge-OI. The strain is biaxial and ~0.17% both transversely (εxx) and longitudinally (εyy), which matches well with our earlier high-resolution X-ray diffractometry (HR-XRD) results1. The accuracy of the model is thus verified. For the w-Ge-OI structure, it can be observed that εyy (Fig. S2b) reveals a negligible change compared to that in the Ge-OI; while the εxx (Fig. S2c) exhibits a certain extent of relaxation. The εxx close to the waveguide sidewall becomes even compressive (~-0.03%). This can be explained by the shrinkage of the contact area to the -OI substrate along the transverse direction, resulting from the waveguide patterning. The reduced contact area relieves the substrate constraint on the w-Ge-OI and thus relaxes its strain. This explanation also fits well to the negligible εyy change longitudinally, where the w-Ge-OI is still in full contact to the substrate. Therefore, the original in-plane biaxial strain changes to a uniaxial-like longitudinal tensile strain upon the waveguide patterning. Furthermore, the εyy was found independent of the use of the SiNx stressor from Fig. S2d and e.

The strain profiles in w-Ge-OI were further experimentally investigated by performing micro-Raman measurements (at 785 nm) on a fabricated identical structure. The fabrication process is discussed in S5. Fig. S3a shows an optical microscope image of a fabricated w-Ge-OI with its cross-sectional SEM image shown in the inset. A more detailed information on the micro-Raman measurement can be found in Methods. It can be clearly seen from Fig. S3b that the peak LO phonon frequency of the w-Ge-OI resides in between that of the bulk Ge and Ge-OI, indicating a strain relaxation in w-Ge-OI compared to Ge-OI. This agrees well with the observation from the simulation. To determine the axial strains, one could get the following relation (Eq. S.4) by summing Eqs. S.1 and S.2, since there is no stress along the vertical direction (σzz = 0):

εxx+εyy=1-vσxx+σyyE			              (S.4)

The term σxx+σyy can be deduced from the stress-induced LO phonon frequency splitting relation2 below:

ωi2-ω02=pS12+qS11+S12(σxx+σyy)			(S.5)

where ωi refers to the peak LO phonon frequency from Ge-OI or w-Ge-OI, and ω0= 300.84 cm-1 corresponds to the peak frequency from bulk Ge. p=-1.45ω02 and q=-1.95ω02 are the phonon deformation potentials. S11 and S12 are the compliance tensors linking between the stress and strain. If we let εyy= 0.17%, εxx~0.04% can be obtained from the following equation by plugging Eq. S.5 into S.4:

εxx=1-ν(ωi2-ω02)2∙E∙pS12+qS11+S12-εyy				(S.6)

The εxx value reasonably matches with the simulation data (Fig. S2c). Therefore, εxx relaxation and εyy~0.17% were verified both theoretically and experimentally in w-Ge-OI without stressor. This again validates the accuracy of the finite element simulation and hence the εyy independence on the use of the SiNx stressor (Fig. S2d and e).

Fig. S2 Simulated Ge mechanical strain profiles of (a) Ge film-OI (in-plane biaxial) and w-Ge-OI along (b) longitudinal (εyy) and (c) transverse (εxx) directions, with (d) un-recessed and (e) recessed SiNx stressor. The Ge thickness and width are 400 nm and 2 µm, respectively, in (a)-(c), and 200 and 500 nm, respectively, in (d) and (e). The strain profiles beyond Ge are artificial due to simulation settings. The circled regions in (b) and (c) are without materials.

Fig. S3 (a) An optical microscope image of a fabricated w-Ge-OI structure. The inset shows a tilted SEM image of the structure; (b) Micro-Raman spectra of the fabricated w-Ge-OI, with the spectra of bulk Ge and thin film Ge-OI as reference.

# S4. SiNx stress characterization and optimization

SiNx films were deposited on 150-mm Si (111) wafers for the characterization of its intrinsic stress. All wafers were RCA-cleaned followed by a diluted HF (1:10) dip before the deposition. No film delamination or crack was observed after deposition. The calculation of the intrinsic stress in SiNx was carried out using the Stoney’s equation3 as below:

σ=Bts26tf1ra-1rb			    	              (S.7)

where B= E1-ν is the biaxial modulus of Si. E and ν are the Young’s modulus (~160 GPa) and Poisson’s ratio (~0.27) of Si, respectively. ts and tf are the thickness of the Si wafer and deposited SiNx film, respectively. ra and rb are the radius of curvature for the wafers before and after the SiNx deposition, respectively. The film thickness was measured by a Filmetrics F20 film thickness measurement system, while the wafer curvatures were measured using a KLA-Tencor FLX-2320 stress measurement system.

Multiple deposition parameters can be varied for the SiNx stress optimization, depending on the capability of respective PECVD systems. In general, a high RF frequency (13.56 MHz) benefits a tensile stress and a low frequency (380 kHz) benefits a compressive stress. Table S1 lists the calculated stresses from some films deposited by the Cello Aegis-20 system, along with the respective deposition parameters. Only single RF frequency was applied. The parametric trends agree well with previous studies4, 5, 6. Both highly compressive (~1.04 GPa) and tensile (~750 MPa) films were achieved. Fig. S4 shows the refractive index (n) of the highly-tensile SiNx obtained from the ellipsometry measurement and fitting. It can be inferred that the film is nitrogen-rich due to its lower n (~1.76) than that of stoichiometric Si3N4 (n ~2) at ~1,550 nm. The dual-frequency PECVD deposition used for strained Ge0.99Si0.01 on SOI EA modulators also follows these trends. An increased ratio of the low-frequency and high-frequency plasma makes the film more compressive and tensile, respectively. A maximum compressive and tensile stresses of ~1 GPa and ~600 MPa, respectively, were achieved in the resulting films.

Table S1 Deposition parameters as well as the resulting SiNx stress by Cello Aegis-20 PECVD. The deposition temperature is at 300 ºC. Negative values indicate compressive stress and positive values tensile stress.

| S/N | RF frequency | SiH4/NH3 Ratio | RF power (W) | Chamber pressure (mT) | Film thickness (nm) | Stress (MPa) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 380 kHz | 0.2 | 400 | 250 | 406 | -1040 |
| 2 | 380 kHz | 0.40 | 400 | 300 | 494 | -792 |
| 3 | 380 kHz | 0.36 | 400 | 270 | 1322 | -477 |
| 4 | 380 kHz | 0.38 | 200 | 300 | 343 | -216 |
| 6 | 13.56 MHz | 0.1 | 100 | 1000 | 183 | 301 |
| 7 | 13.56 MHz | 0.025 | 50 | 1500 | 281 | 750 |

Fig. S4 Refractive index n of PECVD-deposited highly-tensile SiNx in this work, obtained from ellipsometry measurements and fitting.

# S5. Fabrication and supplementary characterization of SiNx-strained w-Ge-OI

The process flow for the fabrication of SiNx-strained w-Ge-OIs is shown in the schematics in Fig. S5. First, the as-fabricated Ge-OI (Supplementary S1) was patterned into strip waveguides, using either optical lithography or electron-beam lithography (EBL) depending on their feature sizes, followed by a chlorine (Cl2)-based RIE. Afterwards, to form the recessed sidewall trenches, carbon tetrafluoride (CF4)-based gases were used to continue the RIE by ~ 500 nm into the underlying SiO2, utilizing the same resist hard mask in patterning the w-Ge-OI. This step is termed as self-aligned dry etching (SADE) and makes the process design of recessed SiNx stressor simple. After the resist removal by O2 plasma ashing, the sample was dipped in buffered oxide etchant (BOE, 6:1) to remove the residual surface oxides and organics. A layer of ~20 nm aluminium oxide (Al2O3) was then deposited via atomic layer deposition (ALD) at 250 ºC. This was followed by an O2 plasma treatment at 100 W for 15 s to enhance the stressor adhesion to w-Ge sidewalls (Supplementary S6. II). SiNx with a tensile stress of ~580 MPa was then deposited by PECVD up to the thickness of w-Ge-OI. The characterization of the SiNx stress is discussed in Supplementary S4. After the deposition, a second EBL patterning was performed, followed by the CF4-based RIE to remove the SiNx layer at the waveguide top. The purpose for the top SiNx removal is elaborated in Supplementary S6. I. The CF4-based RIE stopped when the residual SiNx was ~50 nm, after which a sulfur hexafluoride (SF6)-based RIE was taken over to complete the SiNx removal. The etching was implemented with a low RF power and high chamber pressure to mitigate the ion-bombardment damage to Al2O3. The high etching selectivity between the SiNx and Al2O3 makes the Al2O3 an etch-stop layer to protect the Ge underneath. Finally, the samples were capped with a ~200 nm SiO2 by PECVD. As a comparison, w-Ge-OIs without using SADE were also prepared, by skipping the step III in Fig. S5.

Fig. S5 Process flow schematics for the fabrication of SiNx-strained w-Ge-OI.

Fig. S6 shows the cross-sectional SEM images of the fabricated w-Ge-OIs. Both structures are fabricated with good quality. The Ge layers are ~100 nm thick as expected. Sidewall trenches in (b) are clearly identifiable to accommodate the recessed SiNx stressors and are ~460 nm deep into SiO2. Interestingly, similar micro-Raman measurements on 2 µm-wide w-Ge-OIs (Fig. S7a) do not exhibit the compressive shoulder as that seen in the 0.5 µm-wide w-Ge-OIs. This makes the accumulation of the compressive strain plausible near both sidewalls of w-Ge-OI, since the incident laser spot is near the waveguide center and its diameter (~1 µm) is smaller than the waveguide width (2 µm). To verify this point, micro-Raman measurements were performed at both the centre and edge of the 2 µm-wide w-Ge-OIs (Fig. S7b top insets). As shown in Fig. S7b top, the structure without the recessed stressor shows distinct LO phonon peaks at the centre and edge, and the strain at the edge is lower. With the use of the recessed stressor, the strain at the edge is enhanced (Fig. S7b bottom). Hence, these results verify that the compressive strain is accumulated at the bottom of the w-Ge-OI close to sidewalls. A qualitative two-dimensional (2-D) strain mapping was thus constructed, by combining and comparing all the measurement results (Fig. S7c). Hence, it can be concluded that, with tensile SiNx stressor, the recessed stressor configuration turns the compressive strain induced by the non-recessed SiNx stressor into tensile near both sidewalls of w-Ge-OI at the bottom, thus improving the tensile strain uniformity in Ge with an enhanced average strain magnitude.

Fig. S6 Cross-sectional SEM images of fabricated w-Ge-OIs (a) without and (b) with the use of the recessed SiNx stressor, prepared by focused-ion beam (FIB) cutting. Scale bar: 500 nm.

Fig. S7 Micro-Raman spectra of strained w-Ge-OIs with a width of 2 µm at (a) the centre of the waveguides at both 532 and 785 nm and (b) both the centre and edge of the waveguides at 785 nm. The insets in (b) show optical microscope images of the waveguides under the measurement with the incident laser spots at the waveguide (I) centre and (II) edge. Scale bar: 10 µm. For a clear visual effect, the images were captured at the laser wavelength of 532 nm; (c) Schematic strain distribution in Ge without (left) and with (right) the sidewall SiNx stressor recessed, inferred from the micro-Raman results in Fig. 2h, Fig. S7a and b. The colour coded bar indicates the magnitude of strain, where a higher magnitude of tensile (T) and compressive (C) strain correspond to a warmer and colder colour code, respectively.

# S6. Key process development

## Effect of top SiNx removal on Ge strain

Fig. S8 shows the micro-Raman spectrum at 532 nm of a w-Ge-OI deposited with tensile SiNx without its removal at the waveguide top (step VI in Fig. S5). The waveguide and the SiNx film are both ~400 nm thick, as shown in the insets. A spectrum from bulk Ge is also included as a reference. The Raman peak of the w-Ge-OI is blue-shifted, compared to bulk Ge, indicating a compressive strain at the top of Ge. This is in significant contrast to the red-shift of the spectra for w-Ge-OIs with top SiNx removal (Fig. 2g). This can be easily understood as a material with a tensile stress at the top could induce a compressive strain to the material underneath. Therefore, to maximize the strain in Ge, it is necessary to remove the SiNx at the top of w-Ge-OI.

Fig. S8 Micro-Raman spectrum of a w-Ge-OI deposited with tensile SiNx without its removal at the waveguide top. Insets: tilted-angle SEM images of the w-Ge-OI (a) before and (b) after the SiNx deposition. Scale bars: (a) 200 nm and (b) 1 µm. In this study, the stressors are not recessed.

## Stressor adhesion enhancement to Ge sidewalls

A common phenomenon following the top SiNx removal, observed from early-batch tensile SiNx-strained w-Ge-OIs, is the gradual detachment of the stressors from the w-Ge-OI sidewalls. The SiNx from these batches was directly deposited on w-Ge-OI after the BOE treatment (as in Supplementary S5), followed by the top SiNx removal. As can be seen from Fig. S9a, the corresponding Ge Raman peak exhibits a blue-shift progressively with time and matches with that of w-Ge-OI without SiNx stressor after 9 days, indicating a gradual relaxation of the tensile strain in Ge. The corresponding tilted SEM image in the inset reveals a complete delamination of the stressor from the waveguide sidewalls, explaining the reason for the strain relaxation. This would result in a reliability limitation to the tensile-strained Ge photodetectors and EA modulators, as only the tensile stress would lead to the delamination. Therefore, as discussed in Supplementary S5, O2 plasma treatment was adopted before the SiNx deposition to enhance the stressor adhesion, along with the incorporation of the Al2O3 interlayer and SiO2 capping layer. The Al2O3 and SiO2 layers are believed to passivate the Ge from oxidation to form volatile GeOx that could weaken the SiNx/Ge adhesion. The O2 plasma treatment has been widely used in direct wafer bonding for a higher bonding strength7. The treatment increases the number of activated surface groups for the formation of interfacial covalent bonds. It is thus similarly anticipated that the treatment could populate the covalent bond formation between SiNx and Ge. The resulting w-Ge-OI structure, as shown in Fig. S7b, exhibits consistent Raman peaks independent of time. There is no observable tensile strain relaxation even after 2 years. The corresponding tilted SEM image also verifies that the stressor is not delaminated from the sidewalls. Hence, this method can be useful for an improved reliability of tensile SiNx-strained Ge photodetectors and EA modulators.

Fig. S9 Micro-Raman spectra of tensile SiNx-strained w-Ge-OIs (a) without and (b) with the use of the O2 plasma treatment, together with the incorporation of Al2O3 interlayer and SiO2 capping layer, with respect to the time since they were fabricated. The spectra of a w-Ge-OI without SiNx stressor and bulk Ge were included as reference. Insets show tilted SEM images of the corresponding w-Ge-OIs after (a) 9 days and (b) 2 years. Scale bar: 1 µm. In this study, the laser wavelength is 532 nm and the stressors are not recessed.

# S7. Ge strain with respect to SiNx stressor width

Form factor has become a key performance indicator for integrated electronic systems, with the continuing downscaling of transistors. A system with a small form factor could facilitate a compact integration with a reduced cost and an improved performance, which also applies to PICs. Fig. S10b shows the micro-Raman spectra of recessed SiNx-strained w-Ge-OIs with respect to the widths (2, 5 and 10 µm) of the respective SiNx stressor at Ge sidewalls. The width for all w-Ge-OIs is 1 µm. A cross-sectional schematic of the structure under test is depicted in Fig. S10a, specifying the stressor width w. The spectra highly overlap at these widths, exhibiting a consistent Ge tensile strain irrespective of the stressor width. It is noteworthy that the width of 10 µm is also the width adopted in the fabrication of all w-Ge-OIs and the corresponding MSM photodetectors in this work. This means that the effect of the SiNx stressor can be sustained very well down to a SiNx width (i.e. w-Ge spacing) of 2 µm, enabling a highly-compact electronic-photonic integration. This result is encouraging especially for the integration of arrays of Ge photodetectors and EA modulators for broadband applications.

Fig. S10 (a) A cross-sectional schematic of SiNx-strained w-Ge-OI indicating the SiNx stressor width w; (b) The corresponding micro-Raman spectra at the stressor widths of 2, 5 and 10 µm. In this study, the stressors are recessed.

# S8. Fabrication of SiNx-strained w-Ge-OI MSM photodetectors

The fabrication of SiNx-strained w-Ge-OI MSM photodetectors follows the process for the SiNx-strained w-Ge-OI fabrication (Fig. S5) at the initial two steps, as shown in Fig. S11. In step III, a second EBL patterning and RIE was performed, instead of a direct SADE in Fig. S5, for the recessed trenches. This is to avoid the over-etching of the resist hard mask by SADE and provide more design flexibility for the trench depth. After a short dip in BOE, a ~1 nm Al2O3 was deposited on the sample by ALD for the depinning of the Fermi-level at metal/Ge Schottky contacts. Metal stacks of Ti (bottom layer)/TiN/Al (top layer) were then sputtered sequentially, followed by lift-off, to form and pattern the contacts. Afterwards, as discussed in Supplementary S5, a ~20 nm Al2O3 was deposited again by ALD to serve as an etch-stop layer during the subsequent top SiNx removal. Subsequently, surface treatment by O2 plasma was carried out for a stronger adhesion of the SiNx stressor (Supplementary S6. II). The stressor was then deposited, followed by the top SiNx removal and SiO2 capping. Finally, the dielectrics covering the probing metal pads were etched away for the ease of device characterization. For comparison, control devices of SiNx-strained w-Ge-OI MSM photodetectors without using the recessed stressor, as well as un-strained w-Ge-OI MSM photodetectors without stressor (used 50-nm SiO2 as device passivation), were fabricated. The fabrication skips step III for the non-recessed devices, and steps VI, VII and VIII for the un-strained devices.

Fig. S11 Process flow schematics for the fabrication of SiNx-strained w-Ge-OI MSM photodetectors.

# S9. Supplementary characterization of SiNx-strained w-Ge-OI MSM photodetectors

## Ti/Al2O3/Ge metal-insulator-semiconductor (MIS) contact

To study the Ti/Al2O3/Ge MIS contact, the fabricated w-Ge-OI MSM photodetectors were characterized by temperature-dependent I-V measurements ranging from 293 to 353 K. The Schottky barrier height (ϕB) of the contact can thus be extracted using the following relation:

lnIT2=-qϕBkBT+lnA∙A*				(S.8)

where I is the dark current, T is the temperature in Kelvin, kB is the Boltzmann constant, A is the diode area and A* is the effective Richardson’s constant. Fig. S12a and b show the plots of lnIT2 versus 1kT for w-Ge-OI MSM photodetectors without and with the use of the SiNx stressor, respectively, at a bias of 1 V. The slope of the linear fitting indicates the ϕB of the corresponding MIS contact. The detector without stressor exhibits a ϕB of ~0.33 eV, which indicates that the Fermi level of Ti has been successfully unpinned from ~0.08 eV above the Ge valence band maximum, where the Fermi level of a direct metal/Ge contact is located. Cross-sectional transmission electron microscopic (TEM) imaging reveals a ~1.3 nm interlayer between Ti and Ge (Fig. S13b), which is further verified as Al2O3 via energy-dispersive X-ray (EDX) elemental mapping (Fig. S13a). In contrast, the ϕB is ~0.12 eV for the SiNx-strained detector. The decrease in ϕB can be explained by the image dipoles and fixed charge at the MIS interfaces induced by the trap states in SiNx, since it has been reported that interfacial dipoles and charge are liable for the ϕB lowering of MIS contacts.

Fig. S12 Schottky barrier height (ϕB) extraction of the Ti/Al2O3/Ge Schottky contact in w-Ge-OI MSM photodetectors (a) without and (b) with the use of the SiNx stressor at 1 V, via temperature-dependent dark current-voltage measurement.

Fig. S13 (a) Elemental mapping profiles at the Ti/Al2O3/Ge interface via an EDX line scan, with a cross-sectional TEM image of the interface shown in (b).

## IQE and dark current conduction mechanism

Fig. S14 shows the IQE of w-Ge-OI MSM photodetectors as a function of the corresponding voltage bias. The IQE increases with the bias for both devices. However, the photodetector with SiNx stressor exhibits a higher IQE with a faster increasing rate, compared to that without SiNx stressor. This can be attributed to the trap states in SiNx that induces additional interfacial dipoles and fixed charges at the MIS contact, which enhances the electric field across the Al2O3 interlayer for a more significant transport of photon-generated carriers. A support to this conclusion comes from the analysis of dark current conduction mechanism of the devices, as shown in Fig. S15, which reveals a dominance of Poole-Frenkel (PF) emission at different bias ranges. The PF emission describes the emission of trapped carriers in the dielectric into current conduction, which typically occurs at a high electric field. It can be expressed by the following equation8:

lnJE= q3πεrε0kBTE-qϕtkBT+ln(qNcμ)			(S.9)

where q is the charge of an electron, J is the dark current density, E is the electric field across the Al2O3 interlayer, εr and ε0 are the relative and vacuum permittivity, respectively, ϕt is the potential barrier of the trap, Nc is the density of state in the conduction band and μ is the mobility in the dielectric. It is observed that the photodetectors with and without stressor exhibit a linear fitting from their lnJE-E plots starting from ~0.26 and ~1.46 V, respectively. The values of relative permittivity εr of Al2O3 (~5.87 and ~8.68, respectively, with and without stressor), extracted from the fitted slopes, match well with that reported in literature8, 9, 10. This verifies the involvement of PF emission in the current conduction of both devices. The earlier onset of PF emission for the SiNx-strained device indicates a higher electric field across the corresponding Al2O3 interlayer, which explains its higher IQE (Fig. S14) since the higher electric field could assist the transport of photon-generated carriers through the Al2O3 barrier. The higher electric field can be originated from the trap states in the SiNx stressor, as discussed in the main body of this article, which induces interfacial dipoles and fixed charge at the Ti/Al2O3/Ge MIS interface. This also explains the reduced Schottky barrier height of the MIS contact with the use of the SiNx stressor (Fig. S12).

Fig. S14 IQE as a function of voltage bias for w-Ge-OI MSM photodetectors with and without the use of SiNx stressor. Inset shows the plot in linear scale.

Fig. S15 Poole-Frenkel emission fitting for the dark current (at room temperature) of w-Ge-OI MSM photodetectors (a) without and (b) with the use of the SiNx stressor.

## Gain mechanism

Fig. S16 displays Ge electric field profiles in w-Ge-OI MSM photodetectors at different voltage biases (0.3, 1.3, and 2 V). The profiles share a same inter-digitated electrode on-Ge for an easy comparison of the electric field. Both the electrode width and the spacing between electrodes are 1 µm. The thickness of Ge is ~400 nm as that in the fabricated device. It can be found that the electric field has reached ~2×105 V/m at 0.3 V, even at the bottom portion of Ge away from electrodes, where the carriers have reached a saturation drift velocity (6×106 cm/s, ref. 1). Therefore, one can infer that the IQE would also saturate due to the predictable full carrier collection. However, as depicted in Fig. S14, the IQE is far below 100% at this bias and continues to increase with the bias. This discrepancy indicates that the photocurrent transport is hindered by the Al2O3 interlayer and this layer does not provide a gain mechanism to the photocurrent. Therefore, the gain is unlikely originated from the interfacial trapped carriers at the MIS interface related to the Al2O3 interlayer, which is resulted from the SiNx incorporation and accounts for the Schottky barrier lowering in Fig. S12. Increasing the potential to 1.3 and 2 V results in electric field strength more than 6.3×106 V/m, where avalanche multiplication has been observed11. Hence, the gain can be attributed to the avalanche effect, as the electric field close to electrodes, beyond 1.5 V, has reached the threshold for avalanche multiplication, as shown in Fig. S16.

Fig. S16 Electric field simulation of w-Ge-OI inter-digitated MSM photodetector. The respective biases were applied on the 1-nm Al2O3 interlayers.

# S10. Extraction of Ge effective absorption coefficient from photocurrent spectra

The effective absorption coefficient of SiNx-strained Ge is deduced based on the photocurrent spectra in Fig. 3d (left y-axis). This is because the fabricated devices are with identical illumination and capping material structure at the w-Ge top, irrespective of the use of the (recessed) stressor. The photocurrent Iph for all devices can thus be considered proportional to the light absorption as follows:

Iph∝ 1-e-αd					(S.10)

where α is the Ge effective absorption coefficient and d is the Ge thickness (~400 nm). Since αd≪1, the approximation of 1-e-αd≈αd can be satisfied using the Maclaurin series of exponential function. Hence, both Iph and the corresponding device IQE η∝αd, as shown below:

η2η1=α2α1=Ein-Eg2Ein-Eg1					(S.11)

where Ein denotes the incident photon energy and Eg the Ge bandgap energy. The relation satisfies for any two (as subscripts 1 and 2) among the three types of fabricated devices. Here, we specifically determine the α at the Ein of 1,500 nm using Eq. S.11, where the photocurrent spectra started. The α beyond across the C- and L-band can thus be obtained from the photocurrent spectra based on the α at 1,500 nm. Using α=5,096 cm-1 at 1,500 nm for bulk Ge (Eg~0.801 eV) determined from ellipsometry measurement and fitting, as well as considering the tensile strain-induced Eg shrinkage, the α for w-Ge-OI without stressor, with non-recessed and recessed stressor were deduced as 6,021, 7,767 and 8,376 cm-1 at 1,500 nm, respectively. The photocurrent spectra were thus converted to Ge effective absorption coefficient spectra as the right y-axis in Fig. 3d.

# S11. Bandgap edge wavelength calculation

Deformation potential theory12 was used for the Ge bandgap edge wavelength calculation as a function of εxx. The theory analytically describes the correlation between the bandgap of a semiconductor material and its applied mechanical strain. The strain causes a volumetric and lattice symmetry deformation of the material, where the former is termed as hydrostatic deformation (δEhy) and the latter shear deformation (δEsh), as expressed below13:

δEhy=-a(εxx+εyy+εzz)					(S.12)

δEsh=-b(εxx+εyy-2εzz)				(S.13)

where a is defined as hydrostatic deformation potential and b shear deformation potential. εxx, εyy and εzz are the axial strain in the material along x, y, and z directions, respectively, induced by the stress along the respective directions. Note that (εxx+εyy+εzz) represents the fractional volumetric change of the material.

Considering the coupling with the spin-orbit split-off (SO) band, Ge Γ-valley conduction-band-light-hole bandgap EgLH and conduction-band-heavy-hole bandgap EgHH can be expressed as follows, respectively:

EgLH=Eg-δEhy-14δEsh+12∆-12∆2+∆∙δEsh+94δEsh2		(S.14)

EgHH=Eg-δEhy+12δEsh					(S.15)

where Eg is the bandgap of stress-free Ge (0.801 eV) and ∆ is the spin-orbit energy splitting between the degenerated topmost valence band and the SO valence band of Ge. The ∆, a and b values for Ge used in calculation are 0.289 eV, -8.97 and -1.88, respectively, from the experimental determination by Liu et al.14. As mentioned, εxx is the strain variable and εyy= 0.17% is a constant according to Supplementary S3; εzz is thus determined from Eqs. S.1 to S.3 based on Poisson effect. The obtained EgLH and EgHH values were then converted to bandgap edge wavelengths via λ=hcEg, where h and c are the Planck’s constant and speed of light, respectively.

# S12. Fabrication and characterization of tensile SiNx-strained Ge0.99Si0.01 EA modulator arrays

The strained modulators array was fabricated in MIT Nano and the Substrate Engineering Lab Cleanroom Facilities. First, boron implantation was performed on Si on 150-mm (100) SOI (250-nm Si on 3-µm SiO2) wafers. After dopant activation and a standard RCA cleaning, the wafers were immediately loaded into the ultrahigh vacuum chemical vapor deposition (UHV-CVD) system for Ge0.99Si0.01 epitaxial growth. The growth comprises of a 40-nm buffer layer grown at 350 ºC, followed by a ~300-nm layer of high material quality at 730 ºC. The wafers were then cyclically-annealed, followed by a poly-Si layer growth on top of the Ge0.99Si0.01 layer. Phosphorous was implanted with subsequent thermal annealing for an n-type poly-Si contact layer. Ge0.99Si0.01 waveguide-mesa of various widths were patterned using EBL and etched using reactive-ion etching. A SiNx stressor layer was deposited via multi-frequency plasma-enhanced chemical vapor deposition. Finally, trenches were opened for the metal contacts. Multiple modulators with various widths from 400 nm to 4 µm are simultaneously fabricated in the same process flow to achieve the modulator arrays.

Fig. S17 (a) Dark current-voltage characteristics of a fabricated modulator; (b) A cross-sectional SEM image of a fabricated modulator.

# S13. Optical transmission and ER measurement set-up

Fig. S18 A schematic of the optical transmission measurement stage (left) with the electrical probe station (right) for the modulator performance characterization.

# S14. Ge strain and bandgap extraction from optical transmission spectra using Franz-Keldysh model

Table S2 Extraction of Ge strain (εxx measured) and bandgap (Eg measured) from the optical transmission spectra using the FK model and compare with the simulated εxx from finite element simulation. ER peak wavelengths are also listed, which are consistent with the corresponding strain and bandgap variation.

References

1.	Lin Y, Lee KH, Bao S, Guo X, Wang H, Michel J, et al. High-efficiency normal-incidence vertical p-i-n photodetectors on a germanium-on-insulator platform. Photonics Research 2017, 5(6): 702-709.

2.	Ossikovski R, Picardi G, Ndong G, Chaigneau M. Raman spectroscopy and polarization: Selected case studies. Comptes Rendus Physique 2012, 13(8): 837-852.

3.	Stoney GG. The Tension of Metallic Films Deposited by Electrolysis. Proceedings of the Royal Society of London Series A 1909, 82(553): 172.

4.	Temple-Boyer P, Rossi C, Saint-Etienne E, Scheid E. Residual stress in low pressure chemical vapor deposition SiNx films deposited from silane and ammonia. Journal of Vacuum Science & Technology A: Vacuum, Surfaces, and Films 1998, 16(4): 2003-2007.

5.	Belyansky M, Chace M, Gluschenkov O, Kempisty J, Klymko N, Madan A, et al. Methods of producing plasma enhanced chemical vapor deposition silicon nitride thin films with high compressive and tensile stress. Journal of Vacuum Science & Technology A: Vacuum, Surfaces, and Films 2008, 26(3): 517-521.

6.	R. Arghavani LX, H. M’Saad, M. Balseanu, G. Karunasiri, A. Mascarenhas, and S. E. Thompson. A Reliable and Manufacturable Method to Induce a Stress of >1 GPa on a P-Channel MOSFET in High Volume Manufacturing. IEEE Electron Device Letters 2006, 27(2).

7.	Lee KH, Bao S, Chong GY, Tan YH, Fitzgerald EA, Tan CS. Fabrication and characterization of germanium-on-insulator through epitaxy, bonding, and layer transfer. Journal of Applied Physics 2014, 116(10): 103506.

8.	Spahr H, Montzka S, Reinker J, Hirschberg F, Kowalsky W, Johannes H-H. Conduction mechanisms in thin atomic layer deposited Al2O3 layers. Journal of Applied Physics 2013, 114(18).

9.	Yip G, Qiu J, Ng WT, Lu ZH. Effect of metal contacts on the electrical characteristics of Al2O3 dielectric thin films. Applied Physics Letters 2008, 92(12): 122911.

10.	Tanner CM, Perng Y-C, Frewin C, Saddow SE, Chang JP. Electrical performance of Al2O3 gate dielectric films deposited by atomic layer deposition on 4H-SiC. Applied Physics Letters 2007, 91(20): 203510.

11.	Assefa S, Xia F, Vlasov YA. Reinventing germanium avalanche photodetector for nanophotonic on-chip optical interconnects. Nature 2010, 464(7285): 80-84.

12.	Van de Walle CG. Band lineups and deformation potentials in the model-solid theory. Physical Review B 1989, 39(3): 1871-1883.

13.	Chuang SL. Physics of photonic devices, vol. 80. John Wiley & Sons, 2012.

14.	Liu J, Cannon DD, Wada K, Ishikawa Y, Danielson DT, Jongthammanurak S, et al. Deformation potential constants of biaxially tensile stressed Ge epitaxial films on Si(100). Phys Rev B: Condens Matter Mater Phys 2004, 70: 155309.