Supplemental Methods

Chemicals & analytical standards: The solvents methanol, water with 1% formic acid (FA), acetonitrile with 1% FA (all LC/MS grade) were purchased from Fisher Scientific (Waltham, MA). Analytical standards of neotame (>98% purity, Sigma Aldrich, St. Louis, MO), (+/-)-6-methylnicotine (97%, Santa Cruz Biotechnology, Dallas, TX), (S)-nicotine (99%), menthol (99%, both Fisher Scientific), WS-3 (N-Ethyl-p-menthane-3-carboxamide, >98%), and WS-23 (2-Isopropyl-N,2,3-trimethylbutyramide, >98%, both TCI, Portland, OR) were used for calibration.

Liquid Chromatography-Mass Spectroscopy (Yale): LC-MS-MS was carried out on a Thermo Vanquish LC coupled to a Quantis MS. Compound separation was achieved via an established procedure using an Accucore RP-MS column (50 mm x 2.1 mm id, 2.6 µm particle size, Thermo Fisher Scientific) using a binary mobile phase program of (A) water + 0.1% FA and (B) acetonitrile + 0.1% FA at a flowrate of 0.4 mL/min and a column temperature of 40 °C. The linear gradient was as follows 5-95% (B) from 0.5-4.5min with a total run time of 8 min; injection volume: 10 μL.

MS/MS was carried out in heated electrospray ionization (H-ESI) mode at spray voltage: 3500 V (positive), 2500 V (negative); ion transfer tube: 300 °C; vaporizer: 350 °C; selected reaction monitoring (SRM) mode was used for the following compounds:

| Compound | Polarity | Precursor (m/z) | Product 1 (m/z) (Quantifying) | Product 2 (m/z) (Confirming) | Product 3 (m/z) (Confirming) |
| --- | --- | --- | --- | --- | --- |
| Acesulfame K | Negative | 161.9 | 81.9 | 77.9 |  |
| Advantame | Negative | 443.1 | 425.0 | 243.9 |  |
| Aspartame | Positive | 295.1 | 120.0 | 180.0 | 235.0 |
| Glucose | Negative | 76.9 | 60.1 |  |  |
| Glycyrrhizin | Negative | 821.4 | 351.0 | 113.0 |  |
| Neotame | Positive | 380.2 | 172.0 | 320.0 |  |
| Saccharin | Negative | 181.9 | 105.9 | 42.0 |  |
| Sodium cyclamate | Negative | 177.9 | 79.9 |  |  |
| Sorbitol | Negative | 181 | 88.9 | 71.0 | 100.9 |
| Stevioside | Negative | 803.3 | 641.2 | 479.2 |  |
| Sucralose | Negative | 397 | 361.0 | 359.0 |  |
| Sucrose | Positive | 365 | 202.9 | 184.9 |  |

For neotame, calibration curves with n ≥ 4 points were constructed and used for quantification. Limit of quantification: 5 ng/mL, limit of detection: 1 ng/mL.

Liquid Chromatography-Mass Spectroscopy (Duke) for neotame only: LC-MS was carried out on an Agilent 6224 TOF LC-MS system consisting of a 1200 HPLC (degasser, binary pump, thermostated column compartment, diode array detector) coupled to a 6224 accurate-mass time-of-flight mass spectrometer. HPLC separations were achieved on a Kinetix C18 column (150 mm x 4.6 mm id, 4 µm particle size, Phenomenex, Torrance, CA) using a linear gradient of (A) water + 0.3% FA and (B) acetonitrile + 0.3% FA at a flow rate of 0.5 mL/min, and a column temperature of 40 °C.  The linear gradient was 0-100% (B) over 4 min, with a total run time of 10 min.  Neotame was detected in negative ion mode, and the MS signal for the [M-H]- ion (377.2 m/z) was used for quantitation.  Samples were quantitated relative to a standard curve (n=6) ranging in concentration from 0.1-50 μg/mL.  The limit of detection (LOD) was 0.07 μg/mL, and the limit of quantitation (LOQ) was 0.2 μg/mL.

Gas chromatography-flame ionization detection (only Yale): GC-FID was carried out on a GC-2010 (Shimadzu, Columbia, MD) fitted with a J&W DB-5 column (length 60 m, id 0.25 mm, 0.25 μm film; Agilent, Santa Clara, CA). Injection volume: 1 μL; split ratio: 300; program: 30 °C for 7 min; ramp 10 °C/min to 50 °C then hold for 20 min; ramp 10 °C/min to 310 °C then hold for 12 min. Injector temperature: 250 °C; detector temperature: 325 °C. Calibration curves for nicotine, 6-methylnicotine, menthol, WS-3, and WS-23 with n ≥ 4 points were constructed and used for quantification. Limit of quantification: 30 ug/mL; limit of detection: 10 ug/mL.