# S. 2. Representative catchments

| ID | area_km2 | RepCatch | RepCatch_name | RepCatch_area_km2 | RepCatch_Factor | Relevant to calculation of allocations in: |
| --- | --- | --- | --- | --- | --- | --- |
| Murray_usHume_wet | 5063.87 | 401208 | Cudgewa Creek at Berringama (401208) | 356.2 | 18.92 | Murray system |
| Murray_usHume_dry | 5756.06 | 401013 | Jingellic Creek at Jingellic (401013) | 392 | 16.22 | Murray system |
| SnowyScheme_SwampyPlainsRiv | 634.65 | 401572 | Tooma River at Tooma Reservoir (401572) | 150.757 | 1.32 | Murray / Snowy system** |
| Mitta_usDart_wet | 1521.87 | 401210 | Snowy Creek at Below Granite Flat (401210) | 412.9 | 3.64 | Murray system |
| Mitta_usDart_dry | 1985.83 | 401215 | Morass Creek at Uplands (401215) | 536.7 | 8.80 | Murray system |
| Kiewa_wet | 858.48 | 401216 | Big River at Jokers Creek (401216) | 358.1 | 1.98 |  |
| Kiewa_dry | 870.57 | 402204 | Yackandandah Creek at Osbornes Flat (402204) | 280.5 | 3.92 |  |
| Ovens_wet | 3379.12 | 403233 | Buckland River at Harris Lane (403233) | 457.1 | 5.50 |  |
| Ovens_dry | 2857.04 | 403221 | Reedy Creek at Woolshed (403221) | 211.5 | 11.90 |  |
| Broken | 1910.12 | 404207 | Holland Creek at Kelfeera (404207) | 454.7 | 2.44 |  |
| Goulb_usEild_wet | 2285.96 | 405219 | Goulburn River at Dohertys (405219) | 699.1 | 2.82 | Goulburn system |
| Goulb_usEild_dry | 1596.23 | 405245 | Ford Creek at Mansfield (405245) | 117.3 | 35.37 | Goulburn system |
| Goulb_dsEild_wet | 1969.76 | 405209 | Acheron River at Taggerty (405209) | 625.8 | 2.22 | Goulburn system* |
| Goulb_dsEild_dry | 8814.13 | 405228 | Hughes Creek at Tarcombe Rd (405228) | 480.6 | 12.01 | Goulburn system* |
| CampLodd_wet | 1978.27 | 407217 | Loddon River at Vaughan D/S Fryers Creek (407217) | 294.7 | 7.10 |  |
| CampLodd_dry | 4304.48 | 406226 | Mount Ida Creek at Derrinal (406226) | 173 | 17.15 |  |
| Murray_toMulwala | 2915.66 | 403221 | Reedy Creek at Woolshed (403221) | 211.5 | 2.03 |  |
| SnowyScheme | 4235.14 | 401572 | Tooma River at Tooma Reservoir (401572) | 150.757 | 11.78 | Snowy system** |
| Murrumb_usBurrin_wet | 1977.77 | 410088 | Goodradigbee River at Brindabella (No.2&No.3-Cab (410088) | 428.1 | 3.75 | Murrimbidgee System |
| Murrumb_usBurrin_dry | 9322.95 | 410141 | Micaligo Creek at Michelago (410141) | 190.6 | 107.18 | Murrimbidgee System |
| Canberra_Cotter | 475.83 | 410730 | Cotter River at Gingera (410730) | 129.2 | 2.00 | Murrimbidgee System |
| Canberra_Googong | 891.12 | 410734 | Queanbeyan River at Tinderry (410734) | 564.4 | 1.33 | Murrimbidgee System |
| Murrumb_dsBurrin_wet | 2966.56 | 410038 | Adjungbilly Creek at Darbalara (410038) | 389.2 | 11.20 | Murrimbidgee System* |
| Murrumb_dsBurrin_dry | 8929.59 | 412029 | Boorowa River at Prossers Crossing (412029) | 1550.6 | 9.34 | Murrimbidgee System* |
| BillabongCk | 2798.84 | 410091 | Billabong Creek at Walbundrie (410091) | 2636.3 | 0.97 | Murrimbidgee System* |
| Notes * denotes that an area that is part of a system but downstream of the major reservoir(s) of that system. |  |  |  |  |  |  |  |  |  |  |
| ** the Snowy system does not itself have allocations, but water released from the Snowy into the Murray and the Murrumbidgee systems affects their allocations |  |  |  |  |  |  |  |  |  |  |

# S.2.  Rainfall runoff calibration

# S. 3. Allocation modelling objective function calibration steps

The specific step to calculate the objective function value is outlined as follows.

- Calculate the weights of extreme and non-extreme (i.e., midway) categories

we=extremeextreme+midway

wm=midwayextreme+midway=1-we

where extreme is the number of extreme allocations and midway is the number of non-extreme allocations.

- Calculate SI based on the weighted number of successful detections of extreme and non-extreme cases.

SI=we×hitsextreme+wm×(correctmidway)

where hits is the number of successful detection of extreme allocations and any correct detection of non-extreme allocations will be counted as a correct.

- Calculate NNSE based on the prediction accuracy of non-extreme cases.

NNSE=12-NSE=11+(Amo-Amm)2(Amo-Amo)2

where Amo are the observed midway allocations, Amo is the average of the observed midway allocations, and Amm are the modelled midway allocations.

- The objective function is defined as

Obj= we×SI+wm×NNSE