Table 1 Specifications of PM2.5, PM10, VOCs, CO2, temperature and humidity sensors used in the BIAB setup

|  | Sensors |  |  |  |
| --- | --- | --- | --- | --- |
| Specification | PM2.5 & PM10 | VOCs | CO2 | Temperature & Humidity |
| Brand | Renke | Taqu | Renke | Renke |
| Model | RS-PM-*-2 | T29-PICRST15 | RS-CO2*-*-2 | RS-WS-N01-8 |
| Reference | https://www.renkeer.com/product/pm-sensor/ | https://taqu.coowor.com/ | https://www.renkeer.com/product/co2-sensor/ | https://www.renkeer.com/product/modbus-rtu-temperature-sensor/ |
| Power Supply | 10-30V DC | 10-30V DC | 10-30V DC | 10-30V DC |
| Range | 0-1000ug/m3 | 0~2000 CO2-ppm-equivalent | 0 ~ 5000 ppm | Humidity: 0%RH~ 80%RH |
|  |  |  |  | Temperature: -40℃~+60℃, |
| Resolution | 1ug/m3 | Not specified | 1ppm | Humidity: 0.1%RH |
|  |  |  |  | Temperature: 0.1℃ |
| Accuracy | ±3%FS (@100μg/m3,25℃, 50%RH) | ± 100, CO2-ppm-eq | ± (50ppm+ 3%FS) @ (25℃,400~5000ppm) | Humidity: ±3%RH (60%RH,25℃) |
|  |  |  |  | Temperature: ±0.5℃ (25℃) |
| Response time | ≤90s | Not specified | 2s | Humidity: ≤8s(1m/s wind speed ) |
|  |  |  |  | Temperature: ≤25s(1m/s wind speed ) |
| Output signal | RS485/0-5v/0-10v/4-20mA | RS485 | RS485/0-5V/0-10V/4-20mA | RS485 |

Table 3 Experimental setup of each filtration case

| Case | Setup | Type of Filtration | Targeted Pollutant |
| --- | --- | --- | --- |
| 1 |  | No filter | Baseline Case |
| 2 |  | Botanical Plant (Golden Pothos) | Volatile Organic Compounds (VOCs) |
| 3 |  | - Botanical Plant (Golden Pothos) - Primary Carbon Filter | - Volatile Organic Compounds (VOCs) - Particulate Matter (PM 2.5 and 10) |
| 4 |  | - Botanical Plant (Golden Pothos) - Primary Carbon Filter - Coconut Husk | - Volatile Organic Compounds (VOCs) - Particulate Matter (PM 2.5 and 10) - Particulate Matter (PM 2.5 and 10) |
| 5 |  | - Botanical Plant (Golden Pothos) - Primary Carbon Filter - Coconut Husk - Granular Activated Carbon | - Volatile Organic Compounds (VOCs) - Particulate Matter (PM 2.5 and 10) - Particulate Matter (PM 2.5 and 10) - Volatile Organic Compounds (VOCs) |