# Appendix

Table 2 Summarize of all articles included in this review

| Domain | Article | Summarize |
| --- | --- | --- |
| QM | (J. Wang, Sanchez, Ayesta, et al., 2018) | Present an approach for tolerance monitoring in wire electrical discharge machining (WEDM). Employ K-means and clustering to classify fir-tree slot based on ionization time distribution with an accuracy of 100% for specific cluster with a maximum error of +20 μm. This methodology identifies tolerance deviations, improve process traceability and elimate the need for time-consuming experiment. |
| QM | (Crawford et al., 2021) | Introduce a ML framework for manufacturing of advanced composites, more specially on autoclave cure process of the Hexcel AS4/8552 carbon prepreg material, focusing on dataset knowledgeability assessment and a novel Decision Boundary Crispness Score (DBCS) for evaluating black-box model. This approach combines It combines interpretable surrogate models with domain expertise to enhance dataset quality before training ML models and it enables both global and granular performance evaluation. This study shows how the framework improves decision-making in real-time environments. |
| QM | (Amini & Gan, 2022) | Introduce a machine learning-based model for detecting drilling-induced defects in composite materials during manufacturing, utilizing Terahertz imaging data and the Faster R-CNN Inception v2 model. This approach integrates deep learning with transfer learning to localize, classify, and detect defects with high accuracy (confidence level on average superior than 98%) and efficiency and reduce inspection times and reliance on manual processes. Demonstrate significant potential quality management by enabling real-time defect monitoring and reducing production costs. |
| QM | (Ramezankhani, Narayan, et al., 2021) | Introduce an Active Transfer Learning (ATL) framework for smart manufacturing with limited data with an accuracy of 91,9%. This method focuses on autoclave curing of composites, integrates active learning (AL) to maximize information gain from limited experimental trials and transfer learning (TF) to adapt knowledge from close processes to reduce the dependency on data generation. |
| QM | (Y. Wang et al., 2020) | Present a real-time intelligent fault diagnosis algorithm for the manufacturing process of oxygen decompressors. Integrate a fault tree model with adaptive data analysis to detect low export pressure and serious leakage of safety valve. This approach shown significant improvements in production efficiency and troubleshooting accuracy. |
| QM | (J. Hu et al., 2023) | Introduce a novel AR-based Mask R-CNN system that integrates augmented reality (AR) with deep learning for real-time inspection of cable brackets in aircraft on mobile devices. The system combines AR for generating precise region proposals with Mask R-CNN for segmentation, enabling automatic CAD model checking and intuitive visualization of inspection results with an accuracy of 97,1%. This approach is deployed on C919 assembly workshop and tackle the challenges of portability, adaptability and instantaneity in an industrial environment. |
| QM | (J. Wang et al., 2019) | Explore the use of UL ML techniques, especially K-means and hierarchical clustering (HC) to monitor part tolerances in wire electrical discharge machining (WEDM). Highlights limitations of using average voltage for clustering and shows better accuracy by incorporating skewness and kurtosis features. |
| QM | (Bautista-Hernández & Martín-Prats, 2024) | Introduce an AI-driven approach to mitigate errors in electrical harness assembly process. This study employs different techniques like logistic regression (LR), support vector machines (SVM) and random forests (RF) with a risk matrix to identify, evaluate and minimize risks with SVM demonstrating the highest accuracy (0,98). Achieve significant time reduction and error compared to traditional methods. This framework demonstrates important improvements in efficiency, safety and decision-making accuracy for aerospace manufacturing environments. |
| QM | (Meister et al., 2021a) | Introduce a parallel classification framework combining Convolutional Neural Networks (CNN) and Support Vector Machines (SVM) for automated and explainable defect detection in Automated Fiber Placement (AFP) manufacturing processes with an accuracy superior to 90% exact for two classes. This study enhances trust and reliability in safety-critical aerospace application and insights into classifier decision-making by integrating explainable artificial intelligence (xAI). |
| QM | (Pereira et al., 2018) | Present a data-driven framework for quality prognostics in automated riveting processes for skin panel assembly to identify defects and to enhance quality control. This study demonstrates the effectiveness of using specific ML techniques such as random forest (RF) and extreme gradient boost to predict deviations in rivet hole diameters. Offer real-time implementation and reduce the reliance on manual quality control. |
| QM | (Shafi et al., 2023) | Introduce an AI-based defect detection framework for optimizing aerospace manufacturing processes, focusing on reducing rework delays and associated costs. This uses a convolutional neural network (CNN) and statistical process control tools to identify defects during the early manufacturing stages with an accuracy of 0,7250, avoiding rework time later in assembly phases. It demonstrates a consequent reduction in rework time and cost highlighting the potential use of AI to enhance operational management. |
| QM | (Tyystjärvi et al., 2024) | Explore the deployment of machine learning for radiographic inspection of aerospace welds focusing on automatic defect recognition to improve quality control. The study develops a multi-model approach with weld segmentation for better performances, the creation of a user interface for printing evaluations results and statistical analysis of defect data to detect trends and predict weld defect formation. Demonstrate the potential of integrating AI into non-destructive evaluation (NDE) processes. |
| QM | (Ruiz et al., 2020) | Propose a convolutional neural network (CNN) for detecting and classifying fixation fasteners during aerospace manufacturing processes in uncontrolled industrial environments. This study provides a fast prediction (0,8ms) and an excellent accuracy (98,3%) to detect components such as drills, rivets, and screws. Demonstrate the potential use of AI-driven vision to automate and improve large-scale manufacturing processes related with structural parts. |
| QM | (Rabby et al., 2024) | Introduce a novel non-destructive testing method combining frequency-dependent dielectric measurements with M to assess the curing state and predict the mechanical performance of fiber-reinforced polymer (FRP) composites with an accuracy of 96.7%. This approach achieves a satisfying accuracy for classifying curing states and a good accuracy in predicting tensile strength. Reduce the need for destructive testing and enable prepreg aging monitoring to ensure material quality. |
| QM | (Ruiz et al., 2022) | Present a ML approach using convolutional neural networks (CNN) to classify and localize fixation elements in aerospace manufacturing processes. This system achieves 95,7% of classification accuracy with an average processing time of 0,8 milliseconds per image. Improve the reliability and speed of vision systems in drilling and riveting machines. |
| QM | (J. Wang, Sanchez, Iturrioz, et al., 2018) | Introduce a deep learning-based methodology for detecting geometrical defects during the Wire Electrical Discharge Machining (WEDM) of fir-tree slots in turbine components. The system achieves 80% agreement with traditional Coordinate Measuring Machine (CMM) methods within a strict ±5 µm tolerance. Offer a more efficient, real-time solution for defect detection. |
| QM | (Akhil et al., 2020) | Explore a machine learning-based approach to surface texture characterization and roughness prediction for Ti-6Al-4V selective laser melted (SLM) components using image texture analysis. This approach integrates statistical methods to extract key texture features with Gaussian Process Regression (GPR) in ML models. The highest accuracy is R² > 0.9 across various roughness parameters. Enhance real-time and cost-effective monitoring systems for additive manufacturing. |
| QM | (Mueller et al., 2019) | Present a semi-automated solution for aircraft riveting and quality inspection using Human-Robot Collaboration (HRC), ML, and mixed reality. This system combines human expertise with robotic precision, leveraging sensors and machine learning models to classify rivet quality in real-time and offline. Achieve 97% accuracy in image inspections. Improve production efficiency and inspection reliability and ergonomics. |
| QM | (Meister et al., 2021b) | Explore the use of Explainable Artificial Intelligence (xAI) techniques to improve the transparency and reliability of convolutional neural network (CNN) classifiers in detecting fiber layup defects in automated composite manufacturing. It evaluates three xAI methods namely, Smoothed Integrated Gradients, DeepSHAP, and Guided Grad-CAM. It demonstrates their effectiveness in visualizing the relevance of image regions for CNN decisions and validate their robustness with metrics like Sensitivity and Infidelity scores. Highlight the potential uses of these techniques to enhance defect classification system while providing insights. |
| QM | (Eddy et al., 2023b) | Introduce a method called Learned Manufacturing Inspection Inferences from Image Recognition Capabilities (LeMIIIRC) designed to enhance automated inspections of complex assemblies through ML. This approach is based on 2D and 3D schema image recognition data with process capability indices (Cpk) to predict the whether features meet tolerances specifications. A case study on cable harness assembly is conducted demonstrating its potential, however, this approach still needs to be tested with real manufacturing data. |
| QM | (Fernandes et al., 2018) | Introduce a novel non-destructive evaluation method combining Pulsed Thermal Ellipsometry (PTE) and ML with artificial neural networks (ANN) to assess fiber orientation in randomly-oriented strand (ROS) composite materials. This method uses a flying laser spot (FLS) for line heating and advanced infrared image processing techniques (PCT, PPT, and DTT) and can inspect large areas with significantly reduced time compared to another traditional point-by-point inspections with an accuracy of 71,6% in real settings. |
| QM | (Niccolai et al., 2021) | Present a ML-based automated system for non-destructive testing (NDT) focusing on fluorescent penetrant inspection (FPI) for defect detection in aerospace components. This study compares two system architectures, direct and indirect classifiers, showing that indirect classifiers provide better performance. It combines deterministic feature extraction with neural networks to achieve high classification accuracy with limited datasets. Improve inspection reliability and efficiency and reduce human effort. |
| QM | (Z. Li et al., 2023) | Introduce a machine learning-enabled method for estimating the thickness of thin coatings (50–500 µm) on carbon fiber composites using a microwave cavity resonator sensor. The approach employs principal component analysis for data preprocessing and a neural network for rapid (0,01s), accurate (R2 99%), and non-destructive thickness estimation showing better performances than traditional calibration techniques. |
| QM | (Rolls-Royce Deutschland Ltd & Co KG et al., 2021) | Explore the use of ML models to predict geometric characteristics of blisk blades leveraging milling machine data without additional sensors to reduce reliance on time-intensive coordinate measuring machine (CMM) inspections. This study identifies Random Forest Regressor (RFR) and Partial Least Squares Regressor (PLSR) as the most effective models for accurate predictions showing potential for significant manufacturing efficiency improvements. |
| QM | (Dasari et al., 2020) | Investigate the use of Polar Transformation (PT) for feature extraction in classifying melt-pool defects in additive manufacturing using random forest (RF) model. It demonstrates that PT provides better performances (~94%) than other feature extraction methods such as HOG, HARALICK, and LBP. Highlight the potential use of PT on small datasets and as an efficient feature descriptor for shaped object images. |
| QM | (Nalajam & V, 2021) | Develop ML models to detect and segment microstructural porosity in wire arc additively manufactured (WAAM) AA6061 aluminum alloy parts. These models are based on pixel texture and color features extracted with gabor filters and explore several ML models. Random forest (RF) achieves the best performance (99,49%) for pores larger than 5 µm. |
| QM | (Knittel et al., 2019) | Develop a data-driven surface quality diagnostic system using supervised ML methods like support vector machine (SVM), k-nearest neighbors (kNN), and decision trees to improve milling diagnosis of honeycomb composite materials. SVM achieved the highest accuracy (99,8%) in predicting milling outcomes based on cutting force measurements. |
| QM | (Manohar et al., 2018) | Present PIXI-DUST, a ML-based approach to predict shim gaps in aircraft assembly using sparse sensing and robust principal component analysis (RPCA). This method identifies low-dimensional patterns and optimizes sensor placement based on historical production data. Enable accurate prediction of 99% of shim gaps within tolerance using only 3% of typical laser scan points reducing data collection and processing time. |
| QM | (Kim & Lee, 2022) | Explore the use of ML techniques to predict burr types during the drilling of Al-7075 using acoustic emission (AE) signals. Introduce a convolutional neural network (CNN) and backpropagation (BP) neural network to classify burr types based on AE signals and drilling parameters. The CNN achieve the best accuracy (93,75%). |
| QM | (Yang et al., 2020) | Propose a vibration fault diagnosis model for rotating machinery using autoencoders, including undercomplete, denoising, and sparse autoencoders. Experimental results on four types of faults (unbalanced, misalignment, bump and oil vortex) demonstrate that the sparse autoencoder achieved the highest accuracy (93,48%). Highlight the potential use of deep learning in fault diagnosis. |
| QM | (Eddy et al., 2023a) | Introduce an approach to implement a semiautonomous inspection system using image recognition and machine learning to improve visual inspections. It integrates process capability indices (Cpk) and image recognition data to generate probabilistic insights to help inspectors determine whether assemblies meet tolerance specifications. Demonstrate the feasibility of this solutions through an experimental design for cable harness assembly inspections. |
| QM | (Isaza et al., 2024) | Introduce a vision-based neural network model using YOLO for diagnosing non-conformities in turbine trench-fillers. This method automates defect detection, achieving over 85% accuracy for one type of critical defects. It reported an inspection time from 1.5 hours to 6 minutes enhancing efficiency and precision in production lines. |
| QM | (Martinez et al., 2021) | Present the development and deployment of SAVANNA, a ML system using artificial neural networks (ANNs) to automate the seam validation process in F-35 aircraft manufacturing. This model achieves classification accuracy of 94.3% and 90.8% for respectively gap and mismatch components despite unbalanced data which is mitigated through different pre-processing steps. Highlight the potential of this approach to reduce cost and increase production rate. |
| QM | (Salary et al., 2020) | Propose a sparse representation classification (SRC) model for near real-time to predict functional properties of printed electronics with an accuracy superior to 90%based on in situ and real-time image-based data on printed line morphology and process parameters. Highlight the advantages of SRC over traditional classifiers. |
| ED | (Tong, 2023) | Development of a user-interface to deploy ML for assessing aircraft engine concepts, focusing on turbofan performance metrics. This approach is based on deep learning and K-nearest neighbor algorithm. This study demonstrates a good accuracy (~96%) and efficiency in engine design-space exploration for the conceptual phase. |
| ED | (Zhan & Li, 2021a) | Integrate Continuum Damage Mechanics (CDM) theory with ML to predict the fatigue life of aerospace alloys fabricated through additive manufacturing. Use ANN and Random Forest models combined with elastoplastic fatigue damage models. Provide an efficient tool for fatigue life prediction, reduce computational costs and offer opportunities for optimizing design of aerospace components. |
| ED | (Huang et al., 2023) | Explore the impact of fiber shapes on the transverse modulus of unidirectional fiber-reinforced composites using finite element analysis (FEA) and ML. Show that irregular fiber geometries can affect transverse modulus prediction and provide an accurate ML model (coefficient of determination R2 equals to 0.9987), saving time for predicting mechanical properties. |
| ED | (Hsu et al., 2023) | Present a deep learning-based Smart Representative Volume Element (RVE) method to predict mechanical properties of long-fiber woven composites. This approach streamlines the traditional approach of finite element analysis (FEA) by integrating a dual-input Convolutional Neural Network (CNN) with several inputs such as fiber geometry, weaving methods and material properties to predict mechanical properties (Young's modulus). This model provides a good accuracy (coefficient of determination equals to 0,99) and reduce the computational time to design and analyze composites. |
| ED | (Prudviraj et al., 2021) | Introduce a machine learning-based approach to predict the orifice size of Reaction Control System (RCS) thrusters in aerospace vehicles during cold flow calibration. This study evaluates multiples models such as Artificial Neural Networks (ANN), Support Vector Machines (SVM), and Relevance Vector Machines (RVM), with the Least-Squares Support Vector Machine (LSSVM) using a Gaussian kernel achieving the best performance (minimum mean absolute error MAE equal to 0,057). Demonstrate a significant improvement in accuracy and efficiency to optimize thruster performance. |
| ED | (Zhao et al., 2024) | This article introduces a novel cascaded conditional generative adversarial network (BT-GAN) for efficiently predicting stress fields in CFRP bolted joints during assembly with an MAE of 2,14 and MSE of 4,12. This method improve computational efficiency in comparison with the traditional finite element methods (FEM) by leveraging data augmentation, attention mechanisms, and a U-net-based generator. Offer potential applications in real-time assembly analysis, digital twin modeling and optimization of composite structures. |
| ED | (Román et al., 2023) | Develop a ML-based framework to predict the viscoelastic behavior of natural rubber blends and optimize their formulations for industrial applications. This study employs ANN with sulfur, plasticizer, and foaming content as inputs to assess key viscoelastic properties namely like stress relaxation, tan δ, and durometer hardness with an average accuracy of 96,75%. Enable reverse engineering of material formulations with potential for sustainable polymer design and to decrease manufacturing cost. |
| ED | (Grozav et al., 2023) | Develop a neural network-based predictive model to estimate the mechanical properties of Fused Deposition Modeling (FDM) 3D-printed polylactic acid (PLA) components. The model achieves a good accuracy (~93%) in predicting tensile strength based on process parameters and finite element analysis simulations. It optimizes design in additive manufacturing by predicting mechanical behavior allowing a cost reduction and performance improvement. |
| ED | (Ali et al., 2022) | Deploy a deep learning / ML model to predict stress-strain behavior of various lattice structures based on experimental data. The model is highly accurate (correlation coefficient R2 = 0,999, mean square error MSE = 0,0017, mean absolute error MAE = 0,0312) and offers potential to optimize lattice structures. |
| ED | (Zhan & Li, 2021b) | Present a ML framework for fatigue life prediction of stainless steel 316L in an additive manufacturing context. This study evaluates different ML models based on process parameters (laser power, scan speed and powder layer thickness) and on continuum damage mechanics (CDM) such as artificial neural network (ANN), random forests (RF) and support vector machines (SVM). RF model provides the best accuracy with a coefficient of determination R2 of 0,942 and mean square error MSE of 0,112. It demonstrates the feasibility of integrating CDM with ML to predict fatigue life prediction allowing a reduction in computational cost while improving the reliability of the components. |
| ED | (Doodi & Gunji, 2023) | Investigate he design and performance of novel hybrid bio-inspired 3D-printed lattice structures optimized for energy absorption applications. This approach uses artificial neural network (ANN) combined with the Levenberg-Marquardt algorithm to predict energy absorption capacities. Highlight the potential of combining bio-inspired designs with additive manufacturing and ML to improve lattice structures. |
| ED | (Castro et al., 2023) | Develop a predictive model using a Long Short-Term Memory (LSTM) deep learning algorithm to estimate compressive strength of Fused Deposition Modeling (FDM) 3D-printed parts. The model uses process variables like infill density, print orientation, raster orientation, and layer thickness and provide satisfying accuracy (maximum error percentage of 0,93). Allow a reduction of destructive testing and highlight its potential use to optimize additive manufacturing processes and its integration with sensor-based monitoring to enhance the accuracy. |
| ED | (Merayo et al., 2021) | Develop a methodology to optimize the topology of artificial neural network (ANN) to predict the ultimate tensile strength (UTS) of aluminum alloys used in metal forming. This study identifies a balance between computational efficiency and model accuracy (stabilized below 4%). Provide a framework to apply ML in material characterization allowing an improvement in the prediction of mechanical properties in industrial manufacturing processes. |
| ED | (Varol Özkavak et al., 2023) | Investigate different ML techniques such as convolutional neural network (CNN), artificial neural network (ANN) and random forest regression (RFR) to predict mechanical properties (hardness and bending strength) of 2024 aluminum alloys. The best model achieved good performances (coefficient of determination R2 = 0,94166, root mean square error RMSE 0,08578, mean absolute error MAE = 0,06212) is based on aging temperature and time and reduces the need for costly and time-consuming experiments. CNN provided the most accurate predictions. This approach shows that the material design can be accelerated and the experimental costs can be reduced. |
| ED | (Caiazzo & Caggiano, 2018) | Investigate the laser direct metal deposition (DMD) of 2024 aluminum alloy, focusing on predicting the geometry of deposited metal traces. This approach uses an artificial neural network (ANN) to estimate optimal process parameters (laser power, scanning speed, and powder feed rate) with a good accuracy (~5%). Highlight the potential use of ML in optimizing additive manufacturing for industrial applications. |
| ED | (A. Kumar et al., 2024) | Explore the use of a die-less forming to manufacture lightweight aluminum alloy parts (single point incremental forming (SPIF)) to improve energy efficiency and to decrease cost compared to conventional methods. The study uses an artificial neural network (ANN) to predict peak forming forces with a high accuracy (coefficient of determination R2 0,99, root mean square error RMSE 5,59, mean square error MSE 43,52 and mean absolute percentage error 4,35). This approach offers an alternative to extensive experimental processes, showing the impact of tool geometry and sheet thickness on forming forces and providing insights to optimize SPIF processes. |
| ED | (Thanusha & Balguri, 2024) | Explore the optimization of wear and mechanical characteristics of jute fiber composites reinforced with silicon (SiC) nanoparticles. This approach uses a combination of experimental testing and an artificial neural network (ANN) providing a high accuracy in predicting wear and mechanical properties (coefficient of correlation R 0,99). Provide a tool to optimize composite materials. |
| ED | (Yüce et al., 2023) | Compare random forest regression (RFR) and artificial neural networks (ANN) for predicting the crack growth life of fighter aircraft wing joints under spectrum loading. The RFR provides better accuracy performance and computational cost. Highlight the impact of permutation feature importance and hyperparameter optimization and offer insights for fatigue life estimation. |
| ED | (Yadam et al., 2020) | Explore the challenges of making ML models interpretable for experts by developing visually interpretable frameworks. Use NASA's airfoil noise dataset as a case study and compare several ML method such as random forest (RF) and XGBoost regression, emphasing on RF's interpretability with different techniques. XgBoost achieved the highest performances with a root mean square error of 1,96, a coefficient of determination R2 0,9195. This study proposes that visual explanations can fulfill the gap between ML experts and domain experts with potential for interactive UI-based applications to enhance decision making. |
| ED | (Choi et al., 2023) | Propose a ML-assisted two-step homogenization framework for short fiber-reinforced plastics (SFRPs) to predict mechanical properties (elastic modulus). The approach integrates a series-parallel artificial neural network (ANN) system with the Mori-Tanaka and Voigt models into an ABAQUS UMAT subroutine with an average error within 2%. This framework reduces computational costs providing an alternative for iterative procedures for orientation distribution function (ODF) reconstruction and pseudo grain decomposition. Adress the limitations of traditional homogenization methos and offer an accurate solution to simulate mechanical behavior. |
| ED | (Al-Haddad & Mahdi, 2024) | Introduce a novel integration of Finite Element Analysis (FEA) with a Naïve Bayes (NB) ML model for simulating, predicting stress and strain, as well as deformation in aircraft landing gear during the landing phase with a good accuracy (coefficient of determination R2 1,00 and root mean square error RMSE 0,9%). This approach achieves at the same time an important reduction in computational time and an excellent accuracy. |
| ED | (K. Zhang et al., 2022) | Present an artificial neural network (ANN) model for predicting the mechanical deformation, strength, and progressive damage behaviors of carbon fiber reinforced polymer (CFRP) laminates with circular holes. The ML model is trained on a database generated through finite element (FE) simulations, demonstrating high accuracy (~95%) and significantly reducing computational costs compared to traditional FE methods. |
| ED | (Frey Marioni et al., 2022) | Develop ML-driven turbulence closures for low-pressure turbine (LPT) wake mixing predictions. This method is based on a direct numerical simulation (DNS)-ML framework to train explicit algebraic Reynolds stress models (EARSM) with artificial neural networks (ANNs). DNS data are generated from a solver, validated through experiment, gathered into clusters then used by the ML model to predict aerodynamic features. The model's predictions are then implemented into Rolls-Royce solver HYDRA and compared with Wilcox’s k − ω SST model. Provide a model which is accurate and good in terms of computational cost. |
| PPC | (Matitopanum et al., 2023) | Develop a novel predictive model for Ultimate Tensile Strength (UTS), Maximum Hardness (MH), and Heat Input (HI) in AA7075 Friction Stir Welding (FSW) using a Heterogeneous Ensemble Machine Learning (He-EM) approach. This model uses four key welding parameters ((Tilt Angle, Rotation Speed, Welding Speed, and Shoulder Diameter) to realize its predictions and a long short-term memory (LSTM) ML model. The artificial multiple intelligence system (AMIS) He-EM is then used to improve model's effectiveness (correlation coefficient CC 0,995, root mean square error RMSE 3,030). This study offers better performances than another technique allowing optimization in welding procedures, product performance and manufacturing processes improvements. |
| PPC | (Veeramani et al., 2020) | Propose two novel AI-based planners for multi-head path planning for SwarmltFIX robotic agents in sheet metal manufacturing. While the first planner is based on value iteration and policy iterations (PI) algorithm the second planner uses Monte Carlo reinforcement learning. This study finds that the PI algorithm is the fastest (0,23s) whereas the Monte Carlo offers greater adaptability. Highlight the potential for AI-driven optimization in robotic manufacturing. |
| PPC | (Massimo et al., 2023) | Propose a lightweight and accurate ML-based pipeline to estimate the mass and lengths of sintered workpieces in powder metallurgy (PM) based on features derived from press-curve and environmental data. Several ML models are explored; however, gradient boosting (GB) provides the best performance in terms of accuracy and time with the lowest root-mean-squared error (~0.0886%). Highlight the importance of feature selection and correlation analysis. |
| PPC | (Adeniji & Schoop, 2022) | Introduce a novel Digital Process Twin (DPT) framework for machining processes components made from gamma titanium aluminide alloy (γ-TiAl) integrating physics-based models and ML to optimize manufacturing efficiency and sustainability. Demonstrate significant improvements in energy efficiency (84%), process queuing time (93%), scrap cost (2%), and queuing cost (93%) compared to traditional empirical methods. Enable real-time process adaptation and multi-objective optimization. |
| PPC | (Z. Zhang et al., 2022) | Demonstrate the integration of ML-enabled closed-loop control into ceramic additive manufacturing. This approach uses an artificial neural network (ANN) to optimize robocasting process based on a database of relationships between processing load and extrusion length. The model predicts extrusion parameters in real-time improving print quality and consistency in comparison with open-loop systems. Highlights the potential use of data-driven approaches to avoid complex physics-based models. |
| PPC | (Natarajan et al., 2023) | Explore the machinability of Titanium Grade 5 alloy using Wire Electrical Discharge Machining (WEDM) enhanced with a hybrid learning algorithm combining Artificial Neural Networks (ANN) and Adaptive Neuro-Fuzzy Inference System (ANFIS). This study identifies optimal machining parameters to improve KPI like material removal rate, surface roughness, dimensional deviation and GD&T errors. The ANFIS-based model demonstrates good performances (Correlation coefficient R 99%) and offers a practical tool to optimize WEDM processes. |
| PPC | (Higgins et al., 2023) | Investigate ML methods such as artificial neural networks (ANN) and support vector regression (SVR) in order to improve the positional accuracy of industrial robots for aerospace applications. This approach demonstrates significant error reductions however it doesn't meet the aerospace tolerances. Highlight the importance of input data type (TCP coordinates vs joint angles.) and feature scaling (normalization and standardization) to enhance ML model. Explore the use of Grid Search hyperparameter optimization. |
| PPC | (Ramezankhani, Crawford, et al., 2021) | Introduce a transfer learning framework tailored for smart manufacturing processes addressing challenges in aerospace composite autoclave processing under limited data scenarios. This method combines abundant historical data with limited new data to adapt machine learning models for predicting process parameters like thermal lag and exotherm, demonstrating an 88% reduction in generalization loss compared to conventional models. Highlight the potential of transfer learning to reduce computational costs while maintaining ML model performances. |
| PPC | (Verma et al., 2022) | Investigate the application of ML techniques such as random forest regression (RFR), M5P tree regression, and artificial neural networks (ANN) in order to model the tensile strength of friction stir welded AA6082-T6 aluminum alloy. RFR shown the highest accuracy (correlation coefficient of 0.93) and this identify rotational speed as the most critical parameter affecting ultimate tensile strength (UTS). |
| PPC | (Fysikopoulos et al., 2015) | Introduce the MetaCAM tool, a decision-support system designed to optimize manufacturing processes for sustainability by focusing on energy efficiency, waste reduction, cost minimization, and time savings across four manufacturing levels (process, machine, production line, and factory). demonstrate significant improvements in energy consumption, cost, and environmental impact especially in real industrial case such as loading ramp hinge in aeronautics. |
| PPC | (Shaban et al., 2017) | Present a new process control technique for machining carbon fiber reinforced polymer (CFRP) using Logical Analysis of Data (LAD), a pattern-based ML approach. Demonstrate how LAD identifies characteristic patterns that differentiate conforming from nonconforming products in a milling process and shows that its accuracy is better than artificial neural networks (ANN). |
| PPC | (Jwo et al., 2022) | Introduce a data twin service (DTS) for optimizing the autoclave curing process, combining human intelligence with artificial intelligence through a user-friendly GUI application. It employs a two-stage ML model to simulate mold placement and predict curing times to improve efficiency. Empirical studies on historical data and real-world implementation demonstrate the system's accuracy. |
| PPC | (Adizue et al., 2023) | Investigate the optimization of ultra-precision hard-turning (UPHT) of AISI D2 steel using cubic boron nitride (CBN) tools. It explores Response Surface Methodology (RSM) with several ML models such as Support Vector Machines (SVM), Gaussian Process Regression (GPR), Artificial Neural Networks (ANN), and Adaptive Neuro-Fuzzy Inference Systems (ANFIS), finding that ANFIS and ANN to be the most accurate, respectively with a correlation coefficient of 98% and 99% for surface roughness prediction. |
| PPC | (Zhu et al., 2024) | Introduce a Bayesian learning-enabled explainable AI (xAI) framework to predict thermal distributions in additive friction stir deposition (AFSD) of Al-Mg-Si alloy. It integrates multi-physics modeling with data-driven approaches in order to achieve accurate temperature predictions with limited datasets (average error 3,23%) and computational time. |
| PPC | (Perani et al., 2023) | Present a novel approach to predict track geometry in Laser Metal Deposition (LMD) of Inconel 718 using a deep-learning system that combines convolutional neural networks (CNNs) for melt pool image analysis with process input data. This approach provides a good accuracy in comparison with other ML models and is suitable for on-line prediction. |
| PPC | (Sanchez et al., 2018) | Explore the application of deep learning techniques such as convolutional and gated recurrent unit (CGRU) architectures to predict unexpected events in wire electrical discharge machining (WEDM) processes. This approach achieves a 97.4% accuracy in predicting thickness variations at least 2 mm in advance before process degradation occurs. This method is based on raw voltage signals and a combination of convolutional layers with GRUs. |
| PPC | (Fotovvati & Chou, 2022) | Investigate the influence of key process parameters in selective laser melting (SLM) on surface roughness for single-layer Ti6Al4V samples and develop a ML model. This study uses an artificial neural network (ANN) based on SLM process parameters achieving a coefficient of determination of (98,8%) to predict surface roughness. Enable in-situ monitoring and process optimization of surface quality. |
| MM | (Garcia-Perez et al., 2024) | Introduce a vision-based tool wear monitoring system for vertical lathes. This approach leverages on generative AI models (SCWGAN-GP and Focal Stable Diffusion) to augment datasets with synthetic images and on hardware (K¦Lens optics). This study shows the feasibility of reducing dataset creation time from months to days with 100 real images while achieving good accuracy (94,6%). |
| MM | (W. Liu et al., 2022) | Propose a novel milling cutter state recognition (tool state recognition) addressing challenges like small, imbalanced and heterogeneous multi-source data. This study goes through different steps in its solution such as compressing methods and the use of a stack sparse auto-encoder network based on improved D-S evidence theory and the Dropout method achieving better recognition accuracy and generalization performances (accuracy of 99%). |
| MM | (Filz et al., 2021) | Introduce introduces a data-driven Failure Mode and Effect Analysis (FMEA) methodology leveraging deep learning to improve maintenance planning for industrial investment goods via a case study in the aviation sector. The framework achieves a good fault prediction accuracy (0,9887) based on operational and historical data to dynamically calculate failure probabilities, reducing subjective assessments and enhancing decision-making for maintenance activities. Enable more efficient resource allocation and support less experienced employees. |
| MM | (H.-M. Chen et al., 2023) | Present a predictive maintenance system for aerospace hot-pressing furnaces using ML to monitor three-phase current abnormalities, improving fault detection and reducing downtime costs. Introduce a monitoring algorithm, ML models like XGBoost and real-time visualization of the equipment. The ML model achieve 97% of accuracy. Enable proactive maintenance and reduce cost. |
| MM | (Siyaev et al., 2023) | Introduce neuro-symbolic reasoning (NSR) as a method for interacting with industrial digital twins using natural language. It enables users to perform operations on 3D models, retrieve information from manuals, and execute maintenance procedures autonomously. Achieve 96.2% accuracy in translating user requests into symbolic programs. Offer a robust solution for intuitive and efficient communication with complex digital twins. |
| MM | (Abidi et al., 2022) | Present a predictive maintenance (PdM) planning model for Industry 4.0, leveraging machine learning techniques like Support Vector Machines (SVM) and Recurrent Neural Networks (RNN) optimized with a novel Jaya-based Sea Lion Optimization (J-SLnO) algorithm. Experimental results on aircraft engine shown that this method offers better performance with metrics like MAE or RMSE. |
| MM | (Azar et al., 2022) | Introduce a novel hybrid Maintenance Decision Support System (MDSS) framework that integrates Machine Learning (ML) and statistical techniques. It integrates a time-dependent Proportional Hazard Model (PHM) with semi-supervised clustering and Reinforcement Learning (RL) to optimize maintenance decisions for systems with stochastic degradation. This framework processes high-dimensional Condition Monitoring (CM) data, constructs system states (healthy, warning, failure) using unsupervised clustering, and employs RL to minimize maintenance costs. The proposed MDSS is validated using NASA's C-MAPSS dataset and demonstrates superior performance across various maintenance policies and cost scenarios. |
| MM | (Azar & Naderkhani, 2020) | Introduce a novel hybrid Maintenance Decision Support System (MDSS) for condition-based maintenance (CBM), leveraging a time-dependent Proportional Hazards Model (PHM) augmented with a semi-supervised machine learning approach to optimize maintenance decisions using high-dimensional and multi-modal streaming data. Experimental evaluations using NASA's C-MAPSS dataset demonstrate the model's effectiveness in fault diagnostics and prognostics. |
| MM | (Koch et al., 2022) | Explore the implementation of a tool wear classification system for automated drilling processes using different ML models such as Fine Decision Trees (FDT), Ensemble Bagged Trees (EBT) and Support Vector Machines (SVM), sensors like accelerometer and microphone and data with time series and frequency domain. SVM provided the highest accuracy 87,7% to classify drill bit types. |
| MM | (Llasag Rosero et al., 2024) | Introduce FedLabSync, a novel Hybrid Federated Learning (FL) algorithm designed to address challenges in decentralized machine learning by implementing label synchronization. It enables collaboration among parties with diverse data structures while preserving privacy. It demonstrates that FedLabSync achieves performance comparable to centralized learning methods and improves local model accuracy via two industrial cases specially on remaining useful life (RUL) estimation of aircraft components. |
| MM | (Jaw & Lee, 2014) | Explore the application of ML techniques to aircraft engine diagnostics using a high-fidelity engine simulation dataset. This study identifies nonlinear support vector machines (NSVM) and a hierarchical SVM with kernel sliced inverse regression (HSVMkSIR) as the most effective ML approaches. Highlight the potential of ML to complement physics-based methods in diagnostics of complex systems. |
| SCM | (Loyer et al., 2016) | Compare different models for estimating manufacturing cost of jet engine components during the early design phase. This study shows superior performance with techniques like gradient boosted trees and support vector regression (SVM) in comparison with other ones such as multiple linear regression and artificial neural network. Gradient boosted trees shown the highest performances with coefficient of determination R2 0,96 and mean absolute percentage error 6,40. Highlight the importance of combining multiple models to gain deeper insights into cost drivers. |
| SCM | (Van Nguyen et al., 2023) | Develop a digital cost estimation system for the aerospace manufacturing industry by integrating meta-learning, Bayesian hyper-parameter tuning, domain knowledge, and automated market data collection. The purpose of this approach is to improve cost estimation accuracy and decision-making efficiency. A study case is conducted with an aerospace manufacturer demonstrating significant improvement in prediction accuracy and execution time compared with another methods. |
| SCM | (Dou & Peng, 2023) | Introduce an intelligent tool cabinet system leveraging the YOLO object detection algorithm to address challenges in tool management, such frequent loss and lack of traceability. This approach used a YOLOv8 algorithm to achieve real-time recognition and tracking of object detection based on deep learning and transfer learning. Demonstrate the effectiveness in improving tool management. |
| CS | (Hojjati et al., 2016) | Present a novel side-channel attack using smartphone sensors (audio and magnetometer) to capture manufacturing equipment's emissions in order to reconstruct product designs and manufacturing processes with high accuracy (accuracy of segment lengths within 1mm and 1 degree of angle errors). Demonstrate the feasibility of attacking 3D printers and CNC mills with mobile devices on factory floors. Emphasize the need of enhancing security protocols on manufacturing floors especially for sensitive manufacturing data. |