Supplementary material

Life cycle assessment of synthetic graphite: Assessment results and the importance of transparency, clarity, and reproducibility

Bening Mayanti1

1 Industrial Management, School of Technology and Innovations, University of Vaasa, Vaasa, Finland

Corresponding Author: Bening Mayanti, bening.mayanti@uwasa.fi

# LCA Inventory data

Tables below show the complete inventory of foreground data in the study.

## Coal mining (including washing)  (Tao et al., 2022)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Coal, hard, in ground | t |  | 1.15 |  |
|  | Steel | Kg |  | 1.3 |  |
|  | Cement | kg |  | 5.65 |  |
|  | Wood | kg |  | 0.56 |  |
|  | Explosive | kg |  | 4.36E-02 |  |
|  | Water | m3 |  | 0.29 |  |
|  | Other chemicals | kg |  | 0.14 |  |
|  | Electricity | kWh |  | 33.07 |  |
|  | Diesel | MJ |  | 6.93E-02 |  |
|  | Transportation | t-km |  | 1.16 |  |
| Products/co-products | Hard coal | t |  | 1 | Intermediate product |
| Emissions to air | Methane | kg |  | 18.48 |  |
|  | Carbon dioxide | kg |  | 44.35 |  |
|  | Carbon monoxide | g |  | 73.92 |  |
|  | Hydrogen dioxide | g |  | 22.18 |  |
|  | Sulphur dioxide | g |  | 31.46 |  |
|  | Nitrogen dioxide | g |  | 12.24 |  |
|  | Ammonia | g |  | 66.52 |  |
|  | PM 2.5 | g |  | 8.14 |  |
|  | PM 10 | g |  | 16.71 |  |
| Emissions to water | COD | g |  | 5.87 |  |
|  | Ammonia as N | g |  | 0.42 |  |
| Emissions to soil | Lead | mg |  | 5.58E-02 |  |
|  | Chromium | mg |  | 2.07E-02 |  |
|  | Arsenic | mg |  | 0.25 |  |
|  | Cadmium | mg |  | 9.00E-03 |  |
|  | Mercury | mg |  | 7.20E-03 |  |
| Waste | Wastewater | t |  | 0.35 |  |
|  | Coal gangue | t |  | 0.15 |  |

## Coking

### Coal transportation from mining site (Ecoinvent, 2021)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Transport, freight train | t-km |  | 0.645 |  |
|  | Transport, freight, conveyor belt | t-km |  | 0.005 |  |
|  | Transport, freight, inland waterways, barge | t-km |  | 0.09 |  |
|  | Transport, freight, lorry, unspecified | t-km |  | 0.005 |  |
| Output | Coal, transported | kg |  | 1 |  |

### Coking process (Banerjee et al., 2021; Li et al., 2020)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Coal | t |  | 1.32 |  |
|  | Water | t |  | 0.74 |  |
|  | C3 hydrocarbon mixture as washing oil | kg |  | 43 |  |
|  | Sulphuric acid | kg |  | 9.68 |  |
|  | Sodium hydroxide | kg |  | 3.00E-03 |  |
|  | Compressed air | Nm3 |  | 5.1 |  |
|  | Steam | kg |  | 148 |  |
|  | Coke oven gas | Nm3 |  | 215.8 |  |
|  | Electricity | kWh |  | 45.28 |  |
| Products/co-products | Coke | kg |  | 1000 |  |
|  | Coal tar pitch | kg |  | 36 | Intermediate product |
|  | Crude benzene | kg |  | 11 |  |
|  | Ammonium sulphate | kg |  | 13 |  |
|  | Coke oven gas | Nm3 |  | 432.2 |  |
|  | Sulphur | kg |  | 1 |  |
| Emissions to air | PM | kg |  | 0.03 |  |
|  | Nitrogen oxide | kg |  | 0.17 |  |
|  | Sulphur dioxide | kg |  | 0.07 |  |
|  | Benzo(a)pyrene | kg |  | 2.20E-04 |  |
| Waste | Coke tar residue | kg |  | 0.06 |  |
|  | Coal tar pitch residue | kg |  | 1.00E-04 |  |

## Oil mining, onshore (Meili et al., 2021)

| Input/output flows |  | Unit | Amount | Note |
| --- | --- | --- | --- | --- |
| Inputs | Chemicals, inorganic | kg | 5.53E-4 |  |
|  | Chemicals, organic | kg | 4.22E-4 |  |
|  | Electricity | kWh | 5.46E-2 |  |
|  | Diesel, burned | MJ | 5.23E-2 |  |
|  | Sweet gas, burned in gas turbine | MJ | 2.56 |  |
|  | Onshore well | m | 4.39E-5 |  |
|  | Onshore pipeline | km | 6.95E-9 |  |
|  | Onshore petroleum field | item | 1.24E-10 |  |
|  | Crude, oil, resource in ground | kg | 1 |  |
|  | Water, fossil | m3 | 8.35E-4 |  |
|  | Water, resource in ground | m3 | 4.65E-4 |  |
|  | Transport, lorry | t-km | 9.75E-5 |  |
|  | Transport, freight, rail | t-km | 5.85E-4 |  |
| Products/co-products | Crude oil | kg | 1 | Intermediate product |
| Emissions to air | Methane (as natural gas vented) | kg | 5.56E-03 |  |
|  | Sulphur dioxide | Kg | 1.03E-4 |  |
|  | Nitrogen oxides | kg | 5.27E-4 |  |
| Emissions to water | Water | m3 | 1.30E-3 |  |
|  | Oils | kg | 2.47E-7 |  |
|  | BOD5 | kg | 7.77E-7 |  |
|  | COD | kg | 7.77E-7 |  |
|  | DOC | kg | 2.13E-7 |  |
|  | TOC | kg | 2.13E-7 |  |
|  | AOX as Cl | kg | 2.45E-12 |  |
|  | Nitrogen | kg | 1.91E-10 |  |
|  | Sulphur | kg | 6.6E-10 |  |
| Emissions to soil | Oils | kg | 3.79E-6 |  |
| Waste | Natural gas, sweet | Nm3 | 1.14E-2 |  |
|  | Low level radioactive waste | m3 | 2.00E-7 |  |
|  | Municipal solid waste | kg | 1.00E-4 |  |
|  | Discharge, produced waster | kg | 0.941 |  |

## Oil refining

### Crude oil transportation from mining site (distance was estimated from the US to China)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Transport, freight, lorry, unspecified | t-km |  | 100 |  |
|  | Transport, freight, sea tanker for petroleum | t-km |  | 2.59E4 |  |
| Output | Crude oil, transported | t |  | 1 |  |

### Oil refining (Askari et al., 2021; Liu et al., 2020; NCCP, 2017)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Crude oil | t | 10,000 |  |  |
|  | Methyl alcohol | t | 44.41 |  |  |
|  | Natural gas | t | 1.86 |  |  |
|  | Water | t | 74,27.3 |  |  |
|  | NaOH | t | 6.43 |  |  |
|  | HCl | t | 5.92 |  |  |
|  | Zeolite as catalyst | t | 5.32 |  |  |
|  | Sodium hypochlorite | t | 1.23 |  |  |
|  | Polyacrylamide | t | 0.74 |  |  |
|  | Liquid ammonia | t | 0.46 |  |  |
|  | Imidazole as corrosion inhibitor | t | 0.22 |  |  |
|  | Chemicals organic as others | t | 2.2 |  |  |
|  | Electricity | kWh | 565,007.51 |  |  |
|  | Steam | t | -154.17 |  |  |
| Products/co-products | Diesel | t | 3,912 |  |  |
|  | Petroleum | t | 3,379 |  |  |
|  | LPG | t | 678 |  |  |
|  | Petroleum coke | t | 667 | As green coke, intermediate product |  |
|  | Coke burning | t | 396 |  |  |
|  | Propylene | t | 273 |  |  |
|  | Naphtha | t | 186 |  |  |
|  | Benzene | t | 47 |  |  |
|  | Sulphur | t | 37 |  |  |
| Emissions to air | PM 2.5 | t | 0.16 |  |  |
|  | PM 10 | t | 0.17 |  |  |
|  | Sulphur dioxide | t | 0.71 |  |  |
|  | Nitrogen oxides | t | 1.54 |  |  |
|  | VOCs | t | 3.21 |  |  |
|  | Carbon monoxide | t | 7.22 |  |  |
|  | Carbon dioxide | t | 453.21 |  |  |
|  | Ammonia | kg | 4.32 |  |  |
|  | Nickel | kg | 31.18 |  |  |
| Emission to water | Wastewater | t | 2,961 |  |  |
|  | Oils | kg | 1.63 |  |  |
|  | COD | kg | 118 |  |  |
|  | Total phosphorus | kg | 9.34 |  |  |
|  | Total nitrogen | kg | 20.91 |  |  |
|  | Sulphide | kg | 0.08 |  |  |
|  | Cyanide | kg | 0.06 |  |  |
|  | Nickel | kg | 0.30 |  |  |
|  | Arsenic | kg | 0.01 |  |  |
|  | Methylbenzene | kg | 0.17 |  |  |
|  | Ethylbenzene | kg | 0.08 |  |  |
|  | Xylene | kg | 0.54 |  |  |
|  | Benzene | kg | 0.18 |  |  |
| Waste | Waste zeolite as dead catalysts | t | 5.94 |  |  |
|  | Refinery sludge | t | 2.56 |  |  |

## Calcination (Edwards et al., 2020; Tang et al., 2018)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Green coke | t |  | 1.35 |  |
|  | Hydrated lime | t |  | 3.97E-02 |  |
|  | Lubricating oil as dedusting oil | t |  | 2.06E-03 |  |
|  | Electricity | MWh |  | 1.01E-01 |  |
|  | Heat | MWh |  | 1.19E-02 |  |
|  | Water | m3 |  | 9.88E-01 |  |
| Products/co-products | Calcined petroleum coke | t |  | 1 | As needle coke, Intermediate product |
|  | Electricity | MWh |  | 6.13E-01 |  |
| Emission to air | Carbon dioxide | t |  | 7.23E-01 |  |
|  | Sulphur dioxide | t |  | 2.89E-04 |  |
| Waste | Fly ash and scrubber sludge | t |  | 6.32E-02 |  |

## Baking (including grinding, sieving, mixing moulding, and baking)  (Tang et al., 2018)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Needle coke | kg |  | 984.2 |  |
|  | Coal tar pitch | kg |  | 235.3 |  |
|  | Electricity | kWh |  | 119.12 |  |
|  | Coke | MJ |  | 600 |  |
|  | Lime | kg |  | 2.22 |  |
|  | Natural gas | m3 |  | 60 |  |
|  | Sodium hydroxide | kg |  | 0.44 |  |
| Products/co-products | Prebaked anode | t |  | 1 | As graphitisation precursor, Intermediate product |
|  | Calcium sulphate | kg |  | 5.4 |  |
| Emission to air | Nitrogen oxides | mg |  | 95.94 |  |
|  | Particulates | mg |  | 26.06 |  |
|  | Sulphur dioxide | mg |  | 91.46 |  |

## Graphitisation (Lan et al., 2022; Shang et al., 2024)

| Input/output flows |  | Unit | Amount |  | Note |
| --- | --- | --- | --- | --- | --- |
| Inputs | Graphitisation precursor | t |  | 1.02 |  |
|  | Electricity | kWh |  | 8380 |  |
| Products/co-products | Synthetic graphite, anode grade | t |  | 1 |  |

## Intermediate products needed (Surovtseva et al., 2022; Whattoff, 2022)

Table below shows the summary of the inputs of main materials required (after adjusted by the allocation of co-products) to manufacture 1 kg of anode-grade synthetic graphite.

| Activity | Output | Amount required (kg) |
| --- | --- | --- |
| Mining coal | Hard coal, washed | 0.514 |
| Coal coking | Tar pitch | 0.240 |
| Mining oil | Oil crude | 0.482 |
| Oil refining | Green coke | 1.300 |
| Calcination | Needle coke | 1.004 |
| Baking | Graphitization precursor | 1.020 |
| Graphitisation | >99% pure graphite | 1 |

# Economic allocation inventory data

The market price data was taken from year 2023.

## Economic allocation for coking

| Product | Price (USD/t) | Ref |
| --- | --- | --- |
| Coke | 346.68 | Li et al. (2020), SunSirs (2024a) |
| Coal tar pitch | 673.54 | Li et al. (2020), SunSirs (2023a) |
| Crude benzene | 1103.6 | National Bureau of Statistics of China (2023), Procurement Resource (2024) |
| Ammonium sulfate | 106.97 | Li et al. (2020) SunSirs (2023b) |
| Sulphur | 170.8 | Abdalla (2024), Li et al. (2020) |
| Coke oven gas | 150.77 | Li et al. (2020) |

## Economic allocation for oil refining

| Product | Price (USD/t) | Ref |
| --- | --- | --- |
| Petroleum coke (as Green coke) | 359.13 | Mysteel (2023) |
| Benzene | 1103.6 | National Bureau of Statistics of China (2023), Procurement Resource (2024) |
| Diesel | 1113.9 |  |
| Gasoline | 1273.5 |  |
| Liquefied petroleum gas | 728.7 |  |
| Naphtha | 1032.0 | SunSirs (2024b) |
| Propylene | 122.54 | Procurement Resource (2024) |
| Sulphur | 170.8 | Abdalla (2024) |
| Coke burning | 0 | Eser (2021) (Coked catalyst during cracking is burned) |

## Economic allocation for calcination

| Product | Price (USD/t or USD/MWh) | Ref |
| --- | --- | --- |
| Calcined coke (as needle coke) (t) | 1365.48 | Mysteel (2023) |
| Electricity (MWh) | 89 | Statista (2023) |

## Economic allocation for baking

| Product | Price (USD/t) | Ref |
| --- | --- | --- |
| Graphitisation precursor | 707.5 | Shanghai Metals Market (2023), Tang et al. (2018) |
| Calcium sulfate | 0 | Tang et al. (2018) |

# Results

## Contribution analysis across impact categories per FU

| Activity | Contribution |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AD | AD-FF | AC | EU | Ecotox, FW | CC | HT | Ecotox, MA | ODP | PO | Ecotox, Tr |
|  | (kg Sb eq) | (MJ) | (kg SO2 eq) | (kg PO4 eq) | (kg 1,4-DB eq) | (kg CO2 eq) | (kg 1,4-DB eq) | (kg 1,4-DB eq) | (kg CFC-11 eq) | (kg C2H4 eq) | (kg 1,4-DB eq) |
| Coal mining | 2.50E-08 | 1.79E-01 | 1.60E-04 | 8.41E-05 | 1.41E-02 | 3.09E-01 | 1.16E-02 | 4.60E+01 | 1.63E-10 | 6.22E-05 | 3.76E-05 |
| Coal coking | 6.29E-07 | 5.45E+00 | 1.14E-03 | 6.33E-04 | 1.18E-01 | 1.64E-01 | 3.39E-01 | 2.87E+02 | 3.56E-08 | 1.37E-04 | 1.96E-04 |
| Oil mining | 3.18E-07 | 4.27E+01 | 6.61E-04 | 1.99E-04 | 4.76E-02 | 2.29E-01 | 6.90E-02 | 1.05E+02 | 5.79E-09 | 3.95E-05 | 4.77E-04 |
| Oil refining | 2.10E-07 | 1.36E+00 | 2.05E-03 | 3.20E-04 | 2.40E-02 | 1.32E-01 | 1.00E-01 | 7.85E+01 | 1.40E-08 | 6.57E-05 | 3.43E-04 |
| Calcination | 2.68E-07 | 2.80E+00 | 1.14E-03 | 1.53E-04 | 4.74E-02 | 9.67E-01 | 5.51E-02 | 1.85E+02 | 2.42E-08 | 4.98E-05 | 2.52E-04 |
| Baking | 1.54E-07 | 3.96E+00 | 7.30E-04 | 2.10E-04 | 5.42E-02 | 1.55E-01 | 9.93E-02 | 2.46E+02 | 1.39E-08 | 4.36E-05 | 2.26E-04 |
| Graphitization | 7.95E-06 | 7.57E+01 | 3.78E-02 | 8.29E-03 | 2.36E+00 | 8.49E+00 | 2.95E+00 | 1.34E+04 | 4.03E-08 | 1.40E-03 | 1.37E-02 |

## Contribution analysis for climate change impact

### Contribution analysis for climate change impact based on activity

| Activity | CC (kg CO2 eq) |
| --- | --- |
| Coal mining | 0.309 |
| Coal coking | 0.164 |
| Oil mining | 0.229 |
| Oil refining | 0.132 |
| Calcination | 0.967 |
| Baking | 0.155 |
| Graphitization | 8.487 |

### Contribution analysis for climate change impact based on flows

| Activity | CC (kg CO2 eq) |
| --- | --- |
| Electricity | 8.784 |
| Direct emission | 1.084 |
| Waste treatment | 0.142 |
| Natural gas (incl. sweet gas) | 0.101 |
| Transportation | 0.098 |
| Coal coking gas | 0.095 |
| Others | 0.083 |
| Lime | 0.037 |
| Steam | 0.017 |

### Scenario analysis

| Activity | CC (kg CO2 eq) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CN, Baseline | CN, mass-alloc. | DE | US | JP | KR | FI |
| Coal mining | 0.309 | 0.150 | 0.309 | 0.309 | 0.309 | 0.309 | 0.309 |
| Coal coking | 0.164 | 0.079 | 0.202 | 0.222 | 0.148 | 0.146 | 0.216 |
| Oil mining | 0.229 | 0.415 | 0.229 | 0.229 | 0.229 | 0.229 | 0.229 |
| Oil refining | 0.132 | 0.238 | 0.072 | 0.061 | 0.097 | 0.099 | 0.067 |
| Calcination | 0.967 | 0.620 | 0.918 | 0.920 | 0.933 | 0.936 | 0.889 |
| Baking | 0.155 | 0.154 | 0.111 | 0.107 | 0.141 | 0.116 | 0.086 |
| Graphitization | 8.487 | 8.487 | 4.650 | 4.49 | 5.564 | 5.820 | 2.140 |

### Sensitivity ratio

| Activity | Sensitivity ratio |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Electricity | Direct emission | Transport | Natural gas | Waste |
| Coal mining | 0.0558 | 0.9357 | 0.0000 | 0.0000 | 0.0003 |
| Coal coking | 0.1092 | 0.0000 | 0.1108 | 0.0000 | 0.0003 |
| Oil mining | 0.0573 | 0.3861 | 0.0000 | 0.3662 | 0.0602 |
| Oil refining | 0.2094 | 0.1659 | 0.6083 | 0.0003 | 0.0019 |
| Calcination | 0.1020 | 0.7226 | 0.0000 | 0.0000 | 0.1322 |
| Baking | 0.7951 | 0.0000 | 0.0000 | 0.1086 | 0.0000 |
| Graphitization | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |

References

Abdalla, A., 2024. Sulfur price in China [WWW Document]. URL https://sulfur-price.com/today/china (accessed 7.26.24).

Askari, M., Aliofkhazraei, M., Jafari, R., Hamghalam, P., Hajizadeh, A., 2021. Downhole corrosion inhibitors for oil and gas production – a review. Appl. Surf. Sci. Adv. 6, 100128. https://doi.org/10.1016/j.apsadv.2021.100128

Banerjee, C., Agarwal, S., Dash, P.S., Roy, A., 2021. Effect of wash oil inlet temperature in naphthalene scrubber on the absorptivity of naphthalene in coke oven by product plant. Therm. Sci. Eng. Prog. 25, 101025. https://doi.org/10.1016/j.tsep.2021.101025

Ecoinvent, 2021. Ecoinvent v3.8 [WWW Document]. URL https://ecoinvent.org/the-ecoinvent-database/data-releases/ecoinvent-3-8/ (accessed 1.26.23).

Edwards, L., Hunt, M., Verma, P., Weyell, P., Koop, J., 2020. Sustainable CPC Production at the Vizag Calciner. pp. 16–18.

Eser, S., 2021. Fluid Catalytic Cracking (FCC) [WWW Document]. URL https://www.e-education.psu.edu/fsc432/content/fluid-catalytic-cracking-fcc (accessed 7.29.24).

Lan, Y., Zhao, X., Zhang, W., Mu, L., Wang, S., 2022. Investigation of the waste heat recovery and pollutant emission reduction potential in graphitization furnace. Energy 245. https://doi.org/10.1016/j.energy.2022.123292

Li, J., Zhang, S., Nie, Y., Ma, X., Xu, L., Wu, L., 2020. A holistic life cycle evaluation of coking production covering coke oven gas purification process based on the subdivision method. J. Clean. Prod. 248. https://doi.org/10.1016/j.jclepro.2019.119183

Liu, Y., Lu, S., Yan, X., Gao, S., Cui, X., Cui, Z., 2020. Life cycle assessment of petroleum refining process: A case study in China. J. Clean. Prod. 256, 120422. https://doi.org/10.1016/J.JCLEPRO.2020.120422

Meili, C., Jungbluth, N., Bussa, M., 2021. Life cycle inventories of crude oil and natural gas extraction Life cycle inventories of crude oil and natural gas extraction R. Schaffhausen. https://doi.org/10.13140/RG.2.2.29142.78409

Mysteel, 2023. Needle coke price wanes on weaker downstream demand [WWW Document]. URL https://www.mysteel.net/news/5045743-needle-coke-price-wanes-on-weaker-downstream-demand (accessed 7.26.24).

National Bureau of Statistics of China, 2023. Market Prices of Important Means of Production in Circulation, November 11-20, 2023 [WWW Document]. URL https://www.stats.gov.cn/english/PressRelease/202311/t20231129_1944997.html (accessed 7.26.24).

NCCP, 2017. Zeolite catalysts | NCCP [WWW Document]. URL http://www.nccp.ru/en/products/zeolite_catalysts/?PAGEN_1=3 (accessed 6.26.24).

Procurement Resource, 2024. Procurement Resource [WWW Document]. URL https://www.procurementresource.com/ (accessed 7.26.24).

Shang, T., Zhan, H., Gong, Q., Zeng, T., Li, P., Zeng, Z., 2024. Insights into the thermal and electric field distribution and the structural optimization in the graphitization furnace. Energy 297, 131269. https://doi.org/10.1016/j.energy.2024.131269

Shanghai Metals Market, 2023. Prices of Prebaked Anode Unlikely To Stop Falling In Short Term amid Weak Raw Material Market [WWW Document]. URL https://news.metal.com/newscontent/102216471/Prices-of-Prebaked-Anode-Unlikely-To-Stop-Falling-In-Short-Term-amid-Weak-Raw-Material-Market (accessed 7.26.24).

Statista, 2023. Average electricity prices for enterprises in China from September 2019 to September 2023 [WWW Document]. URL https://www.statista.com/statistics/1373596/business-electricity-price-china/ (accessed 7.26.24).

SunSirs, 2024a. SunSirs: China Coking Coal Price Fell and Rose in 2023, or May Fluctuate in 2024 [WWW Document]. URL https://www.sunsirs.com/uk/detail_news-16705.html (accessed 7.26.24).

SunSirs, 2024b. SunSirs: Analysis of Naphtha Trends in 2023 and Future Prospects [WWW Document]. URL https://www.sunsirs.com/uk/detail_news-16958.html (accessed 7.26.24).

SunSirs, 2023a. SunSirs: It is Expected that There is Currently Room for a Decline in Coal Tar Prices [WWW Document]. URL https://www.sunsirs.com/uk/detail_news-14267.html (accessed 7.26.24).

SunSirs, 2023b. SunSirs: The Ammonium Sulfate Market Stopped Falling and Rose [WWW Document]. URL https://www.sunsirs.com/uk/detail_news-15968.html (accessed 7.26.24).

Surovtseva, D., Crossin, E., Pell, R., Stamford, L., 2022. Toward a life cycle inventory for graphite production. J. Ind. Ecol. 26, 964–979. https://doi.org/10.1111/jiec.13234

Tang, Y., Li, Y., Shi, Y., Wang, Q., Yuan, X., Zuo, J., 2018. Environmental and economic impacts assessment of prebaked anode production process: A case study in Shandong Province, China. J. Clean. Prod. 196, 1657–1668. https://doi.org/10.1016/J.JCLEPRO.2018.06.121

Tao, M., Cheng, W., Nie, K., Zhang, X., Cao, W., 2022. Life cycle assessment of underground coal mining in China. Sci. Total Environ. 805. https://doi.org/10.1016/j.scitotenv.2021.150231

Whattoff, P., 2022. Prospective life cycle assessment study of Epsilon Carbon Ltd synthetic graphite anode manufacturing.