Evaluating the multifunctional resilience of river health to human service demand: A comparison among fuzzy logic, entropy and AHP based MCDM models

Raj Kumar Bhattacharya1*, Nilanjana Das Chatterjee1, Kousik Das1

Department of Geography, Vidyasagar University, West Bengal, India

*rajgeovu10@gmail.com

Suppl. materials

Assessment indicator system

Determination of assessment criteria has more significant role to find out the influence intensity on river health (Deng et al. 2015). However, human demand changed the assessment criteria day by day, and no one consistent standard has been properly setup (Karr, 1999). Several typical methods include field surveys, ground truth verification, comparative analyses, reference system for relevant research results and national standards are used to determine the river health assessment (Zhao and Yang 2009; Sadat et al. 2019). With the assignment of different criteria assessment, threshold ranges of physical, ecological, social indicators are easily determined amongst different river sites. In this study, standard evaluation methods, expert consultation method, surface water quality index (WHO 2011), overall pollution index (Yang et al. 2018) and biodiversity indices are used to find out the threshold limit in each indicators, which may help to classify their range i.e. very well, well, critical, poor and very poor, respectively.

Human activity index in riparian zone (A1)

Trends of several human development activities including fishing, sand mining, road and bridge construction, vegetation depletion, domestic waste stacking along the riparian sites are considered as human activity intensity (Yang et al. 2018). Very poor activity index is regarded where losses of biotic habitat diversity, degradation of structure and function of river ecosystem, and cut off the connectivity of river channel-floodplain are mostly occurred. While, very well activity index is considered when all of those human development activities have not found up to 25m-50m width along the left and right riparian sites (Zheng et al. 2007; Ding et al. 2015; Yang et al. 2017).

Riparian vegetation buffer width (A2)

With the following of vegetation protection condition, large volume of riparian vegetation coverage not only initiated the habitat potentiality for organisms but also favourable for biological diversity formation to conserve the vegetation water as well as protect the bank erosion (Yang et al. 2018). Human activities can threats as more hindrances for disturbing the native species in the riparian vegetation zone where production modes are directly reflected by various land use types. In Kangsabati River, sand mining, agricultural practice in riparian zone causes of adverse effects for changing the vegetation composition and structure; hence intensity of erosion becomes highly increases (Bhattacharya et al. 2019a). Consequently, sediment supply from delivery outlets to channel is gradually amplified (Bhattacharya et al., 2020a).

Bank erosion degree (A3)

Degradation of river habitat, estuary silting and riverbed emerging are occurred by prime ecological factors such as bank erosion, soil erosion and water loss (Yang et al. 2018).  Bank erosion is one of them to degrade the riparian habitat and biodiversity. Maximum bank erosion and shifting in Kangsabati River is progressively grown up near the higher bank slope along the upper course but it become accelerated in sand mining sites along the middle and lower course. Contrastingly, sediment accumulation across the riparian zone can restrict the bank erosion, as a result, minimum erosion is observed in sandbar sites but erosion expanded in mining and pit sites. Assessment level is inversely related between bank erosion and river health connotation that means very well level is assigned as <0.20 and very poor assigned as >0.80 ( Zheng et al. 2007; Yang et al. 2017).

Water flow condition (B1)

Water flow is most effective indicators for preserving the basic ecological process of key species along with maintain the entire river ecosystem structure and function i.e. biotic and energy exchange, biotic spawn, sediment cycle, and pollution dilution (Song et al. 2007; Yang et al. 2013a; Yang et al. 2018). In this study, flow condition for river health connotation is determined through the corresponding criteria of Ganga River, India (Dutta et al. 2017). Moreover, annual water yield including flow regime is greatly influenced on habitat complexity, biotic diversity and lateral connectivity of river ecosystem.

Mean flow velocity (B2)

Flow velocity is an important physiographic indicator, which has been controlled to develop of particular habitat potentiality in the every river segment level (Dutta et al. 2017). In particular upstream movement, high flow velocity can significantly disturbed the biological community; hence river habitat changed as red zones caused by suddenly increases the water discharge. Contrastingly, low to moderate flow velocity in deep pool along the middle and downstream, which is supported the maximum diversity as well as provided the health connotation. In Kangsabati River, maximum flow velocity is observed along the upper course during monsoonal flow while minimum flow velocity occurred along the middle course during post monsoon season (Mittal et al. 2014).

Substrate compositions (C1)

River substrate characteristic and structure are denoted as substrate composition to provide the diverse habitat complexity especially aquatic organisms through the sheltering for benthonic organisms and offered the vast spawn site for fish species (Dutta et al. 2017; Yang et al. 2018). Based on particle size, substrate is to be classified into four classes i.e. boulders (ø>60 mm), gravel (ø=3-60 mm), sand (ø= 3-0.25 mm), mud (ø<0.25 mm), respectively (Kristensen et al. 2011; Wang et al. 2019). In addition, hard clay and peat both are included under substrate classification; however grain size of those particles has very finer.

Channel average width (C2)

Channel width is determined by quantitative analysis of hydraulic geometry to use for identifying the morphological attributes of river segments namely, routine of sediment and flow regime, physical habitats, interaction between channel and floodplain (Stewardson 2005; Yang et al. 2013b). Wider channel always provided the extensive physical habitat potentiality than narrow channel (Maddock 1999). In Kangsabati River, channel width including riparian zone has more extensive along the middle course segments but width become narrowing along the upper and lower segments (Bhattacharya et al. 2019a). Riparian vegetation buffer has progressively grown up with the channel width throughout the course.

Channel average depth (C3)

Water depth in open channel, shallow water and deep pools sustained primary habitat potentiality of river ecosystem, and successively contributed to habitat complexity including maximum diversity (Dutta et al. 2017). Maximum aquatic species diversity including fish community has observed in more deep pools with moderate to low water diversity of alluvial channel while water depth (<1m) has negatively affected on species diversity. After the construction of Mukutmonipur dam (1956), channel depth in Kangsabati River has been drastically reduces; hence minimum flow discharge is now more dominated over the season (Mittal et al. 2014). Recently, massive sand mining in this alluvial river creates numerous pit pools that help to survive the ecological richness, and greatly influences on flora and fauna composition (Bhattacharya et al. 2019b).

Habitat complexity (D5)

Habitat complexity indicates characteristic and composition of river habitat along with microhabitat and biotic habitat association that is grown up in gravel river bed, shrinking hole across the bank margin, pool, riffle, dry branches, embankment, natural leaves, and hydrophytes (Yang et al. 2018; Wang et al. 2019). Habitat complexity has negative impact on species competition in one hand and can more influence the suitable space for all living species in other hand. Based on substrate composition, sand and mud composed habitat has zero complexity while gravel composed habitat has maximum complexity. In this study, maximum habitat complexity concentrated in coarse grain river bed along the upper course but complexity has been declined in finer grain bed along the lower course, respectively. Human activity (E1)

Human activities have great impact on river health through the controlling of river ecosystem likes altering of water flow, degradation of river habitat and biotic condition (Aazami et al. 2015). Various anthropogenic activities including riverine land cover can changes the hydrological systems, stream water quality, structure of flora and fauna assemblages in the entire course (Lee et al. 2009; Ai et al. 2015; Shi et al. 2017; Cheng et al. 2018). In Kangsabati River, sand mining, channelization, urban expand in floodplain and bank margin, and inverse riverine land use change are declined the river health over the years (Bhattacharya et al. 2019a). Moreover, huge human service demand crossed the resilience capacity of river ecosystem integrity.

Groundwater depletion level (E2)

Groundwater hydrology and ecology has great impact on river ecosystem function; however over exploitation and surface water intrusion has been depleted the groundwater level since last decade (Boulton 2000). In addition, most of the exchange is occurred in hyporehic zone situated in the bottom layer of saturated sediments and the river bank, which zone alters the water flow, microbial and chemistry processes through the transforming the decomposing organic matter, consuming oxygen, nutrients, and other activities (Boulton 1993). Over exploitation of groundwater can hampered the hyporehic zone, algal growth and invertebrate composition. In this study area, groundwater health has been more depleted caused by over water utilization and presence of resistant for recharge in the dense settlement sites, crop land sites across the river bank (Bhattacharya et al. 2020b). Land use change and land cover alteration has negative impact on groundwater potentiality, which causes the degradation of hyporehic zone as well as river health.

Developmental work (E3)

Dynamic process has been identified in the entire evolution of river ecosystem, which is observed as regular process for achieving a dynamic balance between river ecosystem protection and human development (Luo et al. 2018). Socioeconomic development is to be a key hindrance for river health, and raises a contradiction between river ecosystem protection and human activities. With the following of human service demand, long-term harmonious relationship depicted the generation of several problems from the ongoing human development and river health. In Kangsabati River, construction of Mukutmonipur dam and Anicut dam, bridge construction, weirs, rapid industrial development and urban pollution caused the gradually deteriorating of river ecosystem and basin water environment (Das et al. 2020).

Assessment index system

Water quality (B3) and pollution level (B4)

Chemical, biological and physical components interaction helps to comprehensively determine the various attributes like water quality (Yang et al., 2018). Physicochemical parameters (PP) i.e. salinity, dissolved oxygen (DO), electric conductivity (EC), total dissolved solids (TDS), PH along with major cations such as Mg2+ and anions such as NO3- controlled the composition of surface and groundwater water quality (Sharma et al., 2014; Bhattacharya et al. 2019c). Moreover, PPs like DO, PH, NH3-N are used to measure the pollution indices to find out the intensity of pollution level along with indentify the maximum major pollutants (Yang et al. 2018). Substrate quality in water can change the habitat suitability of biological community (Kuriqi 2014), bank structure, riparian stability, and channel morphological responses (Bayram and Onsoy 2015). However, overgrowing anthropogenic activities have negative impacts on water quality, and expanded the consequences to river ecosystem. Sand mining is a big challenging problem to deteriorate the water quality in Kangsabati River; hence habitat suitability gradually degraded in particularly middle and lower course (Bhattacharya et al. 2020c). Therefore, it is point that polluted water composition has negative impact on river health throughout the course.

River meandering degree (C4)

Meandering degree reflects natural curvature of rivers as well as flow continuity of different structure and functions; however meandering has been intensively modified caused by navigation, flooding, over agricultural irrigation, and straightening construction for railways and roads (Yang et al. 2018). On the other hand, channelization has been interrupted the natural flow pattern in stream since past decades. Meandering does not only demonstrated the channel planform change but also reveals the entire morphological responses namely, sediment transition, biotic and energy exchange, longitudinal connectivity, and aquatic habitat diversity. Meandering degree is measured with the taking of two points from channel the bed along the straight and middle lines. In this study, maximum meandering degree is concentrated along the middle and lower course than upper course (Bhattacharya et al. 2019a). Moreover, intensive sand mining can inversely changes of meandering degree to interrupt the erosion and deposition sequences.

Biodiversity index (D1, 2, 3, 4, 6)

River ecosystem health is effectively determined using of biological indices (Meng et al. 2009), in particular aquatic organisms i.e. micro invertebrates (Smith et al., 1999), algae (Poikane et al. 2016), fish (Karr, 1981), plankton (Reynolds 2003) are considered as crucial indicators for accessing or evaluating the health connotation (Cheng et al. 2018). Micro invertebrates are important biological indicators due to providing their sedentary behaviour, ubiquitous nature, quick sampling and identification, and long life cycle (Birk et al. 2012; Shan et al. 2016). According to Bhattacharya et al (2019c), species diversity and richness in aquatic community has been deteriorated caused by massive instream sand mining in Kangsabati River. Maximum habitat degradation is occurred in mining sites than natural sandbar.

Multi-criteria decision making models

VIKOR method

Decision matrix is prime step of VIKOR method, and then best and worst values availability of each indicator is determined from decision matrix table using Eqs. (1-4)

In the case of criterion is positive

fi*=jmaxfij and fi-=jminfij                        1

In the case of benchmark is negative

fi*=jminfij and fi-=jmaxfij                        2

Where, fi* denotes the best values, and fi- denotes the worst values

This step is comprises to calculate the maximum group utility values (Si) and minimum individual loneliness values from the opposite group side (R) following respective Eqs. (3) and (4)

Sj=i=1nWi(fi*-fij)(fi*-fi-)                                                    (3)

Rj=maxi=1nWi(fi*-fij)(fi*-fi-)                                      (4)

In this relation, Wi stated the ith criterion weight obtaining from relative importance of criteria. Then, advantage function or hybrid function value of Q is determined by converge the function of S and R equations including each weight.

SAW method

This method is measured rating option using five consecutive steps i.e. (Bhattacharya et al., 2020a)

Step 1: Normalized initial matrix (Pij) of jth criterion is calculated based on decision matrix as follow Eq. (5)

Pij=MijbaMij                                                                    (5)

Where Mij denotes initial weights, and a indicates criteria number

Step 2: This step is comprises for determining the relationship as Pij = Pij max or Pij= Pij min

Normalized weight is used to obtain these two different relations, where maximum value in efficient index denotes Pij = Pij max, and minimum value in efficient index indicates Pij= Pij min

Step 3: In this context, consistency index is crucial tool to estimate the normalize value given by Eq. (5.1) for maximum efficiency index and Eq. (5.2) for minimum efficiency index

Pij=MijPijmax                                                                       (5.1)

Pij=MijminMij                                                               (5.2)

Step 4: This step is involved to measure the weighted normalized decision matrix (Qij) obtaining by Eq. (6)

Qij=Pij×Wj                                           (6)

Where, Wj indicates weight of Analytical Hierarchical Process (AHP) model

COPRAS method

COPRAS method is expressed step by step as below (Podvezko, 2011):

Step 1: This step is comprises to establish the decision matrix

Step 2: Second step is involved to make the normalization of decision matrix given by Eq. (7)

Gij=giji=1mgij      i=1, 2, …, n;j=1, 2, …, m                                                   (7)

Where, Gij express the normalized value of jth criterion; gij denotes ith alternative performance according to jth criterion; m means alternative numbers

Step 3: To compute the normalized weighted matrix obtaining by Eq. (8)

hij=Wj×Gij      i=1, 2, …, n;j=1, 2, …, m                                        8

Where, Wj represent criterion weight; Gij stated efficiency of ith alternative

Step 4: This step is calculated the maximum and minimum indices for alternatives using sum of the weighted normalized value

Sj+=j=1ny+ij j=1, 2, 3, .., n;                                                     (9)

Sj-=j=1ny-ij j=k+1, k+2, , .., n;                                                    (10)

WASPAS method

WASPAS calculation is analyzed as below (Zolfani et al., 2013)

Step 1: In case of optimum value as maxi, normalizing decision making matrix is determined according to Eq. (11)

xij=xij1optxij     i=1, m;  j=1, n                                              (11)

In case of optimum value is min:

xij=1optxijxij         i=1, m;  j=1, n                                         (12)

Step 2: To calculating WASPAS weighted and normalized decision making matrix for summing up part:

xij sum=xijqj  i=1, m;  j=1, n                                      (13)

Step 3: To calculating WASPAS weighted and normalized decision making matrix for multiplication part:

xij mult=xijaj  i=1, m;  j=1, n                                      14

References

Aazami J, Esmaili-Sari, A., Abdoli, A., Sohrabi, H. and Van den Brink, P.J., (2015) Monitoring and assessment of water health quality in the Tajan River, Iran using physicochemical, fish and macroinvertebrates indices. Journal of Environmental Health Science and Engineering, 13(1), pp.1-12.

Ai, L., Shi, Z.H., Yin, W. and Huang, X., (2015) Spatial and seasonal patterns in stream water contamination across mountainous watersheds: Linkage with landscape characteristics. Journal of Hydrology, 523, pp.398-408.

Bayram, A. and Önsoy, H., (2015) Sand and gravel mining impact on the surface water quality: a case study from the city of Tirebolu (Giresun Province, NE Turkey). Environmental earth sciences, 73(5), pp.1997-2011

Bhattacharya, R. K., Chatterjee, N. D., & Das, K (2020a) Estimation of erosion susceptibility and sediment yield in ephemeral channel using RUSLE and SDR model: Tropical plateau fringe region, India. In Gully erosion studies from India and surrounding regions (pp. 163-185). Springer, Cham.

Bhattacharya, R.K., Chatterjee, N.D. and Das, K. (2020b) An integrated GIS approach to analyze the impact of land use change and land cover alteration on ground water potential level: A study in Kangsabati Basin, India. Groundwater for Sustainable Development, 11, p.100399.

Bhattacharya, R.K., Chatterjee, N.D. and Das, K (2020c) Impact of instream sand mining on habitat destruction or transformation using coupling models of HSI and MLR. Spatial Information Research, 28(1), pp.67-85.

Bhattacharya, R.K., Chatterjee, N.D. and Das, K (2019a) Geomorphic response to riverine land cover dynamics in a quarried alluvial river Kangsabati, South Bengal, India. Environmental Earth Sciences, 78(22), pp.1-18.

Bhattacharya, R.K., Chatterjee, N.D. and Dolui, G. (2019b) Consequences of sand mining on water quality and instream biota in alluvial stream: a case-specific study in South Bengal River, India. Sustainable Water Resources Management, 5(4), pp.1815-1832.

Birk, S., Bonne, W., Borja, A., Brucet, S., Courrat, A., Poikane, S., Solimini, A., Van De Bund, W., Zampoukas, N. and Hering, D (2012) Three hundred ways to assess Europe's surface waters: an almost complete overview of biological methods to implement the Water Framework Directive. Ecological indicators, 18, pp.31-41

Boulton, A.J (1993) Stream ecology and surface-hyporheic hydrologic exchange: implications, techniques and limitations. Marine and Freshwater Research, 44(4), pp.553-564.

Boulton, A.J (2000) River ecosystem health down under: assessing ecological condition in riverine groundwater zones in Australia. Ecosystem Health, 6(2), pp.108-118.

Cheng, X., Chen, L., Sun, R. and Kong, P (2018) Land use changes and socio-economic development strongly deteriorate river ecosystem health in one of the largest basins in C hina. Science of the Total Environment, 616, pp.376-385.

Das, B.C., Ghosh, S., Islam, A. and Roy, S. Eds (2020) Anthropogeomorphology of Bhagirathi-Hooghly River System in India. CRC Press.

Ding, Y., Shan, B. and Zhao, Y (2015) Assessment of river habitat quality in the hai river basin, northern China. International journal of environmental research and public health, 12(9), pp.11699-11717

Dutta, V., Sharma, U. and Kumar, R (2017) Assessment of river ecosystems and environmental flows: Role of flow regimes and physical habitat variables. Climate Change and Environmental Sustainability, 5(1), pp.20-34

Karr, J.R (1981) Assessment of biotic integrity using fish communities. Fisheries, 6(6), pp.21-27.

Karr, J.R (1999) Defining and measuring river health. Freshw. Biol. 41, 221–234.

Kristensen, E.A., Baattrup-Pedersen, A. and Thodsen, H (2011) An evaluation of restoration practises in lowland streams: Has the physical integrity been re-created?. Ecological Engineering, 37(11), pp.1654-1660.

Kuriqi, A (2014) Simulink application on dynamic modeling of biological waste water treatment for aerator tank case. Int. J. Sci. Technol. Res, 3(11), pp.69-72

Lee, S.W., Hwang, S.J., Lee, S.B., Hwang, H.S. and Sung, H.C (2009) Landscape ecological approach to the relationships of land use patterns in watersheds to water quality characteristics. Landscape and Urban Planning, 92(2), pp.80-89

Luo, Z., Zuo, Q. and Shao, Q (2018) A new framework for assessing river ecosystem health with consideration of human service demand. Science of the Total Environment, 640, pp.442-453

Maddock, I (1999) The importance of physical habitat assessment for evaluating river health. Freshwater biology, 41(2), pp.373-391

Mittal, N., Mishra, A., Singh, R., Bhave, A.G. and van der Valk, M (2014) Flow regime alteration due to anthropogenic and climatic changes in the Kangsabati River, India. Ecohydrology & Hydrobiology, 14(3), pp.182-191

Podvezko, V (2011) The comparative analysis of MCDA methods SAW and COPRAS. Inzinerine Ekonomika-Engineering Economics 22, 134–146

Poikane, S., Kelly, M. and Cantonati, M (2016) Benthic algal assessment of ecological status in European lakes and rivers: Challenges and opportunities. Science of the Total Environment, 568, pp.603-613

Reynolds, C.S (2003) Planktic community assembly in flowing water and the ecosystem health of rivers. Ecological Modelling, 160(3), pp.191-203

Sadat, M.A., Guan, Y., Zhang, D., Shao, G., Cheng, X. and Yang, Y (2020) The associations between river health and water resources management lead to the assessment of river state. Ecological Indicators, 109, p.105814

Shan, B., Ding, Y. and Zhao, Y (2016) Development and preliminary application of a method to assess river ecological status in the Hai River Basin, north China. Journal of Environmental Sciences, 39, pp.144-154

Sharma, P., Meher, P.K., Kumar, A., Gautam, Y.P. and Mishra, K.P (2014) Changes in water quality index of Ganges river at different locations in Allahabad. Sustainability of Water Quality and Ecology, 3, pp.67-76

Shi, P., Zhang, Y., Li, Z., Li, P. and Xu, G (2017) Influence of land use and land cover patterns on seasonal water quality at multi-spatial scales. Catena, 151, pp.182-190

Smith, M.J., Kay, W.R., Edward, D.H.D., Papas, P.J., Richardson, K.S.J., Simpson, J.C., Pinder, A.M., Cale, D.J., Horwitz, P.H., Davis, J.A. and Yung, F.H (1999) AusRivAS: using macroinvertebrates to assess ecological condition of rivers in Western Australia. Freshwater Biology, 41(2), pp.269-282

Song, J.X., Xu, Z.X., Liu, C.M. and Li, H.E (2007) Ecological and environmental instream flow requirements for the Wei River—the largest tributary of the Yellow River. Hydrological Processes: An International Journal, 21(8), pp.1066-1073

Stewardson, M (2005) Hydraulic geometry of stream reaches. Journal of hydrology, 306(1-4), pp.97-111

Wang, S., Zhang, Q., Yang, T., Zhang, L., Li, X. and Chen, J (2019) River health assessment: Proposing a comprehensive model based on physical habitat, chemical condition and biotic structure. Ecological indicators, 103, pp.446-460

Wei, M., Zhang, N., Zhang, Y. and Zheng, B (2009) Integrated assessment of river health based on water quality, aquatic life and physical habitat. Journal of Environmental Sciences, 21(8), pp.1017-1027

Yang, T., Liu, J. and Chen, Q (2013) Assessment of plain river ecosystem function based on improved gray system model and analytic hierarchy process for the Fuyang River, Haihe River Basin, China. Ecological modelling, 268, pp.37-47

Yang, T., Liu, J., Chen, Q., Zhang, J. and Yang, Y (2013) Estimation of environmental flow requirements for the river ecosystem in the Haihe River Basin, China. Water science and technology, 67(4), pp.699-707

Yang, T., Liu, J., Li, X. and Zhao, X (2017) Plain river habitat assessment of Haihe River basin. Environmental Science & Technology (China), 40(3), pp.190-197

Yang T., Wang, S., Li, X., Wu, T., Li, L. and Chen, J (2018) River habitat assessment for ecological restoration of Wei River Basin, China. Environmental Science and Pollution Research, 25(17), pp.17077-17090

Zhao, Y.W., Yang, Z.F (2009) Integrative fuzzy hierarchical model for river health assessment: a case study of Yong River in Ningbo City, China. Commun. Nonlinear Sci. Numer. Simul. 14, 1729–1736. https://doi.org/10.1016/j.cnsns.2007.09.019

Zheng, B.H., Zhang, Y. and Li, Y.B (2007) Study of indicators and methods for river habitat assessment of Liao River Basin. Acta Scientiae Circumstantiae, 27(6), pp.928-936

Zolfani, S.H., Aghdaie, M.H., Derakhti, A., Zavadskas, E.K. and Varzandeh, M.H.M (2013) Decision making on business issues with foresight perspective; an application of new hybrid MCDM model in shopping mall locating. Expert systems with applications, 40(17), pp.7111-7121

Supplementary Table

Table 1    River health prioritization at segment level using fuzzy AHP

|  | CD | AB | EF | GH | IJ | KL | MN | OP | QR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 0.107 | 0.016 | 0.221 | 0.240 | 0.002 | 0.179 | 0.126 | 0.091 | 0.016 |
| A2 | 0.163 | 0.217 | 0.000 | 0.033 | 0.044 | 0.089 | 0.074 | 0.132 | 0.249 |
| A3 | 0.226 | 0.087 | 0.102 | 0.198 | 0.087 | 0.102 | 0.094 | 0.053 | 0.050 |
| B1 | 0.042 | 0.000 | 0.341 | 0.000 | 0.079 | 0.411 | 0.000 | 0.000 | 0.128 |
| B2 | 0.277 | 0.165 | 0.250 | 0.198 | 0.000 | 0.110 | 0.000 | 0.000 | 0.000 |
| B3 | 0.136 | 0.278 | 0.029 | 0.000 | 0.000 | 0.014 | 0.157 | 0.278 | 0.109 |
| B4 | 0.104 | 0.127 | 0.164 | 0.197 | 0.152 | 0.000 | 0.050 | 0.052 | 0.153 |
| C1 | 0.197 | 0.123 | 0.211 | 0.095 | 0.000 | 0.123 | 0.089 | 0.059 | 0.103 |
| C2 | 0.182 | 0.058 | 0.000 | 0.333 | 0.164 | 0.000 | 0.164 | 0.070 | 0.028 |
| C3 | 0.147 | 0.147 | 0.050 | 0.051 | 0.143 | 0.008 | 0.228 | 0.115 | 0.111 |
| C4 | 0.083 | 0.000 | 0.137 | 0.224 | 0.224 | 0.137 | 0.137 | 0.000 | 0.060 |
| D1 | 0.160 | 0.153 | 0.151 | 0.032 | 0.168 | 0.168 | 0.000 | 0.126 | 0.042 |
| D2 | 0.083 | 0.225 | 0.177 | 0.000 | 0.167 | 0.167 | 0.000 | 0.097 | 0.083 |
| D3 | 0.203 | 0.067 | 0.145 | 0.067 | 0.203 | 0.192 | 0.000 | 0.124 | 0.000 |
| D4 | 0.000 | 0.106 | 0.214 | 0.113 | 0.335 | 0.186 | 0.000 | 0.045 | 0.000 |
| D5 | 0.009 | 0.123 | 0.000 | 0.077 | 0.182 | 0.000 | 0.225 | 0.301 | 0.083 |
| D6 | 0.043 | 0.215 | 0.138 | 0.028 | 0.185 | 0.097 | 0.168 | 0.097 | 0.028 |
| E1 | 0.029 | 0.029 | 0.029 | 0.122 | 0.122 | 0.122 | 0.192 | 0.174 | 0.181 |
| E2 | 0.011 | 0.005 | 0.032 | 0.203 | 0.203 | 0.191 | 0.134 | 0.111 | 0.111 |
| E3 | 0.000 | 0.000 | 0.000 | 0.082 | 0.082 | 0.082 | 0.255 | 0.234 | 0.265 |
| Final score | 0.110 | 0.107 | 0.120 | 0.115 | 0.127 | 0.119 | 0.105 | 0.108 | 0.090 |
| Rank | 5 | 7 | 2 | 4 | 1 | 3 | 8 | 6 | 9 |

Table 2    River health prioritization at segment level using fuzzy TOPSIS

| Sections | Si+ | Si- | Ci | Priority rank |
| --- | --- | --- | --- | --- |
| CD | 7.541251 | 7.6453721 | 0.503428051 | 5 |
| AB | 8.8305352 | 6.603555 | 0.42785515 | 7 |
| EF | 6.7214201 | 8.7894217 | 0.566663103 | 2 |
| GH | 8.9325634 | 6.489291 | 0.42078539 | 8 |
| IJ | -2.194326 | 7.4559357 | 1.417044661 | 1 |
| KL | 7.1278033 | 8.3792201 | 0.54035 | 3 |
| MN | 7.5490224 | 7.526274 | 0.499245509 | 6 |
| OP | 7.3666248 | 8.2132943 | 0.52717182 | 4 |
| QR | 9.5295853 | 5.8309137 | 0.379604443 | 9 |

Table 3    River health prioritization at segment level using entropy VIKOR

| Sections | Si | Ri | Qi | Priority rank |
| --- | --- | --- | --- | --- |
| CD | 0.424824 | 0.044123 | 0.033182 | 1 |
| AB | 0.404499 | 0.054809 | 0.113933 | 2 |
| EF | 0.424064 | 0.05673 | 0.166363 | 3 |
| GH | 0.556037 | 0.069548 | 0.518499 | 6 |
| IJ | 0.535001 | 0.069548 | 0.484156 | 5 |
| KL | 0.453462 | 0.069548 | 0.351035 | 4 |
| MN | 0.710758 | 0.091016 | 1 | 9 |
| OP | 0.592941 | 0.091016 | 0.807651 | 7 |
| QR | 0.678103 | 0.091016 | 0.946686 | 8 |

Table 4    River health prioritization at segment level using AHP VIKOR

| Sections | Si | Ri | Qi | Priority rank |
| --- | --- | --- | --- | --- |
| CD | 0.524344 | 0.111408 | 0.405777 | 5 |
| AB | 0.534835 | 0.160323 | 0.727462 | 8 |
| EF | 0.46339 | 0.100071 | 0.250426 | 3 |
| GH | 0.541948 | 0.111408 | 0.43008 | 6 |
| IJ | 0.442183 | 0.098425 | 0.210807 | 2 |
| KL | 0.357125 | 0.083556 | 0 | 1 |
| MN | 0.719294 | 0.109919 | 0.665566 | 7 |
| OP | 0.581139 | 0.16317 | 0.809267 | 9 |
| QR | 0.602473 | 0.083676 | 0.339472 | 4 |

Table 5   River health prioritization at segment level using entropy COPRAS

| Sections | S+ | S- | 1/S- | Q | U | Priority rank |
| --- | --- | --- | --- | --- | --- | --- |
| CD | 0.07 | 0.029069 | 34.40088 | 0.134803 | 100 | 1 |
| AB | 0.07 | 0.032349 | 30.91327 | 0.131808 | 97.77857 | 3 |
| EF | 0.07 | 0.027778 | 35.99925 | 0.134082 | 99.46541 | 2 |
| GH | 0.06 | 0.044305 | 22.57101 | 0.106171 | 78.76008 | 5 |
| IJ | 0.06 | 0.044638 | 22.40254 | 0.104735 | 77.69502 | 6 |
| KL | 0.06 | 0.041784 | 23.93243 | 0.11031 | 81.83072 | 4 |
| MN | 0.06 | 0.065325 | 15.30809 | 0.091646 | 67.98543 | 9 |
| OP | 0.07 | 0.065004 | 15.38368 | 0.094641 | 70.20724 | 7 |
| QR | 0.06 | 0.061668 | 16.21577 | 0.091805 | 68.10305 | 8 |

Table 6   River health prioritization at segment level using AHP COPRAS

| Sections | S+ | S- | 1/S- | Q | U | Priority rank |
| --- | --- | --- | --- | --- | --- | --- |
| CD | 0.06 | 0.058163 | 17.19306 | 0.114681 | 89.37489 | 5 |
| AB | 0.06 | 0.06433 | 15.54489 | 0.110478 | 86.099547 | 6 |
| EF | 0.05 | 0.051781 | 19.31221 | 0.120157 | 93.64229 | 2 |
| GH | 0.05 | 0.051567 | 19.39243 | 0.116329 | 90.659395 | 4 |
| IJ | 0.05 | 0.049234 | 20.3113 | 0.119408 | 93.058894 | 3 |
| KL | 0.05 | 0.045598 | 21.93076 | 0.128315 | 100 | 1 |
| MN | 0.05 | 0.073465 | 13.6119 | 0.093581 | 72.930837 | 9 |
| OP | 0.05 | 0.075327 | 13.27547 | 0.097063 | 75.644512 | 8 |
| QR | 0.048 | 0.065865 | 15.18253 | 0.100344 | 78.201516 | 7 |

Table 7    River health prioritization at segment level using entropy SAW

| Sections | Final Preference Value (Vi) | Final Preference Value (Vi) | Priority rank |
| --- | --- | --- | --- |
| CD | 0.755133959 | 0.755133959 | 2 |
| AB | 0.767786864 | 0.767786864 | 1 |
| EF | 0.742697688 | 0.742697688 | 3 |
| GH | 0.619070075 | 0.619070075 | 5 |
| IJ | 0.613980191 | 0.613980191 | 6 |
| KL | 0.642652432 | 0.642652432 | 4 |
| MN | 0.522037574 | 0.522037574 | 9 |
| OP | 0.559665964 | 0.559665964 | 7 |
| QR | 0.534332802 | 0.534332802 | 8 |

Table 8    River health prioritization at segment level using AHP SAW

| Sections | Final Preference Value (Vi) | Final Preference Value (Vi) | Priority rank |
| --- | --- | --- | --- |
| CD | 0.668461297 | 0.668461297 | 5 |
| AB | 0.668374354 | 0.668374354 | 6 |
| EF | 0.688489083 | 0.688489083 | 3 |
| GH | 0.684213299 | 0.684213299 | 4 |
| IJ | 0.703752283 | 0.703752283 | 2 |
| KL | 0.725963396 | 0.725963396 | 1 |
| MN | 0.539240164 | 0.539240164 | 9 |
| OP | 0.59635113 | 0.59635113 | 7 |
| QR | 0.59192496 | 0.59192496 | 8 |

Table 9    River health prioritization at segment level using entropy WASPASS

| Sections | Qi1 | Qi2 | Qi | Priority rank |
| --- | --- | --- | --- | --- |
| CD | 0.755133959 | 0.720159176 | 0.737646568 | 2 |
| AB | 0.767786864 | 0.724784353 | 0.746285609 | 1 |
| EF | 0.742697688 | 0.675862753 | 0.70928022 | 3 |
| GH | 0.619070075 | 0.477835034 | 0.548452555 | 5 |
| IJ | 0.613980191 | 0.478289 | 0.546134595 | 6 |
| KL | 0.642652432 | 0.496343814 | 0.569498123 | 4 |
| MN | 0.522037574 | 0.390663203 | 0.456350388 | 9 |
| OP | 0.559665964 | 0.405176624 | 0.482421294 | 7 |
| QR | 0.534332802 | 0.393844829 | 0.464088815 | 8 |

Table 10    River health prioritization at segment level using AHP WASPASS

| Sections | Qi1 | Qi2 | Qi | Priority rank |
| --- | --- | --- | --- | --- |
| CD | 0.668461297 | 0.624812744 | 0.646637021 | 5 |
| AB | 0.668374354 | 0.610059425 | 0.639216889 | 6 |
| EF | 0.688489083 | 0.626370965 | 0.657430024 | 3 |
| GH | 0.684213299 | 0.612040664 | 0.648126982 | 4 |
| IJ | 0.703752283 | 0.628682769 | 0.666217526 | 2 |
| KL | 0.725963396 | 0.65910212 | 0.692532758 | 1 |
| MN | 0.539240164 | 0.473511377 | 0.506375771 | 9 |
| OP | 0.59635113 | 0.4959919 | 0.546171515 | 8 |
| QR | 0.59192496 | 0.505970513 | 0.548947737 | 7 |

Table 11    River health prioritization at segment level using entropy TOPSIS

| Sections | Si+ | Si- | Ci | Priority rank |
| --- | --- | --- | --- | --- |
| CD | 0.034181741 | 0.068878987 | 0.668333985 | 1 |
| AB | 0.035461207 | 0.069844313 | 0.663254053 | 2 |
| EF | 0.042386504 | 0.069791479 | 0.622149523 | 3 |
| GH | 0.053087428 | 0.055096084 | 0.509283559 | 5 |
| IJ | 0.051812493 | 0.05345587 | 0.507805654 | 6 |
| KL | 0.052146096 | 0.05503691 | 0.513485413 | 4 |
| MN | 0.066207532 | 0.032992406 | 0.332584948 | 9 |
| OP | 0.067364384 | 0.035993118 | 0.348239047 | 8 |
| QR | 0.065793337 | 0.037272517 | 0.361637883 | 7 |

Table 12    River health prioritization at segment level using AHP TOPSIS

| Sections | Si+ | Si- | Ci | Priority rank |
| --- | --- | --- | --- | --- |
| CD | 0.053652329 | 0.046131552 | 0.462314666 | 5 |
| AB | 0.067176491 | 0.042395847 | 0.386921076 | 7 |
| EF | 0.04794786 | 0.059077069 | 0.551993536 | 4 |
| GH | 0.044824609 | 0.067708125 | 0.601674933 | 3 |
| IJ | 0.041273634 | 0.068325906 | 0.623414165 | 2 |
| KL | 0.036453624 | 0.065605176 | 0.642817432 | 1 |
| MN | 0.062714832 | 0.031128956 | 0.331710356 | 8 |
| OP | 0.073997454 | 0.036494107 | 0.330288634 | 9 |
| QR | 0.054956601 | 0.046477243 | 0.45820252 | 6 |