Supporting information

Spatial Distribution of multielements including technology critical elements in sediments of the Danube River

Otilia Ana Culicov1,2, TatjanaTrtić-Petrović3*, Roman Balvanović3, Anđelka Petković4, Slavica Ražić5

1Frank Laboratory of Neutron Physics, Joint Institute for Nuclear Research, Dubna, Russian Federation

2National Institute for R&D in Electrical Engineering ICPE-CA, Bucharest, Romania

3Laboratory of Physics, Vinča Institute of Nuclear Sciences, National Institute of the Republic of Serbia, University of Belgrade, Belgrade, Serbia

4“Jaroslav Černi” Institute for the Development of Water Resources, Belgrade, Serbia

5University of Belgrade, Faculty of Pharmacy - Department of Analytical Chemistry, Belgrade, Serbia

*Corresponding author address:

Laboratory of Physics, Vinča Institute of Nuclear Sciences, National Institute of the Republic of Serbia, University of Belgrade, P.O. Box 522, 11001 Belgrade, Serbia, ttrtic@vin.bg.ac.rs

Table S1(a). Experimental (in mg kg-1 and in %) and statistic data (mean concentration, standard deviation (SD) and relative standard deviation (RSD)) of the quantified major elements and total organic carbon (TOC) in the sediments at 13 monitoring sites along the River Danube and three tributaries.

| Sample code | Concentration of major elements, mg kg-1(%) |  |  |  |  |  |  |  | TOC, % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Al | Ca | Fe | K | Na | Ti | Mg | Mn |  |
| 1-Rit | 56900 (5.7) | 70800 (7.1) | 33100 (3.31) | 16200 (1.6) | 9090 (0.9) | 5050 (0.51) | 3110 (0.31) | 1250 (0.13) | 4.41 |
| 2-SmedDS | 64700 (6.5) | 52300 (5.2) | 33300 (3.33) | 14100 (1.4) | 5940 (0.60) | 4320 (0.43) | 5330 (0.53) | 704 (0.07) | 3.20 |
| 3-SmedRB | 48800 (4.9) | 99500 (9.9) | 33500 (3.35) | 13400 (1.3) | 5880 (0.59) | 3310 (0.33) | 4190 (0.42) | 1460 (0.15) | 8.29 |
| 4-Smed | 68400 (6.8) | 60600 (6.1) | 43400 (4.34) | 20300 (2.0) | 7870 (0.8) | 5300 (0.53) | 3210 (0.32) | 1220 (0.12) | 6.15 |
| 5-Ram | 75500 (7.6) | 53200 (5.3) | 47100 (4.71) | 21100 (2.1) | 7370 (0.7) | 5380 (0.54) | 3470 (0.35) | 1650 (0.17) | 7.77 |
| 6-VelGrad | 99400 (9.9) | 68700 (6.9) | 42200 (4.22) | 20100 (2.0) | 9310 (0.9) | 8270 (0.83) | 4680 (0.47) | 1890 (0.19) | 7.54 |
| 7-DoMil | 90100 (9.8) | 53000 (5.3) | 48400 (4.84) | 20500 (2.1) | 6920 (0.7) | 6660 (0.67) | 3740 (0.37) | 1930 (0.20) | 7.03 |
| 8-Tek | 75200 (7.5) | 55300 (5.5) | 48200 (4.82) | 21400 (2.1) | 7340 (0.7) | 5720 (0.57) | 3740 (0.37) | 1540 (0.15) | 7.94 |
| 9-Klad | 70500 (7.1) | 51300 (5.1) | 46600 (4.66) | 20100 (2.0) | 6890 (0.7) | 5060 (0.51) | 3340 (0.33) | 1570 (0.16) | 5.77 |
| 10-Kus | 69100 (6.9) | 36800 (3.7) | 48000 (4.80) | 21500 (2.2) | 7340 (0.7) | 4770 (0.48) | 3040 (0.30) | 1310 (0.13) | 4.38 |
| 11-Sava | 66300 (6.6) | 73600 (7.4) | 41200 (4.12) | 15200 (1.5) | 7000 (0.7) | 5440 (0.54) | 3220 (0.32) | 1810 (0.18) | 1.95 |
| 12-VelMor | 67600 (6.8) | 37300 (3.7) | 44700 (4.47) | 18100 (1.8) | 13000 (1.3) | 5900 (0.59) | 3110 (0.31) | 1560 (0.16) | 5.12 |
| 13-Pek | 68900 (6.9) | 22900 (2.3) | 30900 (3.09) | 14800 (1.5) | 17500 (1.8) | 5880 (0.59) | 2360 (0.24) | 3627 (0.36) | 2.49 |
| Average | 70877 (7.2) | 56562 (5.7) | 41585 (4.2) | 18215 (1.8) | 8573 (8.6) | 5466 (5.5) | 3580 (0.36) | 1477 (0.15) | 5.38 |
| SD | 12875 | 19293 | 6587 | 3040 | 3257 | 1174 | 777 | 330 | 2.16 |
| RSD | 18.2 | 34.1 | 15.8 | 16.7 | 38.0 | 21.5 | 21.7 | 22.3 | 40.1 |

Table S1(b). Experimental (in mg kg-1) and statistic data of the quantified minor and trace elements in the River Danube’s sediments by INAA

| Sample code | Concentration of minor and trace elements, mg kg-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ba | Zn | Cr | Sr | V | Rb | Ni | Cu | Co | As | Sc | Th | Cs | Hf | Sb | U | W | Ta |
| 1-Rit | 420.0 | 226 | 109 | 131.0 | 90.2 | 95.9 | 47 | 54.6 | 15 | 11.5 | 12.1 | 11.0 | 6.3 | 4.7 | 2.0 | 2.1 | 1.8 | 0.9 |
| 2-SmedDS | 503.0 | 67 | 149 | 168.0 | 98.5 | 92.6 | 86 | 67.9 | 16 | 16.1 | 12.1 | 10.5 | 6.2 | 5.8 | 1.4 | 2.6 | 1.8 | 1.0 |
| 3-SmedRB | 368.0 | 62 | 153 | 158.0 | 77.0 | 75.6 | 87 | 40.4 | 14 | 4.2 | 10.7 | 7.8 | 5.1 | 4.3 | 1.4 | 2.7 | 1.3 | 0.7 |
| 4-Smed | 559.0 | 647 | 254 | 178.0 | 111.0 | 112.0 | 98 | 49.4 | 21 | 26.2 | 14.9 | 14.7 | 8.7 | 5.5 | 10.1 | 2.5 | 2.4 | 1.0 |
| 5-Ram | 505.0 | 371 | 193 | 174.0 | 135.0 | 127.0 | 113 | 41.1 | 24 | 21.1 | 16.7 | 13.9 | 10.2 | 4.9 | 3.4 | 2.4 | 2.4 | 1.0 |
| 6-VelGrad | 487.0 | 261 | 191 | 183.0 | 174.0 | 111.0 | 108 | 45.5 | 23 | 17.2 | 15.0 | 11.9 | 9.0 | 5.7 | 2.8 | 2.3 | 2.0 | 1.0 |
| 7-DoMil | 503.0 | 292 | 184 | 168.0 | 160.0 | 129.0 | 121 | 74.6 | 24 | 21.4 | 17.5 | 13.5 | 10.9 | 4.8 | 3.3 | 2.3 | 2.3 | 1.0 |
| 8-Tek | 535.0 | 299 | 185 | 167.0 | 136.0 | 132.0 | 110 | 78.7 | 25 | 19.0 | 17.3 | 13.4 | 10.8 | 5.1 | 3.5 | 2.4 | 2.3 | 1.0 |
| 9-Klad | 518.0 | 297 | 168 | 169.0 | 124.0 | 125.0 | 90 | 66.2 | 23 | 21.4 | 16.4 | 14.9 | 9.8 | 4.8 | 3.1 | 2.2 | 2.2 | 1.0 |
| 10-Kus | 545.0 | 329 | 175 | 162.0 | 119.0 | 130.0 | 107 | 55.9 | 24 | 19.9 | 17.3 | 15.4 | 10.3 | 4.8 | 4.4 | 2.3 | 2.2 | 1.0 |
| 11-Sava | 397.0 | 307 | 294 | 162.0 | 118.0 | 88.5 | 146 | 44.9 | 25 | 21.6 | 13.2 | 11.8 | 7.3 | 5.1 | 5.1 | 1.9 | 1.9 | 0.8 |
| 12-VelMor | 556.0 | 666 | 363 | 218.0 | 120.0 | 101.0 | 174 | 51.3 | 29 | 24.4 | 15.8 | 12.1 | 10.0 | 6.9 | 4.7 | 2.3 | 2.1 | 1.0 |
| 13-Pek | 459.0 | 425 | 75 | 219.0 | 114.0 | 58.6 | 24 | 512.6 | 15 | 11.0 | 12.9 | 7.7 | 2.3 | 4.6 | 5.2 | 1.4 | 1.9 | 0.7 |
| Average | 488.8 | 327 | 192 | 173.6 | 121.3 | 106.0 | 101 | 91 | 21 | 18 | 14.8 | 12.2 | 8.2 | 5.2 | 3.9 | 2.2 | 2.1 | 0.9 |
| SD | 61.2 | 179 | 76 | 23.4 | 26.3 | 23.1 | 38 | 127.3 | 5 | 6 | 2.3 | 2.5 | 2.6 | 0.7 | 2.2 | 0.3 | 0.3 | 0.1 |
| RSD | 12.5 | 55 | 39 | 13.5 | 21.7 | 21.8 | 38 | 140 | 22 | 34 | 15.7 | 20.3 | 31.8 | 13.2 | 57.9 | 14.7 | 14.7 | 13.3 |

Table S1(c). Experimental (in mg kg-1) and statistic data of the quantified lanthanide elements in the River Danube’s sediments by INAA

| Sample code | Concentration of lanthanide elements, mg kg-1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | La | Ce | Nd | Sm | Eu | Gd | Tb | Dy | Tm | Yb | ΣLn |
| 1-Rit | 31.6 | 56.6 | 37.3 | 3.8 | 1.5 | 3.9 | 0.7 | 4.1 | 0.3 | 2.5 | 142.31 |
| 2-SmedDS | 33.0 | 71.5 | 26.9 | 5.4 | 1.1 | 5.7 | 0.8 | 4.2 | 0.4 | 2.7 | 151.63 |
| 3-SmedRB | 24.4 | 55.5 | 16.2 | 4.2 | 1.2 | 4.1 | 0.7 | 2.9 | 0.5 | 2.6 | 112.23 |
| 4-Smed | 36.5 | 64.2 | 46.6 | 5.4 | 1.6 | 6.4 | 0.8 | 7.1 | 0.5 | 2.8 | 171.95 |
| 5-Ram | 36.1 | 63.1 | 39.7 | 3.6 | 1.9 | 4.7 | 0.8 | 6.5 | 0.4 | 3.0 | 160.03 |
| 6-VelGrad | 34.4 | 60.3 | 34.0 | 5.0 | 1.6 | 4.9 | 0.7 | 9.2 | 0.4 | 2.5 | 152.94 |
| 7-DoMil | 36.9 | 65.2 | 43.1 | 3.6 | 1.6 | 4.8 | 0.8 | 6.1 | 0.4 | 2.7 | 165.19 |
| 8-Tek | 36.8 | 65.2 | 35.2 | 4.9 | 1.8 | 4.6 | 0.9 | 5.7 | 0.4 | 3.3 | 158.60 |
| 9-Klad | 34.3 | 59.7 | 36.9 | 4.2 | 1.8 | 4.7 | 0.8 | 6.1 | 0.4 | 2.6 | 151.56 |
| 10-Kus | 35.7 | 63.7 | 37.9 | 3.2 | 1.9 | 4.2 | 0.8 | 5.5 | 0.4 | 2.8 | 156.11 |
| 11-Sava | 29.1 | 49.5 | 28.8 | 2.5 | 1.6 | 3.9 | 0.6 | 3.4 | 0.4 | 2.4 | 122.21 |
| 12-VelMor | 33.9 | 59.6 | 30.5 | 5.1 | 1.5 | 4.8 | 0.8 | 6.6 | 0.5 | 2.7 | 145.92 |
| 13-Pek | 21.7 | 37.5 | 31.9 | 3.5 | 1.5 | 4.0 | 0.6 | 5.9 | 0.3 | 2.2 | 108.97 |
| Average | 32.6 | 59.4 | 34.2 | 4.2 | 1.6 | 4.7 | 0.7 | 5.6 | 0.4 | 2.7 | 146.13 |
| SD | 4.8 | 8.5 | 7.7 | 0.9 | 0.2 | 0.7 | 0.1 | 1.7 | 0.1 | 0.3 | 20.6 |
| RSD | 14.8 | 14.3 | 22.6 | 21.6 | 15.5 | 15.5 | 12.5 | 29.6 | 16.9 | 10.5 | 14.1 |

Table S2. Limit values (in mg kg-1) for assessing the status and trend of sediment quality according Regulation on limit values of pollutants in surface and groundwater and sediment and deadlines for reaching them, “Sl. glasnik RS", br. 50/2012

| Element | Target Value | Maximum allowed concentration | Remediation value |
| --- | --- | --- | --- |
| As | 29 | 42 | 55 |
| Cd | 0.8 | 6.4 | 12 |
| Cr | 100 | 240 | 380 |
| Cu | 36 | 110 | 190 |
| Hg | 0.3 | 1.6 | 10 |
| Pb | 85 | 310 | 530 |
| Ni | 35 | 44 | 210 |
| Zn | 140 | 430 | 720 |

Table S3. Concentrations of the selected elements in the Danube sediments quantified in 1985*, 2002* and 2016**.

| Sample location | Fe, g kg-1 |  |  | Mn, g kg-1 |  |  | Zn, mg kg-1 |  |  | Cr, mg kg-1 |  |  | Ni, mg kg-1 |  |  | Cu, mg kg-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1985 | 2002 | 2016 | 1985 | 2002 | 2016 | 1985 | 2002 | 2016 | 1985 | 2002 | 2016 | 1985 | 2002 | 2016 | 1985 | 2002 | 2016 |
| 4-Smed | - | 18.0 | 43.4 | - | 0.75 | 1.22 | - | 219.1 | 647 | - | 51.8 | 254 | - | 46.8 | 98 | - | 23.9 | 49.4 |
| 5-Ram | - | 29.9 | 47.1 | - | 1.12 | 1.65 | - | 328.4 | 371 | - | 112.5 | 193 | - | 116.4 | 113 | - | 36.8 | 41.1 |
| 6-VelGrad | 17.62 | 32.0 | 42.2 | 0.65 | 0.93 | 1.89 | 230 | 389.5 | 261 | 54 | 105.9 | 191 | 39.0 | 99.9 | 108 | 25 | 41.0 | 45.5 |
| 7-DoMil | - | 25.3 | 48.4 | - | 0.90 | 1.93 | - | 285.7 | 292 | - | 68.0 | 184 | - | 69.9 | 121 | - | 45.3 | 74.6 |
| 8-Tek | 22.59 | 29.5 | 48.2 | 0.54 | 0.88 | 1.54 | 210 | 307.8 | 299 | 50 | 93.3 | 185 | 34.0 | 74.5 | 110 | 35 | 57.6 | 78.7 |
| 9-Klad | - | 22.0 | 46.6 | - | 0.52 | 1.57 | - | 197.5 | 297 | - | 71.1 | 168 | - | 59.2 | 90 | - | 31.6 | 66.2 |

*M.R.N. Milenkovic, M. Damjanovic, M. Ristić, Study of heavy metal pollution in sediments from the Iron Gate (, Danube River), Serbia Montenegro. Polish J Env. Stud. 14 (2005) 781–787.

***This study.

Figure S1. Spatial distribution of the studied lanthanides.

Figure S2. Component Eigen values, Na - U

Table S4. Eigen analysis of the Correlation Matrix

| Eigenvalue | 15.704 | 6.765 | 4.153 | 2.997 | 2.844 | 1.295 | 0.896 | 0.575 | 0.293 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proportion | 0.436 | 0.188 | 0.115 | 0.083 | 0.079 | 0.036 | 0.025 | 0.016 | 0.008 |
| Cumulative | 0.436 | 0.624 | 0.740 | 0.823 | 0.902 | 0.938 | 0.963 | 0.979 | 0.987 |

Table S5. Rotated component matrixa

|  | Component |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Rb | 0.937 |  |  |  |  |  |
| Th | 0.857 |  |  |  |  | 0.376 |
| Tb | 0.857 |  |  | 0.455 |  |  |
| La | 0.854 |  |  |  |  |  |
| Ta | 0.847 |  |  | 0.336 |  |  |
| K | 0.837 | 0.419 |  |  |  |  |
| Cs | 0.828 |  | 0.421 |  |  |  |
| Sc | 0.811 | 0.389 |  |  |  |  |
| Fe | 0.806 | 0.302 | 0.430 |  |  |  |
| Yb | 0.791 |  |  |  |  |  |
| Br | 0.786 |  |  |  | -0.323 | -0.305 |
| W | 0.758 | 0.329 |  |  |  | 0.457 |
| Ba | 0.695 |  |  | 0.431 | 0.445 |  |
| Ce | 0.690 |  |  | 0.566 | -0.352 |  |
| Eu | 0.690 |  |  | -0.495 |  |  |
| As | 0.558 |  | 0.469 |  |  | 0.527 |
| Ti |  | 0.963 |  |  |  |  |
| Al | 0.310 | 0.911 |  |  |  |  |
| V | 0.367 | 0.890 |  |  |  |  |
| Mg | 0.407 | 0.758 |  |  |  | 0.331 |
| Dy |  | 0.756 |  | 0.315 | 0.310 |  |
| Mn |  | 0.672 | 0.408 | -0.500 |  |  |
| Cr |  |  | 0.929 |  |  |  |
| Ni |  |  | 0.923 |  |  |  |
| Co | 0.531 | 0.362 | 0.719 |  |  |  |
| Tm | 0.336 |  | 0.628 | 0.545 |  |  |
| Sm |  |  |  | 0.934 |  |  |
| Gd |  |  |  | 0.841 |  | 0.379 |
| Hf |  |  | 0.607 | 0.610 |  |  |
| U | 0.384 |  |  | 0.563 | -0.538 |  |
| Ca |  |  |  |  | -0.898 |  |
| Sr |  |  | 0.327 |  | 0.815 |  |
| Na | -0.433 |  |  |  | 0.811 |  |
| Cu | -0.427 |  | -0.425 |  | 0.732 |  |
| Sb |  |  |  |  |  | 0.925 |
| Zn |  |  | 0.406 |  | 0.487 | 0.696 |
| Nd | 0.621 | 0.348 |  |  |  | 0.623 |
| aExtraction Method: Principal Component Analysis, Rotation Method: Varimax with Kaiser Normalization. aRotation converged in 16 iterations. |  |  |  |  |  |  |

Normalization.

Figure S3. Principal component analysis of the studied lanthanides.