Annex 1:

Philosophy of Ligament Balance : Functional Alignment (FA) with "Flexion First"

• Recreate the HKA angle for normal alignment of the patient's leg (avoiding valgus greater than 3°).

• Perform perpendicular or restricted varus tibial resection (up to 3°) for pre-operative varus knees.

• Achieve precise femoral joint line reconstruction in flexion and extension.

• Slightly greater laxity on the lateral side in flexion compared to the medial side.

• Adjust the femoral valgus angle based on extension balance, then adjust femoral height cut.

• Limit ligament releases (except in cases of fixed deformities or extreme tibial varus).

2. Step-by-step Planning:

- Assess the morphology of the proximal tibia with the medial proximal tibial angle (MPTA)

- If MPTA ≥ 90° (valgus) : set at 0° varus/valgus.

- If MPTA < 90° (varus): set up-to 3° of varus according to balance gap in flexion and extension.

- Check the medial slope and put 3° slope for the PS implant.

- Adjust femoral rotation until space balance is achieved in flexion (lateral space may be laxer by 1-2 mm compared to medial based on surgeon preference).

- Check the posterior medial femoral resection (Always 10 mm resection).

- Adjust tibial resection until a minimal space is achieved on the medial side.

- Adjust the femoral valgus angle until a balanced space is achieved in extension.

- Adjust distal femoral resection until a minimal space is achieved.

- Verify femoral implant size, avoid the "Notch," and add femoral flexion if necessary.