1  Calculation of ratio of inductances LR/LL

LR/LL can be calculated based on the position of the electrode and shape of circuits. The sample was installed as shown in Figure 3. The left semicircle and the right semicircle are uniform by themselves. Let the inductance of the unit length in the left semicircle is kL, and in the right semicircle is kR. Since the 6 electrodes and the two Hall detectors are evenly distributed on the loop circuit, the arc length between neighboring electrodes 2, 3, 4, 5 and 6 can be set to be s , and the arc length between electrodes 1 and 2 or 1 and 6 is 2s. Taking the current flowing through electrodes 1 and 2 as an example, then LL=2skL ，LR=4skR+2skL , LRLL=4skR+2skL 2skL =2kR+kL kL . If the current flows through electrodes 1 and 5, then LL=4skL+skR ，LR=3skR , LRLL=3skR4skL+skR=3kR4kL+kR . For the connections between any two electrodes, the LR/LL can be calculated by the same way.

Since the cross-section of the loop sample is geometrically symmetrical, when calculating the magnetic field excited by the current through it, it can be assumed that the current passes through the center of the cross-section, like a straight wire passing through the center of the cross-section, and the magnetic field B at point A in the figure below is proportional to the width of the cross-section. B∝a,

a

b

A

For sample 1, since the width of the left and right semicircle is the same, both are 4.38mm, it is reasonable to take kL=kR . For sample 2, the width of the left and the right semicircle is 2.82mm and 4.38 mm respectively, so  kLkR=4.382.82=1.55.  The  LR/LL of samples 1 and 2 can be calculated for various modes of connections between electrodes 1 to 6.

| electrodes |  | LR/LL |  | Sample 1 (kL=kR) |  | Sample 2 (kL=1.55kR) |
| --- | --- | --- | --- | --- | --- | --- |
| 1--2 |  | 4skR+2skL 2skL |  | 3.00 |  | 2.28 |
| 1--3 |  | 4skR+skL 3skL |  | 1.67 |  | 1.19 |
| 1--4 |  | 4skR 4skL |  | 1.00 |  | 0.65 |
| 1--5 |  | 3skR4skL+skR |  | 0.60 |  | 0.42 |
| 1--6 |  | 2skR4skL+2skR |  | 0.33 |  | 0.24 |

2  Calculation of ratio of electrical resistance RR/RL

Let the electrical resistance of the unit length in the left semicircle is rL, and in the right semicircle is rR. Since the 6 electrodes and the two Hall detectors are evenly distributed on the loop circuit, the arc length between neighboring electrodes 2, 3, 4, 5 and 6 can be set to be s , and the arc length between electrodes 1 and 2 or 1 and 6 is 2s.

Taking the current flowing through electrodes 1 and 2 as an example, then RL=2srL ，RR=4srR+2srL , RRRL=4srR+2srL 2srL =2rR+rL rL . If the current flows through electrodes 1 and 5, then RL=4srL+srR ，RR=3srR , RRRL=3srR4srL+srR=3rR4rL+rR . For the connections between any two electrodes, the RR/RL can be calculated by the same way.

For sample 1, rLrR=2.34*4.381.36*4.38=1.72,

For sample 2, rLrR=2.34*4.381.36*2.82=2.67,

| electrodes |  | RR/RL |  | Sample 1 (rL=1.72rR) |  | Sample 2 (rL=2.67rR) |
| --- | --- | --- | --- | --- | --- | --- |
| 1--2 |  | 4srR+2srL 2srL |  | 2.16 |  | 1.74 |
| 1--3 |  | 4srR+srL 3srL |  | 1.11 |  | 0.83 |
| 1--4 |  | 4srR 4srL |  | 0.58 |  | 0.37 |
| 1--5 |  | 3srR4srL+srR |  | 0.38 |  | 0.26 |
| 1--6 |  | 2srR4srL+2srR |  | 0.23 |  | 0.16 |