Supporting Information

Occurrence, distribution, sources and bioaccumulation of polycyclic aromatic hydrocarbons (PAHs) of multi environmental media in estuaries and coast of the Beibu Gulf, China: A health risk assessment through seafood consumption

Minwei Han a, Fang Liu a, Yaru Kang a, Ruijie Zhang a,b,*, Kefu Yu a,b,*, Yinghui Wang a, Ruixuan Wang a

a Guangxi Laboratory on the Study of Coral Reefs in the South China Sea; Coral Reef Research Center of China; School of Marine Sciences, Guangxi University, Nanning 530004, China

b Southern Marine Science and Engineering Guangdong Laboratory (Zhuhai), Zhuhai 519080, China

*Corresponding Author, Dr. Ruijie Zhang, Tel: +86-771-3227522, Email: rjzhang@gxu.edu.cn

*Corresponding Author, Dr. Kefu Yu, Tel: +86-771-3231358, Email: kefuyu@gxu.edu.cn

Total number of Pages: 25

Total number of Tables: 13

Total number of Figures: 5

Contents

Text S1 Chemicals	3

Text S2 Extraction of PAHs	3

Text S3. Calculation of bioaccumulation factors of PAHs in marine organisms	3

Text S4. Calculation of the PAHs flux into the sea	4

Text S5. Determination of TOC in sediment	4

Table S1. Sampling location and sample information of aquatic ecosystem in Beibu Gulf	6

Table S2. Optimized GC-MS/MS parameters and instrumental quantitative limits (IQLs) of the target PAHs and surrogates	7

Table S3. Physicochemical characteristics, standards producers of the target PAHs	8

Table. S4. Total pollution levels (ng/L) of PAHs in the summer water of the Beibu Gulf estuary and coastal.	9

Table. S5. Total pollution levels (ng/L) of PAHs in the winter waters of the Beibu Gulf estuary and coastal	10

Table. S6. Flux of PAHs into the the Beibu Gulf through the river discharge	11

Table. S7. Total pollution levels (ng/g) of PAHs in the sediment of the Beibu Gulf estuary and coastal	12

Table. S8. Total pollution levels (ng/g) of PAHs in the marine organisms of the Beibu Gulf.	13

Table. S9. The average Log BWAFs of PAHs in the marine organisms from the Beibu Gulf.	15

Table. S10. The average Log BSAFs of PAHs in the the marine organisms from the Beibu Gulf.	16

Table. S11. The average PAHs’ concentration (ng/g; wet weight) in marine organisms from the Beibu Gulf.	17

Table. S12. The Diagnosis ratios to characterize sources of soil PAHs.	18

Table. S13. The toxicity equivalent quotients (TEQ: pg-TEQ/g) of PAHs for marine organisms from Beibu Gulf.	19

Table. S14. Excess cancer risk (×10-9) induced by exposure to PAHs via seafood ingestion for different age groups	20

Fig. S1. Sampling sites of aquatic ecosystem of Beibu Gulf, Guangxi.	21

Fig. S2. The molecular formula of the 16 target PAHs.	22

Fig. S3. Concentration distribution of PAHs in rivers and bays in summer (A) and winter (B).	23

Fig. S4. The Log BWAFs (bio-water accumulation factors, group A) and Log BSAFs (bio-sediment accumulation factors, group B) of 16 PAHs by marine organisms.	24

Fig. S5. Relationship between the Log BWAFs of detected PAHs in the various marine organisms and their log KOW.	25

References	26

# Text S1 Chemicals

Sixteen mixed solutions of target PAHs were purchased from pestek. Hexamethylbenzene, as an internal standard, was purchased from Aldrich. Chromatographic pure dichloromethane was purchased from Anhui Fushi professional solvent Reagent Co., Ltd., and n-hexane and methanol (CH3OH) were purchased from Honeywell Co., Ltd. All glassware must be soaked in ABS detergent for two days, then washed with tap water, washed with distilled water, dried in an oven at 120°C and burned in a muffle furnace at 450°C for 4 hours. Inactive neutral alumina (70-230 mesh, Merck, Germany) was baked in an oven at 450°C for 12 hours, and then activated with 3% distilled water. The inactivated neutral silica gel (80-200 mesh, Merck, Germany) was baked in an oven at 450°C for 12 hours, then activated by adding 3% distilled water; Anhydrous sodium sulfate was also baked in the oven at 450°C for 12 hours. After cooling, all of the three reagents were put into in a desiccator.

# Text S2 Extraction of PAHs

Before extraction, the water quality of each water sample was determined, including salinity, pH and other parameters. Then 1000 ml water sample was filtered with 0.7 μm glass fiber filter membrane to remove suspended solids and other impurities in the water sample. After filtration, 80 ng substitutes were added to all filtered water samples to determine the program recovery, dichloromethane was used as extractant for liquid-liquid extraction (15mL/time, extraction for three times). All extracts were reduced by rotary evaporation and solvent-exchanged into a volume of 4 mL hexane. ENVI™-Florisil cartridges (500 mg, 3 mL, Supleco, Bellefonte, PA, USA) were used for cleanup and fractionations. The first fraction was eluted with 10 mL mixture of hexane and dichloromethane (8:2, V/V), and the second fraction was eluted with 10 mL ethyl acetate. The first fraction which included PAHs was further purified with a 10 mm i.d. alumina/silica column chromatography filled with 3 cm neutral alumina, 3 cm neutral silica gel, and 1 cm anhydrous sodium sulfate from bottom to top. The PAH fractions were eluted with 15 mL mixture of dichloromethane and hexane (1:1, V/V). The elution was concentrated to 0.5 mL and added with 200 ng of hexamethylbenzene as an internal standard before instrumental analysis.

# Text S3. Calculation of bioaccumulation factors of PAHs in marine organisms

Generally speaking, the bioaccumulation factors (BAFs) can be expressed on the basis of wet weight, dry weight and lipid weight. The most common is that the weight of organism is based on wet weight and the unit of the BAFs are L/kg. In this study, the BWAFs (bio-water accumulation factors) were calculated based on the wet weight concentration of PAHs in marine organisms (Cm) divided by the concentration of PAHs in water (CW), the BSAFs (bio-sediment accumulation factors) were calculated on an organic carbon and lipid normalized basis (Burkhard, 2003; Moermond et al., 2005).

BWAF = CmCW×1000    (1)

BWAF=Cbio/flipCS/foc×1000   (2)

Where the C bio is the concentration in biota, the f lip is the lipid fraction in biota, the Cs is the concentration of PAHs in the sediment, the foc is the total organic carbon (TOC) fraction in sediment.

# Text S4. Calculation of the PAHs flux into the sea

Based on the concentration of PAHs in different rivers multiplied by the river flow, the annual flux of PAHs into the Beibu Gulf coastal waters was estimated. The calculation formula is as follows:

Fi=CW,i×Wi  (3)

F=inFi    (4)

Where F is the total PAHs flux of all seagoing rivers into the ocean, n is the total number of seagoing rivers, Fi is the PAHs flux from a single seagoing river into the ocean, CW,i is the average concentration of PAHs in seagoing river.

# Text S5. Determination of TOC in sediment

The inorganic carbon in the sediment was removed by acid washing method, and then the total organic carbon (TOC) content was determined by element analyzer (Leong and Tanner, 1999; Yang et al., 2014). Specifically, weigh 1g of the dried homogenized sediment and put it into a 50ml centrifuge tube, then add 1mol/L hydrochloric acid solution and vibrate for 1.5 hours by ultrasonic oscillation. Repeat this step 2-3 times to ensure that the inorganic carbon in the sample is completely consumed. Then wash off the excess hydrochloric acid with Milli Q ultrapure water until neutral, add 30ml ultrapure water into the centrifuge tube each time, centrifuge with centrifuge, take the supernatant, compare with pH test paper, and remove the supernatant. Repeat several times until the supernatant is neutral. Finally, after removing the supernatant, the centrifuge tube was dried at 40 ℃ in the oven, and 100 mg of the supernatant was taken for analysis. The percentage of TC measured was the content of TOC.

# Table S1. Sampling location and sample information of aquatic ecosystem in Beibu Gulf

| Water | longitude | latitude | Sediment | longitude | latitude | organisms | number | species | moisture content b | Lipid (%, dw) c | sampling site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1C | 108.231 | 21.600 | 1C | 108.231 | 21.600 | Tilapia mossambica | 3 | fish |  |  | Qinzhou |
| 2C | 108.200 | 21.537 | 2C | 108.200 | 21.537 | Trachinotus ovatus | 3 |  | 75% | 8.5% |  |
| 3C | 108.381 | 21.603 | 3C | 108.381 | 21.603 | Siganus fuscescens | 1 |  |  |  |  |
| 4C | 108.342 | 21.549 | 4C | 108.342 | 21.549 | Crab | 6 | crab | 78% | 17.35% |  |
| 5E | 108.477 | 21.928 | 5E | 108.477 | 21.928 | Oysters | 3 | shellfish | 86% | 6.65% |  |
| 6E | 108.466 | 21.887 | 6E | 108.466 | 21.887 | Trachinotus ovatus | 2 | fish | 75% | 8.5% | Beihai |
| 7E | 108.485 | 21.838 | 7E | 108.485 | 21.838 | perches | 2 |  |  |  |  |
| 8E | 108.557 | 21.922 | 8E | 108.557 | 21.922 | Crab | 4 | crab | 78% | 17.35% |  |
| 9E | 108.621 | 21.943 | 9E | 108.621 | 21.943 | shrimp | 3 | shrimp | 76% | 4.39% |  |
| 10E | 108.603 | 21.907 | 10E | 108.603 | 21.907 | Oysters | 3 | shellfish | 86% | 6.65% |  |
| 11E | 108.571 | 21.885 | 11E | 108.571 | 21.885 | Rhabdosargus sarba | 3 | fish |  |  | Fangcheng |
| 12C | 108.542 | 21.781 | 12C | 108.542 | 21.781 | Megalobrama amblycephala | 1 |  | 75% | 8.5% |  |
| 13C | 108.613 | 21.635 | 13C | 108.629 | 21.632 | Crab | 4 | crab | 78% | 17.35% |  |
| 14C | 108.793 | 21.591 | 14C | 108.793 | 21.591 | Scylla | 2 |  |  |  |  |
| 15C | 108.836 | 21.575 | 15C | 108.836 | 21.575 | shrimp | 3 | shrimp | 76% | 4.39% |  |
| 16C | 108.890 | 21.582 | 16C | 108.890 | 21.582 | Oysters | 3 | shellfish | 86% | 6.65% |  |
| 17E | 108.863 | 21.656 | 17E | 108.863 | 21.656 | Scallop | 2 |  |  |  |  |
| 18E | 108.865 | 21.762 | 18E | 108.865 | 21.762 | — | — | — |  |  | — |
| 19E | 109.118 | 21.675 | 19E | 109.138 | 21.670 | — | — | — |  |  | — |
| 20E | 109.073 | 21.655 | 20E | 109.073 | 21.655 | — | — | — |  |  | — |
| 21E | 109.049 | 21.605 | 21C | 108.996 | 21.536 | — | — | — |  |  | — |
| 22E | 109.138 | 21.670 | 22C | 109.114 | 21.499 | — | — | — |  |  | — |
| 23E | 109.080 | 21.635 | 23C | 109.024 | 21.499 | — | — | — |  |  | — |
| 24E | 109.072 | 21.603 | — | — | — | — | — | — |  |  | — |
| 25C | 108.996 | 21.536 | — | — | — | — | — | — |  |  | — |
| 26C | 109.024 | 21.499 | — | — | — | — | — | — |  |  | — |
| 27C | 109.114 | 21.499 | — | — | — | — | — | — |  |  | — |

Note: “—” represent no sample or no site; b the dates of moisture content refer to previous study (Ding et al., 2020; Zeng, 2019); b the dates of lipid refer to previous study (Ding et al., 2020; Zeng, 2019).

# Table S2. Optimized GC-MS/MS parameters and instrumental quantitative limits (IQLs) of the target PAHs and surrogates

| PAHs | Retention time (min) | Precursor ion (m/z) | Product ion (m/z) | Collision energy (V) | Dwell | Polarity | IQLs b (ng/mL) | Time slot | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAP-D8 | 6.77 | 136 a | 108 | 30 | 50 | Positive | 0.07 | 1 | 10 |
| NAP-D8 |  | 136 | 84 | 30 | 50 | Positive |  |  | 10 |
| NAP | 6.799 | 128 | 102 | 20 | 50 | Positive | 0.24 | 1 | 10 |
| NAP |  | 128 | 78 | 30 | 50 | Positive |  |  | 10 |
| ISTD | 9.297 | 147 | 91 | 30 | 30 | Positive | 0.06 | 2 | 10 |
| ISTD |  | 147 | 117 | 30 | 30 | Positive |  |  | 10 |
| ACEY | 9.348 | 152 | 151 | 20 | 30 | Positive | 0.17 | 2 | 10 |
| ACEY |  | 152 | 126 | 20 | 30 | Positive |  |  | 10 |
| ACE-D10 | 9.635 | 164 | 162 | 20 | 30 | Positive | 0.06 | 2 | 10 |
| ACE-D10 |  | 164 | 160 | 40 | 30 | Positive |  |  | 10 |
| ACE | 9.696 | 153 | 152 | 20 | 30 | Positive | 0.19 | 2 | 10 |
| ACE |  | 153 | 126 | 50 | 30 | Positive |  |  | 10 |
| FLU | 10.859 | 165 | 163 | 20 | 30 | Positive | 0.32 | 3 | 10 |
| FLU |  | 165 | 164 | 40 | 30 | Positive |  |  | 10 |
| PHE-D10 | 13.674 | 188 | 160 | 40 | 30 | Positive | 0.06 | 3 | 10 |
| PHE-D10 |  | 188 | 184 | 30 | 30 | Positive |  |  | 10 |
| PHE | 13.908 | 178 | 152 | 30 | 30 | Positive | 0.23 | 3 | 10 |
| PHE |  | 178 | 176 | 40 | 30 | Positive |  |  | 10 |
| ANTH | 13.751 | 178 | 152 | 20 | 20 | Positive | 0.16 | 3 | 10 |
| ANTH |  | 178 | 176 | 20 | 40 | Positive |  |  | 10 |
| PYR | 18.343 | 202 | 200 | 50 | 20 | Positive | 0.13 | 4 | 10 |
| PYR |  | 202 | 201 | 30 | 20 | Positive |  |  | 10 |
| FLUA | 19.225 | 202 | 200 | 40 | 20 | Positive | 0.22 | 4 | 10 |
| FLUA |  | 202 | 201 | 30 | 20 | Positive |  |  | 10 |
| CHR | 24.602 | 228 | 226 | 40 | 20 | Positive | 0.07 | 4 | 10 |
| CHR |  | 228 | 202 | 30 | 20 | Positive |  |  | 10 |
| CHR-D12 | 24.66 | 240 | 236 | 50 | 20 | Positive | 0.15 | 4 | 10 |
| CHR-D12 |  | 240 | 212 | 30 | 20 | Positive |  |  | 10 |
| BaA | 24.769 | 228 | 226 | 40 | 20 | Positive | 0.11 | 4 | 10 |
| BaA |  | 228 | 202 | 50 | 20 | Positive |  |  | 10 |
| BbF | 29.654 | 252 | 250 | 50 | 20 | Positive | 0.11 | 5 | 10 |
| BbF |  | 252 | 226 | 20 | 20 | Positive |  |  | 10 |
| BjF | 29.724 | 252 | 250 | 40 | 20 | Positive | 0.08 | 5 | 10 |
| BjF |  | 252 | 226 | 40 | 20 | Positive |  |  | 10 |
| BkF | 30.996 | 252 | 250 | 40 | 20 | Positive | 0.14 | 5 | 10 |
| BkF |  | 252 | 226 | 40 | 20 | Positive |  |  | 10 |
| BaP | 31.245 | 252 | 250 | 40 | 20 | Positive | 0.32 | 5 | 10 |
| BaP |  | 252 | 226 | 20 | 20 | Positive |  |  | 10 |
| Pery-D12 | 37.045 | 264 | 260 | 20 | 20 | Positive | 0.13 | 5 | 10 |
| Pery-D12 |  | 264 | 234 | 40 | 20 | Positive |  |  | 10 |
| Ind | 31.559 | 276 | 274 | 50 | 35 | Positive | 0.15 | 6 | 10 |
| Ind |  | 276 | 275 | 30 | 35 | Positive |  |  | 10 |
| DiB | 37.286 | 278 | 276 | 50 | 35 | Positive | 0.11 | 6 | 10 |
| DiB |  | 278 | 263 | 40 | 35 | Positive |  |  | 10 |
| BghiP | 38.231 | 276 | 274 | 30 | 35 | Positive | 0.27 | 6 | 10 |
| BghiP |  | 276 | 275 | 40 | 35 | Positive |  |  | 10 |

Note: a product ions in bold were quantitative ions; b instrumental quantitative limits

| Compound | Abbreviation | CAS No. a | Molecular weight a | Melting Point a | Boiling Point a | Water Solubility (mg/L) a | Log Kow a |  | Henry's Law a | Ring No. | MDLs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | water (ng/L) | sediment (ng/g) | organism (ng/L) |
| Naphthalene | NAP | 90-20-3 | 128.3 | 80.3 | 218 | 2.42E-04 | 3.34 | 4.40E-04 |  | 2 | 0.24 | 0.03 | 0.48 |
| Acenaphthylene | ACEY | 208-96-8 | 152.2 | 90 | 280 | 1.06E-04 | 3.85 | 1.14E-04 |  | 3 | 0.17 | 0.02 | 0.33 |
| Acenaphthene | ACE | 83-32-9 | 154.2 | 93.9 | 279 | 2.53E-05 | 3.89 | 1.84E-04 |  | 3 | 0.19 | 0.03 | 0.38 |
| Fluorene | FLU | 86-73-7 | 166.2 | 115 | 295 | 1.14E-05 | 4.14 | 9.62E-05 |  | 3 | 0.32 | 0.04 | 0.65 |
| Phenanthrene | PHE | 1985/1/8 | 178.2 | 99.2 | 339 | 6.45E-06 | 4.39 | 4.23E-05 |  | 3 | 0.23 | 0.03 | 0.46 |
| Anthracene | ANTH | 120-12-7 | 178.2 | 215 | 340 | 2.44E-07 | 4.39 | 5.56E-05 |  | 3 | 0.16 | 0.02 | 0.33 |
| Fluoranthene | FLUA | 206-44-0 | 202.3 | 108 | 380 | 1.14E-06 | 4.9 | 8.86E-06 |  | 4 | 0.13 | 0.02 | 0.25 |
| Pyrene | PYR | 129-00-0 | 202.3 | 150 | 399 | 6.67E-07 | 4.9 | 1.19E-05 |  | 4 | 0.22 | 0.03 | 0.43 |
| Chrysene | CHR | 218-01-9 | 228.3 | 225 | 448 | 8.76E-09 | 5.32 | 5.23E-06 |  | 4 | 0.07 | 0.01 | 0.13 |
| Benzo[a]anthracene | BaA | 56-55-3 | 228.3 | 159 | 437 | 4.12E-08 | 5.32 | 1.20E-05 |  | 4 | 0.11 | 0.01 | 0.22 |
| Benzo[b]fluoranthene | BbF | 205-99-2 | 252.3 | 166 | 446* | 5.94E-09 | 5.95 | 6.57E-07 |  | 5 | 0.11 | 0.01 | 0.21 |
| Benzo[k]fluoranthene | BkF | 207-08-9 | 252.3 | 217 | 480 | 3.17E-09 | 5.95 | 5.84E-07 |  | 5 | 0.08 | 0.01 | 0.16 |
| Benzo[a]pyrene | BaP | 50-32-8 | 252.3 | 177 | 495 | 6.42E-09 | 5.95 | 4.57E-07 |  | 5 | 0.14 | 0.02 | 0.28 |
| Indeno[1,2,3-cd]pyrene | Ind | 193-39-5 | 276.3 | 164 | 536 | 6.88E-10 | 6.45 | 3.48E-07 |  | 6 | 0.13 | 0.02 | 0.26 |
| Dibenz [a,h]anthracene | DiB | 53-70-3 | 278.4 | 268 | 524 | 1.22e-08* | 6.5 | 4.50e-07* |  | 5 | 0.11 | 0.01 | 0.22 |
| Benzo[g,h,i]perylene | BghiP | 191-24-2 | 276.3 | 277 | 500 | 9.41E-10 | 6.45 | 3.31E-07 |  | 6 | 0.27 | 0.04 | 0.53 |

# Table S3. Physicochemical characteristics, standards producers of the target PAHs

a Data are from the United States Environmental Protection Agency; we took their average experimental https://comptox.epa.gov/dashboard

* The experimental average lacks data and we took the predicted average https://comptox.epa.gov/dashboard

# Table. S4. Total pollution levels (ng/L) of PAHs in the summer water of the Beibu Gulf estuary and coastal.

| Sample No. | NAP | 2-ring | ACEY | ACE | FLU | PHE | ANTH | 3-ring | FLUA | PYR | 4-ring | BaP | 5-ring | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1C | 4.02 | 4.02 | 1.12 | 2.61 | 5.09 | 11.9 | 1.90 | 22.7 | 1.67 | 1.29 | 2.96 | nd c | nd | 29.6 |
| 2C | 7.88 | 7.88 | 1.69 | 4.95 | 6.88 | 13.0 | 1.71 | 28.2 | 1.53 | 1.27 | 2.80 | nd | nd | 38.9 |
| 3C | 9.41 | 9.41 | 1.89 | 4.87 | 7.85 | 19.0 | 3.56 | 37.2 | 2.97 | 2.46 | 5.43 | nd | nd | 52.0 |
| 4C | 10.2 | 10.2 | 1.68 | 3.88 | 7.85 | 16.2 | 3.12 | 32.8 | 3.86 | 2.48 | 6.35 | nd | nd | 49.4 |
| 5E | 28.2 | 28.2 | 2.68 | 3.76 | 9.95 | 24.7 | 2.72 | 43.8 | 1.76 | 1.83 | 3.59 | nd | nd | 75.6 |
| 6E | 17.9 | 17.9 | 2.66 | 6.16 | 14.1 | 26.0 | 3.47 | 52.4 | 2.24 | 2.22 | 4.47 | nd | nd | 74.7 |
| 7E | 18.1 | 18.1 | 2.39 | 4.71 | 11.8 | 22.6 | 2.88 | 44.4 | 1.51 | 1.58 | 3.09 | nd | nd | 65.5 |
| 8E | 24.5 | 24.5 | 3.36 | 6.60 | 11.8 | 27.4 | 4.40 | 53.6 | 2.35 | 2.56 | 4.90 | nd | nd | 83.0 |
| 9E | 29.1 | 29.1 | 2.33 | 3.85 | 9.42 | 20.8 | 3.08 | 39.5 | 1.94 | 2.18 | 4.11 | nd | nd | 72.7 |
| 10E | 25.5 | 25.5 | 2.72 | 4.23 | 11.3 | 23.1 | 3.46 | 44.8 | 1.59 | 1.56 | 3.15 | nd | nd | 73.4 |
| 11E | 25.6 | 25.6 | 1.88 | 2.70 | 7.96 | 15.2 | 2.40 | 30.1 | 1.44 | 1.26 | 2.69 | nd | nd | 58.4 |
| 12C | 19.0 | 19.0 | 1.90 | 5.54 | 5.93 | 14.4 | 2.06 | 29.8 | 1.25 | 1.28 | 2.53 | nd | nd | 51.3 |
| 13C | 19.0 | 19.0 | 1.63 | 5.16 | 5.73 | 11.0 | 1.87 | 25.4 | 1.07 | 0.95 | 2.01 | nd | nd | 46.5 |
| 14C | 22.1 | 22.1 | 1.53 | 3.72 | 5.57 | 13.6 | 2.00 | 26.4 | 1.48 | 1.45 | 2.93 | nd | nd | 51.5 |
| 15C | 22.0 | 22.0 | 1.77 | 5.29 | 6.63 | 10.6 | 1.64 | 26.0 | 1.04 | 0.72 | 1.77 | nd | nd | 49.8 |
| 16C | 38.1 | 38.1 | 1.85 | 4.88 | 6.88 | 12.5 | 1.96 | 28.1 | 1.03 | 0.85 | 1.88 | 0.59 | 0.59 | 68.6 |
| 17E | 31.4 | 31.4 | 1.37 | 2.43 | 5.42 | 10.7 | 2.35 | 22.3 | 2.05 | 1.73 | 3.78 | 2.52 | 2.52 | 60.0 |
| 18E | 31.2 | 31.2 | 1.69 | 5.11 | 5.89 | 10.2 | 1.47 | 24.4 | 1.31 | 1.03 | 2.34 | nd | nd | 57.9 |
| 19E | 36.9 | 36.9 | 2.75 | 5.24 | 9.84 | 17.3 | 2.64 | 37.8 | 1.42 | 1.32 | 2.74 | nd | nd | 77.4 |
| 20E | 31.2 | 31.2 | 1.59 | 2.17 | 8.73 | 20.8 | 4.01 | 37.3 | 2.99 | 3.07 | 6.06 | 4.40 | 4.40 | 78.9 |
| 21E | 33.8 | 33.8 | 1.76 | 3.34 | 8.21 | 18.2 | 3.04 | 34.5 | 1.59 | 1.45 | 3.04 | nd | nd | 71.4 |
| 22E | 37.2 | 37.2 | 2.95 | 5.89 | 12.6 | 26.2 | 2.16 | 49.8 | 1.84 | 1.88 | 3.72 | nd | nd | 90.8 |
| 23E | 32.1 | 32.1 | 1.14 | 1.46 | 6.85 | 13.9 | 1.27 | 24.7 | 1.62 | 1.48 | 3.11 | nd | nd | 59.9 |
| 24E | 33.9 | 33.9 | 2.67 | 4.07 | 9.63 | 16.1 | 2.55 | 35.0 | 1.41 | 1.23 | 2.64 | nd | nd | 71.6 |
| 25C | 26.9 | 26.9 | 1.58 | 2.23 | 6.81 | 12.1 | 2.01 | 24.8 | 1.43 | 1.12 | 2.54 | nd | nd | 54.1 |
| 26C | 29.6 | 29.6 | 1.31 | 3.50 | 5.52 | 10.7 | 1.84 | 22.9 | 1.09 | 1.04 | 2.13 | nd | nd | 54.6 |
| 27C | 29.1 | 29.1 | 1.89 | 4.04 | 7.13 | 13.3 | 1.32 | 27.6 | 1.25 | 1.03 | 2.28 | nd | nd | 59.0 |
| DR a | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 11% | 11% | 100% |
| Min | 4.02 | 4.02 | 1.12 | 1.46 | 5.09 | 10.2 | 1.27 | 22.3 | 1.03 | 0.72 | 1.77 | 0.59 | nd | 29.6 |
| Max | 38.1 | 38.1 | 3.36 | 6.60 | 14.1 | 27.4 | 4.40 | 53.6 | 3.86 | 3.07 | 6.35 | 4.40 | 4.40 | 90.8 |
| Median | 26.9 | 26.9 | 1.85 | 4.07 | 7.85 | 15.2 | 2.35 | 30.1 | 1.53 | 1.45 | 2.96 | 2.52 | nd | 59.9 |
| Mean | 25.0 | 25.0 | 1.99 | 4.16 | 8.20 | 16.7 | 2.48 | 33.6 | 1.73 | 1.57 | 3.30 | 2.50 | 0.28 | 62.1 |
| SD b | 9.10 | 9.10 | 0.58 | 1.30 | 2.42 | 5.33 | 0.81 | 9.32 | 0.65 | 0.58 | 1.20 | 1.55 | 0.94 | 13.9 |

Note: DR a = detection rate; SD b = standard deviation; nd c = not detection. The PAHs of CHR, BaA, BbF, BkF, Ind, DiB and BghiP were not detected or below the MDLs.

# Table. S5. Total pollution levels (ng/L) of PAHs in the winter waters of the Beibu Gulf estuary and coastal

| Sample No. | NAP | 2-ring | ACEY | ACE | FLU | PHE | ANTH | 3-ring | FLUA | PYR | 4-ring | BaP | 5-ring | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1C | 63.5 | 63.5 | 1.30 | 8.94 | 5.58 | 10.2 | 0.83 | 26.8 | 1.29 | 1.11 | 2.40 | 0.43 | 0.43 | 93.1 |
| 2C | 60.1 | 60.1 | 0.93 | 7.59 | 5.50 | 9.37 | 1.31 | 24.7 | 1.51 | 0.79 | 2.29 | nd c | nd | 87.1 |
| 3C | 69.8 | 69.8 | 1.43 | 7.92 | 5.67 | 8.57 | 1.35 | 24.9 | 1.78 | 0.83 | 2.61 | nd | nd | 97.4 |
| 4C | 64.4 | 64.4 | 1.43 | 1.45 | 5.07 | 7.62 | 1.03 | 16.6 | 1.08 | 0.66 | 1.74 | nd | nd | 82.7 |
| 5E | 57.3 | 57.3 | 1.41 | 1.56 | 5.65 | 6.81 | 0.88 | 16.3 | 1.93 | 0.52 | 2.45 | nd | nd | 76.1 |
| 6E | 59.9 | 59.9 | 1.30 | 1.54 | 5.78 | 8.75 | 1.09 | 18.5 | 1.21 | 0.93 | 2.14 | nd | nd | 80.5 |
| 7E | 61.3 | 61.3 | 1.75 | 1.59 | 5.90 | 9.87 | 0.97 | 20.1 | 2.23 | 1.07 | 3.31 | nd | nd | 84.6 |
| 8E | 60.8 | 60.8 | 1.63 | 1.85 | 6.72 | 9.19 | 0.98 | 20.4 | 1.12 | 1.01 | 2.14 | 0.57 | 0.57 | 83.9 |
| 9E | 59.3 | 59.3 | 1.41 | 2.04 | 5.50 | 7.24 | 0.93 | 17.1 | 1.46 | 0.48 | 1.94 | nd | nd | 78.4 |
| 10E | 68.8 | 68.8 | 1.64 | 1.87 | 5.92 | 8.16 | 0.88 | 18.5 | 1.75 | 0.84 | 2.60 | nd | nd | 89.9 |
| 11E | 77.3 | 77.3 | 1.87 | 1.83 | 7.12 | 10.2 | 1.34 | 22.4 | 3.86 | 1.01 | 4.87 | nd | nd | 105 |
| 12C | 92.7 | 92.7 | 2.22 | 3.00 | 8.07 | 9.31 | 0.94 | 23.5 | 1.41 | 1.09 | 2.50 | nd | nd | 119 |
| 13C | 47.0 | 47.0 | 1.04 | 1.89 | 5.72 | 8.87 | 0.76 | 18.3 | 0.95 | 0.75 | 1.69 | nd | nd | 67.0 |
| 14C | 45.0 | 45.0 | 1.31 | 1.50 | 5.33 | 8.85 | 1.45 | 18.4 | 1.30 | 0.90 | 2.20 | nd | nd | 65.6 |
| 15C | 52.8 | 52.8 | 1.10 | 1.47 | 5.10 | 8.81 | 1.14 | 17.6 | 1.51 | 1.02 | 2.54 | nd | nd | 72.9 |
| 16C | 95.7 | 95.7 | 1.61 | 3.32 | 9.48 | 11.6 | 1.45 | 27.4 | 2.79 | 2.26 | 5.04 | 0.56 | 0.56 | 129 |
| 17E | 115 | 115 | 2.09 | 3.99 | 11.2 | 15.3 | 1.62 | 34.2 | 5.05 | 2.86 | 7.91 | 0.79 | 0.79 | 158 |
| 18E | 112 | 112 | 1.98 | 3.22 | 10.5 | 16.2 | 1.40 | 33.3 | 3.32 | 2.27 | 5.59 | 0.61 | 0.61 | 152 |
| 19E | 65.3 | 65.3 | 1.25 | 5.21 | 6.60 | 8.08 | 0.95 | 22.1 | 0.92 | 0.75 | 1.67 | 0.52 | 0.52 | 89.5 |
| 20E | 66.5 | 66.5 | 1.30 | 1.71 | 6.27 | 8.16 | 1.05 | 18.5 | 1.16 | 0.51 | 1.67 | nd | nd | 86.7 |
| 21E | 69.3 | 69.3 | 1.42 | 6.05 | 5.18 | 8.35 | 1.11 | 22.1 | 3.29 | 0.80 | 4.09 | nd | nd | 95.5 |
| 22E | 66.1 | 66.1 | 1.52 | 2.26 | 5.69 | 9.39 | 1.27 | 20.1 | 1.41 | 0.70 | 2.10 | nd | nd | 88.3 |
| 23E | 65.2 | 65.2 | 1.07 | 4.53 | 6.82 | 11.1 | 1.31 | 24.9 | 1.04 | 0.82 | 1.86 | nd | nd | 91.9 |
| 24E | 63.1 | 63.1 | 1.38 | 9.75 | 5.23 | 8.85 | 1.43 | 26.6 | 0.90 | 0.88 | 1.77 | nd | nd | 91.5 |
| 25C | 62.4 | 62.4 | 1.41 | 6.11 | 5.87 | 12.1 | 1.67 | 27.2 | 2.80 | 0.88 | 3.68 | nd | nd | 93.2 |
| 26C | 74.3 | 74.3 | 1.10 | 3.05 | 5.78 | 8.54 | 0.76 | 19.2 | 1.31 | 0.98 | 2.29 | nd | nd | 95.8 |
| 27C | 71.7 | 71.7 | 1.38 | 5.64 | 6.56 | 9.72 | 1.37 | 24.7 | 1.17 | 0.73 | 1.90 | nd | nd | 98.3 |
| DR a | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 19% | 19% | 100% |
| Min | 45.0 | 45.0 | 0.93 | 1.45 | 5.07 | 6.81 | 0.76 | 16.3 | 0.90 | 0.48 | 1.67 | 0.43 | nd | 65.6 |
| Max | 115 | 115 | 2.22 | 9.75 | 11.2 | 16.2 | 1.67 | 34.2 | 5.05 | 2.86 | 7.91 | 0.79 | 0.79 | 158 |
| Median | 65.2 | 65.2 | 1.41 | 3.00 | 5.78 | 8.87 | 1.11 | 22.1 | 1.41 | 0.88 | 2.29 | 0.57 | nd | 89.9 |
| Mean | 69.1 | 69.1 | 1.45 | 3.74 | 6.44 | 9.60 | 1.16 | 22.4 | 1.84 | 1.02 | 2.85 | 0.58 | 0.13 | 94.5 |
| SD b | 16.6 | 16.6 | 0.31 | 2.49 | 1.56 | 2.11 | 0.26 | 4.69 | 1.02 | 0.55 | 1.45 | 0.11 | 0.25 | 21.6 |

Note: DR a = detection rate; SD b = standard deviation; nd c = not detection. The PAHs of CHR, BaA, BbF, BkF, Ind, DiB and BghiP were not detected or below the MDLs.

| River | ∑16PAHs (ng/L) |  |  | River flow a |  | Flux of 16 PAHs into the sea |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Representative site | summer | winter | summer and autumn (t/0.5a) | winter and spring (t/0.5a) | summer and autumn (kg/0.5a) | winter and spring (kg/0.5a) | Total PAHs( kg/a) |
| Nanliu River | 22 | 90.76 | 88.29 | 451533 | 55808 | 409.8 | 49.3 | 459.1 |
| Dafeng River | 18 | 57.89 | 151.76 | 74454 | 15250 | 43.1 | 23.1 | 66.2 |
| Qin River | 9 | 72.67 | 78.4 | 196845 | 40318 | 143.0 | 31.6 | 174.7 |
| Maoling River | 5 | 75.58 | 76.1 | 138101 | 28286 | 104.4 | 21.5 | 125.9 |
| Total |  |  |  | 860933 | 139662 | 700.3 | 125.5 | 825.9 |

# Table. S6. Flux of PAHs into the the Beibu Gulf through the river discharge

a Data are from the China Meteorological Data Network http://data.cma.cn/.

# Table. S7. Total pollution levels (ng/g) of PAHs in the sediment of the Beibu Gulf estuary and coastal

| Sample No. | NAP | ACEY | ACE | FlU | PHE | ANTH | FLUA | PYR | CHR | BaA | BbF | BkF | BaP | DiB | Ind | BghiP | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1C | 0.81 | 0.05 | 0.11 | 0.52 | 1.84 | nd | 0.82 | 1.08 | 0.38 | 0.82 | 0.46 | 0.72 | 0.96 | 0.15 | 2.01 | 0.80 | 11.5 |
| 2C | 0.44 | 0.06 | 0.18 | 0.72 | 3.03 | 0.31 | 1.40 | 1.81 | 0.80 | 1.27 | 0.68 | 1.00 | 1.58 | 0.32 | 3.06 | 1.07 | 17.7 |
| 3C | 10.9 | 1.59 | 4.87 | 13.5 | 50.2 | 4.16 | 24.9 | 29.3 | 11.6 | 24.5 | 8.82 | 11.7 | 21.0 | 3.97 | 26.7 | 14.0 | 262 |
| 4C | 17.9 | 0.63 | 3.23 | 15.8 | 46.6 | 2.75 | 12.1 | 8.49 | 7.40 | 20.6 | 5.10 | 4.54 | 19.0 | 3.69 | 14.3 | 12.1 | 194 |
| 5E | 3.84 | 0.41 | 0.94 | 2.50 | 20.6 | 2.92 | 36.0 | 41.3 | 19.6 | 27.8 | 13.8 | 23.7 | 33.2 | 5.82 | 64.8 | 22.2 | 319 |
| 6E | 2.03 | 0.22 | 0.30 | 1.31 | 5.53 | 0.52 | 6.20 | 4.85 | 2.48 | 4.92 | 2.05 | 2.87 | 5.02 | 0.88 | 8.16 | 3.22 | 50.6 |
| 7E | 1.06 | 0.28 | 0.60 | 1.66 | 11.4 | 2.48 | 17.4 | 20.3 | 9.92 | 12.8 | 6.98 | 11.0 | 15.8 | 2.82 | 31.0 | 10.6 | 156 |
| 8E | 2.31 | 1.39 | 0.64 | 3.49 | 21.4 | 3.78 | 29.8 | 32.8 | 10.8 | 19.0 | 9.97 | 15.3 | 22.4 | 3.17 | 42.4 | 15.9 | 235 |
| 9E | 1.65 | 0.51 | 0.44 | 1.04 | 16.1 | 2.75 | 35.7 | 46.0 | 24.4 | 30.1 | 17.2 | 28.9 | 39.4 | 7.77 | 79.9 | 26.9 | 359 |
| 10E | 5.51 | 0.68 | 0.32 | 2.90 | 12.0 | 0.99 | 8.30 | 11.2 | 1.94 | 4.28 | 2.20 | 3.59 | 4.31 | 0.79 | 9.54 | 3.06 | 71.7 |
| 11E | 1.80 | 0.71 | 0.37 | 1.84 | 16.3 | 2.52 | 23.7 | 25.8 | 8.13 | 14.3 | 7.21 | 10.7 | 16.3 | 2.67 | 32.0 | 11.7 | 176 |
| 12C | 1.97 | 0.20 | 0.20 | 1.12 | 5.39 | 0.60 | 4.03 | 4.11 | 1.52 | 2.91 | 1.60 | 2.40 | 3.53 | 0.46 | 6.53 | 2.29 | 38.9 |
| 13C | 22.2 | 1.50 | 2.45 | 11.6 | 51.4 | 4.08 | 22.2 | 26.7 | 8.22 | 24.4 | 12.4 | 14.9 | 29.4 | 5.21 | 43.4 | 16.9 | 297 |
| 14C | 1.02 | 0.05 | 0.07 | 0.50 | 1.62 | nd | 0.29 | 0.30 | nd | 0.19 | nd | 0.23 | 0.22 | nd | 0.60 | 0.23 | 5.32 |
| 15C | 0.47 | nd c | 0.05 | 0.22 | 0.55 | nd | 0.11 | 0.11 | nd | 0.09 | nd | 0.16 | 0.14 | nd | 0.36 | 0.11 | 2.36 |
| 16C | 0.09 | nd | 0.04 | 0.22 | 1.04 | nd | 0.23 | 0.24 | 0.05 | 0.13 | nd | nd | 0.15 | nd | 0.40 | 0.18 | 2.76 |
| 17E | 2.03 | 0.22 | 0.33 | 2.01 | 6.93 | 0.57 | 3.39 | 4.02 | 2.08 | 2.11 | 2.31 | 3.64 | 3.84 | 0.44 | 10.8 | 3.63 | 48.4 |
| 18E | 1.15 | 0.12 | 0.18 | 1.01 | 3.54 | 0.43 | 1.83 | 1.87 | 0.60 | 1.08 | 0.71 | 1.10 | 1.36 | 0.23 | 3.24 | 1.18 | 19.6 |
| 19E | 3.10 | 0.26 | 0.45 | 2.87 | 9.38 | 1.18 | 9.15 | 4.55 | 1.67 | 4.38 | 1.71 | 2.55 | 3.91 | 0.54 | 7.04 | 2.61 | 55.3 |
| 20E | 10.5 | 1.00 | 1.25 | 1.86 | 16.9 | 1.96 | 13.1 | 13.3 | 3.85 | 8.33 | 3.49 | 5.19 | 7.84 | 1.04 | 15.6 | 6.75 | 112 |
| 21C | 3.01 | 0.42 | 0.20 | 1.56 | 7.96 | 0.67 | 6.74 | 5.88 | 1.44 | 2.45 | 1.57 | 2.66 | 3.41 | 0.38 | 8.37 | 3.54 | 50.3 |
| 22C | 0.91 | 0.05 | 0.06 | 0.33 | 1.09 | 0.16 | 0.55 | 0.50 | 0.22 | 0.39 | 0.24 | 0.42 | 0.51 | nd | 1.26 | 0.50 | 7.20 |
| 23C | 1.40 | 0.12 | 0.12 | 1.69 | 3.35 | 0.72 | 2.52 | 2.41 | 1.38 | 2.24 | 1.39 | 2.33 | 3.30 | 0.53 | 7.16 | 2.66 | 33.3 |
| DR a | 100% | 91% | 100% | 100% | 100% | 83% | 100% | 100% | 91% | 100% | 87% | 96% | 100% | 83% | 100% | 100% | 100% |
| Min | 0.09 | nd | 0.04 | 0.22 | 0.55 | nd | 0.11 | 0.11 | nd | 0.09 | nd | nd | 0.14 | nd | 0.36 | 0.11 | 2.36 |
| Max | 22.2 | 1.59 | 4.87 | 15.8 | 51.4 | 4.16 | 36.0 | 46.0 | 24.4 | 30.1 | 17.2 | 28.9 | 39.4 | 7.77 | 79.9 | 26.9 | 359 |
| Median | 1.97 | 0.26 | 0.32 | 1.66 | 7.96 | 0.72 | 6.74 | 4.85 | 1.94 | 4.28 | 2.05 | 2.87 | 3.91 | 0.54 | 8.37 | 3.22 | 50.6 |
| Mean | 4.18 | 0.45 | 0.76 | 3.06 | 13.7 | 1.46 | 11.3 | 12.5 | 5.15 | 9.10 | 4.34 | 6.50 | 10.3 | 1.78 | 18.2 | 7.05 | 110 |
| SD b | 5.66 | 0.48 | 1.17 | 4.24 | 15.2 | 1.38 | 11.6 | 13.9 | 6.42 | 9.98 | 4.88 | 7.69 | 11.6 | 2.16 | 21.2 | 7.57 | 112 |

Note: DR a = detection rate; SD b = standard deviation; nd c = not detection.

# Table. S8. Total pollution levels (ng/g) of PAHs in the marine organisms of the Beibu Gulf.

| Sample. No | NAP | ACEY | ACE | FLU | PHE | ANTH | FLUA | PYR | CHR | BaA | BbF | BkF | BaP | DiB | Ind | BghiP | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q1 | 7.61 | 0.76 | 1.23 | 3.95 | 10.7 | 1.01 | 2.23 | 2.03 | 0.73 | 1.00 | 0.62 | 1.66 | 1.79 | 3.72 | 0.67 | 1.29 | 41.0 |
| Q2 | 7.92 | 0.84 | 1.26 | 3.67 | 11.9 | 1.21 | 2.70 | 2.39 | 0.61 | 0.86 | 0.57 | 1.38 | 1.41 | 3.63 | 0.37 | 1.20 | 41.9 |
| Q3 | 8.57 | 0.93 | 1.64 | 5.75 | 14.7 | 1.27 | 3.22 | 2.75 | 0.62 | 0.90 | 0.45 | 1.25 | 1.21 | 3.39 | 0.24 | 1.23 | 48.1 |
| Q4 | 14.8 | 1.47 | 2.39 | 9.21 | 25.9 | 2.15 | 3.78 | 6.06 | 0.43 | 1.14 | 0.26 | 0.58 | 0.61 | 1.93 | 0.26 | 0.74 | 71.7 |
| Q1 | 14.5 | 1.13 | 2.15 | 8.40 | 22.7 | 2.83 | 3.06 | 4.93 | 0.22 | 0.55 | 0.13 | 0.29 | 0.31 | 1.08 | 0.13 | 0.38 | 62.7 |
| Q2 | 17.6 | 1.11 | 1.79 | 7.02 | 18.1 | 2.07 | 2.68 | 4.09 | 0.23 | 0.53 | 0.11 | 0.25 | 0.29 | 1.03 | 0.09 | 0.42 | 57.4 |
| Q3 | 10.2 | 0.91 | 1.97 | 7.25 | 15.3 | 0.98 | 3.65 | 3.66 | 0.41 | 0.74 | 0.18 | 0.27 | 0.43 | 0.38 | 0.04 | 0.37 | 46.7 |
| Q4 | 16.5 | 0.58 | 1.08 | 3.51 | 9.09 | 0.78 | 0.91 | 1.16 | 0.17 | 0.31 | 0.12 | 0.27 | 0.34 | 0.97 | 0.14 | 0.55 | 36.5 |
| Q1 | 16.0 | 0.67 | 1.14 | 4.63 | 9.16 | 0.54 | 1.21 | 1.16 | 0.23 | 0.30 | 0.16 | 0.24 | 0.37 | 1.36 | 0.12 | 0.59 | 37.9 |
| Q2 | 12.1 | 0.41 | 0.61 | 3.05 | 10.5 | 1.24 | 0.75 | 1.04 | 0.16 | 0.21 | 0.09 | 0.13 | 0.19 | 0.25 | 0.10 | 0.35 | 31.2 |
| Q3 | 10.1 | 0.52 | 0.90 | 2.80 | 6.80 | 0.71 | 0.98 | 0.99 | 0.17 | 0.21 | 0.06 | 0.17 | 0.22 | 0.29 | 0.07 | 0.39 | 25.4 |
| Q4 | 7.86 | 0.55 | 0.84 | 3.31 | 7.01 | 0.69 | 0.82 | 0.87 | 0.16 | 0.19 | 0.08 | 0.20 | 0.21 | 0.37 | 0.04 | 0.38 | 23.6 |
| Q1 | 9.07 | 0.95 | 1.57 | 10.7 | 21.8 | 4.78 | 10.0 | 12.4 | 1.23 | 7.60 | 1.34 | 0.96 | 2.77 | 0.63 | 0.16 | 1.00 | 86.9 |
| Q2 | 9.67 | 0.97 | 1.18 | 5.70 | 18.1 | 1.80 | 11.1 | 14.1 | 1.36 | 8.04 | 1.09 | 0.74 | 2.34 | 0.55 | 0.24 | 0.91 | 77.9 |
| Q3 | 10.6 | 0.86 | 1.33 | 5.37 | 16.6 | 2.10 | 10.7 | 13.7 | 1.65 | 9.44 | 1.77 | 1.77 | 3.65 | 0.86 | 0.53 | 1.36 | 82.4 |
| Q4 | 7.23 | 0.85 | 0.94 | 4.55 | 26.3 | 2.00 | 17.8 | 19.1 | 1.59 | 4.64 | 0.62 | 0.79 | 1.95 | 0.49 | 0.01 | 1.15 | 89.9 |
| B1 | 10.3 | 1.37 | 2.11 | 7.13 | 19.0 | 1.88 | 2.00 | 2.44 | 0.29 | 0.51 | 0.13 | 0.22 | 0.02 | 0.05 | 0.03 | 0.25 | 47.7 |
| B2 | 12.2 | 1.29 | 1.80 | 5.81 | 13.8 | 1.61 | 1.10 | 1.48 | 0.17 | 0.30 | 0.07 | 0.15 | 0.02 | 0.03 | 0.01 | 0.01 | 39.8 |
| B3 | 29.5 | 1.64 | 1.78 | 5.93 | 11.7 | 1.18 | 1.16 | 1.46 | 0.23 | 0.38 | 0.02 | 0.27 | 0.02 | 0.05 | 0.06 | 0.02 | 55.4 |
| B4 | 23.6 | 0.89 | 1.58 | 5.38 | 10.7 | 1.02 | 1.10 | 1.43 | 0.20 | 0.36 | 0.13 | 0.20 | 0.02 | 0.08 | 0.08 | 0.01 | 46.8 |
| B5 | 22.1 | 1.21 | 1.03 | 4.03 | 6.93 | 0.84 | 0.75 | 0.80 | 0.02 | 0.27 | 0.05 | 0.12 | 0.01 | 0.06 | 0.03 | 0.04 | 38.3 |
| B6 | 17.6 | 0.81 | 0.78 | 2.07 | 3.80 | 0.37 | 0.42 | 0.51 | 0.01 | 0.09 | 0.01 | 0.13 | 0.01 | 0.07 | 0.03 | 0.05 | 26.7 |
| B7 | 17.2 | 1.22 | 2.56 | 5.24 | 10.8 | 0.62 | 1.57 | 1.45 | 0.02 | 0.36 | 0.00 | 0.24 | 0.01 | 0.09 | 0.05 | 0.01 | 41.5 |
| B8 | 17.5 | 1.28 | 3.23 | 1.08 | 12.6 | 1.06 | 1.62 | 1.65 | 0.04 | 0.71 | 0.00 | 0.35 | 0.15 | 0.09 | 0.03 | 0.23 | 41.6 |
| B9 | 9.56 | 0.95 | 3.34 | 0.69 | 13.9 | 1.69 | 1.66 | 1.85 | 0.30 | 0.57 | 0.00 | 0.02 | 0.02 | 0.17 | 0.03 | 0.37 | 35.1 |
| B10 | 8.73 | 0.68 | 1.07 | 3.91 | 8.75 | 1.15 | 0.64 | 0.73 | 0.02 | 0.01 | 0.01 | 0.02 | 0.01 | 0.03 | 0.02 | 0.03 | 25.8 |
| B11 | 8.58 | 0.44 | 1.88 | 1.37 | 15.0 | 3.84 | 1.20 | 0.03 | 0.10 | 0.01 | 0.01 | 0.02 | 0.03 | 0.09 | 0.09 | 0.14 | 32.8 |
| B12 | 17.4 | 0.59 | 2.47 | 10.1 | 23.4 | 10.00 | 15.1 | 1.38 | 1.83 | 2.02 | 0.06 | 0.09 | 9.66 | 0.15 | 0.53 | 2.12 | 96.9 |
| B13 | 24.7 | 2.01 | 3.68 | 23.3 | 199 | 19.9 | 26.8 | 30.0 | 10.5 | 6.40 | 0.07 | 1.00 | 12.9 | 0.20 | 0.42 | 1.61 | 363 |
| B14 | 21.0 | 1.90 | 3.43 | 1.36 | 195 | 24.3 | 23.9 | 16.5 | 10.1 | 5.44 | 0.13 | 1.09 | 15.0 | 0.36 | 0.75 | 2.34 | 323 |
| F1 | 7.41 | 1.37 | 2.55 | 1.69 | 154 | 10.9 | 6.75 | 27.4 | 3.52 | 4.93 | 0.26 | 1.00 | 3.05 | 0.08 | 0.07 | 0.04 | 225 |
| F2 | 1.18 | 0.25 | 0.80 | 0.24 | 12.6 | 1.31 | 1.60 | 0.28 | 0.11 | 0.03 | 0.01 | 0.04 | 0.30 | 0.03 | 0.07 | 0.12 | 19.0 |
| F3 | 5.51 | 0.55 | 0.83 | 4.66 | 9.32 | 0.83 | 1.26 | 0.06 | 0.04 | 0.01 | 0.00 | 0.02 | 0.21 | 0.10 | 0.04 | 0.25 | 23.7 |
| F4 | 5.16 | 0.61 | 1.15 | 0.38 | 10.4 | 0.58 | 0.89 | 0.02 | 0.00 | 0.01 | 0.01 | 0.01 | 0.04 | 0.01 | 0.00 | 0.01 | 19.3 |
| F5 | 2.90 | 0.32 | 1.31 | 0.23 | 8.12 | 0.82 | 1.17 | 0.05 | 0.01 | 0.02 | 0.01 | 0.02 | 0.07 | 0.02 | 0.02 | 0.18 | 15.3 |
| F6 | 4.22 | 0.28 | 1.24 | 0.17 | 7.02 | 0.89 | 1.04 | 0.05 | 0.01 | 0.02 | 0.00 | 0.02 | 0.12 | 0.06 | 0.01 | 0.12 | 15.3 |
| F7 | 8.85 | 0.71 | 1.97 | 0.87 | 15.1 | 0.90 | 1.84 | 0.12 | 0.03 | 0.06 | 0.01 | 0.01 | 1.36 | 0.14 | 0.04 | 0.82 | 32.8 |
| F8 | 9.12 | 0.86 | 2.00 | 0.32 | 14.4 | 1.54 | 2.01 | 0.07 | 0.01 | 0.01 | 0.01 | 0.02 | 0.51 | 0.06 | 0.01 | 0.68 | 31.6 |
| F9 | 14.5 | 0.79 | 46.9 | 1.37 | 10.7 | 1.26 | 1.15 | 0.04 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 | 0.04 | 0.02 | 0.08 | 76.9 |
| F10 | 11.9 | 0.73 | 29.8 | 0.50 | 13.3 | 1.70 | 1.50 | 0.04 | 0.01 | 0.04 | 0.00 | 0.01 | 0.03 | 0.07 | 0.04 | 0.21 | 60.0 |
| F11 | 9.78 | 0.42 | 2.35 | 0.35 | 9.53 | 1.23 | 1.12 | 0.08 | 0.04 | 0.01 | 0.01 | 0.01 | 0.27 | 0.07 | 0.02 | 0.11 | 25.4 |
| F12 | 18.6 | 0.66 | 2.30 | 0.63 | 23.0 | 2.74 | 6.22 | 0.58 | 0.58 | 0.03 | 0.03 | 0.06 | 3.23 | 0.41 | 0.07 | 1.59 | 60.7 |
| F13 | 375 | 18.4 | 46.7 | 23.5 | 508 | 38.3 | 168 | 22.2 | 1.64 | 2.36 | 0.62 | 0.73 | 51.9 | 6.61 | 0.65 | 17.5 | 1283 |
| F14 | 26.6 | 3.58 | 4.91 | 2.98 | 357 | 24.4 | 29.4 | 78.7 | 3.14 | 6.28 | 1.34 | 1.10 | 18.3 | 0.11 | 0.09 | 0.96 | 559 |
| F15 | 9.18 | 1.09 | 1.31 | 0.50 | 102 | 7.05 | 5.79 | 16.5 | 0.14 | 0.39 | 0.38 | 0.58 | 3.09 | 0.24 | 0.04 | 0.25 | 149 |
| F16 | 5.99 | 0.43 | 0.89 | 3.33 | 8.96 | 1.53 | 1.37 | 0.17 | 0.01 | 0.02 | 0.01 | 0.02 | 0.53 | 0.18 | 0.03 | 0.28 | 23.7 |
| F17 | 15.8 | 1.19 | 2.18 | 0.62 | 63.4 | 6.27 | 2.41 | 8.00 | 0.36 | 1.08 | 0.10 | 0.19 | 1.25 | 0.46 | 0.10 | 0.12 | 104 |
| F18 | 16.1 | 1.43 | 3.14 | 2.29 | 131 | 15.9 | 7.17 | 15.2 | 0.72 | 1.96 | 0.46 | 0.37 | 2.29 | 0.47 | 0.03 | 0.07 | 198 |
| DR a | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Min | 1.18 | 0.25 | 0.61 | 0.17 | 3.80 | 0.37 | 0.42 | 0.02 | 0.00 | 0.01 | 0.00 | 0.01 | 0.01 | 0.01 | 0.00 | 0.01 | 15.3 |
| Max | 375 | 18.4 | 46.9 | 23.5 | 508 | 38.3 | 168 | 78.7 | 10.5 | 9.44 | 1.77 | 1.77 | 51.9 | 6.61 | 0.75 | 17.5 | 1283 |
| Median | 10.5 | 0.86 | 1.79 | 3.42 | 13.8 | 1.42 | 1.75 | 1.45 | 0.21 | 0.37 | 0.08 | 0.21 | 0.33 | 0.17 | 0.06 | 0.36 | 44.3 |
| Mean | 20.2 | 1.32 | 4.27 | 4.39 | 46.8 | 4.45 | 8.25 | 6.70 | 0.92 | 1.49 | 0.24 | 0.40 | 2.97 | 0.66 | 0.14 | 0.89 | 104 |
| SD b | 52.2 | 2.56 | 9.77 | 4.83 | 93.7 | 7.60 | 24.3 | 13.0 | 2.11 | 2.43 | 0.40 | 0.47 | 8.12 | 1.24 | 0.19 | 2.48 | 199 |

Note: DR a = detection rate; SD b = standard deviation; Q = Qinzhou, B = Beihai, F = Fangcheng Port.

| Region | Classes | NAP | ACEY | ACE | FLU | PHE | ANTH | FLUA | PYR | CHR | BaA | BbF | BkF | BaP | Ind | DiB | BghiP | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qinzhou | Fish | 2.10 | 2.03 | 1.97 | 2.22 | 2.32 | 2.15 | 2.66 | 2.74 | 2.79 | 3.23 | 2.67 | 3.32 | 2.48 | 2.59 | 2.62 | 3.23 | 2.30 |
|  | Carb | 2.06 | 1.76 | 1.68 | 2.03 | 2.06 | 2.04 | 2.51 | 2.58 | 2.61 | 3.44 | 2.59 | 2.88 | 2.33 | 2.01 | 2.19 | 3.00 | 2.13 |
|  | Shellfish | 1.74 | 1.71 | 1.53 | 1.87 | 2.14 | 1.97 | 3.03 | 3.10 | 3.04 | 3.93 | 2.96 | 3.20 | 2.71 | 1.80 | 2.37 | 3.12 | 2.24 |
| Beihai | Fish | 2.32 | 2.13 | 1.98 | 2.19 | 2.23 | 2.09 | 2.30 | 2.40 | 2.47 | 2.91 | 2.10 | 2.73 | 0.82 | 0.99 | 1.84 | 2.19 | 2.25 |
|  | Carb | 2.25 | 2.02 | 1.95 | 1.85 | 1.97 | 1.74 | 2.16 | 2.16 | 1.44 | 2.82 | 1.28 | 2.68 | 1.14 | 1.09 | 1.67 | 2.18 | 2.09 |
|  | Shrimp | 1.98 | 1.85 | 2.03 | 1.70 | 2.17 | 2.27 | 2.23 | 2.09 | 2.26 | 2.60 | 1.00 | 1.69 | 0.87 | 1.22 | 1.88 | 2.56 | 2.06 |
|  | Shellfish | 2.10 | 1.94 | 1.97 | 2.21 | 2.97 | 2.93 | 3.26 | 3.11 | 3.73 | 3.73 | 1.83 | 3.02 | 3.38 | 1.37 | 2.71 | 3.37 | 2.73 |
| Fang cheng | Fish | 1.67 | 1.93 | 1.95 | 1.40 | 3.01 | 2.72 | 2.80 | 3.31 | 3.38 | 3.72 | 2.29 | 3.13 | 2.77 | 1.00 | 2.05 | 2.21 | 2.66 |
|  | Carb | 1.76 | 1.71 | 1.82 | 1.40 | 2.06 | 1.85 | 2.26 | 0.91 | 1.33 | 1.64 | 0.96 | 1.57 | 2.08 | 1.03 | 1.45 | 2.81 | 1.88 |
|  | Shrimp | 2.11 | 1.82 | 3.13 | 1.27 | 2.12 | 2.07 | 2.26 | 0.88 | 1.45 | 1.63 | 1.06 | 1.51 | 1.54 | 1.04 | 1.68 | 2.43 | 2.30 |
|  | Shellfish | 2.72 | 2.45 | 2.51 | 1.94 | 3.13 | 2.84 | 3.54 | 3.28 | 2.90 | 3.32 | 2.57 | 2.85 | 3.47 | 2.17 | 2.20 | 3.67 | 2.93 |
| Min |  | 2.44 | 1.67 | 1.71 | 1.53 | 1.27 | 1.97 | 1.74 | 2.16 | 0.88 | 1.33 | 1.63 | 0.96 | 1.51 | 0.82 | 0.99 | 1.45 | 2.18 |
| Max |  | 4.04 | 2.72 | 2.45 | 3.13 | 2.22 | 3.13 | 2.93 | 3.54 | 3.31 | 3.73 | 3.93 | 2.96 | 3.32 | 3.47 | 2.59 | 2.71 | 3.67 |
| Median |  | 2.88 | 2.07 | 1.94 | 2.05 | 1.82 | 2.38 | 2.24 | 2.64 | 2.41 | 2.49 | 3.00 | 1.94 | 2.60 | 2.15 | 1.48 | 2.06 | 2.80 |
| Mean |  | 2.76 | 2.10 | 1.93 | 1.97 | 1.87 | 2.17 | 2.09 | 2.51 | 2.58 | 2.61 | 3.23 | 2.10 | 2.85 | 2.33 | 1.22 | 2.05 | 2.81 |
| SD a |  | 0.41 | 0.28 | 0.21 | 0.41 | 0.33 | 0.41 | 0.39 | 0.45 | 0.82 | 0.77 | 0.75 | 0.71 | 0.65 | 0.90 | 0.54 | 0.38 | 0.50 |

# Table. S9. The average Log BWAFs of PAHs in the marine organisms from the Beibu Gulf.

Note: SD a = standard deviation; the bold represents >3.7 (are bioaccumulative).

| Region | Classes | NAP | ACEY | ACE | FLU | PHE | ANTH | FLUA | PYR | CHR | BaA | BbF | BkF | BaP | Ind | DiB | BghiP | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qinzhou | Fish | 2.56 | 2.36 | 2.57 | 2.49 | 2.15 | 2.02 | 1.34 | 1.36 | 0.85 | 0.89 | 0.78 | 0.98 | 0.83 | 0.95 | 1.06 | 0.97 | 1.61 |
|  | Carb | 2.26 | 1.83 | 2.03 | 2.04 | 1.64 | 1.66 | 0.94 | 0.95 | 0.42 | 0.83 | 0.44 | 0.27 | 0.42 | 0.12 | 0.37 | 0.49 | 1.19 |
|  | Shellfish | 2.56 | 2.41 | 2.49 | 2.51 | 2.33 | 2.21 | 2.09 | 2.09 | 1.47 | 1.95 | 1.43 | 1.22 | 1.42 | 0.53 | 1.17 | 1.23 | 1.92 |
| Beihai | Fish | 2.77 | 2.46 | 2.59 | 2.47 | 2.06 | 1.96 | 0.99 | 1.02 | 0.53 | 0.56 | 0.20 | 0.39 | -0.83 | -0.64 | 0.28 | -0.07 | 1.57 |
|  | Carb | 2.45 | 2.09 | 2.29 | 1.87 | 1.54 | 1.36 | 0.59 | 0.53 | -0.76 | 0.22 | -0.87 | 0.08 | -0.77 | -0.80 | -0.15 | -0.33 | 1.15 |
|  | Shrimp | 2.73 | 2.48 | 2.93 | 2.27 | 2.30 | 2.44 | 1.21 | 1.02 | 0.62 | 0.55 | -0.59 | -0.36 | -0.48 | -0.11 | 0.61 | 0.61 | 1.67 |
|  | Shellfish | 2.92 | 2.63 | 2.94 | 2.85 | 3.17 | 3.17 | 2.31 | 2.10 | 2.16 | 1.75 | 0.30 | 1.04 | 2.10 | 0.10 | 1.51 | 1.48 | 2.41 |
| Fang cheng | Fish | 2.13 | 2.26 | 2.55 | 1.67 | 2.84 | 2.59 | 1.48 | 1.93 | 1.44 | 1.37 | 0.39 | 0.78 | 1.12 | -0.64 | 0.48 | -0.04 | 1.98 |
|  | Carb | 1.96 | 1.78 | 2.17 | 1.42 | 1.64 | 1.46 | 0.68 | -0.72 | -0.87 | -0.97 | -1.19 | -1.03 | 0.17 | -0.86 | -0.37 | 0.30 | 0.94 |
|  | Shrimp | 2.86 | 2.45 | 4.03 | 1.84 | 2.25 | 2.24 | 1.25 | -0.19 | -0.19 | -0.42 | -0.53 | -0.54 | 0.19 | -0.29 | 0.42 | 0.48 | 1.91 |
|  | Shellfish | 3.54 | 3.14 | 3.48 | 2.58 | 3.32 | 3.08 | 2.59 | 2.27 | 1.33 | 1.34 | 1.04 | 0.87 | 2.19 | 0.91 | 1.00 | 1.79 | 2.62 |
| Min |  | 2.76 | 1.96 | 1.78 | 2.03 | 1.42 | 1.54 | 1.36 | 0.59 | -0.72 | -0.87 | -0.97 | -1.19 | -1.03 | -0.83 | -0.86 | -0.37 | -0.33 |
| Max |  | 4.06 | 3.54 | 3.14 | 4.03 | 2.85 | 3.32 | 3.17 | 2.59 | 2.27 | 2.16 | 1.95 | 1.43 | 1.22 | 2.19 | 0.95 | 1.51 | 1.79 |
| Median |  | 3.41 | 2.61 | 2.35 | 2.73 | 2.18 | 2.29 | 2.20 | 1.41 | 1.12 | 0.64 | 0.73 | 0.13 | 0.34 | 0.58 | -0.07 | 0.58 | 0.63 |
| Mean |  | 3.39 | 2.56 | 2.41 | 2.57 | 2.27 | 2.25 | 2.21 | 1.25 | 1.02 | 0.62 | 0.83 | 0.30 | 0.39 | 0.42 | -0.11 | 0.48 | 0.49 |
| SD a |  | 0.36 | 0.41 | 0.36 | 0.56 | 0.43 | 0.57 | 0.57 | 0.63 | 0.93 | 0.91 | 0.85 | 0.79 | 0.70 | 1.01 | 0.63 | 0.54 | 0.64 |

# Table. S10. The average Log BSAFs of PAHs in the the marine organisms from the Beibu Gulf.

Note: SD a = standard deviation.

| Region | Classes | NAP | ACEY | ACE | FLU | PHE | ANTH | FLUA | PYR | CHR | BaA | BbF | BkF | BaP | Ind | DiB | BghiP | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qinzhou | Fish | 2.91 | 0.26 | 0.45 | 1.63 | 4.28 | 0.41 | 0.77 | 0.93 | 0.12 | 0.21 | 0.08 | 0.20 | 0.22 | 0.54 | 0.06 | 0.20 | 13.3 |
|  | Carb | 2.66 | 0.14 | 0.23 | 1.04 | 2.39 | 0.32 | 0.54 | 0.65 | 0.08 | 0.33 | 0.07 | 0.07 | 0.15 | 0.14 | 0.02 | 0.12 | 8.96 |
|  | Shellfish | 1.27 | 0.12 | 0.16 | 0.72 | 2.81 | 0.27 | 1.82 | 2.16 | 0.21 | 1.02 | 0.16 | 0.15 | 0.36 | 0.09 | 0.04 | 0.16 | 11.5 |
| Beihai | Fish | 4.75 | 0.33 | 0.46 | 1.52 | 3.46 | 0.36 | 0.34 | 0.43 | 0.06 | 0.10 | 0.02 | 0.05 | 0.00 | 0.01 | 0.01 | 0.02 | 11.9 |
|  | Carb | 4.13 | 0.25 | 0.42 | 0.69 | 1.90 | 0.16 | 0.24 | 0.25 | 0.01 | 0.08 | 0.00 | 0.05 | 0.01 | 0.02 | 0.01 | 0.02 | 8.23 |
|  | Shrimp | 2.19 | 0.17 | 0.51 | 0.49 | 3.07 | 0.54 | 0.29 | 0.21 | 0.03 | 0.05 | 0.00 | 0.00 | 0.01 | 0.02 | 0.01 | 0.04 | 7.64 |
|  | Shellfish | 2.90 | 0.21 | 0.44 | 1.60 | 19.2 | 2.49 | 3.03 | 2.20 | 1.03 | 0.64 | 0.01 | 0.10 | 1.73 | 0.03 | 0.08 | 0.28 | 36.0 |
| Fang cheng | Fish | 1.08 | 0.20 | 0.42 | 0.24 | 21.0 | 1.54 | 1.05 | 3.48 | 0.46 | 0.62 | 0.03 | 0.13 | 0.42 | 0.01 | 0.02 | 0.02 | 30.7 |
|  | Carb | 1.33 | 0.12 | 0.31 | 0.25 | 2.38 | 0.21 | 0.30 | 0.01 | 0.00 | 0.01 | 0.00 | 0.00 | 0.09 | 0.01 | 0.00 | 0.08 | 5.11 |
|  | Shrimp | 2.95 | 0.16 | 6.45 | 0.18 | 2.73 | 0.34 | 0.31 | 0.01 | 0.01 | 0.01 | 0.00 | 0.00 | 0.02 | 0.02 | 0.01 | 0.03 | 13.2 |
|  | Shellfish | 12.0 | 0.67 | 1.55 | 0.85 | 27.6 | 2.04 | 5.83 | 3.26 | 0.15 | 0.25 | 0.07 | 0.07 | 2.13 | 0.21 | 0.02 | 0.57 | 57.3 |

# Table. S11. The average PAHs’ concentration (ng/g; wet weight) in marine organisms from the Beibu Gulf.

# Table. S12. The Diagnosis ratios to characterize sources of soil PAHs.

| Isomeric ratios |  | Source apportionment | Reference | This study |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | water |  | Sediment |
|  |  |  |  | Summer | Winter | Summer |
| ANTH/(PHE+ANTH) | <0.1 | Petrogenic | (Chen et al., 2012; Pies et al., 2008; Yunker et al., 2002) | 0.109 | 0.131 | 0.09 |
|  | >0.1 | Pyrogenic |  |  |  |  |
| FLUA/(FLUA+PYR) | <0.4 | Petroleum | (Yunker et al., 2002; Zhang et al., 2008) | 0.633 | 0.526 | 0.493 |
|  | 0.4-0.5 | Petroleum combustion |  |  |  |  |
|  | >0.5 | Coal, and biomass combustion |  |  |  |  |
| BaA/(BaA+CHR) | <0.2 | Petroleum | (Yunker et al., 2002; Zhang et al., 2008) | —a | — | 0.592 |
|  | 0.2-0.35 | Mixed origins |  |  |  |  |
|  | >0.35 | Coal, and biomass combustion |  |  |  |  |
| Ind/(Ind+BghiP) | <0.2 | Petroleum | (Ma et al., 2014; Tobiszewski and Namiesnik, 2012) | — | — | 0.718 |
|  | 0.2-0.5 | Mixed origins |  |  |  |  |
|  | >0.5 | Coal, and biomass combustion |  |  |  |  |
| ANTH/PHE | >0.1 | coal, and wood combustion | (Chen et al., 2012; Duodu et al., 2017) | 0.122 | 0.151 | 0.103 |
|  | <0.1 | Petrogenic |  |  |  |  |
| FLUA/PTR | >1 | coal, and wood combustion | (Chen et al., 2012; Duodu et al., 2017) | 1.926 | 1.122 | 1.001 |
|  | <1 | Petrogenic |  |  |  |  |

Note: —a represent data missing.

| Region | Classes | NAP | ACEY | ACE | FLU | PHE | ANTH | FLUA | PYR | CHR | BaA | BbF | BkF | BaP | Ind | DiB | BghiP | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Nisbet and LaGoy, 1992) | TEF | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.01 | 0.001 | 0.001 | 0.01 | 0.1 | 0.1 | 0.1 | 1 | 0.01 | 5 | 0.01 |  |
| Qinzhou | Fish | 2.91 | 0.26 | 0.45 | 1.63 | 4.28 | 4.14 | 0.77 | 0.93 | 1.17 | 20.5 | 8.35 | 20.4 | 217 | 5.44 | 323 | 2.02 | 613 |
|  | Carb | 2.66 | 0.14 | 0.23 | 1.04 | 2.39 | 3.24 | 0.54 | 0.65 | 0.79 | 32.7 | 6.87 | 7.30 | 152 | 1.44 | 119 | 1.21 | 332 |
|  | Shellfish | 1.27 | 0.12 | 0.16 | 0.72 | 2.81 | 2.71 | 1.82 | 2.16 | 2.11 | 102 | 16.0 | 15.2 | 365 | 0.87 | 180 | 1.57 | 694 |
| Beihai | Fish | 4.75 | 0.33 | 0.46 | 1.52 | 3.46 | 3.57 | 0.34 | 0.43 | 0.56 | 9.78 | 2.21 | 5.25 | 4.71 | 0.14 | 53.6 | 0.18 | 91.3 |
|  | Carb | 4.13 | 0.25 | 0.42 | 0.69 | 1.90 | 1.61 | 0.24 | 0.25 | 0.05 | 7.96 | 0.34 | 4.68 | 9.88 | 0.17 | 36.1 | 0.18 | 68.8 |
|  | Shrimp | 2.19 | 0.17 | 0.51 | 0.49 | 3.07 | 5.44 | 0.29 | 0.21 | 0.35 | 4.74 | 0.18 | 0.47 | 5.29 | 0.23 | 57.5 | 0.44 | 81.6 |
|  | Shellfish | 2.90 | 0.21 | 0.44 | 1.60 | 19.2 | 24.9 | 3.03 | 2.20 | 10.3 | 63.8 | 1.19 | 10.0 | 1725 | 0.33 | 392 | 2.79 | 2260 |
| Fang cheng | Fish | 1.08 | 0.20 | 0.42 | 0.24 | 21.0 | 15.4 | 1.05 | 3.48 | 4.56 | 62.4 | 3.42 | 13.1 | 421 | 0.14 | 85.2 | 0.20 | 633 |
|  | Carb | 1.33 | 0.12 | 0.31 | 0.25 | 2.38 | 2.06 | 0.30 | 0.01 | 0.04 | 0.52 | 0.16 | 0.36 | 85.4 | 0.15 | 21.4 | 0.77 | 116 |
|  | Shrimp | 2.95 | 0.16 | 6.45 | 0.18 | 2.73 | 3.42 | 0.31 | 0.01 | 0.05 | 0.51 | 0.20 | 0.31 | 24.8 | 0.15 | 36.8 | 0.32 | 79.3 |
|  | Shellfish | 12.0 | 0.67 | 1.55 | 0.85 | 27.6 | 20.4 | 5.83 | 3.26 | 1.52 | 25.1 | 6.61 | 6.85 | 2126 | 2.09 | 120 | 5.67 | 2367 |
| min |  | 1.08 | 0.12 | 0.16 | 0.18 | 1.90 | 1.61 | 0.24 | 0.01 | 0.04 | 0.51 | 0.16 | 0.31 | 4.71 | 0.14 | 21.4 | 0.18 | 68.8 |
| max |  | 12.0 | 0.67 | 6.45 | 1.63 | 27.6 | 24.9 | 5.83 | 3.48 | 10.3 | 102 | 16.0 | 20.4 | 2126 | 5.44 | 392 | 5.67 | 2367 |
| mean |  | 2.90 | 0.20 | 0.44 | 0.72 | 3.07 | 3.57 | 0.54 | 0.65 | 0.79 | 20.5 | 2.21 | 6.85 | 152 | 0.23 | 85.2 | 0.77 | 332 |
| median |  | 3.47 | 0.24 | 1.04 | 0.84 | 8.25 | 7.91 | 1.32 | 1.24 | 1.96 | 30.0 | 4.14 | 7.63 | 467 | 1.01 | 130 | 1.40 | 667 |
| SD a |  | 2.92 | 0.15 | 1.75 | 0.52 | 9.00 | 7.89 | 1.64 | 1.24 | 2.94 | 31.2 | 4.74 | 6.24 | 706 | 1.53 | 117 | 1.58 | 811 |
| Fish |  | 2.91 | 0.26 | 0.44 | 1.13 | 9.58 | 7.71 | 0.72 | 1.61 | 2.10 | 30.9 | 4.66 | 12.9 | 214 | 1.91 | 154 | 0.80 | 446 |
| Carb |  | 2.71 | 0.17 | 0.32 | 0.66 | 2.22 | 2.30 | 0.36 | 0.30 | 0.29 | 13.7 | 2.46 | 4.11 | 82.5 | 0.59 | 58.7 | 0.72 | 172 |
| Shrimp |  | 2.57 | 0.16 | 3.48 | 0.33 | 2.90 | 4.43 | 0.30 | 0.11 | 0.20 | 2.62 | 0.19 | 0.39 | 15.0 | 0.19 | 47.1 | 0.38 | 80.4 |
| Shellfish |  | 5.40 | 0.33 | 0.72 | 1.06 | 16.5 | 16.0 | 3.56 | 2.54 | 4.66 | 63.5 | 7.94 | 10.7 | 1405 | 1.10 | 231 | 3.34 | 1774 |

# Table. S13. The toxicity equivalent quotients (TEQ: pg-TEQ/g) of PAHs for marine organisms from Beibu Gulf.

Note: SD a = standard deviation

# Table. S14. Excess cancer risk (×10-9) induced by exposure to PAHs via seafood ingestion for different age groups

| Age group | Gender | Organ. | NAP | ACEY | ACE | FLU | PHE | ANTH | FLUA | PYR | CHR | BaA | BbF | BkF | BaP | Ind | DiB | BghiP | ∑PAHs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-5 | Male | Fish | 3.8E+02 | 3.5E+01 | 5.8E+01 | 1.5E+02 | 1.3E+03 | 1.0E+03 | 9.5E+01 | 2.1E+02 | 2.8E+02 | 4.1E+03 | 6.2E+02 | 1.7E+03 | 2.8E+04 | 2.5E+02 | 2.0E+04 | 1.1E+02 | 5.9E+04 |
|  |  | carb | 2.9E+01 | 1.8E+00 | 3.5E+00 | 7.1E+00 | 2.4E+01 | 2.5E+01 | 3.9E+00 | 3.3E+00 | 3.1E+00 | 1.5E+02 | 2.6E+01 | 4.4E+01 | 8.9E+02 | 6.3E+00 | 6.3E+02 | 7.7E+00 | 1.8E+03 |
|  |  | shrimp | 6.6E+01 | 4.2E+00 | 8.9E+01 | 8.5E+00 | 7.4E+01 | 1.1E+02 | 7.6E+00 | 2.9E+00 | 5.1E+00 | 6.7E+01 | 4.9E+00 | 1.0E+01 | 3.8E+02 | 5.0E+00 | 1.2E+03 | 9.8E+00 | 2.1E+03 |
|  |  | shellfish | 5.5E+02 | 3.4E+01 | 7.3E+01 | 1.1E+02 | 1.7E+03 | 1.6E+03 | 3.6E+02 | 2.6E+02 | 4.7E+02 | 6.5E+03 | 8.1E+02 | 1.1E+03 | 1.4E+05 | 1.1E+02 | 2.3E+04 | 3.4E+02 | 1.8E+05 |
|  | Female | Fish | 3.9E+02 | 3.5E+01 | 5.9E+01 | 1.5E+02 | 1.3E+03 | 1.0E+03 | 9.5E+01 | 2.1E+02 | 2.8E+02 | 4.1E+03 | 6.2E+02 | 1.7E+03 | 2.8E+04 | 2.5E+02 | 2.0E+04 | 1.1E+02 | 5.9E+04 |
|  |  | carb | 1.9E+01 | 1.2E+00 | 2.2E+00 | 4.6E+00 | 1.5E+01 | 1.6E+01 | 2.5E+00 | 2.1E+00 | 2.0E+00 | 9.5E+01 | 1.7E+01 | 2.9E+01 | 5.7E+02 | 4.1E+00 | 4.1E+02 | 5.0E+00 | 1.2E+03 |
|  |  | shrimp | 4.2E+01 | 2.7E+00 | 5.7E+01 | 5.5E+00 | 4.8E+01 | 7.3E+01 | 4.9E+00 | 1.9E+00 | 3.3E+00 | 4.3E+01 | 3.2E+00 | 6.5E+00 | 2.5E+02 | 3.2E+00 | 7.8E+02 | 6.3E+00 | 1.3E+03 |
|  |  | shellfish | 3.5E+02 | 2.1E+01 | 4.6E+01 | 6.8E+01 | 1.1E+03 | 1.0E+03 | 2.3E+02 | 1.6E+02 | 3.0E+02 | 4.1E+03 | 5.1E+02 | 6.9E+02 | 9.0E+04 | 7.0E+01 | 1.5E+04 | 2.1E+02 | 1.1E+05 |
| 6-18 | Male | Fish | 9.3E+01 | 8.4E+00 | 1.4E+01 | 3.6E+01 | 3.1E+02 | 2.5E+02 | 2.3E+01 | 5.2E+01 | 6.7E+01 | 9.9E+02 | 1.5E+02 | 4.1E+02 | 6.9E+03 | 6.1E+01 | 4.9E+03 | 2.6E+01 | 1.4E+04 |
|  |  | carb | 1.0E+01 | 9.1E-01 | 1.5E+00 | 3.9E+00 | 3.3E+01 | 2.7E+01 | 2.5E+00 | 5.6E+00 | 7.3E+00 | 1.1E+02 | 1.6E+01 | 4.5E+01 | 7.4E+02 | 6.6E+00 | 5.3E+02 | 2.8E+00 | 1.5E+03 |
|  |  | shrimp | 2.5E+01 | 2.2E+00 | 3.7E+00 | 9.6E+00 | 8.1E+01 | 6.5E+01 | 6.1E+00 | 1.4E+01 | 1.8E+01 | 2.6E+02 | 3.9E+01 | 1.1E+02 | 1.8E+03 | 1.6E+01 | 1.3E+03 | 6.8E+00 | 3.8E+03 |
|  |  | shellfish | 9.8E+01 | 8.8E+00 | 1.5E+01 | 3.8E+01 | 3.2E+02 | 2.6E+02 | 2.4E+01 | 5.4E+01 | 7.0E+01 | 1.0E+03 | 1.6E+02 | 4.3E+02 | 7.2E+03 | 6.4E+01 | 5.2E+03 | 2.7E+01 | 1.5E+04 |
|  | Female | Fish | 9.6E+01 | 8.6E+00 | 1.5E+01 | 3.7E+01 | 3.1E+02 | 2.5E+02 | 2.4E+01 | 5.3E+01 | 6.9E+01 | 1.0E+03 | 1.5E+02 | 4.2E+02 | 7.0E+03 | 6.3E+01 | 5.1E+03 | 2.6E+01 | 1.5E+04 |
|  |  | carb | 6.1E+00 | 3.9E-01 | 7.3E-01 | 1.5E+00 | 5.0E+00 | 5.2E+00 | 8.2E-01 | 6.9E-01 | 6.6E-01 | 3.1E+01 | 5.6E+00 | 9.3E+00 | 1.9E+02 | 1.3E+00 | 1.3E+02 | 1.6E+00 | 3.9E+02 |
|  |  | shrimp | 1.4E+01 | 9.2E-01 | 2.0E+01 | 1.9E+00 | 1.6E+01 | 2.5E+01 | 1.7E+00 | 6.3E-01 | 1.1E+00 | 1.5E+01 | 1.1E+00 | 2.2E+00 | 8.4E+01 | 1.1E+00 | 2.6E+02 | 2.1E+00 | 4.5E+02 |
|  |  | shellfish | 1.2E+02 | 7.4E+00 | 1.6E+01 | 2.3E+01 | 3.7E+02 | 3.6E+02 | 7.9E+01 | 5.6E+01 | 1.0E+02 | 1.4E+03 | 1.8E+02 | 2.4E+02 | 3.1E+04 | 2.4E+01 | 5.1E+03 | 7.4E+01 | 3.9E+04 |
| >18 | Male | Fish | 8.8E+00 | 7.9E-01 | 1.3E+00 | 3.4E+00 | 2.9E+01 | 2.3E+01 | 2.2E+00 | 4.9E+00 | 6.3E+00 | 9.3E+01 | 1.4E+01 | 3.9E+01 | 6.5E+02 | 5.8E+00 | 4.6E+02 | 2.4E+00 | 1.3E+03 |
|  |  | carb | 3.2E-01 | 2.0E-02 | 3.8E-02 | 7.8E-02 | 2.7E-01 | 2.7E-01 | 4.3E-02 | 3.6E-02 | 3.5E-02 | 1.6E+00 | 2.9E-01 | 4.9E-01 | 9.8E+00 | 7.0E-02 | 7.0E+00 | 8.6E-02 | 2.1E+01 |
|  |  | shrimp | 7.6E-01 | 4.8E-02 | 1.0E+00 | 9.9E-02 | 8.6E-01 | 1.3E+00 | 8.8E-02 | 3.3E-02 | 5.9E-02 | 7.8E-01 | 5.7E-02 | 1.2E-01 | 4.4E+00 | 5.7E-02 | 1.4E+01 | 1.1E-01 | 2.4E+01 |
|  |  | shellfish | 6.3E+00 | 3.9E-01 | 8.4E-01 | 1.2E+00 | 1.9E+01 | 1.9E+01 | 4.2E+00 | 3.0E+00 | 5.4E+00 | 7.4E+01 | 9.3E+00 | 1.2E+01 | 1.6E+03 | 1.3E+00 | 2.7E+02 | 3.9E+00 | 2.1E+03 |
|  | Female | Fish | 8.9E+00 | 8.0E-01 | 1.3E+00 | 3.4E+00 | 2.9E+01 | 2.3E+01 | 2.2E+00 | 4.9E+00 | 6.4E+00 | 9.4E+01 | 1.4E+01 | 3.9E+01 | 6.5E+02 | 5.8E+00 | 4.7E+02 | 2.4E+00 | 1.4E+03 |
|  |  | carb | 3.4E-01 | 2.2E-02 | 4.1E-02 | 8.4E-02 | 2.8E-01 | 2.9E-01 | 4.6E-02 | 3.9E-02 | 3.7E-02 | 1.7E+00 | 3.1E-01 | 5.2E-01 | 1.1E+01 | 7.5E-02 | 7.5E+00 | 9.2E-02 | 2.2E+01 |
|  |  | shrimp | 8.4E-01 | 5.4E-02 | 1.1E+00 | 1.1E-01 | 9.5E-01 | 1.4E+00 | 9.7E-02 | 3.7E-02 | 6.5E-02 | 8.6E-01 | 6.3E-02 | 1.3E-01 | 4.9E+00 | 6.3E-02 | 1.5E+01 | 1.2E-01 | 2.6E+01 |
|  |  | shellfish | 6.9E+00 | 4.3E-01 | 9.2E-01 | 1.4E+00 | 2.1E+01 | 2.1E+01 | 4.6E+00 | 3.3E+00 | 6.0E+00 | 8.2E+01 | 1.0E+01 | 1.4E+01 | 1.8E+03 | 1.4E+00 | 3.0E+02 | 4.3E+00 | 2.3E+03 |

# Fig. S1. Sampling sites of aquatic ecosystem of Beibu Gulf, Guangxi.

# Fig. S2. The molecular formula of the 16 target PAHs.

# Fig. S3. Concentration distribution of PAHs in rivers and bays in summer (A) and winter (B).

# Fig. S4. The Log BWAFs (bio-water accumulation factors, group A) and Log BSAFs (bio-sediment accumulation factors, group B) of 16 PAHs by marine organisms.

# Fig. S5. Relationship between the Log BWAFs of detected PAHs in the various marine organisms and their log KOW.

# References

Burkhard, L. P., 2003. Factors influencing the design of bioaccumulation factor and biota-sediment accumulation factor field studies. Environmental Toxicology and Chemistry. 22, 351-360.

Chen, H.-y., et al., 2012. Source apportionment of polycyclic aromatic hydrocarbons (PAHs) in surface sediments of the Rizhao coastal area (China) using diagnostic ratios and factor analysis with nonnegative constraints. Science of the Total Environment. 414, 293-300.

Ding, Y., et al., 2020. Bioaccumulation and trophic transfer of organophosphate esters in tropical marine food web, South China Sea. Environment international. 143, 105919-105919.

Duodu, G. O., et al., 2017. Source apportionment and risk assessment of PAHs in Brisbane River sediment, Australia. Ecological Indicators. 73, 784-799.

Leong, L. S., Tanner, P. A., 1999. Comparison of methods for determination of organic carbon in marine sediment. Marine Pollution Bulletin. 38, 875-879.

Ma, C., et al., 2014. Source apportionment of polycyclic aromatic hydrocarbons in soils of wetlands in the Liao River Delta, Northeast China. Marine Pollution Bulletin. 80, 160-167.

Moermond, C. T. A., et al., 2005. Black carbon and ecological factors affect in situ biota to sediment accumulation factors for hydrophobic organic compounds in flood plain lakes. Environmental Science & Technology. 39, 3101-3109.

Nisbet, I. C., LaGoy, P. K., 1992. Toxic equivalency factors (TEFs) for polycyclic aromatic hydrocarbons (PAHs). Regulatory toxicology and pharmacology : RTP. 16, 290-300.

Pies, C., et al., 2008. Characterization and source identification of polycyclic aromatic hydrocarbons (PAHs) in river bank soils. Chemosphere. 72, 1594-1601.

Tobiszewski, M., Namiesnik, J., 2012. PAH diagnostic ratios for the identification of pollution emission sources. Environmental Pollution. 162, 110-119.

Yang, J., et al., 2014. Spatial distributions and sources of heavy metals in sediment from public park in Shanghai, the Yangtze River Delta. Applied Geochemistry. 44, 54-60.

Yunker, M. B., et al., 2002. PAHs in the Fraser River basin: a critical appraisal of PAH ratios as indicators of PAH source and composition. Organic Geochemistry. 33, 489-515.

Zeng, W., Occurrence of organophosphorusesters  and risk assessment of seafood  in mariculture area of the Beibu Gulf Vol. master. Guangxi University, 2019.

Zhang, W., et al., 2008. Source diagnostics of polycyclic aromatic hydrocarbons in urban road runoff, dust, rain and canopy throughfall. Environmental Pollution. 153, 594-601.